3GPP TSG-RAN WG4 Meeting # 101b-e	R4-2202020

# Electronic Meeting, January 17-25, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	MSD update needed for PC2 and PC1.5 UL configurations

Agenda Item:	5.32	Power Class 2 UE for NR inter-band CA and SUL configurations with x (x>2) bands DL and y (y=1, 2) bands UL	[NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL-Core/Perf]

Document for:	Approval

# Introduction

In R4#101 e-meeting two CRs [1,2] added single UL PC2 and PC1.5 configurations to the intra-band and inter-band RC CA combinations and configurations. At this point it is unclear if a new or increased MSD has been considered for these higher power UL configurations.

# Discussion

## Band combinations with increased UL power

In [1,2], there ares case where it is unclear if the potential for increased MSD has been considered when adding PC2 and/or PC1.5 single UL configurations.

In [1,2] the following configurations should be assessed for potential new or increased MSD:

- CA_n25A-n41C or CA_ n25A-n41(2A) DL with PC2 or PC1.5 n41 UL or PC2 CA_n25A-n41A UL

- CA_n41(2A)-n66A DL with PC2 or PC1.5 n41 UL or PC2 CA_ n41A-n66A UL

- CA_n41C-n66A DL with PC2 or PC1.5 n41 UL or PC2 CA_ n41A-n66A UL

- CA_n41C-n71A DL with PC2 or PC1.5 n41 UL or PC2 CA_ n41A-n71A UL

- CA_n1A-n78A DL with PC2 n78 UL or PC2 CA_ n1A-n78A UL

- CA_n2A-n77/78A or CA_n2A-n77(2A) DL with PC2 n77/n78 UL or PC2 CA_n2A-n77A UL

- CA_n3A-n41A DL with PC2 n41 UL or PC2 CA_n3A-n41A UL

- CA_n3A-n78A DL with PC2 n78 UL or PC2 CA_n3A-n78A UL

- CA_n5A-n77A or CA_n5A-n77(2A) DL with PC2 n77 UL or PC2 CA_n5A-n77A UL

- CA_n12A-n77A or CA_n12A-n77(2A) DL with PC2 n77 UL or PC2 CA_n12A-n77A UL

- CA_n14A-n77A or CA_n14A-n77(2A) DL with PC2 n77 UL or PC2 CA_n14A-n77A UL

- CA_n25A-n77A DL with PC2 n77 UL

- CA_n28A-n41A DL with PC2 n41 UL or PC2 CA_n28A-n41A UL

- CA_n28A-n79A DL with PC2 n79 UL or PC2 CA_n28A-n79A UL

- CA_n30A-n77A or CA_n30A-n77(2A) DL with PC2 n77 UL or PC2 CA_n30A-n77A UL

- CA_n40A-n41A with PC2 n41 UL or PC2 CA_n40A-n41A UL (MSD unless non-simultaneous Tx/Rx is granted, note is missing)

- CA_n41A-n77A DL with PC2 or PC1.5 n41 UL or PC2 n77 UL or PC2 CA_ n41A-n77A UL (unclear if both can be PC2 and simultaneously, MSD unless non-simultaneous Tx/Rx is granted, note is missing)

- CA_n41A-n79A DL with PC2 n41 or n79 UL or PC2 CA_ n41A-n79A UL (unclear if both can be PC2 and simultaneously, MSD unless non-simultaneous Tx/Rx is granted, note is missing)

- CA_n66A-n77A or CA_n66A-n77(2A) DL with PC2 n77 UL or PC2 CA_ n66A-n77A UL

- CA_n71A-n77A DL with PC2 n77 UL or PC2 CA_ n71A-n77A UL

It should be noted that there are overlapping changes between the two CRs.

Proposal: Potential MSD increase or new MSD should be checked for the added HPUE UL configurations listed above before the end of Release 17 to maintain those cases in the specification.

# Conclusions

In this contribution, we look into the list of added HPUE UL configurations in [1, 2] and propose to verify the potential MSD issues generated.

Proposal: Potential MSD increase or new MSD should be checked for the added HPUE UL configurations listed above before the end of Release 17 to maintain those cases in the specification.

# References

- R4-2119885 Draft CR for 38.101-1: PC2 Corrections and improvements, T-Mobile USA, R4#101e

- R4-2119886 Draft CR for 38.101-1: PC2 and PC1.5 combinations, T-Mobile USA, R4#101e