3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215597

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	4.5.3.1.2

Source:	Apple

Title:	On PDCCH demod requirements for multi-TRP

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#104-e demodulation and CSI requirements FeMIMO was discussed and way forward [1] was agreed. In this contribution we present simulation results for PDCCH requirements with mTRP.

# 2. Discussion

Based on the agreed simulation assumptions, we present simulation results for PDCCH demod with mTRP.

Table 1: Simulation Parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| Repetition transmission schemes | FDM |  |
| CBW | 10 MHz | 40 MHz |
| CORESET RB | 24 | 48 |
| CORESET duration | 2 |  |
| Aggregation level | 2 |  |
| Propagation Condition | TDLA30-10 |  |
| Antenna configuration | 2x2, 2x4 |  |
| CCE to REG mapping type | nonInterleaved |  |
| REG bundle size | 6 |  |
| Payload bits(without CRC) | 39 | 41 |
| Test metric | 1% of Pm-dsg (%) |  |

Table 2: Simulation Results

|  | Ant Config | AL2 |
| --- | --- | --- |
| FDD | 2x2 | -1.4 |
|  | 2x4 | -5.2 |
| TDD | 2x2 | -1.2 |
|  | 2x4 | -4.8 |

In [1] it was agreed that we would define requirements for PDCCH demod without specific receiver assumption and include extra margin to account for span in companies’ results with and without soft combining.

The additional margin should be decided based on the span in results, rather than picking a number from the proposed options.

Proposal #1: Define the requirements with an extra margin based on the span in results due to different receiver assumption.

# 3. Conclusion

In this paper, we provide simulation results for requirements for PDCCH for multi-TRP. Our observations and proposals are captured below:

Proposal #1: Define the requirements with an extra margin based on the span in results due to different receiver assumption.

# Reference

- R4-2214397, “WF on UE demodulation and CSI requirements for FeMIMO”, Samsung.