3GPP TSG RAN WG4 Meeting #104bis		R4-2215651

Electronic meeting, October 10 – 19th, 2022

Agenda item:	5.28.3

Source:	Apple Inc.

Title:	NR-U MPR and A-MPR for PC3

WI/SI:	NR_unlic_enh-Core

Release:	Rel-18

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz. Since not all the countries and regions were addressed during the corresponding Rel-16 and Rel-17 WI, RAN#96 meeting approved a new WI [2] with the main motivation of introducing PC3 and enabling LPI and VLP in countries not covered by previous efforts

In this paper we present the simulation results for PC3 MPR for 27dB and 30dB ACLR. Also, the results values for VLP and SP for US and South Korea are provided.

# 2	Discussion

Simulations were done for unlicensed access in band n96 with PC3. The PA was calibrated with the agreed QPSK DFT-s-OFDM waveform. The detailed list of the setup is provided in the following:

- Power Class: PC5

- Calibration: QPSK DFT-s-OFDM 100RB with 1dB backoff

- ACLR: 27dB & 30dB

- SEM: NR-U

- Image rejection: -28dBc

- Carrier leakage rejection: -28dBc

- MPR simulations include EVM and IBE requirements

The PA was simulated with the listed impairments and the results evaluated with the given spectral requirements. The 0dB reference was obtained by finding the maximum inband power with 1MHz aggregation bandwidth. The mask was applied accordingly. Before identifying the reference level, the carrier leakage was removed to avoid an offset in the     NR-U SEM mask. Simulations were conducted for full carrier and interlace operation.

##### 2.1 PC3 MPR

Below the simulations results are presented for ACLR of 27dBc and 30dBc. The results for ACLR of 27dBc are presented in Figure 1 and Table 2 while the results for 30dBc are found in Figure 2 and Table 3.

For lower order modulations of up to 16QAM the 27dBc ACLR is found to provide almost the same power back-off limitation as NR-U SEM. For higher order modulations the EVM represents the limit for the output power. With 30dBc ACLR the main limitation for lower order modulations is ACLR and the proposed values are 0.5 to 1.0dB increased.

Table1: Signal Setup for MPR

| ID | Modulation | Waveform | Allocation |
| --- | --- | --- | --- |
| 1 | Pi/2 BPSK | DFT-s-OFDM | Full |
| 2 | Pi/2 BPSK | DFT-s-OFDM | Interlaced |
| 3 | QPSK | DFT-s-OFDM | Full |
| 4 | QPSK | CP-OFDM | Full |
| 5 | QPSK | DFT-s-OFDM | Interlaced |
| 6 | QPSK | CP-OFDM | Interlaced |
| 7 | 16QAM | DFT-s-OFDM | Full |
| 8 | 16QAM | CP-OFDM | Full |
| 9 | 16QAM | DFT-s-OFDM | Interlaced |
| 10 | 16QAM | CP-OFDM | Interlaced |
| 11 | 64QAM | DFT-s-OFDM | Full |
| 12 | 64QAM | CP-OFDM | Full |
| 13 | 64QAM | DFT-s-OFDM | Interlaced |
| 14 | 64QAM | CP-OFDM | Interlaced |
| 15 | 256QAM | DFT-s-OFDM | Full |
| 16 | 256QAM | CP-OFDM | Full |
| 17 | 256QAM | DFT-s-OFDM | Interlaced |
| 18 | 256QAM | CP-OFDM | Interlaced |

Figure 1: Required power backoff for PC3 with 27dB ACLR

Table 2: Proposed MPR for PC3 with 27dBc ACLR

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 0.5 | ≤ 2.0 |
|  | QPSK | ≤ 1.0 | ≤ 2.0 |
|  | 16 QAM | ≤ 1.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 2.5 | ≤ 3.5 |
|  | 256 QAM | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 2.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 3.0 | ≤ 3.5 |
|  | 64 QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 6.5 | ≤ 7.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

Figure 2: Required power backoff for PC3 with 30dB ACLR

Table 3: Proposed MPR for PC3 with 30dBc ACLR

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 1.0 | ≤ 2.5 |
|  | QPSK | ≤ 1.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 2.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 2.5 | ≤ 3.5 |
|  | 256 QAM | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 3.5 | ≤ 4.0 |
|  | 64 QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 6.5 | ≤ 7.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

Proposal 1: Dependent on the final decision for ACLR requirements either consider table 2 or table 3 to specify PC3 MPR.

##### 2.2 PC3 A-MPR

The signal setup for A-MPR simulations is provided in the table below. Full and interlaced allocations are simulated for all modulations and the channel bandwidth sizes 20MHz. 40MHz, 60MHz, 80MHz and 100MHz. Wideband operation covers the case where the lowest 20MHz sub-band is fully allocated while the other sub-bands are empty e.i. the sub-band configurations are 10, 100, 1000 and 10000.

Table 4: Signal Setup

| ID | Modulation | Waveform | Allocation |
| --- | --- | --- | --- |
| 1 | Pi/2 BPSK | DFT-s-OFDM | Full |
| 2 | Pi/2 BPSK | DFT-s-OFDM | Interlaced |
| 3 | Pi/2 BPSK | DFT-s-OFDM | Wideband |
| 4 | Pi/2 BPSK | DFT-s-OFDM | Wideband Interlaced |
| 5 | QPSK | DFT-s-OFDM | Full |
| 6 | QPSK | CP-OFDM | Full |
| 7 | QPSK | DFT-s-OFDM | Interlaced |
| 8 | QPSK | CP-OFDM | Interlaced |
| 9 | QPSK | DFT-s-OFDM | Wideband |
| 10 | QPSK | CP-OFDM | Wideband |
| 11 | QPSK | DFT-s-OFDM | Wideband Interlaced |
| 12 | QPSK | CP-OFDM | Wideband Interlaced |
| 13 | 16QAM | DFT-s-OFDM | Full |
| 14 | 16QAM | CP-OFDM | Full |
| 15 | 16QAM | DFT-s-OFDM | Interlaced |
| 16 | 16QAM | CP-OFDM | Interlaced |
| 17 | 16QAM | DFT-s-OFDM | Wideband |
| 18 | 16QAM | CP-OFDM | Wideband |
| 19 | 16QAM | DFT-s-OFDM | Wideband Interlaced |
| 20 | 16QAM | CP-OFDM | Wideband Interlaced |
| 21 | 64QAM | DFT-s-OFDM | Full |
| 22 | 64QAM | CP-OFDM | Full |
| 23 | 64QAM | DFT-s-OFDM | Interlaced |
| 24 | 64QAM | CP-OFDM | Interlaced |
| 25 | 64QAM | DFT-s-OFDM | Wideband |
| 26 | 64QAM | CP-OFDM | Wideband |
| 27 | 64QAM | DFT-s-OFDM | Wideband Interlaced |
| 28 | 64QAM | CP-OFDM | Wideband Interlaced |
| 29 | 256QAM | DFT-s-OFDM | Full |
| 30 | 256QAM | CP-OFDM | Full |
| 31 | 256QAM | DFT-s-OFDM | Interlaced |
| 32 | 256QAM | CP-OFDM | Interlaced |
| 33 | 256QAM | DFT-s-OFDM | Wideband |
| 34 | 256QAM | CP-OFDM | Wideband |
| 35 | 256QAM | DFT-s-OFDM | Wideband Interlaced |
| 36 | 256QAM | CP-OFDM | Wideband Interlaced |

2.2.1 South Korea LPI

Below the simulation results for South Korea LPI are shown. The LPI out-of-band emission requirement is -27dBm/MHz. The maximum transmit power is defined with 24dBm and the maximum in-band PSD is 2dBm/MHz. The power backoff is mostly governed by the PSD limitation. The 100MHz channel bandwidth is limited by the out-of-band emission limits.

Figure 3: Power backoff simulation results for South Korea LPI with 20MHz offset to lower band edge

Table 5: A-MPR for South Korea LPI with 20MHz offset to lower band edge

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 1.5 | ≤ 5.0 |
|  | QPSK | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 7.0 | ≤ 4.0 | ≤ 6.0 | ≤ 3.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 7.0 | ≤ 4.0 | ≤ 6.0 | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 7.0 | ≤ 6.5 | ≤ 7.0 | ≤ 6.5 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: Consider the values provided in Table 5 when specifying the A-MPR for South Korea LPI.

2.2.2 US LPI

Below the simulation results for US LPI are shown. The LPI out-of-band emission requirement is -27dBm/MHz. The maximum transmit power is defined with 24dBm and the maximum in-band PSD is -1dBm/MHz.  The power backoff is strongly governed by the PSD limitation.

Figure 4: Power backoff simulation results for US LPI with 20MHz offset to lower band edge

Table 6: A-MPR for US LPI with 20MHz offset to lower band edge

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | QPSK | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 16 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 64 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 256 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
| CP-OFDM | QPSK | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 16 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 64 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 256 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 6.5 | ≤ 8.5 | ≤ 6.5 | ≤ 7.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

Proposal 3: Consider the values provided in Table 6 when specifying the A-MPR for US LPI.

2.2.2 US SP

Below the simulation results for US SP are shown. The SP out-of-band emission requirement is -27dBm/MHz. The maximum transmit power is defined with 30dBm and the maximum in-band PSD is 17dBm/MHz. The simulations results indicated that MPR should be sufficient for 20MHz channels. For all channels larger than 20MHz the power backoff is governed by the -27dBm/MHz emission requirement when placed at lower band edge with 20MHz offset.

Please note that the simulation results shown in Figure 1 are for CBW larger than 20MHz and do not consider EVM and IBE requirements.

Figure 5: Power backoff simulation results for US SP with 20MHz offset to lower band edge

Table 7: A-MPR for US SP and power class 3

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK4 | See Table [PC3 MPR] | ≤ 2.0 | ≤ 4.5 |
|  | QPSK |  | ≤ 2.5 | ≤ 4.5 |
|  | 16 QAM |  | ≤ 3.0 | ≤ 4.5 |
|  | 64 QAM |  | ≤ 3.0 | ≤ 4.5 |
|  | 256 QAM |  | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM | QPSK |  | ≤ 4.5 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 4.5 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 4.5 | ≤ 6.0 |
|  | 256 QAM |  | ≤ 6.5 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz]. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |

Proposal 4: Consider the values provided in Table 7 when specifying the A-MPR for US SP.

# Conclusions

In this contribution we presented det MPR and A-MPR simulated results for PC3 for US and South Korea. The following proposals are made:

Proposal 1: Dependent on the final decision for ACLR requirements either consider table 2 or table 3 to specify PC3 MPR.

Proposal 2: Consider the values provided in Table 5 when specifying the A-MPR for South Korea LPI.

Proposal 3: Consider the values provided in Table 6 when specifying the A-MPR for US LPI.

Proposal 4: Consider the values provided in Table 7 when specifying the A-MPR for US SP.

# 4	References

- RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

- RP-221813, "New WID: Introduction of evolved shared spectrum bands", Apple Inc.