3GPP TSG-RAN WG4 Meeting # 106	R4-2300191

Athens, Greece, 27 February – 3 March 2023

Agenda Item:	9.4.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TR 38.877: FR2-1 specific multi-band requirements

Document for:	Approval

# 1.	Introduction

The revised SID on NR BS RF requirement evolution was approved at TSG RAN#97-e [1]. The objective of this study item is to study certain aspects for FR2 multi-band BS, including the following aspects relating to FR2-1 multi-band BS:

    - Investigate whether a generic solution for all combinations within FR2-1 is possible and/or a solution for all or a part of the frequency range should be targeted

      - Frequency range 24-29 GHz which includes n257/n258/n261

      - Frequency range 37-48 GHz which includes n260/n259/n262

This topic on the feasibility of FR2-1 multi-band RIB with band combinations across different frequency groups from RF chain-level performance point of view was discussed at TSG RAN4#105 but no conclusion was made, and the WF on requirements of FR2-1 multi-band RIB across different frequency groups was agreed [2]:

- Agreement:

  - Rel-18 Study Item focus on the multi-band RIB within the same frequency group meanwhile the study on multi-band RIB across different groups not precluded.

With this agreement, together with the agreements on re-using FR1 multi-band methods and exceptions [3], we currently see no need for FR2-1 specific multi-band requirements as the current FR1 multi-band methods and exceptions are sufficient for FR2-1 multi-band RIB for all band combinations within the same frequency group, while FR2-1 specific multi-band requirements may be considered in the future for multi-band RIB for band combinations across different frequency groups taking into account the implementation challenges. This contribution provides a text proposal to record the agreement in the WF and our observation into TR 38.877 [4].

# 2.	Text proposal

<Start of change>

## 6.4	FR2-1 specific multi-band requirements

FR2-1 specific multi-band requirements refer to requirements specific to FR2-1 compared to those currently applied to FR1. It was agreed that this study focuses on the multi-band RIB within the same frequency group meanwhile the study on multi-band RIB across different frequency groups not precluded. With this agreement, together with the agreements on re-using FR1 multi-band methods and exceptions, there is currently no need for FR2-1 specific multi-band requirements as the current FR1 multi-band methods and exceptions can be applied to FR2-1 and are sufficient for FR2-1 multi-band RIB for all band combinations within the same frequency group. FR2-1 specific multi-band requirements may be considered in the future for multi-band RIB for band combinations across different frequency groups taking into account the implementation challenges.

<End of change>

# References

[1]	RP-222501, “SID revision: NR BS RF requirement evolution”, Huawei, HiSilicon.

[2]	R4-2220249, “WF for the feasibility study on FR2 multi-band RIB”, Huawei, HiSilicon.

[3]	R4-2220294, “TP to TR 38.877: Requirements of FR2-1 multi-band BS”, Nokia, Nokia Shanghai Bell, Huawei.

[4]	R4-2219148, “TR 38.877 v0.1.0”, Huawei, HiSilicon.