3GPP TSG-RAN WG4 Meeting # 106     	R4-2300775

Athens, GR, Feb 27 - Mar 03, 2023

Agenda Item:	6.1.5.2

Source: 	CMCC

Title: 			 Discussion on RRM test parameter configuration for NTN

Document for:	Discussion

- Introduction

In RAN4#105 meeting, the NTN RRM test cases configuration and design were discussed, a way forward was agreed in [1]. In this contribution, we further provide our views on koffset configuration.

- koffset configuration

In the approved CR, the kUE,offset has been set as 0, and the configuration of kcell,offset is still TBD.

First of all, the koffset (= kcell,offset - kUE,offset) value should be equal to or greater than twice of the satellite-UE one-way delay. For satellite configuration in RRM test, RAN4 achieved following agreements:

- Satellite trajectory to be used in RRM test cases shall meet the following characteristics. - For GEO at an altitude of 35,786km, an elevation angle relative to a UE position shall not be smaller than 30 deg during entire test time - For LEO at an altitude of 600km on a circular orbit, an elevation angle relative to a UE position shall not be smaller than 30 deg during entire test time

Refer to HW’s contribution R4-2213860 [2], the relationship between twice of the satellite-UE one-way delay, satellite altitude and tilt angle are as follows:

| Tilt angle/° |  | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Delay/ms | LEO-600 | 12.89 | 9.29 | 7.17 | 5.89 | 5.08 | 4.56 | 4.24 | 4.06 | 4.00 |
|  | LEO-1200 | 20.89 | 16.38 | 13.34 | 11.29 | 9.92 | 9.01 | 8.43 | 8.11 | 8.01 |
|  | GEO | 270.74 | 263.87 | 257.58 | 252.04 | 247.37 | 243.64 | 240.94 | 239.30 | 238.75 |

After align the scenario, the value in highlight can be applied to RRM test configuration. Consider the unit of koffset is slot, the kcell,offset should be 8 slots for LEO and 258 slots for GEO.

Proposal 1: Configure kcell,offset as 8 slots for LEO, 258 slots for GEO.

- Conclusion

In this contribution, we discuss the configuration of koffset, the proposal is:

Proposal 1: Configure kcell,offset as 8 slots for LEO, 258 slots for GEO.

- Reference

[1] R4-2220711, WF on Rel-17 NR NTN RRM Performance requirements, Xiaomi

[2] R4-2213860, Discussion on UE NTN demod PDSCH, Huawei, HiSilicon

[3] R4-2217815, WF on performance part for NR NTN, Xiaomi

[4] R4-2214473, WF on performance part for NR NTN, Xiaomi