3GPP TSG-RAN WG4 Meeting #106	R4-2302517

# Athens, Greece, February 27th – March 3rd, 2023

# Title:	RRM test cases methodology and configuration for SAN NTN assistance information parameters

# Source:	THALES

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	6.1.5.2

Release	Rel-17

# Introduction

The following Satellite 5G NR NTN (Non-Terrestrial Networks) technical documents have been approved at the 3GPP RAN-Plenary #96 (Budapest, 6th-9th of June 2022) for the Release-17 NTN satellite connectivity using FR1 S-band (n256) and FR1 L-band (n255):

- Technical Specification TS 38.108 (NR; Satellite Node radio transmission and reception);

- Technical Specification TS 38.101-5 (NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements);

- Technical Report TR 38.863 (Non-terrestrial networks (NTN) related RF and co-existence aspects);

The question remaining now in RAN4#106 is how to test the NTN UE defined in Rel-17 with respect to time and frequency synchronization, more precisely how to test that the NTN UE is correctly performing timing advance and Doppler frequency pre-compensation in UL (for more information please see TS 38.133 and TS 38.101-5 respectively). Different NTN UE requirements have been provided for timing error and frequency offset related to Doppler pre-compensation. Before commercial phase, the NTN UE/smartphone has to be tested against those defined requirements in order to verify compliancy with 3GPP specifications.

One of the proposals at RAN#98-e meeting was to declare Rel-17 WI NR_NTN_solutions completed on the basis that this remaining activity can be carried out as part of the on-going NTN related RAN5 work item given that RAN4 already specified the requirements. On the other hand, some other companies proposed to continue discussion for at least one meeting in RAN4.

For this reason, the purpose of this TDoc is to indicate a potential way forward and give more details with respect to the potential NTN UE RRM test methodology to follow.

In order to explain potential methodology, we would like first to recall current agreements in RAN1 with respect to ephemeris data format:

RAN1 Agreement: Modify bit allocations for orbital parameters ephemeris format as follows: - Orbital parameters are indicated in 21 bytes payload: - Semi-major axis α (m) is 33 bits - Range: from 6500 km to 43000 km - The quantization step is 4.249 ×10-3m - Eccentricity e is 20 bits - Range: ≤ 0.015 - The quantization step is 1.431 ×10-8 - Argument of periapsis ω (rad) is 28 bits - Range: from 0 to 2π - The quantization step is 2.341 ×10-8 rad - Longitude of ascending node (Ω rad) is 28 bits - Range: from 0 to 2π - The quantization step is 2.341 ×10-8 rad - Inclination i (rad) is 27 bits - Range: from - π/2 to + π/2 - The quantization step is 2.341 ×10-8 rad - Mean anomaly M (rad) at epoch time to is 28 bits - Range: from 0 to 2π - The quantization step is 2.341 ×10-8 rad RAN1 Agreement: For ephemeris set 2, RAN1 agree on orbital parameters α , e, ω , Ω , I, and M in Earth Centered Inertial (ECI) Frame - The ECI and ECEF coincide at Epoch time (e.g. x,y,z axis in ECEF are aligned with x,y,z axis in ECI)

This contribution therefore provides material for discussion with respect to RRM test cases methodology for SAN NTN assistance information parameters.

# Discussion with respect to UE NTN terminal parameters

## 2.1	General aspects

The NTN synchronization mechanisms in Rel-17 were explained many times in the past [R4-2119505, On the NTN UL Timing Accuracy, THALES, RAN4#101-e] and further represented in the figure below:

Figure 1. System Architecture Overview

The NTN working hypothesis in Rel-17 are as follow:

- UE (User Equipment) with GNSS

- Ephemeris data send through satellite to UE (broadcast)

- Transparent satellite with Earth Fixed Cells

- Considered constellation types:

  - LEO@600km (worst case)

  - LEO@1200km

  - GEO

## 2.2	The purpose of the test

The purpose of the test is to generate (on a Testing Equipment) the ephemeris data adapted to UE location, as represented in Figure 2.

Figure 2. Purpose of Test

As explained in Figure 2, the UE (User Equipment, e.g. Smartphone) uses its own GNSS to localise and it can be (of course) in different region across the globe. RRM test would require at least an (hypothetical) testing constellation type e.g. LEO@600km available (worst case). Such hypothetical testing constellation will be used to derive required ephemeris data values (values to be broadcasted by Testing Equipment on SIB19 – System Information Block19):

1/ The exact ephemeris values/data are to be broadcasted on relevant System Information Block (values to be broadcasted by Testing Equipment on SIB19 – System Information Block 19). The Testing Equipment (replacing SAN) has to be configured with these values.

2/ Since the test would be available for different regions (depending on UE location), Testing Equipment will have to be able to select relevant satellites from a list. A propagator/interpolator can be also used on Testing Equipment if data provided to the Testing Equipment has higher timing granularity, however this is a detail.

## 2.3	Proposed Methodology

Step 0: Condition prior to test process, UE (Smartphone) performs (SAN test equipment) data acquisition (ephemeris data send by Testing Equipment). Moreover, UE uses its GNSS to localise.

Figure 3. UE data acquisition

Step 1: Testing UE behaviour: UE performs UpLink (UL, UE->SAN) timing and frequency pre-compensation. UE pre-compensates UL transmission (based on own GNSS and transmitted ephemeris data by Testing Equipment)

Figure 4. Testing Equipment receiving pre-compensated UE UL transmission

Step 2: Measurement and Validation process by Testing Equipment

- Testing Equipment measures UL Doppler shift error & UL Timing error

- If measured values < requirements thresholds => test OK

Proposal 1: RAN4 RRM to consider a hypothetical testing constellation for the Testing Equipment used to test UE.

Proposal 2: The testing constellation should allow global coverage (full coverage of the Earth).

As well-known from the satellite state of the art, it is possible to generate satellite constellation with global coverage by using a Rider-type constellation (see for instance “Optimized Polar Orbit Constellations for Redundant Earth Coverage”, L. Rider, 1985).

Proposal 3: Testing Equipment emulating SAN behaviour to consider Rider-type constellation.

Proposal 4: Testing Equipment to emulate SAN ephemeris data on SIB19, with values representative to UE location.

Proposal 5: Testing Equipment can use satellite ECEF information and/or Orbital information to generate SIB19.

Proposal 6: If required, Testing Equipment to consider a propagator in order to generate SIB19.

## 2.4	Constellation Example

A first important condition is that the testing constellation should have global coverage in any point of the Earth. Therefore, a UE sees at least 1 beam in any point of the Earth, and there is also a potential overlap between 2 beams in order to test HandOver (HO) for instance. Such constellation will have different characteristics as the one given below as example (with more than 800 satellites, for a given minimal elevation angle of 30 deg.):

Figure 5. Example of Rider-type constellation definition

The main assumptions are the following:

- Constellation type: Rider;

  - The inclination of the orbit is 90 degrees, and therefore all satellites pass above the poles.

Note: In practice, such configuration could generate some problems with respect to collision risk e.g. when many satellites cross at the same time a pole. A real constellation would have to modify its inclination to avoid the potential collision risk. However, the goal here is to define a theoretical constellation for testing the user equipment and therefore such (theoretical) constellation can still be used for such purpose.

- Minimum altitude = 600 km;

  - This altitude is the most sizing parameter with respect to the number of handovers and it has been retained as a minimum acceptable altitude for NTN communication constellation.

- Minimum elevation angle = 30 degrees;

  - This is a typical minimum elevation for the NTN user equipment.

The data required by the Test Equipment to implement SIB19 is based on 2 potential information: ECEF information and/or Orbital information. The ECEF example below (for one single satellite) was generated by taking into account Rider-type constellation as test constellation.

Example for Satellite-Sat0101 (satellite 1 from Layer 1):

Time (UTCG)             x (km)          y (km)          z (km)       x (km/sec)    y (km/sec)    z (km/sec)

------------------------    ------------    ------------    ------------    ----------    ----------    ----------

24 Jan 2023 11:00:00.000    -1475.707482    -4407.803392     5204.608792     -1.789619     -5.345371     -5.034441

24 Jan 2023 11:01:00.000    -1600.387312    -4711.630588     4892.207441     -1.704167     -5.017112     -5.389409

24 Jan 2023 11:02:00.000    -1720.927517    -4994.638488     4559.209454     -1.608613     -4.668618     -5.721686

24 Jan 2023 11:03:00.000    -1836.635715    -5255.680264     4207.016782     -1.503185     -4.301438     -6.029875

24 Jan 2023 11:04:00.000    -1946.830169    -5493.707157     3837.112190     -1.388170     -3.917195     -6.312677

24 Jan 2023 11:05:00.000    -2050.843521    -5707.772769     3451.053008     -1.263907     -3.517582     -6.568903

24 Jan 2023 11:06:00.000    -2148.026501    -5897.036881     3050.464581     -1.130791     -3.104352     -6.797473

24 Jan 2023 11:07:00.000    -2237.751592    -6060.768807     2637.033422     -0.989268     -2.679315     -6.997424

24 Jan 2023 11:08:00.000    -2319.416628    -6198.350244     2212.500113     -0.839836     -2.244321     -7.167916

24 Jan 2023 11:09:00.000    -2392.448319    -6309.277627     1778.651980     -0.683043     -1.801261     -7.308231

24 Jan 2023 11:10:00.000    -2456.305656    -6393.163975     1337.315560     -0.519483     -1.352053     -7.417777

24 Jan 2023 11:11:00.000    -2510.483212    -6449.740214      890.348921     -0.349797     -0.898638     -7.496094

24 Jan 2023 11:12:00.000    -2554.514293    -6478.855980      439.633832     -0.174667     -0.442964     -7.542851

24 Jan 2023 11:13:00.000    -2587.973931    -6480.479896      -12.932154      0.005183      0.013012     -7.557852

24 Jan 2023 11:14:00.000    -2610.481710    -6454.699327     -465.443696      0.188995      0.467343     -7.541034

24 Jan 2023 11:15:00.000    -2621.704393    -6401.719612     -915.995677      0.375975      0.918093     -7.492468

24 Jan 2023 11:16:00.000    -2621.358347    -6321.862780    -1362.691233      0.565299      1.363349     -7.412358

24 Jan 2023 11:17:00.000    -2609.211741    -6215.565759    -1803.649736      0.756118      1.801228     -7.301040

24 Jan 2023 11:18:00.000    -2585.086521    -6083.378079    -2237.014711      0.947559      2.229885     -7.158985

24 Jan 2023 11:19:00.000    -2548.860120    -5925.959093    -2660.961651      1.138732      2.647523     -6.986790

24 Jan 2023 11:20:00.000    -2500.466925    -5744.074721    -3073.705703      1.328732      3.052397     -6.785180

24 Jan 2023 11:21:00.000    -2439.899465    -5538.593739    -3473.509175      1.516642      3.442826     -6.555004

24 Jan 2023 11:22:00.000    -2367.209328    -5310.483620    -3858.688859      1.701542      3.817194     -6.297230

24 Jan 2023 11:23:00.000    -2282.507782    -5060.805966    -4227.623114      1.882510      4.173964     -6.012945

24 Jan 2023 11:24:00.000    -2185.966118    -4790.711529    -4578.758693      2.058628      4.511678     -5.703345

24 Jan 2023 11:25:00.000    -2077.815681    -4501.434868    -4910.617285      2.228983      4.828968     -5.369733

24 Jan 2023 11:26:00.000    -1958.347618    -4194.288648    -5221.801733      2.392678      5.124559     -5.013513

# Conclusions

Proposal 1: RAN4 RRM to consider a hypothetical testing constellation for the Testing Equipment used to test UE.

Proposal 2: The testing constellation should allow global coverage (full coverage of the Earth).

Proposal 3: Testing Equipment emulating SAN behaviour to consider Rider-type constellation.

Proposal 4: Testing Equipment to emulate SAN ephemeris data on SIB19, with values representative to UE location.

Proposal 5: Testing Equipment can use satellite ECEF information and/or Orbital information to generate SIB19.

Proposal 6: If required, Testing Equipment to consider a propagator in order to generate SIB19.

# References

- R4-2119505, “On the NTN UL Timing Accuracy”, THALES, RAN4#101-e.

- “Optimized Polar Orbit Constellations for Redundant Earth Coverage”, L. Rider, 1985.

- “Link Strategy for the Mobile Satellite System Iridium”, Harald Keller, Horst Salzwedel.

- TS 38.108, Rel-17.

- TS 38.101-5, Rel-17.

- TS 38.133, Rel-17.

-END-