3GPP TSG-RAN WG4 Meeting #106 bis-e                   	                                  R4-2304256

# Electronic meeting, April 17 – April 26, 2023

Agenda item:	5.19.2

Source:	Intel Corporation

Title:	Simulation results of absolute physical layer throughput with link adaptation

Document for:	Discussion

- Introduction

In RAN4#106 requirements for physical layer throughput were discussed and way forward [1] was approved. It was encouraged to bring simulation results for alignment in addition to previous simulation results in the last meeting [2]. In this paper simulation results for alignment under HARQ retransmission is enabled or not.

- Simulation Results

  - Conditions

In RAN4#106, it was agreed that companies bring simulation results for alignment in RAN4 #106bis-e with HARQ retransmission enable or not. For the simulation parameters in the table below we provide we simulation results in the sub-sections below.

Table 1: Simulation Parameters for Physical layer throughput

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U | DDSU S:11D+3G+0U |
| SNR |  | dB | 0:2:20 | 0:2:20 | 0:2:16 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 | 2 |
|  | Length (L) |  | 12 | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 | 2 |
|  | PRB bundling type |  | Static | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 | As specified in Table A.4-1 of TS 38.101-4: Rank 1: TBS.1-1 Rank 2: TBS.1-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) | Row 5, (4) | Row 5, (8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 | 13 |
|  | CSI-RS periodicity and offset | slot | 5/1 | 10/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) | Row 3 (6) | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) | (8,13) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| ReportConfigType |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 2 | Table 2 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 16 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 | 9 | 7 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  | CodebookSubsetRestriction |  | Not configured | Not configured | Not configured |
|  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 | 5.5 | 1.375 |
| Maximum number of HARQ transmission |  |  | 1 or 4 RV: 0-2-3-1 | 1 or 4 RV: 0-2-3-1 | 1 or 4 RV: 0-2-3-1 |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

  - Results

FR1 FDD

Table 2: Simulation results for FDD 2x2

| SNR (dB) | HARQ Re-Tx Disable |  | HARQ Re-Tx Enable |  | Med CQI | Med RI |
| --- | --- | --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER | TP (Mbps) | BLER |  |  |
| 0 | 3.4584 | 0.1141 | 3.7345 | 0.0951 | 3 | 1 |
| 2 | 4.9798 | 0.1433 | 5.3748 | 0.1153 | 4 | 1 |
| 4 | 6.6492 | 0.1473 | 7.1358 | 0.1134 | 5 | 1 |
| 6 | 8.3048 | 0.1471 | 8.8861 | 0.1129 | 6 | 1 |
| 8 | 10.0144 | 0.1410 | 10.6577 | 0.1091 | 7 | 1 |
| 10 | 11.9869 | 0.1194 | 12.6044 | 0.0925 | 8 | 1 |
| 12 | 14.0544 | 0.0918 | 14.5800 | 0.0716 | 9 | 1 |
| 14 | 15.9911 | 0.0710 | 16.4425 | 0.0565 | 9 | 1 |
| 16 | 18.1713 | 0.0583 | 18.6006 | 0.0475 | 6 | 2 |
| 18 | 20.8187 | 0.0567 | 21.3456 | 0.0449 | 6 | 2 |
| 20 | 23.8776 | 0.0638 | 24.6012 | 0.0494 | 8 | 2 |

Table 3: Simulation results for FDD 2x4

| SNR (dB) | HARQ Re-Tx Disable |  | HARQ Re-Tx Enable |  | Med CQI | Med RI |
| --- | --- | --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER | TP (Mbps) | BLER |  |  |
| 0 | 5.9966 | 0.1102 | 6.2782 | 0.0900 | 5 | 1 |
| 2 | 7.8916 | 0.0737 | 8.1042 | 0.0615 | 6 | 1 |
| 4 | 9.3270 | 0.0629 | 9.5613 | 0.0520 | 7 | 1 |
| 6 | 10.6654 | 0.0416 | 10.8491 | 0.0342 | 8 | 1 |
| 8 | 13.0983 | 0.0154 | 13.1745 | 0.0130 | 4 | 2 |
| 10 | 16.8278 | 0.0060 | 16.8598 | 0.0051 | 6 | 2 |
| 12 | 21.3420 | 0.0028 | 21.3606 | 0.0025 | 7 | 2 |
| 14 | 25.9410 | 0.0027 | 25.9616 | 0.0025 | 8 | 2 |
| 16 | 30.7556 | 0.0049 | 30.8002 | 0.0043 | 9 | 2 |
| 18 | 35.5591 | 0.0075 | 35.6467 | 0.0062 | 10 | 2 |
| 20 | 40.2178 | 0.0122 | 40.3616 | 0.0102 | 11 | 2 |

FR1 TDD

Table 4: Simulation results for TDD 2x2

| SNR (dB) | HARQ Re-Tx Disable |  | HARQ Re-Tx Enable |  | Med CQI | Med RI |
| --- | --- | --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER | TP (Mbps) | BLER |  |  |
| 0 | 10.6166 | 0.1754 | 12.0769 | 0.1374 | 3 | 1 |
| 2 | 15.1097 | 0.2192 | 17.1659 | 0.1685 | 4 | 1 |
| 4 | 20.0837 | 0.2142 | 22.4278 | 0.1661 | 5 | 1 |
| 6 | 25.0118 | 0.2104 | 27.8565 | 0.1620 | 6 | 1 |
| 8 | 30.7390 | 0.1930 | 33.7259 | 0.1477 | 7 | 1 |
| 10 | 37.4769 | 0.1515 | 40.0476 | 0.1176 | 8 | 1 |
| 12 | 43.9575 | 0.1132 | 46.0580 | 0.0894 | 9 | 1 |
| 14 | 49.6854 | 0.0795 | 51.3239 | 0.0639 | 9 | 1 |
| 16 | 56.6962 | 0.0503 | 57.8957 | 0.0409 | 5 | 2 |
| 18 | 65.7806 | 0.0357 | 66.7990 | 0.0288 | 6 | 2 |
| 20 | 76.7557 | 0.0378 | 78.0558 | 0.0304 | 7 | 2 |

Table 5: Simulation results for TDD 2x4

| SNR (dB) | HARQ Re-Tx Disable |  | HARQ Re-Tx Enable |  | Med CQI | Med RI |
| --- | --- | --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER | TP (Mbps) | BLER |  |  |
| 0 | 16.5953 | 0.3262 | 20.0177 | 0.2625 | 5 | 1 |
| 2 | 20.8843 | 0.3028 | 24.9804 | 0.2419 | 6 | 1 |
| 4 | 25.2294 | 0.2713 | 29.6311 | 0.2121 | 7 | 1 |
| 6 | 33.7962 | 0.1329 | 36.2154 | 0.1004 | 8 | 1 |
| 8 | 44.6898 | 0.0401 | 45.4230 | 0.0306 | 4 | 2 |
| 10 | 59.1705 | 0.0077 | 59.3256 | 0.0064 | 6 | 2 |
| 12 | 74.2736 | 0.0041 | 74.3607 | 0.0036 | 7 | 2 |
| 14 | 88.9403 | 0.0046 | 89.0488 | 0.0042 | 8 | 2 |
| 16 | 103.0602 | 0.0069 | 103.2391 | 0.0063 | 9 | 2 |
| 18 | 116.3063 | 0.0256 | 117.1301 | 0.0228 | 10 | 2 |
| 20 | 130.0289 | 0.0390 | 131.7085 | 0.0307 | 11 | 2 |

FR2 TDD

Table 6: Simulation results for FR2 TDD 2x2

| SNR (dB) | HARQ Re-Tx Disable |  | HARQ Re-Tx Enable |  | Med CQI | Med RI |
| --- | --- | --- | --- | --- | --- | --- |
|  | TP (Mbps) | BLER | TP (Mbps) | BLER |  |  |
| 0 | 22.0257 | 0.0463 | 22.3884 | 0.0418 | 5 | 1 |
| 2 | 30.0619 | 0.0453 | 30.4744 | 0.0405 | 6 | 1 |
| 4 | 39.6895 | 0.0579 | 40.4675 | 0.0515 | 8 | 1 |
| 6 | 50.2427 | 0.0718 | 51.2822 | 0.0622 | 9 | 1 |
| 8 | 60.8486 | 0.0668 | 61.9450 | 0.0579 | 10 | 1 |
| 10 | 72.0467 | 0.0491 | 72.9787 | 0.0425 | 11 | 1 |
| 12 | 81.0306 | 0.0153 | 81.3024 | 0.0139 | 11 | 1 |
| 14 | 91.5887 | 0.0045 | 91.6735 | 0.0043 | 8 | 2 |
| 16 | 107.3120 | 0.0016 | 107.3310 | 0.0015 | 9 | 2 |

- Conclusion

In this paper, we provide simulation results for alignment for defining physical layer throughput requirements.

- References

- R4-2302930 WF on PHY Layer TP requirements

- R4-2300484 Summary of simulation results for NR UE Application Layer Data Throughput 	    Performance