3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305710

Online, April 17th – 26th, 2023

Agenda Item:	5.20.2.4

Source:	MediaTek Inc.

Title: 	Discussion on UE RF requirement impacts from SBFD

Document for:	Discussion

# Introduction

RAN4 continues the study on duplex evolution, and from UE aspects, a way forward was agreed in RAN4#106 [1]. Some open issues related to UE RF requirement impacts are listed as following:

- For new SBFD aware UE: FFS whether sub-filtering can be considered or not - For AGC and NF modelling for both co-channel and adjacent channel CLI: - Use a fixed value noise figure model for the purpose of system level simulation for SBFD - FR1 noise figure value in the range [7 to 9 dB] - FR2-1 noise figure value in the range [7.5 to 10 dB]

# Discussion

## 2.1 Sub-band filtering for SBFD-aware UEs

As agreed in RAN4, RAN4 shall not introduce new RAN4 requirement for sub-band selectivity for legacy UE till Rel-18 [1]. And for legacy UEs in an SBFD-operating BS, some guard band can be considered to minimize the impacts to legacy UEs.

Observation 1: Guard band can be considered for a legacy UEs in an SBFD-operating BS in order to minimize the impacts to the legacy UE.

However, for SBFD-aware UEs, sub-band filtering can be considered, which means that the sub-band selectivity can be improved via a certain sub-band filtering, e.g., in the digital domain. However, due to the complexity and power consumption constraint, a UE may not be able to conduct a sub-band filtering with a size of any PRB number. To facilitate the use of implemented sub-band filtering, such capability of an SBFD-aware UE should be reported to the network scheduler. Moreover, for a sub-band filtering capable UE, network does not consider to allocate guard band for the UE if its intended RB size can be allocated.

Observation 2: The capability of sub-band filtering of an SBFD-aware UE can be reported to network and network does not need to consider guard band for the UE if its intended RB size can be allocated.

## 2.2 AGC and NF modeling

As agreed in RAN4#106, in order to perform system level evaluation, one fixed value for noise figure should be used. Since a range of noise figure was agreed, i.e., [7~9] dB for FR1, and [7.5~10] dB for FR2-1, respectively. As a starting point, we tend to propose to use higher end for system level simulation, which means 9dB for FR1, and 10dB for FR2-1.

Proposal 1: RAN4 to perform system level evaluation by using NF 9dB for FR1, and 10dB for FR2-1 as a starting point.

# Conclusion

In this contribution we have the following observations and proposal for UE RF requirement impacts from SBFD:

Observation 1: Guard band can be considered for a legacy UEs in an SBFD-operating BS in order to minimize the impacts to the legacy UE.

Observation 2: The capability of sub-band filtering of an SBFD-aware UE can be reported to network and network does not need to consider guard band for the UE if its intended RB size can be allocated.

Proposal 1: RAN4 to perform system level evaluation by using NF 9dB for FR1, and 10dB for FR2-1 as a starting point.

# References

- R4-2302977 WF for SBFD feasibility study and requirements impact from UE aspect, Qualcomm.

- R4-2303755, Topic summary for [106][310] FS_NR_duplex_evo_Part1