3GPP TSG-RAN WG4 Meeting #106bis-e                           		R4-2305725

# Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon

Title: 	TP to TS 36.181: Conducted and OTA Tx spur (6.6.5, 9.7.5)

Agenda Item:	6.8.3.5

Document for:	Approval

# Introduction

Referring to the work-split arranged before this meeting and captured in [3], in this contribution we provide TP to TS 36.181 on conducted and radiated Tx spurious emissions requirements.

Couple of clarifications to the proposed TP:

- All the manufacturer declaration are highlighted to indicate the need to run final consistency check in the stable version of the draft specification.

- Multiple cross-references are in [] to perform final consistency check in the stable version of the draft specification.

# Conclusions

Based on the above discussion, the following proposals were formulated:

Proposal 1: Approve the attached TP to TS 36.181.

# References

[1]	RP-223437	WID on NB-IoT/eMTC core & performance requirements for Non-Terrestrial Networks, WID, RAN#98e

[2]	R4-2302451	TS 36.181 SAN conformance testing skeleton and work-split arrangement	Huawei, HiSilicon, RAN4#106 (Athens)

[3]	R4-2305715	Work-split arrangement for TS 36.181 development	Huawei, HiSilicon

# Annex A: TP to TS 36.181

------------------------------ Modified section ------------------------------

### 6.6.5	Transmitter spurious emissions

##### 6.6.5.1	Definition and applicability

The transmitter spurious emission limits shall apply from 30 MHz to 11 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in table [6.6.1].

The lower limit and upper limit are as per ITU-R recommendation SM.329 [x], Table 1: For systems operating within 600 MHz and 5.2 GHz, the lower limit is 30 MHz and the upper limit is the 5th harmonic of the higher frequency.

The lower limit of 30 MHz can be replaced as per ITU-R SM.329-12 [x]: Systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off wavelength, do not require spurious domain emission measurements below 0.7 times the waveguide cut-off frequency.

The requirements shall apply whatever the type of transmitter considered (e.g. single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer's specification.

The requirements shall apply to SAN that support NR.

Unless otherwise stated, all requirements are measured as mean power (RMS).

##### 6.6.5.2	Minimum requirement

The minimum requirement applies per single-band connector supporting transmission in the operating band.

The minimum requirement for SAN type 1-H is defined in Table 6.6.5.5-1.

##### 6.6.5.3	Test purpose

This test measures conducted spurious emissions while the transmitter is in operation.

##### 6.6.5.4	Method of test

##### 6.6.5.4.1	Initial conditions

Test environment: Normal; see annex B.

RF channels to be tested for single carrier:

-	B when testing the spurious emissions below FDL_low - ΔfOBUE,

-	T when testing the spurious emissions above FDL_high + ΔfOBUE; see clause [4.9.1].

SAN RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW when testing the spurious frequencies below FDL_low - ΔfOBUE;

-	TRFBW when testing the spurious frequencies above FDL_high + ΔfOBUE in single-band operation; see clause [4.9.1].

##### 6.6.5.4.2	Procedure

For SAN type 1-H where there may be multiple TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex [D.1.1]. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the single-band connector under test to measurement equipment as shown in annex [D.1.1] for SAN type 1-H. All connectors not under test shall be terminated with a matched load.

2)	Measurements shall use a measurement bandwidth in accordance to the conditions in clause 6.6.5.5.

The measurement device characteristics shall be: Detection mode: True RMS.

The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table [4.1.2.2-1].

3)	For a connectors declared to be capable of single carrier operation only (D.39), set the representative connectors under test to transmit according to the applicable test configuration in clause [4.8] at rated carrier output power (Prated,c,TABC, D.34). Channel set-up shall be according to [FR1-TM 1.1].

For a connector under test declared to be capable of multi-carrier operation (D.39) set the connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in clauses [4.7] and [4.8] using the corresponding test models or set of physical channels in clause [4.9.2].

4)	Measure the emission at the specified frequencies with specified measurement bandwidth.

##### 6.6.5.5	Test requirements

The limits of Table 6.6.5.5-1 shall apply.

Table 6.6.5.5-1: General SAN transmitter spurious emission limits

| Spurious frequency range | Prated,c,sys (dBm) | Basic limit (dBm) | Measurement bandwidth (kHz) | Notes |
| --- | --- | --- | --- | --- |
| 30 MHz – 5th harmonic of the upper frequency edge of the DL operating band | ≤ 47 | -13 | 4 | NOTE 1, NOTE 2, NOTE 3 |
|  | > 47 | Prated,c,sys – 60dB |  |  |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [x], s2.5 table 1. NOTE 3: The lower frequency limit is replaced by 0.7 times the waveguide cut-off frequency, according to ITU-R SM.329 [x], for systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off. |  |  |  |  |

### 6.6.6	Protection of the own Satellite Access Node receiver

### 6.6.7	Additional spurious emissions requirements

The additional spurious emissions requirement is not applicable for SAN.

------------------------------ Next modified section ------------------------------

### 9.7.5	OTA transmitter spurious emissions

##### 9.7.5.1	Definition and applicability

Unless otherwise stated, all requirements are measured as mean power.

The OTA transmitter spurious emissions limits are specified as TRP per RIB, unless otherwise stated.

The OTA transmitter spurious emission limits for FR1 shall apply from 30 MHz to 11 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in clause 6.6.1.

The lower limit and upper limit are as per ITU-R recommendation SM.329 [x], Table 1: For systems operating within 600 MHz and 5.2 GHz, the lower limit is 30 MHz and the upper limit is the 5th harmonic of the higher frequency.

The lower limit of 30 MHz can be replaced as per ITU-R SM.329-12 [x]: Systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off wavelength, do not require spurious domain emission measurements below 0.7 times the waveguide cut-off frequency.

The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer's specification.

SAN type 1-O requirements consists of OTA transmitter spurious emission requirements based on TRP.

The general OTA transmitter spurious emissions requirements are specified as TRP per RIB, per cell, unless otherwise specified.

##### 9.7.5.2	Minimum requirement

The minimum requirement for SAN type 1-O is specified in TS 36.108 [2], clause [9.7.5.2.2].

##### 9.7.5.3	Test purpose

The test purpose is to verify if the radiated spurious emissions from the SAN at the RIB are within the specified minimum requirements.

##### 9.7.5.4	Method of test

##### 9.7.5.4.1	Initial conditions

Test environment: Normal; see annex B.

RF channels to be tested for single carrier, see clause [4.9.1]:

-	B when testing from 30 MHz to FDL_low - ΔfOBUE

-	T when testing from FDL_high + ΔfOBUE to 5th harmonic

RF bandwidth positions to be tested in single-band multi-carrier operation, see clause [4.9.1]:

-	BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

-	TRFBW when testing from FDL_high + ΔfOBUE to 5th harmonic

Directions to be tested: As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see TS 38.141-2 [x], annex I) as long as the required TRP level is achieved.

##### 9.7.5.4.2	Procedure

The following procedure for measuring TRP is based on directional power measurements as described in TS 38.141-2 [x], annex I. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber if so follow steps 1, 3, 4, 5, 7.

1)	Place the SAN at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the SAN with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in clause 9.7.5.5.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

-	The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table [4.1.2.2-2].

5)	Set the SAN to transmit:

-	For RIB declared to be capable of single carrier operation only, set the RIB to transmit a signal according to the applicable test configuration in clause [4.8] using the corresponding test model in clause [4.9.2] (i.e. [SAN-FR1-TM1.1] for SAN type 1-O), at manufacturer's declared rated output power Prated,c,TRP.

-	For a RIB declared to be capable of multi-carrier operation, set the RIB to transmit according to the corresponding test model in clause [4.9.2] on all carriers configured using the applicable test configuration and corresponding power setting specified in clause [4.7.2] and [4.8].

6)	Orient the positioner (and SAN) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see TS 38.141-2 [x], annex I).

7)	Measure the emission at the specified frequencies with specified measurement bandwidth.

8)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see TS 38.141-2 [x], annex I).

NOTE 1:	The TRP measurement grid may not be the same for all measurement frequencies.

NOTE 2:	The frequency sweep or the TRP measurement grid sweep may be done in any order.

9)	Calculate TRP at each specified frequency using the directional measurements.

##### 9.7.5.5	Test requirement

For a SAN type 1-O, the TRP of any spurious emission shall not exceed the limits in table 9.7.5.5-1.

Table 9.7.5.5-1: General OTA SAN transmitter spurious emission limits for SAN type 1-O

| Spurious frequency range | Prated,c,TRP (dBm) | Basic limit (dBm) | Measurement bandwidth (kHz) | Notes |
| --- | --- | --- | --- | --- |
| 30 MHz – 5th harmonic of the upper frequency edge of the DL operating band | ≤ 47 | -13 | 4 | NOTE 1, NOTE 2, NOTE 3 |
|  | > 47 | Prated,c,TRP – 60dB |  |  |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [x], s2.5 table 1. NOTE 3: The lower frequency limit is replaced by 0.7 times the waveguide cut-off frequency, according to ITU-R SM.329 [x], for systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off. |  |  |  |  |

### 9.7.6		Protection of the SAN receiver

The co-location requirement is not applicable for SAN in this version of the specification.

### 9.7.7	Additional spurious emissions requirements

The additional spurious emissions requirement is not applicable for SAN.

------------------------------ End of modified section ------------------------------