3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305749

Electronic Meeting, April 17 – April 26, 2023

Source:	Skyworks Solutions, Inc.

Title:	TP for TR 38.846 on test burden reduction for multiple MSD in band combinations

Agenda item:			5.1.3 Simplification of specification and reduction of test burden	[FS_SimBC]

Document for:	Approval

# 1. Introduction

This contribution captures the changes to guidelines on test burden reduction [1] previously discussed in [2] at RAN4 #106. During this meeting it was also brought to our attention that the additional MSD test points due to regional frequency band restrictions are not removed automatically. This text proposal (TP) therefore captures both changes to TR [1].

# 2. References

[1] 	TR 38.846 v0.3.0 Study on simplification of band combination specification for NR and LTE, 3GPP TSG-RAN WG4 Meeting # 106 Athens, Greece.

[2]	R4-2302738 Views on BC simplification for two-band combinations, 3GPP TSG-RAN WG4 Meeting # 106 Athens, Greece, Skyworks Solutions, Inc.

# 3. Text proposal

******************************* Start of TP to TR 38.486 *******************************

## 7.4	Test burden reduction for multiple MSD

In current RAN4 spec, there are tables for the reference sensitivity exceptions due to intermodulation interference with 2UL CA. The test points in the reference sensitivity requirements specified for the single band are relaxed by the amount of the corresponding MSD values, as shown in Table 7.4-1. For some CA configurations such as CA_n2-n77, CA_n3-n77 and CA_n3-n78, multiple test points with different order IMD are defined, while for some other CA configurations such as CA_n2-n78, only 2nd order IMD2 having the worst-case MSD are defined, although the corresponding band is subject to the 5th order IMD5. At the end of the table, a “Note 4” is set to indicate that MSD is not specified for the interfered band although IMD5 may fall into the Rx frequencies of the interfered band. To reduce the test burden, the following guidelines for handling multiple MSD should be taken into consideration.

Table 7.4-1: Example for inter-band reference sensitivity with multiple MSD

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3 | n1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | TDD | N/A |
| CA_n1-n8 | n1 | 1965 | 5 | 25 | 2155 | 6.0 | FDD | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| CA_n2-n48 | n2 | 1852.5 | 5 | 25 | 1932.5 | 12 | FDD | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | 8.0 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | TDD | N/A |
| CA_n2-n78 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n3-n7 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | FDD | IMD4 |
| CA_n3-n8 | n3 | 1755 | 10 | 50 | 1850 | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | 8 | FDD | IMD44 |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | FDD | IMD5 |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | FDD | N/A |
| CA_n3-n38 | n3 | 1713 | 5 | 25 | 1808 | 8.2 | FDD | IMD4 |
|  | n38 | 2617 | 5 | 25 | 2617 | N/A | TDD | N/A |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | TDD | N/A |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  | n77 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |
| … | … | … | … | … | … | … | … | … |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

For a given 2 band DL CA combination, MSD test points corresponding to type 1,2,3 UL configuration are captured in the same table entry.

–   Type 1: UL configuration = 2 UL CCs configured with intra-band UL CA configured in one of the two band. Intra-band UL CA may be contiguous (like UL CA_n41C) or non-contiguous (like UL CA_n78(2A)).

–   Type 2: UL configuration = 2 UL CCs configured inter-band UL CA with 1UL CC in each of UL band. Example: UL CA_n3A_n78A.

–   Type 3: UL configuration = 3 UL CCs with 1 CC in one UL band, and 2 UL CCs configured intra-band CA in the other band. Example: UL CA_n3A-n41C.

Guideline 1: It is proposed that for the test points for reference sensitivity exceptions due to intermodulation interference with 2UL CA, the limitation to higher order IMD source could be a solution to reduce test burden.

–   The existing IMD MSD requirements in Rel-17 specifications are kept unchanged.

–   For Rel-18 new introduced band combinations,

–  For type 1 UL configurations (eg. UL_CA_n41C or CA_n78(2A))

- The lowest order IMD is recommended as worst case to represent single band UL transmission with UL configured intra-band CA.

- If the DL band may be affected by a mix of even and odd order IMD products, the MSD value of the lowest even and the lowest odd order IMD, if any, shall be defined in the specifications.

- A footnote shall be attached to the DL band to indicate that MSD may occur for higher order IMD products, and these orders shall be specified in the footnote.

- As an exception to this rule, a second MSD test point may be specified to capture the MSD that may occur due to the next highest odd order or due to the next highest even order IMD product. This exception ensures regional frequency restrictions are accounted for. For example, in CA_n66_n77 the type 1 IMD7 MSD test point (next highest odd order) may be retained in addition to the type 1 IMD5  MSD test point (lowest odd order).

- A footnote shall be attached to the UL band that is configured intra-band UL CA to distinguish the case of intra-band contiguous vs intra-band non-contiguous CA.

–  For type 2 UL configurations (eg. UL_CA_n1A-n3A)

- If only one IMD order occurs per victim band, the MSD value if any shall be defined in the specifications.

- If the DL band may be affected by a mix of even and odd order IMD products, then the MSD value of the lowest even and the lowest odd order IMD, if any, shall be defined in the specifications.

- A footnote shall be attached to the DL band to indicate that MSD may occur for higher order IMD products, and these orders shall be specified in the footnote.

- If the DL band may be affected only by multiple even order IMD products, or only by multiple odd order IMD products, then the MSD value of the lowest even order IMD or the MSD value of the lowest odd order IMD ,if any, shall be defined in the specifications.

•    The lowest order IMD is recommended as worst case to represent the whole spectrum of the inter-band CA combinations.

•    Optionally, a second MSD test point may be specified on a case-by-case basis to account for additional IMD orders. It is recommended this 2nd MSD test point corresponds to the lowest even and the lowest odd order IMD. For example, if DL band is affected by IMD2/3/5, we may consider a maximum of test points: one for IMD2 and one for IMD3.

∎  Any additional IMD order that is not specified shall be indicated by a note in the table.

–  For type 3 UL configurations (e.g. CA_n3A-n41C)

○   For the case when the victim band is affected by a 1st order triple-beat product, an additional IMD3 test point shall be defined per victim band.

******************************* END of TP to TR 38.486 *******************************