3GPP TSG-RAN WG4 Meeting #109	R4-2318052

Chicago, USA, November 13 – November 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on UL 256 QAM BS Demodulation

Agenda item:	8.6.4.1

Document for:	Discussion

Introduction

At RAN#98 meeting, the WI was approved to study the feasibility to support UL256 QAM [1] with the following objectives of the performance part.

UL 256QAM - Specify the BS demodulation performance

In this paper, we present Nokia’s view on the open issues of 256QAM UL demodulation for discussion at RAN4#109.

Discussion

Background

The introduction of FR2 UL 256 QAM for Rel-18 has been outlined in the WID [1], with an “aim to support a variety of research, marketing, and industrial use cases, such as super high-resolution imagery being uploaded to the cloud requiring Gbps data rate”.

RAN4 Demodulation will specify the performance of FR2 UL 256 QAM to enable this feature, specifically RAN4 will focus on the operating SNR, phase noise and other implementation aspects to define the BS demodulation performance of FR2 UL 256 QAM.

Two open issues were presented following discussion at RAN4#108-bis [5] which are discussed below.

Issue 1-6: 60 kHz SCS and corresponding carrier BW

The following was presented in the WF at RAN4#108-bis for Issue 1-6 [5].

Agreement: Companies to use 50 MHz for 60 kHz, however decision whether to include requirements for SCS of 60 kHz to be deferred to RAN4#109.

We propose that a singular 60 kHz SCS requirement is defined within this work item to define the performance for the minimum requirement for SCS, despite the prevailing market drive for 120 kHz SCS.

RAN4 shall use 50 MHz for defining requirements with 60 kHz for 256QAM BS Demodulation

Issue 1-6: 60 kHz SCS and corresponding carrier BW

The following was presented in the WF at RAN4#108-bis for Issue 1-6 [5].

Agreement: Companies to use 50 MHz initially for requirements derivation, with FFS on 100 MHz and 200 MHz. Companies to provide simulations for 50, 100, 200 with a priority in order of 50, 200, 100, with a view on testable SNR. Note: Could use existing applicability rule to extend 50 MHz to other CBW.

In our companion simulation TDoc [3] we show that for all BWs with 120 kHz there is very little performance difference, therefore only one set of requirements should be defined.

There is little difference between different CBWs with 120 kHz

RAN4 shall only define one CBW for SCS 120 kHz SCS for 256QAM BS Demodulation.

RAN4 shall use 50 MHz for defining requirements with 120 kHz SCS for 256QAM BS Demodulation.

Conclusion

Within this contribution we have discussed and introduced the demodulation requirements for UL 256 QAM.

In the paper, the following Observations and Proposals were made:

Proposal 1: RAN4 shall use 50 MHz for defining requirements with 60 kHz for 256QAM BS Demodulation

Observation 1: There is little difference between different CBWs with 120 kHz

Proposal 2: RAN4 shall only define one CBW for SCS 120 kHz SCS for 256QAM BS Demodulation.

Proposal 3: RAN4 shall use 50 MHz for defining requirements with 120 kHz SCS for 256QAM BS Demodulation.

- References

- RP-222909 Revised WID NR RF requirements enhancement for frequency range 2 (FR2), Phase 3, RAN#98-e, December 2022

- TS 38.104 v18.1.0. Technical Specification Group Radio Access Network; NR; Base Station (BS) radio transmission and reception (Release 18). March 2023

- R4-2318053, Supporting simulations for UL 256 QAM BS Demodulation, Nokia, Nokia Shanghai Bell, 3GPP RAN4 #109, November, Chicago

- R4-2113630, “On PUSCH demodulation requirements for FR1 256QAM”, Nokia, Nokia Shanghai Bell, RAN WG4 #100-e, Online, Aug 2021 - 27th Aug 2021

- R4-2316903 “Way Forward for 256 QAM BS Demod”, Nokia, Nokia Shanghai Bell, 3GPP RAN4 #108-bis, Oct 9-13, 2023