3GPP TSG-RAN4 Meeting #109	R4-2318425

# Chicago, USA, November 13th – 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0040 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-5 on the introduction of NTN DMRS bundling feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | One of the objectives of the NTN enhancement WID [RP-223534] is to enable the use of the Rel-17 DMRS bundling feature in NTN networks. This CR introduces the feature with the necessary changes to the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause 6.4.2.1 Phase continuity requirements for DMRS bundling. The terrestrial UE RF requirements on DMRS bundling are captured in Clause 6.4.2.5 of TS38.101-1 and are taken as the starting point. Two NTN specific aspects are captured: - According to the agreement from RAN4 #108bis [R4-2317767], the requirement on phase continuity for NTN DMRS bundling shall be verified under zero Dopppler and constant delay conditions - It is reasonable to limit the applicability of maximum DMRS bundling window length for NTN NGSO UEs to no more than [4] slots. For NTN GSO UEs the same maximum DMRS window bundling length can be reused from the terrestrial specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for the NTN DMRS bundling feature remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Additional information in R4-2318426 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.4	Transmit signal quality

### 6.4.1	Frequency error

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift. For the test case, the location of the UE is explicitly provided to the UE from the test equipment.]

Requirement will be verified for at least two cases of which one has zero Doppler conditions.

### 6.4.2	Transmit modulation quality

The requirements for transmit modulation quality defined in 3GPP TS 38.101-1 [5] clause 6.4.2 shall apply for NTN satellite UE except for the requirements for Pi/2 BPSK modulation.

<< begin change >>

#### 6.4.2.1	Phase continuity requirements for DMRS bundling

For NTN bands that UE indicates the support of DMRS bundling, when the UE is configured with DMRS bundling, the maximum allowable difference between the measured phase value in any slot p-1 and slot p, or slot 0 and any slot p for each antenna connector shall satisfy the requirements as listed in Table 6.4.2.1-1 for the measurement conditions defined in Table 6.4.2.1-2, within a measurement time window limited by the UE capability of maximum duration for DMRS bundling [maxDurationDMRS-Bundling-r17], and defined for each frequency band separately. The phase value for each slot is measured as shown in Annex F.9 of 3GPP TS 38.101-1 [5]. These requirements apply to PUCCH and PUSCH transmissions with DFT-s-OFDM and CP-OFDM waveforms. The requirement is verified under zero Doppler and constant delay test conditions. The delay condition is a constant and derived from the ephemeris information (SIB-19) and UE location associated with the Doppler value under test.

Table 6.4.2.1-1: Maximum allowable phase difference for DMRS bundling in NTN bands

| UL channel | Modulation order | Phase difference between any slot p-1 and slot p (NOTE 2) | Phase difference between slot 0 and any slot p (NOTE 3) |
| --- | --- | --- | --- |
| PUSCH | Pi/2 BPSK, QPSK | [25] degrees | [30] degrees |
| PUCCH | Pi/2 BPSK, BPSK, QPSK |  |  |
| NOTE 1: The UE capability of the length of maximum duration refers to the maximum time duration during which UE is able to meet the phase continuity requirements, assuming no phase consistency violating events defined in TS 38.214 in between. NOTE 2: This requirement applies for NTN bands as follows: if ntn-ScenarioSupport-r17 is present and indicated as GSO, then this requirement applies for the supported DMRS bundling configurations ≤ [8] slots; if ntn-ScenarioSupport-r17 is present and indicated as NGSO or only the IE field nonTerrestrialNetwork-r17 is present, then this requirement applies for the supported DMRS bundling configurations ≤ [4] slots. NOTE 3: This requirement applies for NTN bands as follows: if ntn-ScenarioSupport-r17 is present and indicated as GSO, then this requirement applies for the supported DMRS bundling configurations ≤ [16] slots; if ntn-ScenarioSupport-r17 is present and indicated as NGSO or only the IE field nonTerrestrialNetwork-r17 is present, then this requirement applies for the supported DMRS bundling configurations ≤ [4] slots. |  |  |  |

The above requirements are applicable when all the following conditions are met within the measurement time window:

-	RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated.

-	Modulation order does not change.

-	No network commanded TA takes effect.

-	The TPMI precoder does not change.

-	There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE.

-	UE is not scheduled with uplink transmission of other physical channel/signal in-between the PUSCH or PUCCH transmissions.

-	For TDD, no downlink slot(s) or downlink symbol(s) or flexible symbol(s) with/without DL monitoring occasion configured in-between the PUSCH or PUCCH transmissions.

Table 6.4.2.1-2: Measurement conditions for the maximum allowable phase difference

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | PCMAX,f,c in clause 6.2.4, P-MPR = 0 |
| UE downlink received power |  | Not change |
| Operating conditions |  | Normal conditions |
| Transmission bandwidth |  | Confined within FUL_low + [4] MHz and FUL_high – [4] MHz |
| DL signal frequency |  | Not change before and during the measurement window |
| DL signal timing |  | Maintained constant before and during the measurement window |
| UL slots for testing |  | Tested on consecutive UL slots |
| PUSCH waveform for testing |  | DFT-s-OFDM |

<< end change >>