3GPP TSG-RAN WG4 Meeting #109	R4-2318725
Chicago, USA, 13 – 17 Nov, 2023

Title: 	MSD due to IMD5 in CA_n25-n77 and CA_n5-n77

Source: 	Huawei, HiSilicon

Agenda Item:	8.5.2.2

Document for:	Discussion

# 1	Introduction

In the last meeting it was proposed that for CA_n25-n77 and CA_n5-n77 that the MSD due to IMD5 can be significant[1]. As it was proposed, offline, to bring our opinions on the matter. In the following section we provide our inputs on the topic.

# 2	MSD calculations due to IMD5

First we need to recall that it was agreed to perform the MSD analysis for PC1.5 with the following output powers at the antenna ports, in the WF[2].

Issue 5-1-1: PC1.5 Tx power configuration for IMD WF: 23+27.8 is used as the power configuration in the 3Tx PC1.5 IMD calculation where band A is 23dBm and band B is 24.8dBm+24.8dBm with 2Tx

Below the MSD calculations for IMD 5 of the two band combinations are performed.

| CA_n25-n77 |  |  |  | CA_n5-n77 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna ISO | 10 | dB |  | Antenna ISO | 10 | dB |
| PCB isolation Paout-Pain | 60 | dB |  | PCB isolation Paout-Pain | 60 | dB |
| PCB isolation Paout Div-Pain main | 65 | dB |  | PCB isolation Paout Div-Pain main | 65 | dB |
| Diplexer ISO | 15 | dB |  | Diplexer ISO | 15 | dB |
| n25 TX SAW/duplexer rejection at n77 Tx | 40 | dB |  | n5 TX SAW/duplexer rejection at n77 Tx | 18 | dB |
| n25 RX duplexer rejection at n77 Tx | 50 | dB |  | n5 RX duplexer rejection at n77 Tx | 50 | dB |
| n25 SAW/Duplexer TX rejection at n25 RX band | 53 | dB |  | n5 SAW/Duplexer TX rejection at n25 RX band | 54 | dB |
| filter n77 TX rejection at n25 RX band | 45 | dB |  | filter n77 TX rejection at n5 RX band | 18 | dB |
| n77 Tx attenuation at n25 RX | 45 | dB |  | n77 Tx attenuation at n5 RX | 47 | dB |
| n77 Tx attenuation at n25 TX | 45 | dB |  | n77 Tx attenuation at n5 TX | 47 | dB |
| Diplexer IL | 0.5 | dB |  | Diplexer IL | 0.5 | dB |
| Antenna switch IL | 1 | dB |  | Antenna switch IL | 1 | dB |
| TX/RX switch IL | 0.5 | dB |  | TX/RX switch IL | 0.5 | dB |
| n25 Dup IL | 4 | dB |  | n5 Dup IL | 4 | dB |
| n77 RX SAW filter IL | 3 | dB |  | n77 RX SAW filter IL | 3 | dB |
| Pout n25 | 23 | dBm |  | Pout n5 | 23 | dBm |
| Pout n77 | 24.8 | dBm |  | Pout n77 | 24.8 | dBm |
| Pout n77 Div | 24.8 | dBm |  | Pout n77 Div | 24.8 | dBm |
| REFSENS n25 | -96.6 | dBm |  | REFSENS n5 | -98 | dBm |

Table 3.11: IMD5 level to n25 and n5 n5 Rx main path (PC1.5)

| Main Path | received interference, IMD5 (dBm) on n25 Rx |  | Main Path | received interference, IMD5 (dBm) on n5 Rx |
| --- | --- | --- | --- | --- |
| Antenna switch | -89.93 |  | Antenna switch | -89.93 |
| Diplexer (H-H or L-L) | -94.49 |  | Diplexer (H-H or L-L) | -86.49 |
| n25 SAW/Duplexer | -114.17 |  | n55 SAW/Duplexer | -92.17 |
| n25 PA Forward mixing | -55.50 |  | n5 PA Forward mixing | -63.54 |
| n77 PA Forward mixing | -144.69 |  | n77 PA Forward mixing | -215.35 |
| n25 PA Reverse mixing | -105.20 |  | n5 PA Reverse mixing | -103.24 |
| n77 PA Reverse mixing | -308.69 |  | n77 PA Reverse mixing | -334.85 |
| Total | -55.50 |  | Total | -63.50 |
| n77 LNA via filters | -91.37 |  | n77 LNA via filters | -102.41 |
| n77 LNA via PCB coupling | -116.50 |  | n77 LNA via PCB coupling | -123.54 |
| Total main IMD | -55.50 |  | Total main IMD | -63.50 |

Table 3.12: IMD level to n25 and n5 Rx diversity path (PC1.5)

| Diversity Path | received interference, IMD5 (dBm) on n25 Rx |  | Diversity Path | received interference, IMD5 (dBm) on n5 Rx |
| --- | --- | --- | --- | --- |
| From main path | -65.50 |  | From main path | -73.50 |
| Diplexer | -142.79 |  | Diplexer | -134.79 |
| Antenna switch | -133.79 |  | Antenna switch | -133.79 |
| n25 SAW Filter | -141.99 |  | n25 SAW Filter | -141.99 |
| n77 PA div Forward mixing | -206.69 |  | n77 PA div Forward mixing | -236.85 |
| n25 LNA via antenna ISO | -151.37 |  | n5 LNA via antenna ISO | -140.41 |
| n25 LNA via PCB coupling | -135.50 |  | n5 LNA via PCB coupling | -142.54 |
| Total div IMD | -65.50 |  | Total div IMD | -73.50 |

Table 3.13: CA_n25A-n77A MSD for n25 Rx due to IMDs (PC1.5) and CA_n5A-n77A MSD for n5 Rx due to IMDs (PC1.5)

| MSD calculation parameters | received interference, IMD5 (dB) on n25 Rx | received interference, IMD5 (dB) on n5 Rx |
| --- | --- | --- |
| MSD | 35.86 | 27.57 |
| SNR | -30.88 | -24.09 |
| MSD at SNR=-1 | 29.88 | 23.09 |

Proposal 1: add MSDs due to IMD5 to the MSD tables as shown below.

| : |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 29.9 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n5-n7713 | n5 | 844 | 5 | 25 | 889 | 23.1 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of TS 38.101-1 Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

Proposal 2. Make a mean out of the MSD proposals and remove the brackets for IMD5 of Table 7.3A.5-1b in TS38.101-1.

# 3	Conclusion

Proposal 1: add MSDs due to IMD5 to the MSD tables as shown below.

| : |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 29.9 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n5-n7713 | n5 | 844 | 5 | 25 | 889 | 23.1 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of TS 38.101-1 Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

Proposal 2. Make a mean out of the MSD proposals and remove the brackets for IMD5 of Table 7.3A.5-1b in TS38.101-1.

# 4	Reference

[1]	“R4-2315391, MSD analysis for PC1.5 combinations with 3Tx, Apple.”

[2]	“R4-2314705, WF on 3Tx and 4Rx requirements, Oppo.”