3GPP TSG RAN WG4 Meeting #109							     R4-2319072

Chicago, US, Nov 13th – Nov 17th, 2023

Agenda item:			8.22.2.2

Source:	vivo

Title:	Updated Link-level simulation results for SL-PRS measurement

Document for:	Discussion

Introduction

In RAN4#108bis meeting, the updated link-level simulation assumptions for sidelink positioning were agreed and captured in [1]. We presented simulation results for SL-PRS measurement in the last meeting. In this paper, we update our simulation results for SL-PRS RSTD, SL-PRS RSRP, SL-PRS RSRPP and SL-PRS Rx-Tx based on [1].

Discussion

SL-PRS RSTD simulation results

Table 1-4 shows the results for 5%-ile and 95%-ile of the SL-PRS RSTD errors with Es/Iot (-3,-6,-6) ,(0,-6,-6), (0,-3,-3) and (0,0,0), respectively. For convenience, we provide the absolute accuracy simulation results, Table 5-8 shows the results for the absolute accuracy of the SL-PRS RSTD based on absolute accuracy = max (abs (5%-ile error), abs (95%-ile error)) with Es/Iot (-3,-6,-6), (0,-6,-6), (0,-3,-3) and (0,0,0), respectively.

Table 1 5%-ile and 95%-ile of the SL-PRS RSTD errors with Es/Iot (-3,-6,-6)

| SL-PRS based RSTD [Tc] (-3, -6, -6) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  |  | TDL-A |  |  |  | TDL-B |  |  |  |
|  |  |  |  |  | UE 2 |  | UE 3 |  | UE 2 |  | UE 3 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| 15 | 48 | 4 | 4 | 1 | 47.0 | 69.2 | 47.0 | 69.0 | 19.2 | 111.3 | 21.0 | 121.6 | -19.0 | 139.5 | -9.8 | 130.6 |
|  |  |  |  | 2 | 47.2 | 69.3 | 47.1 | 69.1 | 20.6 | 80.5 | 25.8 | 83.6 | -6.7 | 101.7 | 1.3 | 97.4 |
|  |  |  |  | 3 | 47.4 | 69.1 | 47.0 | 69.1 | 22.1 | 79.2 | 25.6 | 82.9 | -5.2 | 94.6 | 7.0 | 103.2 |
|  |  |  |  | 4 | 47.3 | 69.0 | 47.2 | 69.1 | 13.3 | 78.1 | 17.7 | 83.1 | -1.1 | 98.6 | 1.6 | 103.6 |
|  |  | 4 | 2 | 1 | 42.4 | 74.2 | 33.8 | 65.4 | -19.5 | 359.2 | -22.1 | 521.1 | -72.1 | 302.9 | -61.1 | 357.9 |
|  |  |  |  | 2 | 42.6 | 74.0 | 33.9 | 65.3 | -17.7 | 97.7 | -26.0 | 108.1 | -57.1 | 134.1 | -47.6 | 142.7 |
|  |  |  |  | 3 | 42.5 | 73.8 | 33.9 | 65.9 | -34.7 | 94.4 | -22.5 | 104.9 | -57.0 | 136.5 | -61.5 | 128.1 |
|  |  |  |  | 4 | 42.3 | 73.8 | 33.9 | 65.3 | -81.6 | 94.7 | -74.3 | 103.6 | -57.7 | 137.4 | -101.8 | 127.3 |
|  | 96 | 4 | 4 | 1 | 23.8 | 31.8 | 22.9 | 30.9 | 10.1 | 42.1 | 8.8 | 45.1 | -3.6 | 58.3 | -1.1 | 62.1 |
|  |  |  |  | 2 | 23.8 | 31.8 | 22.9 | 31.0 | 13.2 | 37.8 | 15.9 | 38.0 | -0.5 | 48.4 | 5.0 | 49.3 |
|  |  |  |  | 3 | 23.8 | 31.8 | 22.9 | 31.1 | 13.9 | 36.9 | 16.3 | 37.8 | 3.5 | 46.6 | 7.1 | 46.9 |
|  |  |  |  | 4 | 23.6 | 31.8 | 22.9 | 31.0 | 15.6 | 37.5 | 17.2 | 37.4 | 9.3 | 46.7 | 6.2 | 45.5 |
|  |  | 4 | 2 | 1 | 25.5 | 36.7 | 20.0 | 31.4 | -3.3 | 57.2 | -4.7 | 68.2 | -27.8 | 86.6 | -31.8 | 94.9 |
|  |  |  |  | 2 | 25.5 | 36.5 | 20.1 | 31.4 | 1.2 | 46.8 | -1.3 | 51.2 | -10.8 | 64.9 | -14.9 | 64.9 |
|  |  |  |  | 3 | 25.7 | 36.4 | 20.2 | 31.6 | 2.0 | 45.6 | -1.7 | 52.8 | -10.8 | 68.0 | -4.0 | 59.1 |
|  |  |  |  | 4 | 25.7 | 36.4 | 20.2 | 31.7 | 1.9 | 44.5 | -2.0 | 44.9 | -6.2 | 70.8 | -6.4 | 62.8 |
| 30 | 24 | 4 | 4 | 1 | 48.2 | 80.0 | 44.2 | 74.6 | -17.7 | 353.6 | -24.7 | 182.1 | -65.6 | 308.9 | -61.5 | 183.7 |
|  |  |  |  | 2 | 48.1 | 79.3 | 44.1 | 74.1 | -17.4 | 116.7 | -25.6 | 109.4 | -45.0 | 135.1 | -39.7 | 124.2 |
|  |  |  |  | 3 | 48.0 | 79.2 | 44.6 | 73.8 | -27.7 | 101.9 | -43.6 | 101.3 | -38.3 | 128.3 | -39.9 | 119.4 |
|  |  |  |  | 4 | 48.4 | 79.0 | 44.4 | 73.5 | -34.1 | 97.5 | -55.8 | 98.1 | -33.9 | 130.7 | -39.7 | 123.9 |
|  |  | 4 | 2 | 1 | 42.7 | 90.8 | 36.5 | 90.2 | -752.2 | 381.5 | -328.7 | 1166.6 | -271.0 | 381.6 | -251.2 | 452.2 |
|  |  |  |  | 2 | 42.2 | 85.4 | 36.6 | 81.1 | -1127.6 | 128.9 | -544.8 | 176.2 | -259.5 | 175.5 | -236.3 | 191.4 |
|  |  |  |  | 3 | 42.3 | 84.7 | 36.4 | 80.8 | -1178.1 | 120.6 | -894.7 | 137.3 | -260.2 | 161.6 | -257.8 | 162.1 |
|  |  |  |  | 4 | 41.6 | 84.7 | 36.4 | 80.2 | -1455.6 | 112.7 | -1442.5 | 125.2 | -264.8 | 172.5 | -269.8 | 160.9 |
|  | 48 | 4 | 4 | 1 | 23.2 | 34.2 | 26.1 | 37.0 | 2.4 | 62.8 | 1.9 | 62.1 | -21.9 | 85.0 | -21.8 | 76.4 |
|  |  |  |  | 2 | 23.3 | 34.3 | 26.1 | 36.9 | -0.1 | 46.2 | 2.2 | 46.7 | -9.7 | 57.8 | -10.7 | 59.5 |
|  |  |  |  | 3 | 23.3 | 34.2 | 26.1 | 36.8 | 2.3 | 42.8 | 3.8 | 42.9 | -7.9 | 54.1 | -10.2 | 56.6 |
|  |  |  |  | 4 | 23.3 | 34.1 | 26.3 | 36.9 | 2.2 | 42.4 | 5.9 | 44.3 | -6.1 | 54.0 | -4.8 | 56.4 |
|  |  | 4 | 2 | 1 | 20.4 | 36.2 | 22.5 | 38.4 | -14.5 | 110.4 | -21.0 | 82.2 | -103.2 | 189.6 | -103.8 | 175.9 |
|  |  |  |  | 2 | 20.6 | 36.1 | 22.7 | 38.1 | -12.5 | 63.7 | -20.6 | 64.0 | -48.4 | 79.7 | -44.9 | 84.4 |
|  |  |  |  | 3 | 20.6 | 36.1 | 22.7 | 38.1 | -13.4 | 57.8 | -20.9 | 56.7 | -43.9 | 74.0 | -71.0 | 73.6 |
|  |  |  |  | 4 | 20.5 | 35.9 | 22.6 | 38.1 | -18.3 | 56.0 | -19.0 | 56.9 | -41.8 | 76.7 | -77.4 | 72.4 |
|  | 96 | 4 | 4 | 1 | 12.0 | 16.0 | 11.9 | 15.9 | 6.2 | 23.0 | 6.7 | 22.2 | -5.6 | 33.2 | -4.3 | 32.5 |
|  |  |  |  | 2 | 12.0 | 16.1 | 11.9 | 15.9 | 8.0 | 20.3 | 6.7 | 19.5 | -0.9 | 24.6 | -1.5 | 26.5 |
|  |  |  |  | 3 | 12.1 | 16.1 | 11.9 | 15.9 | 7.9 | 19.5 | 7.0 | 19.5 | -0.3 | 24.0 | -0.4 | 25.4 |
|  |  |  |  | 4 | 12.0 | 16.1 | 11.9 | 15.8 | 8.7 | 19.1 | 7.7 | 19.2 | 1.2 | 23.5 | 0.1 | 24.6 |
|  |  | 4 | 2 | 1 | 12.7 | 18.3 | 11.7 | 17.3 | 0.8 | 29.5 | -1.0 | 30.8 | -34.7 | 48.7 | -18.1 | 66.3 |
|  |  |  |  | 2 | 12.7 | 18.2 | 11.7 | 17.3 | 3.0 | 23.0 | 0.4 | 24.2 | -11.8 | 34.5 | -11.8 | 35.8 |
|  |  |  |  | 3 | 12.7 | 18.1 | 11.7 | 17.3 | 3.0 | 22.5 | 2.4 | 23.9 | -10.1 | 32.3 | -9.5 | 33.6 |
|  |  |  |  | 4 | 12.7 | 18.1 | 11.7 | 17.3 | 3.8 | 22.3 | 4.4 | 23.8 | -7.3 | 32.4 | -9.0 | 33.8 |
| 60 | 24 | 4 | 4 | 1 | 26.6 | 43.7 | 22.6 | 38.5 | -16.7 | 142.2 | -12.1 | 149.6 | -102.2 | 189.1 | -88.6 | 182.1 |
|  |  |  |  | 2 | 26.5 | 41.8 | 22.5 | 38.0 | -22.4 | 70.8 | -19.0 | 69.6 | -55.9 | 77.9 | -53.4 | 74.2 |
|  |  |  |  | 3 | 26.5 | 41.8 | 22.6 | 37.8 | -24.3 | 61.0 | -18.6 | 63.8 | -33.8 | 69.6 | -36.6 | 71.6 |
|  |  |  |  | 4 | 26.3 | 41.8 | 22.6 | 38.0 | -18.7 | 56.8 | -18.5 | 57.9 | -30.1 | 67.2 | -31.4 | 71.0 |
|  |  | 4 | 2 | 1 | 20.5 | 45.1 | 12.4 | 34.5 | -130.8 | 186.4 | -91.3 | 1035.1 | -202.8 | 261.8 | -153.9 | 808.2 |
|  |  |  |  | 2 | 20.3 | 41.9 | 12.7 | 34.5 | -143.4 | 92.5 | -111.9 | 271.6 | -178.6 | 171.3 | -155.1 | 185.3 |
|  |  |  |  | 3 | 20.3 | 41.7 | 12.8 | 34.4 | -144.1 | 77.5 | -117.6 | 164.7 | -164.0 | 142.0 | -150.1 | 153.5 |
|  |  |  |  | 4 | 20.3 | 41.6 | 12.9 | 34.4 | -140.5 | 74.1 | -122.8 | 97.7 | -172.4 | 142.6 | -157.5 | 163.0 |

Table 2 5%-ile and 95%-ile of the SL-PRS RSTD errors with Es/Iot (0,-6,-6)

| SL-PRS based RSTD [Tc] (0, -6, -6) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  |  | TDL-A |  |  |  | TDL-B |  |  |  |
|  |  |  |  |  | UE 2 |  | UE 3 |  | UE 2 |  | UE 3 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| 15 | 52 | 4 | 4 | 1 | 47.6 | 68.4 | 47.5 | 68.4 | 28.5 | 107.7 | 31.0 | 120.1 | -3.6 | 133.4 | 2.2 | 121.6 |
|  |  |  |  | 2 | 47.9 | 68.4 | 47.7 | 68.3 | 31.9 | 79.1 | 34.1 | 78.3 | 4.4 | 86.9 | 13.9 | 80.3 |
|  |  |  |  | 3 | 48.1 | 68.1 | 47.7 | 68.2 | 32.5 | 77.0 | 32.9 | 77.0 | 9.2 | 80.7 | 24.1 | 77.8 |
|  |  |  |  | 4 | 48.0 | 68.1 | 47.8 | 68.4 | 32.2 | 75.6 | 33.5 | 77.0 | 11.4 | 82.1 | 12.9 | 74.8 |
|  |  | 4 | 2 | 1 | 44.4 | 72.3 | 33.5 | 61.5 | 6.8 | 668.6 | 5.4 | 3274.8 | -44.8 | 346.7 | -39.7 | 415.0 |
|  |  |  |  | 2 | 44.4 | 72.1 | 33.4 | 61.6 | 12.5 | 94.0 | 11.8 | 108.5 | -33.9 | 124.5 | -20.1 | 129.3 |
|  |  |  |  | 3 | 44.2 | 72.3 | 33.4 | 61.9 | 13.0 | 89.3 | 6.5 | 99.5 | -33.9 | 117.2 | -22.9 | 112.1 |
|  |  |  |  | 4 | 43.9 | 72.3 | 33.3 | 61.9 | 6.4 | 84.4 | 2.2 | 91.4 | -35.3 | 120.2 | -20.9 | 110.5 |
|  | 104 | 4 | 4 | 1 | 23.7 | 31.4 | 22.8 | 30.3 | 16.2 | 40.1 | 11.1 | 45.0 | 0.7 | 51.5 | 2.9 | 61.5 |
|  |  |  |  | 2 | 23.8 | 31.5 | 22.8 | 30.3 | 18.4 | 36.2 | 18.5 | 37.3 | 8.9 | 44.4 | 8.4 | 44.8 |
|  |  |  |  | 3 | 23.7 | 31.4 | 23.0 | 30.4 | 19.7 | 35.3 | 18.6 | 35.4 | 8.4 | 39.9 | 12.1 | 41.7 |
|  |  |  |  | 4 | 23.8 | 31.4 | 23.0 | 30.3 | 18.3 | 34.7 | 19.9 | 35.6 | 13.2 | 41.4 | 13.0 | 37.6 |
|  |  | 4 | 2 | 1 | 26.5 | 36.5 | 19.7 | 29.9 | 1.1 | 53.2 | -3.6 | 70.0 | -20.6 | 68.6 | -24.3 | 80.4 |
|  |  |  |  | 2 | 26.6 | 36.3 | 19.8 | 30.0 | 6.3 | 41.4 | 2.7 | 44.1 | -1.2 | 54.7 | -8.9 | 55.4 |
|  |  |  |  | 3 | 26.8 | 36.3 | 19.9 | 30.0 | 7.7 | 41.4 | 3.8 | 42.7 | 3.2 | 55.6 | 0.3 | 53.3 |
|  |  |  |  | 4 | 26.7 | 36.1 | 19.9 | 30.2 | 8.3 | 41.1 | 2.7 | 41.4 | 3.3 | 54.7 | 0.8 | 52.9 |
| 30 | 24 | 4 | 4 | 1 | 50.9 | 81.6 | 45.4 | 73.5 | 5.8 | 384.7 | 0.0 | 154.2 | -43.9 | 315.8 | -44.0 | 159.2 |
|  |  |  |  | 2 | 50.7 | 80.1 | 45.4 | 73.0 | 6.2 | 115.4 | 7.4 | 102.8 | -23.4 | 124.5 | -15.9 | 112.0 |
|  |  |  |  | 3 | 50.6 | 79.9 | 45.6 | 72.6 | 5.8 | 100.7 | 1.8 | 91.0 | -9.6 | 113.8 | -11.7 | 101.7 |
|  |  |  |  | 4 | 51.1 | 79.8 | 45.3 | 72.1 | 1.8 | 97.2 | -0.4 | 93.0 | -4.1 | 114.6 | -14.2 | 106.3 |
|  |  | 4 | 2 | 1 | 46.5 | 94.3 | 39.8 | 104.8 | -274.6 | 179.6 | -66.5 | 1137.5 | -246.0 | 346.9 | -204.0 | 452.2 |
|  |  |  |  | 2 | 46.1 | 83.7 | 39.9 | 79.1 | -287.8 | 112.5 | -59.3 | 173.3 | -161.2 | 149.0 | -112.0 | 157.4 |
|  |  |  |  | 3 | 45.9 | 83.1 | 40.0 | 78.9 | -266.2 | 103.2 | -97.8 | 134.4 | -71.6 | 136.9 | -67.7 | 137.6 |
|  |  |  |  | 4 | 45.4 | 83.2 | 40.0 | 78.4 | -258.9 | 103.1 | -132.0 | 119.6 | -92.5 | 135.6 | -68.9 | 135.2 |
|  | 48 | 4 | 4 | 1 | 23.4 | 33.9 | 26.7 | 36.9 | 6.3 | 58.9 | 6.6 | 57.1 | -15.0 | 81.6 | -14.2 | 68.3 |
|  |  |  |  | 2 | 23.5 | 33.8 | 26.7 | 36.9 | 5.7 | 42.7 | 7.8 | 42.7 | -3.2 | 51.1 | -4.0 | 51.5 |
|  |  |  |  | 3 | 23.4 | 33.9 | 26.7 | 36.7 | 7.4 | 39.5 | 11.2 | 39.5 | 2.7 | 46.7 | -2.8 | 47.4 |
|  |  |  |  | 4 | 23.8 | 33.8 | 26.6 | 36.7 | 10.0 | 39.5 | 11.1 | 39.8 | 4.9 | 44.2 | 1.5 | 49.0 |
|  |  | 4 | 2 | 1 | 21.3 | 35.3 | 23.5 | 37.7 | -5.3 | 153.5 | -11.4 | 77.6 | -75.8 | 192.3 | -83.7 | 160.5 |
|  |  |  |  | 2 | 21.5 | 35.3 | 23.6 | 37.6 | -3.3 | 58.9 | -8.5 | 59.4 | -19.4 | 67.1 | -24.8 | 72.8 |
|  |  |  |  | 3 | 21.5 | 35.3 | 23.5 | 37.5 | -1.1 | 50.0 | -7.0 | 51.6 | -14.8 | 64.4 | -20.7 | 64.5 |
|  |  |  |  | 4 | 21.5 | 35.2 | 23.5 | 37.3 | -3.8 | 48.6 | -1.1 | 51.2 | -15.2 | 61.1 | -20.6 | 63.8 |
|  | 96 | 4 | 4 | 1 | 12.0 | 15.8 | 11.9 | 15.7 | 8.3 | 21.9 | 8.9 | 20.8 | -2.7 | 30.5 | -1.2 | 30.0 |
|  |  |  |  | 2 | 12.0 | 15.8 | 11.9 | 15.7 | 9.1 | 18.9 | 9.2 | 18.6 | 2.7 | 20.9 | 1.9 | 21.8 |
|  |  |  |  | 3 | 12.0 | 15.8 | 12.0 | 15.7 | 9.5 | 18.3 | 9.8 | 18.3 | 2.5 | 19.7 | 3.1 | 18.8 |
|  |  |  |  | 4 | 12.0 | 15.8 | 12.0 | 15.6 | 9.9 | 18.0 | 9.9 | 17.9 | 4.6 | 19.6 | 3.6 | 18.5 |
|  |  | 4 | 2 | 1 | 13.2 | 18.2 | 12.0 | 17.0 | 3.2 | 27.5 | 2.4 | 30.3 | -15.7 | 42.8 | -12.9 | 47.7 |
|  |  |  |  | 2 | 13.2 | 18.1 | 12.0 | 17.0 | 5.3 | 21.4 | 3.6 | 22.5 | -7.1 | 29.9 | -7.5 | 31.5 |
|  |  |  |  | 3 | 13.2 | 18.1 | 12.0 | 17.0 | 6.2 | 20.7 | 5.4 | 22.0 | -5.6 | 28.3 | -6.4 | 28.9 |
|  |  |  |  | 4 | 13.2 | 18.1 | 12.1 | 17.0 | 6.5 | 20.8 | 6.5 | 21.3 | -4.3 | 28.2 | -7.0 | 29.0 |
| 60 | 24 | 4 | 4 | 1 | 27.9 | 50.1 | 23.8 | 38.8 | -8.0 | 155.9 | -2.1 | 168.0 | -79.5 | 189.8 | -32.4 | 183.6 |
|  |  |  |  | 2 | 27.6 | 41.8 | 23.8 | 37.8 | -13.3 | 70.4 | -5.3 | 71.5 | -24.4 | 71.7 | -17.1 | 65.5 |
|  |  |  |  | 3 | 27.6 | 41.6 | 23.8 | 37.7 | -12.7 | 59.2 | -6.6 | 60.4 | -14.5 | 61.4 | -12.1 | 62.9 |
|  |  |  |  | 4 | 27.6 | 41.5 | 23.8 | 37.8 | -11.9 | 52.6 | -2.9 | 57.0 | -10.2 | 59.9 | -10.6 | 62.8 |
|  |  | 4 | 2 | 1 | 22.5 | 48.2 | 12.8 | 32.1 | -112.8 | 151.6 | -18.2 | 1360.9 | -179.5 | 220.9 | -112.7 | 1055.8 |
|  |  |  |  | 2 | 22.3 | 41.1 | 13.2 | 32.2 | -125.6 | 74.2 | -26.2 | 812.6 | -130.5 | 132.3 | -104.6 | 177.5 |
|  |  |  |  | 3 | 22.3 | 40.7 | 13.4 | 32.3 | -130.2 | 66.8 | -27.3 | 181.6 | -119.2 | 88.1 | -96.4 | 99.7 |
|  |  |  |  | 4 | 22.3 | 40.6 | 13.4 | 32.4 | -117.4 | 63.7 | -30.1 | 135.5 | -109.1 | 87.3 | -91.0 | 97.4 |

Table 3 5%-ile and 95%-ile of the SL-PRS RSTD errors with Es/Iot (0,-3,-3)

| SL-PRS based RSTD [Tc] (0, -3, -3) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  |  | TDL-A |  |  |  | TDL-B |  |  |  |
|  |  |  |  |  | UE 2 |  | UE 3 |  | UE 2 |  | UE 3 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| 15 | 52 | 4 | 4 | 1 | 48.7 | 67.5 | 48.5 | 67.5 | 30.4 | 89.5 | 32.9 | 89.1 | 3.2 | 115.2 | 12.1 | 101.7 |
|  |  |  |  | 2 | 48.8 | 67.4 | 48.7 | 67.3 | 28.9 | 75.8 | 32.1 | 76.2 | 8.1 | 85.3 | 15.5 | 78.6 |
|  |  |  |  | 3 | 49.3 | 67.1 | 48.4 | 67.4 | 28.5 | 78.9 | 27.9 | 77.3 | 10.9 | 81.4 | 24.1 | 79.4 |
|  |  |  |  | 4 | 49.4 | 67.2 | 48.4 | 67.5 | 24.3 | 78.8 | 25.5 | 79.0 | 7.0 | 88.8 | 18.9 | 78.0 |
|  |  | 4 | 2 | 1 | 44.8 | 71.6 | 36.6 | 63.4 | 8.0 | 123.9 | 6.0 | 138.5 | -32.5 | 155.4 | -33.1 | 171.4 |
|  |  |  |  | 2 | 44.8 | 72.1 | 36.2 | 63.3 | 9.6 | 90.1 | 4.7 | 100.1 | -30.4 | 121.6 | -21.8 | 123.9 |
|  |  |  |  | 3 | 44.0 | 71.4 | 36.4 | 63.5 | -1.8 | 92.0 | -7.7 | 101.5 | -48.8 | 121.1 | -41.0 | 121.6 |
|  |  |  |  | 4 | 44.3 | 72.2 | 36.4 | 63.3 | -22.9 | 92.0 | -21.7 | 99.5 | -52.1 | 122.4 | -40.3 | 127.0 |
|  | 104 | 4 | 4 | 1 | 24.4 | 31.3 | 23.6 | 30.4 | 19.3 | 37.0 | 15.7 | 40.7 | 7.2 | 48.9 | 7.2 | 55.2 |
|  |  |  |  | 2 | 24.4 | 31.3 | 23.6 | 30.4 | 19.5 | 35.2 | 19.3 | 36.3 | 10.9 | 44.1 | 12.2 | 45.8 |
|  |  |  |  | 3 | 24.4 | 31.3 | 23.7 | 30.4 | 19.4 | 34.8 | 18.5 | 35.7 | 11.0 | 44.8 | 12.5 | 44.2 |
|  |  |  |  | 4 | 24.5 | 31.3 | 23.6 | 30.4 | 18.3 | 34.8 | 19.3 | 35.7 | 13.6 | 44.1 | 13.9 | 40.5 |
|  |  | 4 | 2 | 1 | 26.1 | 35.7 | 20.9 | 30.7 | 4.7 | 46.7 | -1.1 | 56.9 | -7.3 | 63.5 | -15.2 | 69.2 |
|  |  |  |  | 2 | 26.1 | 35.6 | 21.0 | 30.7 | 5.7 | 43.4 | 1.6 | 45.3 | 0.9 | 58.6 | -5.0 | 58.5 |
|  |  |  |  | 3 | 26.3 | 35.6 | 21.0 | 30.8 | 5.5 | 44.9 | 2.1 | 45.3 | 2.4 | 60.3 | 1.6 | 58.4 |
|  |  |  |  | 4 | 26.2 | 35.6 | 21.2 | 31.0 | 3.0 | 45.0 | 0.9 | 45.0 | 2.4 | 59.5 | -1.7 | 57.3 |
| 30 | 24 | 4 | 4 | 1 | 50.3 | 77.1 | 46.6 | 72.2 | 5.7 | 129.3 | 1.3 | 109.8 | -28.3 | 152.4 | -26.6 | 131.4 |
|  |  |  |  | 2 | 50.1 | 76.7 | 46.6 | 71.9 | -0.7 | 100.1 | -1.9 | 91.9 | -16.8 | 119.7 | -18.8 | 113.8 |
|  |  |  |  | 3 | 50.0 | 76.5 | 46.5 | 71.6 | -11.7 | 91.3 | -15.2 | 88.6 | -13.6 | 114.5 | -18.2 | 110.4 |
|  |  |  |  | 4 | 50.1 | 76.5 | 46.4 | 71.3 | -26.4 | 90.8 | -22.0 | 93.1 | -14.3 | 117.9 | -21.9 | 116.7 |
|  |  | 4 | 2 | 1 | 45.6 | 84.2 | 39.8 | 79.7 | -240.8 | 128.8 | -70.1 | 193.4 | -201.7 | 189.9 | -190.2 | 301.1 |
|  |  |  |  | 2 | 45.0 | 83.1 | 39.8 | 78.8 | -309.5 | 106.2 | -229.9 | 128.2 | -189.7 | 151.5 | -173.3 | 147.6 |
|  |  |  |  | 3 | 44.8 | 83.1 | 40.0 | 78.4 | -808.3 | 101.7 | -471.0 | 117.5 | -191.5 | 144.3 | -197.8 | 145.2 |
|  |  |  |  | 4 | 44.8 | 83.1 | 39.7 | 78.3 | -1016.5 | 105.2 | -842.9 | 117.5 | -208.7 | 153.3 | -210.9 | 146.0 |
|  | 48 | 4 | 4 | 1 | 24.0 | 33.4 | 26.8 | 36.0 | 10.7 | 48.6 | 10.5 | 47.8 | -9.7 | 66.6 | -8.0 | 61.0 |
|  |  |  |  | 2 | 24.1 | 33.4 | 26.7 | 36.0 | 9.7 | 40.4 | 10.1 | 41.0 | -1.2 | 51.0 | -0.4 | 52.4 |
|  |  |  |  | 3 | 24.0 | 33.4 | 26.7 | 35.9 | 9.0 | 39.3 | 10.7 | 39.3 | 4.2 | 48.8 | -0.4 | 48.5 |
|  |  |  |  | 4 | 24.1 | 33.3 | 26.6 | 35.9 | 10.0 | 39.7 | 9.9 | 40.9 | 5.0 | 48.8 | -0.1 | 49.9 |
|  |  | 4 | 2 | 1 | 21.6 | 35.0 | 23.6 | 37.2 | -3.0 | 68.5 | -8.1 | 66.1 | -29.8 | 141.7 | -45.2 | 98.4 |
|  |  |  |  | 2 | 21.6 | 35.0 | 23.7 | 37.1 | -2.6 | 53.4 | -7.2 | 53.2 | -17.3 | 67.0 | -23.0 | 72.8 |
|  |  |  |  | 3 | 21.7 | 34.9 | 23.6 | 37.0 | -3.8 | 50.9 | -7.0 | 50.0 | -14.8 | 66.0 | -24.0 | 70.1 |
|  |  |  |  | 4 | 21.7 | 34.8 | 23.5 | 37.1 | -7.0 | 53.4 | -6.0 | 52.7 | -17.9 | 66.7 | -26.2 | 68.2 |
|  | 96 | 4 | 4 | 1 | 12.3 | 15.8 | 12.2 | 15.6 | 9.2 | 19.7 | 9.7 | 19.0 | 0.7 | 27.6 | 1.3 | 27.1 |
|  |  |  |  | 2 | 12.3 | 15.7 | 12.2 | 15.6 | 9.4 | 18.4 | 9.7 | 18.1 | 3.3 | 20.9 | 3.5 | 21.8 |
|  |  |  |  | 3 | 12.3 | 15.8 | 12.2 | 15.6 | 9.4 | 18.4 | 9.6 | 18.3 | 1.9 | 20.1 | 3.5 | 18.9 |
|  |  |  |  | 4 | 12.3 | 15.8 | 12.2 | 15.6 | 9.5 | 18.2 | 9.4 | 18.2 | 4.7 | 21.6 | 3.5 | 18.7 |
|  |  | 4 | 2 | 1 | 13.0 | 17.8 | 12.1 | 17.0 | 4.3 | 24.1 | 4.0 | 24.4 | -9.1 | 37.8 | -8.3 | 39.1 |
|  |  |  |  | 2 | 13.0 | 17.7 | 12.1 | 16.9 | 4.9 | 21.0 | 4.5 | 21.9 | -5.7 | 30.8 | -6.6 | 31.6 |
|  |  |  |  | 3 | 13.0 | 17.8 | 12.1 | 17.0 | 4.7 | 21.3 | 5.4 | 22.6 | -5.0 | 30.4 | -6.0 | 32.5 |
|  |  |  |  | 4 | 13.0 | 17.8 | 12.1 | 17.0 | 4.4 | 21.2 | 5.5 | 22.7 | -4.3 | 30.9 | -8.0 | 34.6 |
| 60 | 24 | 4 | 4 | 1 | 27.5 | 41.4 | 23.6 | 37.0 | -4.1 | 73.5 | -0.8 | 72.7 | -36.1 | 135.8 | -24.7 | 95.0 |
|  |  |  |  | 2 | 27.4 | 40.8 | 23.6 | 36.9 | -7.3 | 59.3 | -2.9 | 58.5 | -22.0 | 68.9 | -17.7 | 65.6 |
|  |  |  |  | 3 | 27.4 | 40.8 | 23.6 | 36.8 | -9.3 | 51.8 | -4.5 | 54.8 | -17.1 | 63.1 | -16.4 | 65.2 |
|  |  |  |  | 4 | 27.4 | 40.9 | 23.6 | 36.8 | -9.0 | 51.4 | -2.6 | 53.3 | -14.5 | 64.2 | -19.7 | 69.0 |
|  |  | 4 | 2 | 1 | 22.0 | 41.4 | 14.2 | 33.4 | -80.9 | 82.9 | -22.5 | 214.6 | -140.2 | 180.7 | -117.3 | 265.4 |
|  |  |  |  | 2 | 21.8 | 40.7 | 14.6 | 33.4 | -103.1 | 69.3 | -31.3 | 139.1 | -127.0 | 132.5 | -114.4 | 148.5 |
|  |  |  |  | 3 | 21.8 | 40.6 | 14.6 | 33.5 | -111.4 | 65.0 | -40.6 | 80.0 | -133.1 | 116.4 | -123.2 | 132.9 |
|  |  |  |  | 4 | 21.8 | 40.4 | 14.8 | 33.6 | -103.3 | 66.0 | -66.9 | 73.9 | -134.6 | 137.5 | -126.7 | 143.8 |

Table 4 5%-ile and 95%-ile of the SL-PRS RSTD errors with Es/Iot (0,0,0)

| SL-PRS based RSTD [Tc] (0, 0, 0) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  |  | TDL-A |  |  |  | TDL-B |  |  |  |
|  |  |  |  |  | UE 2 |  | UE 3 |  | UE 2 |  | UE 3 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| 15 | 52 | 4 | 4 | 1 | 48.4 | 67.6 | 48.5 | 67.6 | 29.9 | 83.6 | 30.8 | 82.6 | 1.7 | 113.3 | 11.0 | 103.6 |
|  |  |  |  | 2 | 48.5 | 67.4 | 48.5 | 67.5 | 24.7 | 77.1 | 27.8 | 78.0 | 7.7 | 97.5 | 10.2 | 87.0 |
|  |  |  |  | 3 | 49.1 | 67.2 | 48.5 | 67.5 | 22.3 | 81.3 | 23.3 | 81.3 | 1.6 | 102.3 | 14.4 | 94.5 |
|  |  |  |  | 4 | 49.3 | 67.3 | 48.5 | 67.6 | 1.8 | 82.8 | 17.4 | 83.4 | -1.0 | 111.9 | 2.0 | 95.1 |
|  |  | 4 | 2 | 1 | 43.8 | 72.5 | 37.1 | 65.7 | -2.8 | 103.5 | -9.3 | 117.7 | -31.0 | 144.5 | -30.6 | 153.4 |
|  |  |  |  | 2 | 43.8 | 73.1 | 36.6 | 65.6 | -31.6 | 88.3 | -27.4 | 97.3 | -43.9 | 123.6 | -29.0 | 124.3 |
|  |  |  |  | 3 | 43.2 | 72.4 | 36.9 | 65.6 | -232.0 | 94.1 | -212.8 | 101.8 | -63.1 | 124.3 | -58.0 | 126.4 |
|  |  |  |  | 4 | 43.6 | 73.1 | 36.6 | 65.5 | -892.3 | 93.3 | -889.5 | 100.8 | -76.7 | 130.8 | -79.9 | 138.7 |
|  | 104 | 4 | 4 | 1 | 24.6 | 31.5 | 23.9 | 30.9 | 19.0 | 36.4 | 16.6 | 39.0 | 7.6 | 47.9 | 8.0 | 53.8 |
|  |  |  |  | 2 | 24.6 | 31.5 | 23.9 | 30.8 | 17.6 | 35.9 | 18.5 | 36.6 | 9.8 | 45.8 | 11.6 | 48.9 |
|  |  |  |  | 3 | 24.7 | 31.4 | 24.0 | 30.9 | 17.6 | 36.3 | 16.3 | 37.2 | 7.7 | 46.1 | 11.4 | 47.3 |
|  |  |  |  | 4 | 24.7 | 31.4 | 23.8 | 30.8 | 11.7 | 37.3 | 15.9 | 37.3 | 10.9 | 45.5 | 8.0 | 47.0 |
|  |  | 4 | 2 | 1 | 25.4 | 35.8 | 21.3 | 31.8 | 4.5 | 48.1 | 0.2 | 52.2 | -3.9 | 62.8 | -11.3 | 66.7 |
|  |  |  |  | 2 | 25.3 | 35.7 | 21.3 | 31.8 | 4.0 | 47.4 | 2.7 | 46.5 | -1.1 | 59.9 | -6.6 | 61.0 |
|  |  |  |  | 3 | 25.5 | 35.6 | 21.3 | 31.9 | 0.3 | 49.0 | 0.2 | 48.6 | -3.0 | 65.3 | -2.0 | 62.1 |
|  |  |  |  | 4 | 25.4 | 35.7 | 21.4 | 32.0 | 0.2 | 49.4 | -1.1 | 50.9 | -6.2 | 64.2 | -5.5 | 62.1 |
| 30 | 24 | 4 | 4 | 1 | 48.7 | 75.4 | 46.1 | 72.3 | 0.0 | 110.9 | -5.5 | 101.0 | -27.6 | 143.3 | -27.8 | 128.6 |
|  |  |  |  | 2 | 48.7 | 75.3 | 46.1 | 72.1 | -13.7 | 95.6 | -17.7 | 89.9 | -26.5 | 124.3 | -31.3 | 120.0 |
|  |  |  |  | 3 | 48.8 | 75.5 | 45.9 | 72.0 | -35.9 | 93.1 | -38.1 | 92.7 | -30.4 | 122.8 | -39.6 | 122.6 |
|  |  |  |  | 4 | 48.8 | 75.4 | 45.7 | 71.5 | -187.0 | 93.9 | -220.5 | 96.9 | -37.3 | 129.7 | -45.2 | 126.0 |
|  |  | 4 | 2 | 1 | 42.7 | 83.9 | 37.7 | 79.4 | -333.8 | 121.1 | -249.3 | 151.3 | -218.9 | 183.5 | -211.8 | 213.0 |
|  |  |  |  | 2 | 42.2 | 83.8 | 37.7 | 79.6 | -1013.9 | 106.4 | -781.0 | 123.7 | -253.1 | 162.9 | -242.1 | 167.5 |
|  |  |  |  | 3 | 42.3 | 83.6 | 37.7 | 79.2 | -1748.6 | 105.5 | -1590.5 | 121.2 | -282.7 | 166.6 | -273.2 | 174.8 |
|  |  |  |  | 4 | 41.9 | 83.4 | 37.9 | 79.3 | -2023.5 | 109.2 | -1795.9 | 125.3 | -312.1 | 175.7 | -287.5 | 195.2 |
|  | 48 | 4 | 4 | 1 | 24.0 | 33.5 | 26.4 | 35.8 | 11.4 | 46.1 | 11.1 | 47.1 | -7.9 | 63.1 | -4.7 | 60.3 |
|  |  |  |  | 2 | 24.0 | 33.5 | 26.4 | 35.9 | 8.5 | 41.1 | 9.3 | 42.6 | -1.3 | 53.4 | -2.4 | 56.0 |
|  |  |  |  | 3 | 24.0 | 33.6 | 26.3 | 35.7 | 6.5 | 40.5 | 8.2 | 41.0 | 0.2 | 53.5 | -3.6 | 52.7 |
|  |  |  |  | 4 | 24.1 | 33.6 | 26.3 | 35.7 | 6.8 | 41.9 | 7.1 | 44.5 | 1.1 | 55.4 | -3.6 | 56.9 |
|  |  | 4 | 2 | 1 | 21.1 | 35.4 | 23.1 | 37.5 | -6.7 | 64.7 | -11.3 | 64.0 | -35.0 | 91.2 | -40.0 | 88.4 |
|  |  |  |  | 2 | 21.0 | 35.3 | 23.1 | 37.4 | -9.0 | 55.6 | -16.2 | 56.3 | -29.9 | 73.4 | -29.7 | 76.6 |
|  |  |  |  | 3 | 20.9 | 35.2 | 23.1 | 37.4 | -14.3 | 56.3 | -15.8 | 56.6 | -31.2 | 81.0 | -69.2 | 80.0 |
|  |  |  |  | 4 | 21.0 | 35.2 | 23.1 | 37.5 | -21.3 | 57.3 | -24.4 | 59.4 | -44.3 | 82.0 | -91.5 | 79.6 |
|  | 96 | 4 | 4 | 1 | 12.4 | 15.9 | 12.3 | 15.7 | 9.1 | 19.5 | 9.6 | 18.7 | 1.3 | 27.6 | 1.8 | 26.4 |
|  |  |  |  | 2 | 12.4 | 15.8 | 12.3 | 15.7 | 9.1 | 18.8 | 9.0 | 18.4 | 3.5 | 23.7 | 1.8 | 23.3 |
|  |  |  |  | 3 | 12.4 | 15.8 | 12.3 | 15.7 | 8.6 | 19.0 | 8.3 | 19.1 | 2.2 | 24.4 | 1.5 | 21.1 |
|  |  |  |  | 4 | 12.4 | 15.8 | 12.3 | 15.7 | 8.4 | 19.1 | 7.3 | 19.5 | 3.9 | 25.3 | 1.2 | 21.9 |
|  |  | 4 | 2 | 1 | 12.7 | 17.7 | 11.9 | 17.1 | 3.6 | 23.6 | 3.5 | 23.3 | -8.2 | 37.2 | -7.2 | 38.1 |
|  |  |  |  | 2 | 12.7 | 17.7 | 11.9 | 17.1 | 3.5 | 21.7 | 3.3 | 22.6 | -7.4 | 33.0 | -6.8 | 33.4 |
|  |  |  |  | 3 | 12.7 | 17.8 | 11.9 | 17.1 | 3.2 | 23.1 | 2.5 | 24.4 | -8.7 | 34.5 | -8.2 | 35.1 |
|  |  |  |  | 4 | 12.7 | 17.7 | 11.9 | 17.1 | 1.1 | 24.0 | 0.2 | 26.6 | -7.4 | 35.4 | -10.6 | 36.8 |
| 60 | 24 | 4 | 4 | 1 | 26.7 | 40.5 | 23.0 | 36.7 | -5.7 | 66.3 | -2.9 | 65.8 | -36.0 | 99.2 | -25.8 | 86.3 |
|  |  |  |  | 2 | 26.7 | 40.4 | 23.1 | 36.6 | -10.4 | 56.8 | -6.6 | 55.1 | -28.1 | 74.5 | -24.7 | 70.3 |
|  |  |  |  | 3 | 26.6 | 40.4 | 23.1 | 36.6 | -12.0 | 53.1 | -9.8 | 54.0 | -24.7 | 71.4 | -29.3 | 71.4 |
|  |  |  |  | 4 | 26.6 | 40.5 | 23.0 | 36.5 | -13.2 | 54.0 | -7.9 | 52.4 | -27.8 | 75.4 | -37.9 | 78.5 |
|  |  | 4 | 2 | 1 | 20.4 | 41.3 | 14.1 | 35.1 | -105.4 | 77.2 | -80.7 | 144.1 | -145.5 | 172.7 | -135.4 | 192.5 |
|  |  |  |  | 2 | 20.4 | 41.0 | 14.4 | 35.1 | -119.6 | 69.5 | -99.7 | 79.3 | -154.8 | 158.1 | -146.6 | 157.7 |
|  |  |  |  | 3 | 20.4 | 40.9 | 14.4 | 35.2 | -124.6 | 67.2 | -120.0 | 73.9 | -184.5 | 152.2 | -173.3 | 157.5 |
|  |  |  |  | 4 | 20.5 | 40.7 | 14.5 | 35.2 | -137.9 | 70.9 | -125.5 | 69.3 | -207.7 | 168.2 | -189.3 | 168.2 |

Table 5 absolute accuracy of the SL-PRS RSTD with Es/Iot (-3,-6,-6)

| SL-PRS based RSTD [Tc] (-3, -6, -6) dB |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  | TDL-A |  | TDL-B |  |
|  |  |  |  |  | UE 2 | UE 3 | UE 2 | UE 3 | UE 2 | UE 3 |
| 15 | 48 | 4 | 4 | 1 | 69.2 | 69.0 | 111.3 | 121.6 | 139.5 | 130.6 |
|  |  |  |  | 2 | 69.3 | 69.1 | 80.5 | 83.6 | 101.7 | 97.4 |
|  |  |  |  | 3 | 69.1 | 69.1 | 79.2 | 82.9 | 94.6 | 103.2 |
|  |  |  |  | 4 | 69.0 | 69.1 | 78.1 | 83.1 | 98.6 | 103.6 |
|  |  | 4 | 2 | 1 | 74.2 | 65.4 | 359.2 | 521.1 | 302.9 | 357.9 |
|  |  |  |  | 2 | 74.0 | 65.3 | 97.7 | 108.1 | 134.1 | 142.7 |
|  |  |  |  | 3 | 73.8 | 65.9 | 94.4 | 104.9 | 136.5 | 128.1 |
|  |  |  |  | 4 | 73.8 | 65.3 | 94.7 | 103.6 | 137.4 | 127.3 |
|  | 96 | 4 | 4 | 1 | 31.8 | 30.9 | 42.1 | 45.1 | 58.3 | 62.1 |
|  |  |  |  | 2 | 31.8 | 31.0 | 37.8 | 38.0 | 48.4 | 49.3 |
|  |  |  |  | 3 | 31.8 | 31.1 | 36.9 | 37.8 | 46.6 | 46.9 |
|  |  |  |  | 4 | 31.8 | 31.0 | 37.5 | 37.4 | 46.7 | 45.5 |
|  |  | 4 | 2 | 1 | 36.7 | 31.4 | 57.2 | 68.2 | 86.6 | 94.9 |
|  |  |  |  | 2 | 36.5 | 31.4 | 46.8 | 51.2 | 64.9 | 64.9 |
|  |  |  |  | 3 | 36.4 | 31.6 | 45.6 | 52.8 | 68.0 | 59.1 |
|  |  |  |  | 4 | 36.4 | 31.7 | 44.5 | 44.9 | 70.8 | 62.8 |
| 30 | 24 | 4 | 4 | 1 | 80.0 | 74.6 | 353.6 | 182.1 | 308.9 | 183.7 |
|  |  |  |  | 2 | 79.3 | 74.1 | 116.7 | 109.4 | 135.1 | 124.2 |
|  |  |  |  | 3 | 79.2 | 73.8 | 101.9 | 101.3 | 128.3 | 119.4 |
|  |  |  |  | 4 | 79.0 | 73.5 | 97.5 | 98.1 | 130.7 | 123.9 |
|  |  | 4 | 2 | 1 | 90.8 | 90.2 | 752.2 | 1166.6 | 381.6 | 452.2 |
|  |  |  |  | 2 | 85.4 | 81.1 | 1127.6 | 544.8 | 259.5 | 236.3 |
|  |  |  |  | 3 | 84.7 | 80.8 | 1178.1 | 894.7 | 260.2 | 257.8 |
|  |  |  |  | 4 | 84.7 | 80.2 | 1455.6 | 1442.5 | 264.8 | 269.8 |
|  | 48 | 4 | 4 | 1 | 34.2 | 37.0 | 62.8 | 62.1 | 85.0 | 76.4 |
|  |  |  |  | 2 | 34.3 | 36.9 | 46.2 | 46.7 | 57.8 | 59.5 |
|  |  |  |  | 3 | 34.2 | 36.8 | 42.8 | 42.9 | 54.1 | 56.6 |
|  |  |  |  | 4 | 34.1 | 36.9 | 42.4 | 44.3 | 54.0 | 56.4 |
|  |  | 4 | 2 | 1 | 36.2 | 38.4 | 110.4 | 82.2 | 189.6 | 175.9 |
|  |  |  |  | 2 | 36.1 | 38.1 | 63.7 | 64.0 | 79.7 | 84.4 |
|  |  |  |  | 3 | 36.1 | 38.1 | 57.8 | 56.7 | 74.0 | 73.6 |
|  |  |  |  | 4 | 35.9 | 38.1 | 56.0 | 56.9 | 76.7 | 77.4 |
|  | 96 | 4 | 4 | 1 | 16.0 | 15.9 | 23.0 | 22.2 | 33.2 | 32.5 |
|  |  |  |  | 2 | 16.1 | 15.9 | 20.3 | 19.5 | 24.6 | 26.5 |
|  |  |  |  | 3 | 16.1 | 15.9 | 19.5 | 19.5 | 24.0 | 25.4 |
|  |  |  |  | 4 | 16.1 | 15.8 | 19.1 | 19.2 | 23.5 | 24.6 |
|  |  | 4 | 2 | 1 | 18.3 | 17.3 | 29.5 | 30.8 | 48.7 | 66.3 |
|  |  |  |  | 2 | 18.2 | 17.3 | 23.0 | 24.2 | 34.5 | 35.8 |
|  |  |  |  | 3 | 18.1 | 17.3 | 22.5 | 23.9 | 32.3 | 33.6 |
|  |  |  |  | 4 | 18.1 | 17.3 | 22.3 | 23.8 | 32.4 | 33.8 |
| 60 | 24 | 4 | 4 | 1 | 43.7 | 38.5 | 142.2 | 149.6 | 189.1 | 182.1 |
|  |  |  |  | 2 | 41.8 | 38.0 | 70.8 | 69.6 | 77.9 | 74.2 |
|  |  |  |  | 3 | 41.8 | 37.8 | 61.0 | 63.8 | 69.6 | 71.6 |
|  |  |  |  | 4 | 41.8 | 38.0 | 56.8 | 57.9 | 67.2 | 71.0 |
|  |  | 4 | 2 | 1 | 45.1 | 34.5 | 186.4 | 1035.1 | 261.8 | 808.2 |
|  |  |  |  | 2 | 41.9 | 34.5 | 143.4 | 271.6 | 178.6 | 185.3 |
|  |  |  |  | 3 | 41.7 | 34.4 | 144.1 | 164.7 | 164.0 | 153.5 |
|  |  |  |  | 4 | 41.6 | 34.4 | 140.5 | 122.8 | 172.4 | 163.0 |

Table 6 absolute accuracy of the SL-PRS RSTD with Es/Iot (0,-6,-6)

| SL-PRS based RSTD [Tc] (0, -6, -6) dB |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  | TDL-A |  | TDL-B |  |
|  |  |  |  |  | UE 2 | UE 3 | UE 2 | UE 3 | UE 2 | UE 3 |
| 15 | 48 | 4 | 4 | 1 | 68.4 | 68.4 | 107.7 | 120.1 | 133.4 | 121.6 |
|  |  |  |  | 2 | 68.4 | 68.3 | 79.1 | 78.3 | 86.9 | 80.3 |
|  |  |  |  | 3 | 68.1 | 68.2 | 77.0 | 77.0 | 80.7 | 77.8 |
|  |  |  |  | 4 | 68.1 | 68.4 | 75.6 | 77.0 | 82.1 | 74.8 |
|  |  | 4 | 2 | 1 | 72.3 | 61.5 | 668.6 | 3274.8 | 346.7 | 415.0 |
|  |  |  |  | 2 | 72.1 | 61.6 | 94.0 | 108.5 | 124.5 | 129.3 |
|  |  |  |  | 3 | 72.3 | 61.9 | 89.3 | 99.5 | 117.2 | 112.1 |
|  |  |  |  | 4 | 72.3 | 61.9 | 84.4 | 91.4 | 120.2 | 110.5 |
|  | 96 | 4 | 4 | 1 | 31.4 | 30.3 | 40.1 | 45.0 | 51.5 | 61.5 |
|  |  |  |  | 2 | 31.5 | 30.3 | 36.2 | 37.3 | 44.4 | 44.8 |
|  |  |  |  | 3 | 31.4 | 30.4 | 35.3 | 35.4 | 39.9 | 41.7 |
|  |  |  |  | 4 | 31.4 | 30.3 | 34.7 | 35.6 | 41.4 | 37.6 |
|  |  | 4 | 2 | 1 | 36.5 | 29.9 | 53.2 | 70.0 | 68.6 | 80.4 |
|  |  |  |  | 2 | 36.3 | 30.0 | 41.4 | 44.1 | 54.7 | 55.4 |
|  |  |  |  | 3 | 36.3 | 30.0 | 41.4 | 42.7 | 55.6 | 53.3 |
|  |  |  |  | 4 | 36.1 | 30.2 | 41.1 | 41.4 | 54.7 | 52.9 |
| 30 | 24 | 4 | 4 | 1 | 81.6 | 73.5 | 384.7 | 154.2 | 315.8 | 159.2 |
|  |  |  |  | 2 | 80.1 | 73.0 | 115.4 | 102.8 | 124.5 | 112.0 |
|  |  |  |  | 3 | 79.9 | 72.6 | 100.7 | 91.0 | 113.8 | 101.7 |
|  |  |  |  | 4 | 79.8 | 72.1 | 97.2 | 93.0 | 114.6 | 106.3 |
|  |  | 4 | 2 | 1 | 94.3 | 104.8 | 274.6 | 1137.5 | 346.9 | 452.2 |
|  |  |  |  | 2 | 83.7 | 79.1 | 287.8 | 173.3 | 161.2 | 157.4 |
|  |  |  |  | 3 | 83.1 | 78.9 | 266.2 | 134.4 | 136.9 | 137.6 |
|  |  |  |  | 4 | 83.2 | 78.4 | 258.9 | 132.0 | 135.6 | 135.2 |
|  | 48 | 4 | 4 | 1 | 33.9 | 36.9 | 58.9 | 57.1 | 81.6 | 68.3 |
|  |  |  |  | 2 | 33.8 | 36.9 | 42.7 | 42.7 | 51.1 | 51.5 |
|  |  |  |  | 3 | 33.9 | 36.7 | 39.5 | 39.5 | 46.7 | 47.4 |
|  |  |  |  | 4 | 33.8 | 36.7 | 39.5 | 39.8 | 44.2 | 49.0 |
|  |  | 4 | 2 | 1 | 35.3 | 37.7 | 153.5 | 77.6 | 192.3 | 160.5 |
|  |  |  |  | 2 | 35.3 | 37.6 | 58.9 | 59.4 | 67.1 | 72.8 |
|  |  |  |  | 3 | 35.3 | 37.5 | 50.0 | 51.6 | 64.4 | 64.5 |
|  |  |  |  | 4 | 35.2 | 37.3 | 48.6 | 51.2 | 61.1 | 63.8 |
|  | 96 | 4 | 4 | 1 | 15.8 | 15.7 | 21.9 | 20.8 | 30.5 | 30.0 |
|  |  |  |  | 2 | 15.8 | 15.7 | 18.9 | 18.6 | 20.9 | 21.8 |
|  |  |  |  | 3 | 15.8 | 15.7 | 18.3 | 18.3 | 19.7 | 18.8 |
|  |  |  |  | 4 | 15.8 | 15.6 | 18.0 | 17.9 | 19.6 | 18.5 |
|  |  | 4 | 2 | 1 | 18.2 | 17.0 | 27.5 | 30.3 | 42.8 | 47.7 |
|  |  |  |  | 2 | 18.1 | 17.0 | 21.4 | 22.5 | 29.9 | 31.5 |
|  |  |  |  | 3 | 18.1 | 17.0 | 20.7 | 22.0 | 28.3 | 28.9 |
|  |  |  |  | 4 | 18.1 | 17.0 | 20.8 | 21.3 | 28.2 | 29.0 |
| 60 | 24 | 4 | 4 | 1 | 50.1 | 38.8 | 155.9 | 168.0 | 189.8 | 183.6 |
|  |  |  |  | 2 | 41.8 | 37.8 | 70.4 | 71.5 | 71.7 | 65.5 |
|  |  |  |  | 3 | 41.6 | 37.7 | 59.2 | 60.4 | 61.4 | 62.9 |
|  |  |  |  | 4 | 41.5 | 37.8 | 52.6 | 57.0 | 59.9 | 62.8 |
|  |  | 4 | 2 | 1 | 48.2 | 32.1 | 151.6 | 1360.9 | 220.9 | 1055.8 |
|  |  |  |  | 2 | 41.1 | 32.2 | 125.6 | 812.6 | 132.3 | 177.5 |
|  |  |  |  | 3 | 40.7 | 32.3 | 130.2 | 181.6 | 119.2 | 99.7 |
|  |  |  |  | 4 | 40.6 | 32.4 | 117.4 | 135.5 | 109.1 | 97.4 |

Table 7 absolute accuracy of the SL-PRS RSTD with Es/Iot (0,-3,-3)

| SL-PRS based RSTD [Tc] (0, -3, -3) dB |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  | TDL-A |  | TDL-B |  |
|  |  |  |  |  | UE 2 | UE 3 | UE 2 | UE 3 | UE 2 | UE 3 |
| 15 | 48 | 4 | 4 | 1 | 67.5 | 67.5 | 89.5 | 89.1 | 115.2 | 101.7 |
|  |  |  |  | 2 | 67.4 | 67.3 | 75.8 | 76.2 | 85.3 | 78.6 |
|  |  |  |  | 3 | 67.1 | 67.4 | 78.9 | 77.3 | 81.4 | 79.4 |
|  |  |  |  | 4 | 67.2 | 67.5 | 78.8 | 79.0 | 88.8 | 78.0 |
|  |  | 4 | 2 | 1 | 71.6 | 63.4 | 123.9 | 138.5 | 155.4 | 171.4 |
|  |  |  |  | 2 | 72.1 | 63.3 | 90.1 | 100.1 | 121.6 | 123.9 |
|  |  |  |  | 3 | 71.4 | 63.5 | 92.0 | 101.5 | 121.1 | 121.6 |
|  |  |  |  | 4 | 72.2 | 63.3 | 92.0 | 99.5 | 122.4 | 127.0 |
|  | 96 | 4 | 4 | 1 | 31.3 | 30.4 | 37.0 | 40.7 | 48.9 | 55.2 |
|  |  |  |  | 2 | 31.3 | 30.4 | 35.2 | 36.3 | 44.1 | 45.8 |
|  |  |  |  | 3 | 31.3 | 30.4 | 34.8 | 35.7 | 44.8 | 44.2 |
|  |  |  |  | 4 | 31.3 | 30.4 | 34.8 | 35.7 | 44.1 | 40.5 |
|  |  | 4 | 2 | 1 | 35.7 | 30.7 | 46.7 | 56.9 | 63.5 | 69.2 |
|  |  |  |  | 2 | 35.6 | 30.7 | 43.4 | 45.3 | 58.6 | 58.5 |
|  |  |  |  | 3 | 35.6 | 30.8 | 44.9 | 45.3 | 60.3 | 58.4 |
|  |  |  |  | 4 | 35.6 | 31.0 | 45.0 | 45.0 | 59.5 | 57.3 |
| 30 | 24 | 4 | 4 | 1 | 77.1 | 72.2 | 129.3 | 109.8 | 152.4 | 131.4 |
|  |  |  |  | 2 | 76.7 | 71.9 | 100.1 | 91.9 | 119.7 | 113.8 |
|  |  |  |  | 3 | 76.5 | 71.6 | 91.3 | 88.6 | 114.5 | 110.4 |
|  |  |  |  | 4 | 76.5 | 71.3 | 90.8 | 93.1 | 117.9 | 116.7 |
|  |  | 4 | 2 | 1 | 84.2 | 79.7 | 240.8 | 193.4 | 201.7 | 301.1 |
|  |  |  |  | 2 | 83.1 | 78.8 | 309.5 | 229.9 | 189.7 | 173.3 |
|  |  |  |  | 3 | 83.1 | 78.4 | 808.3 | 471.0 | 191.5 | 197.8 |
|  |  |  |  | 4 | 83.1 | 78.3 | 1016.5 | 842.9 | 208.7 | 210.9 |
|  | 48 | 4 | 4 | 1 | 33.4 | 36.0 | 48.6 | 47.8 | 66.6 | 61.0 |
|  |  |  |  | 2 | 33.4 | 36.0 | 40.4 | 41.0 | 51.0 | 52.4 |
|  |  |  |  | 3 | 33.4 | 35.9 | 39.3 | 39.3 | 48.8 | 48.5 |
|  |  |  |  | 4 | 33.3 | 35.9 | 39.7 | 40.9 | 48.8 | 49.9 |
|  |  | 4 | 2 | 1 | 35.0 | 37.2 | 68.5 | 66.1 | 141.7 | 98.4 |
|  |  |  |  | 2 | 35.0 | 37.1 | 53.4 | 53.2 | 67.0 | 72.8 |
|  |  |  |  | 3 | 34.9 | 37.0 | 50.9 | 50.0 | 66.0 | 70.1 |
|  |  |  |  | 4 | 34.8 | 37.1 | 53.4 | 52.7 | 66.7 | 68.2 |
|  | 96 | 4 | 4 | 1 | 15.8 | 15.6 | 19.7 | 19.0 | 27.6 | 27.1 |
|  |  |  |  | 2 | 15.7 | 15.6 | 18.4 | 18.1 | 20.9 | 21.8 |
|  |  |  |  | 3 | 15.8 | 15.6 | 18.4 | 18.3 | 20.1 | 18.9 |
|  |  |  |  | 4 | 15.8 | 15.6 | 18.2 | 18.2 | 21.6 | 18.7 |
|  |  | 4 | 2 | 1 | 17.8 | 17.0 | 24.1 | 24.4 | 37.8 | 39.1 |
|  |  |  |  | 2 | 17.7 | 16.9 | 21.0 | 21.9 | 30.8 | 31.6 |
|  |  |  |  | 3 | 17.8 | 17.0 | 21.3 | 22.6 | 30.4 | 32.5 |
|  |  |  |  | 4 | 17.8 | 17.0 | 21.2 | 22.7 | 30.9 | 34.6 |
| 60 | 24 | 4 | 4 | 1 | 41.4 | 37.0 | 73.5 | 72.7 | 135.8 | 95.0 |
|  |  |  |  | 2 | 40.8 | 36.9 | 59.3 | 58.5 | 68.9 | 65.6 |
|  |  |  |  | 3 | 40.8 | 36.8 | 51.8 | 54.8 | 63.1 | 65.2 |
|  |  |  |  | 4 | 40.9 | 36.8 | 51.4 | 53.3 | 64.2 | 69.0 |
|  |  | 4 | 2 | 1 | 41.4 | 33.4 | 82.9 | 214.6 | 180.7 | 265.4 |
|  |  |  |  | 2 | 40.7 | 33.4 | 103.1 | 139.1 | 132.5 | 148.5 |
|  |  |  |  | 3 | 40.6 | 33.5 | 111.4 | 80.0 | 133.1 | 132.9 |
|  |  |  |  | 4 | 40.4 | 33.6 | 103.3 | 73.9 | 137.5 | 143.8 |

Table 8 absolute accuracy of the SL-PRS RSTD with Es/Iot (0,0,0)

| SL-PRS based RSTD [Tc] (0, 0, 0) dB |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  | TDL-A |  | TDL-B |  |
|  |  |  |  |  | UE 2 | UE 3 | UE 2 | UE 3 | UE 2 | UE 3 |
| 15 | 48 | 4 | 4 | 1 | 67.6 | 67.6 | 83.6 | 82.6 | 113.3 | 103.6 |
|  |  |  |  | 2 | 67.4 | 67.5 | 77.1 | 78.0 | 97.5 | 87.0 |
|  |  |  |  | 3 | 67.2 | 67.5 | 81.3 | 81.3 | 102.3 | 94.5 |
|  |  |  |  | 4 | 67.3 | 67.6 | 82.8 | 83.4 | 111.9 | 95.1 |
|  |  | 4 | 2 | 1 | 72.5 | 65.7 | 103.5 | 117.7 | 144.5 | 153.4 |
|  |  |  |  | 2 | 73.1 | 65.6 | 88.3 | 97.3 | 123.6 | 124.3 |
|  |  |  |  | 3 | 72.4 | 65.6 | 232.0 | 212.8 | 124.3 | 126.4 |
|  |  |  |  | 4 | 73.1 | 65.5 | 892.3 | 889.5 | 130.8 | 138.7 |
|  | 96 | 4 | 4 | 1 | 31.5 | 30.9 | 36.4 | 39.0 | 47.9 | 53.8 |
|  |  |  |  | 2 | 31.5 | 30.8 | 35.9 | 36.6 | 45.8 | 48.9 |
|  |  |  |  | 3 | 31.4 | 30.9 | 36.3 | 37.2 | 46.1 | 47.3 |
|  |  |  |  | 4 | 31.4 | 30.8 | 37.3 | 37.3 | 45.5 | 47.0 |
|  |  | 4 | 2 | 1 | 35.8 | 31.8 | 48.1 | 52.2 | 62.8 | 66.7 |
|  |  |  |  | 2 | 35.7 | 31.8 | 47.4 | 46.5 | 59.9 | 61.0 |
|  |  |  |  | 3 | 35.6 | 31.9 | 49.0 | 48.6 | 65.3 | 62.1 |
|  |  |  |  | 4 | 35.7 | 32.0 | 49.4 | 50.9 | 64.2 | 62.1 |
| 30 | 24 | 4 | 4 | 1 | 75.4 | 72.3 | 110.9 | 101.0 | 143.3 | 128.6 |
|  |  |  |  | 2 | 75.3 | 72.1 | 95.6 | 89.9 | 124.3 | 120.0 |
|  |  |  |  | 3 | 75.5 | 72.0 | 93.1 | 92.7 | 122.8 | 122.6 |
|  |  |  |  | 4 | 75.4 | 71.5 | 187.0 | 220.5 | 129.7 | 126.0 |
|  |  | 4 | 2 | 1 | 83.9 | 79.4 | 333.8 | 249.3 | 218.9 | 213.0 |
|  |  |  |  | 2 | 83.8 | 79.6 | 1013.9 | 781.0 | 253.1 | 242.1 |
|  |  |  |  | 3 | 83.6 | 79.2 | 1748.6 | 1590.5 | 282.7 | 273.2 |
|  |  |  |  | 4 | 83.4 | 79.3 | 2023.5 | 1795.9 | 312.1 | 287.5 |
|  | 48 | 4 | 4 | 1 | 33.5 | 35.8 | 46.1 | 47.1 | 63.1 | 60.3 |
|  |  |  |  | 2 | 33.5 | 35.9 | 41.1 | 42.6 | 53.4 | 56.0 |
|  |  |  |  | 3 | 33.6 | 35.7 | 40.5 | 41.0 | 53.5 | 52.7 |
|  |  |  |  | 4 | 33.6 | 35.7 | 41.9 | 44.5 | 55.4 | 56.9 |
|  |  | 4 | 2 | 1 | 35.4 | 37.5 | 64.7 | 64.0 | 91.2 | 88.4 |
|  |  |  |  | 2 | 35.3 | 37.4 | 55.6 | 56.3 | 73.4 | 76.6 |
|  |  |  |  | 3 | 35.2 | 37.4 | 56.3 | 56.6 | 81.0 | 80.0 |
|  |  |  |  | 4 | 35.2 | 37.5 | 57.3 | 59.4 | 82.0 | 91.5 |
|  | 96 | 4 | 4 | 1 | 15.9 | 15.7 | 19.5 | 18.7 | 27.6 | 26.4 |
|  |  |  |  | 2 | 15.8 | 15.7 | 18.8 | 18.4 | 23.7 | 23.3 |
|  |  |  |  | 3 | 15.8 | 15.7 | 19.0 | 19.1 | 24.4 | 21.1 |
|  |  |  |  | 4 | 15.8 | 15.7 | 19.1 | 19.5 | 25.3 | 21.9 |
|  |  | 4 | 2 | 1 | 17.7 | 17.1 | 23.6 | 23.3 | 37.2 | 38.1 |
|  |  |  |  | 2 | 17.7 | 17.1 | 21.7 | 22.6 | 33.0 | 33.4 |
|  |  |  |  | 3 | 17.8 | 17.1 | 23.1 | 24.4 | 34.5 | 35.1 |
|  |  |  |  | 4 | 17.7 | 17.1 | 24.0 | 26.6 | 35.4 | 36.8 |
| 60 | 24 | 4 | 4 | 1 | 40.5 | 36.7 | 66.3 | 65.8 | 99.2 | 86.3 |
|  |  |  |  | 2 | 40.4 | 36.6 | 56.8 | 55.1 | 74.5 | 70.3 |
|  |  |  |  | 3 | 40.4 | 36.6 | 53.1 | 54.0 | 71.4 | 71.4 |
|  |  |  |  | 4 | 40.5 | 36.5 | 54.0 | 52.4 | 75.4 | 78.5 |
|  |  | 4 | 2 | 1 | 41.3 | 35.1 | 105.4 | 144.1 | 172.7 | 192.5 |
|  |  |  |  | 2 | 41.0 | 35.1 | 119.6 | 99.7 | 158.1 | 157.7 |
|  |  |  |  | 3 | 40.9 | 35.2 | 124.6 | 120.0 | 184.5 | 173.3 |
|  |  |  |  | 4 | 40.7 | 35.2 | 137.9 | 125.5 | 207.7 | 189.3 |

SL-PRS RSRP simulation results

Table 9-12 shows the results for 5%-ile and 95%-ile of the SL-PRS RSRP errors with Es/Iot (-3,-6,-6), (0,-6,-6), (0,-3,-3) and (0,0,0), respectively. For convenience, we provide the absolute and relative accuracy simulation results, Table 13-16 shows the results for the absolute and relative accuracy of the SL-PRS RSRP based on absolute accuracy = max (abs (5%-ile error), abs (95%-ile error)) and relative accuracy = abs(95%-ile error-5%-ile error) with Es/Iot (-3,-6,-6), (0,-6,-6), (0,-3,-3) and (0,0,0), respectively.

Table 9 5%-ile and 95%-ile of the SL-PRS RSRP errors with Es/Iot (-3,-6,-6)

| SL-PRS based RSRP [dB] (-3, -6, -6) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  |  |  |  | TDL-A |  |  |  |  |  | TDL-B |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| 15 | 48 | 4 | 4 | 1 | -0.5 | 0.7 | -0.6 | 1.0 | -0.7 | 0.9 | -0.7 | 0.7 | -1.0 | 1.1 | -1.8 | 0.7 | -0.7 | 0.7 | -1.0 | 1.0 | -1.3 | 0.8 |
|  |  |  |  | 2 | -0.3 | 0.6 | -0.3 | 0.8 | -0.4 | 0.7 | -0.5 | 0.5 | -0.8 | 0.8 | -1.4 | 0.5 | -0.5 | 0.5 | -0.7 | 0.8 | -1.0 | 0.5 |
|  |  |  |  | 3 | -0.2 | 0.5 | -0.2 | 0.7 | -0.3 | 0.6 | -0.4 | 0.4 | -0.7 | 0.7 | -1.2 | 0.4 | -0.4 | 0.4 | -0.6 | 0.7 | -0.8 | 0.4 |
|  |  |  |  | 4 | -0.1 | 0.5 | -0.2 | 0.7 | -0.2 | 0.6 | -0.4 | 0.4 | -0.6 | 0.6 | -1.1 | 0.3 | -0.3 | 0.4 | -0.5 | 0.6 | -0.8 | 0.3 |
|  |  | 4 | 2 | 1 | -0.7 | 1.1 | -0.5 | 1.5 | -1.8 | 0.8 | -1.2 | 1.1 | -1.3 | 1.8 | -4.4 | 0.9 | -1.1 | 1.0 | -1.1 | 1.6 | -3.1 | 0.9 |
|  |  |  |  | 2 | -0.5 | 0.8 | -0.1 | 1.3 | -1.4 | 0.4 | -0.9 | 0.8 | -0.9 | 1.4 | -3.5 | 0.6 | -0.8 | 0.8 | -0.7 | 1.2 | -2.5 | 0.5 |
|  |  |  |  | 3 | -0.3 | 0.7 | 0.0 | 1.1 | -1.2 | 0.3 | -0.8 | 0.7 | -0.7 | 1.3 | -2.8 | 0.5 | -0.6 | 0.6 | -0.5 | 1.0 | -2.1 | 0.4 |
|  |  |  |  | 4 | -0.2 | 0.7 | 0.1 | 1.1 | -1.1 | 0.2 | -0.7 | 0.6 | -0.6 | 1.2 | -2.4 | 0.4 | -0.5 | 0.5 | -0.4 | 0.9 | -1.7 | 0.3 |
|  | 96 | 4 | 4 | 1 | -0.3 | 0.5 | -0.2 | 0.9 | -0.4 | 0.8 | -0.5 | 0.5 | -0.6 | 0.9 | -0.8 | 0.7 | -0.5 | 0.4 | -0.5 | 0.9 | -0.7 | 0.7 |
|  |  |  |  | 2 | -0.2 | 0.4 | 0.0 | 0.7 | -0.2 | 0.6 | -0.3 | 0.4 | -0.4 | 0.7 | -0.6 | 0.5 | -0.3 | 0.3 | -0.4 | 0.7 | -0.5 | 0.5 |
|  |  |  |  | 3 | -0.1 | 0.4 | 0.0 | 0.6 | -0.1 | 0.5 | -0.3 | 0.3 | -0.3 | 0.6 | -0.5 | 0.5 | -0.3 | 0.3 | -0.2 | 0.6 | -0.4 | 0.4 |
|  |  |  |  | 4 | -0.1 | 0.3 | 0.1 | 0.6 | -0.1 | 0.5 | -0.2 | 0.3 | -0.2 | 0.6 | -0.5 | 0.4 | -0.2 | 0.2 | -0.2 | 0.5 | -0.4 | 0.4 |
|  |  | 4 | 2 | 1 | -0.5 | 0.8 | -0.5 | 1.0 | -0.9 | 0.8 | -0.8 | 0.8 | -0.7 | 1.2 | -1.7 | 1.0 | -0.7 | 0.7 | -0.7 | 1.2 | -1.5 | 0.9 |
|  |  |  |  | 2 | -0.3 | 0.6 | -0.3 | 0.8 | -0.6 | 0.5 | -0.6 | 0.5 | -0.5 | 1.0 | -1.4 | 0.7 | -0.5 | 0.5 | -0.5 | 0.9 | -1.1 | 0.6 |
|  |  |  |  | 3 | -0.2 | 0.5 | -0.2 | 0.7 | -0.5 | 0.4 | -0.5 | 0.4 | -0.3 | 0.9 | -1.2 | 0.6 | -0.4 | 0.4 | -0.3 | 0.8 | -0.9 | 0.5 |
|  |  |  |  | 4 | -0.2 | 0.5 | -0.1 | 0.6 | -0.4 | 0.4 | -0.4 | 0.4 | -0.3 | 0.8 | -1.1 | 0.5 | -0.4 | 0.4 | -0.3 | 0.7 | -0.8 | 0.4 |
| 30 | 24 | 4 | 4 | 1 | -0.6 | 1.0 | -1.1 | 1.2 | -0.5 | 1.6 | -1.1 | 1.0 | -1.9 | 1.4 | -1.6 | 1.6 | -0.9 | 0.9 | -1.8 | 1.4 | -1.3 | 1.5 |
|  |  |  |  | 2 | -0.4 | 0.8 | -0.7 | 1.0 | -0.2 | 1.3 | -0.8 | 0.7 | -1.5 | 1.0 | -1.1 | 1.3 | -0.7 | 0.7 | -1.3 | 1.0 | -0.9 | 1.1 |
|  |  |  |  | 3 | -0.3 | 0.7 | -0.5 | 0.8 | -0.1 | 1.2 | -0.7 | 0.6 | -1.3 | 0.9 | -1.0 | 1.1 | -0.5 | 0.5 | -1.0 | 0.8 | -0.7 | 1.0 |
|  |  |  |  | 4 | -0.2 | 0.6 | -0.4 | 0.7 | 0.1 | 1.1 | -0.6 | 0.5 | -1.2 | 0.8 | -0.9 | 1.0 | -0.5 | 0.5 | -0.9 | 0.7 | -0.6 | 0.9 |
|  |  | 4 | 2 | 1 | -1.1 | 1.4 | -0.6 | 2.1 | -1.6 | 1.7 | -1.9 | 1.4 | -1.8 | 2.4 | -3.1 | 2.4 | -1.5 | 1.3 | -1.7 | 2.1 | -2.4 | 2.1 |
|  |  |  |  | 2 | -0.6 | 1.1 | -0.1 | 1.7 | -1.0 | 1.3 | -1.3 | 1.1 | -1.2 | 1.9 | -2.3 | 1.9 | -1.0 | 1.0 | -1.2 | 1.7 | -1.8 | 1.6 |
|  |  |  |  | 3 | -0.4 | 1.0 | 0.1 | 1.6 | -0.7 | 1.1 | -1.2 | 0.9 | -1.0 | 1.8 | -1.8 | 1.7 | -0.8 | 0.8 | -0.9 | 1.4 | -1.4 | 1.4 |
|  |  |  |  | 4 | -0.3 | 0.9 | 0.2 | 1.5 | -0.6 | 1.1 | -1.0 | 0.8 | -0.9 | 1.6 | -1.5 | 1.5 | -0.7 | 0.7 | -0.7 | 1.3 | -1.2 | 1.2 |
|  | 48 | 4 | 4 | 1 | -0.4 | 0.7 | -0.6 | 1.0 | -0.6 | 1.0 | -0.7 | 0.7 | -1.0 | 1.0 | -1.1 | 1.0 | -0.6 | 0.6 | -1.0 | 1.0 | -1.0 | 1.0 |
|  |  |  |  | 2 | -0.2 | 0.6 | -0.3 | 0.8 | -0.4 | 0.8 | -0.5 | 0.5 | -0.7 | 0.8 | -0.8 | 0.7 | -0.5 | 0.5 | -0.7 | 0.8 | -0.7 | 0.7 |
|  |  |  |  | 3 | -0.2 | 0.5 | -0.2 | 0.7 | -0.2 | 0.7 | -0.4 | 0.4 | -0.6 | 0.7 | -0.7 | 0.6 | -0.4 | 0.4 | -0.6 | 0.6 | -0.6 | 0.6 |
|  |  |  |  | 4 | -0.1 | 0.5 | -0.2 | 0.6 | -0.2 | 0.7 | -0.3 | 0.4 | -0.5 | 0.6 | -0.6 | 0.5 | -0.3 | 0.3 | -0.5 | 0.6 | -0.5 | 0.5 |
|  |  | 4 | 2 | 1 | -0.7 | 1.1 | -0.5 | 1.5 | -1.5 | 1.0 | -1.0 | 1.0 | -1.1 | 1.7 | -2.0 | 1.3 | -1.0 | 0.9 | -1.2 | 1.6 | -1.8 | 1.3 |
|  |  |  |  | 2 | -0.4 | 0.8 | -0.2 | 1.2 | -1.1 | 0.6 | -0.7 | 0.7 | -0.8 | 1.4 | -1.6 | 1.0 | -0.7 | 0.7 | -0.8 | 1.3 | -1.3 | 0.9 |
|  |  |  |  | 3 | -0.3 | 0.7 | 0.0 | 1.1 | -0.9 | 0.5 | -0.6 | 0.6 | -0.6 | 1.2 | -1.4 | 0.9 | -0.5 | 0.6 | -0.6 | 1.1 | -1.1 | 0.7 |
|  |  |  |  | 4 | -0.2 | 0.7 | 0.0 | 1.0 | -0.8 | 0.4 | -0.5 | 0.5 | -0.5 | 1.1 | -1.3 | 0.8 | -0.5 | 0.5 | -0.5 | 1.0 | -0.9 | 0.7 |
|  | 96 | 4 | 4 | 1 | -0.3 | 0.5 | -0.2 | 0.9 | -0.6 | 0.6 | -0.5 | 0.5 | -0.5 | 1.0 | -0.8 | 0.7 | -0.4 | 0.4 | -0.6 | 0.9 | -0.7 | 0.7 |
|  |  |  |  | 2 | -0.2 | 0.4 | 0.0 | 0.7 | -0.4 | 0.5 | -0.3 | 0.4 | -0.3 | 0.8 | -0.6 | 0.5 | -0.3 | 0.3 | -0.4 | 0.7 | -0.5 | 0.5 |
|  |  |  |  | 3 | -0.1 | 0.4 | 0.0 | 0.7 | -0.3 | 0.4 | -0.3 | 0.3 | -0.3 | 0.7 | -0.5 | 0.4 | -0.2 | 0.3 | -0.3 | 0.6 | -0.4 | 0.4 |
|  |  |  |  | 4 | -0.1 | 0.3 | 0.1 | 0.6 | -0.3 | 0.3 | -0.2 | 0.3 | -0.2 | 0.6 | -0.4 | 0.4 | -0.2 | 0.2 | -0.2 | 0.5 | -0.4 | 0.3 |
|  |  | 4 | 2 | 1 | -0.5 | 0.8 | -0.5 | 1.0 | -0.7 | 1.0 | -0.8 | 0.8 | -0.7 | 1.3 | -1.3 | 0.9 | -0.7 | 0.7 | -0.8 | 1.2 | -1.2 | 0.9 |
|  |  |  |  | 2 | -0.3 | 0.6 | -0.3 | 0.8 | -0.4 | 0.8 | -0.5 | 0.6 | -0.5 | 1.1 | -1.0 | 0.7 | -0.5 | 0.5 | -0.5 | 0.9 | -0.8 | 0.7 |
|  |  |  |  | 3 | -0.2 | 0.5 | -0.2 | 0.7 | -0.3 | 0.7 | -0.4 | 0.5 | -0.4 | 1.0 | -0.9 | 0.6 | -0.4 | 0.4 | -0.4 | 0.8 | -0.7 | 0.5 |
|  |  |  |  | 4 | -0.2 | 0.5 | -0.1 | 0.6 | -0.2 | 0.6 | -0.4 | 0.4 | -0.4 | 0.9 | -0.8 | 0.5 | -0.3 | 0.4 | -0.3 | 0.7 | -0.6 | 0.5 |
| 60 | 24 | 4 | 4 | 1 | -0.7 | 1.0 | -0.9 | 1.4 | -0.4 | 1.7 | -1.0 | 0.9 | -1.7 | 1.4 | -1.3 | 1.6 | -0.9 | 0.9 | -1.6 | 1.4 | -1.2 | 1.6 |
|  |  |  |  | 2 | -0.4 | 0.8 | -0.5 | 1.1 | -0.1 | 1.5 | -0.7 | 0.7 | -1.4 | 1.1 | -0.9 | 1.3 | -0.6 | 0.7 | -1.1 | 1.1 | -0.8 | 1.2 |
|  |  |  |  | 3 | -0.3 | 0.7 | -0.3 | 1.0 | 0.1 | 1.3 | -0.6 | 0.6 | -1.2 | 1.0 | -0.7 | 1.1 | -0.5 | 0.6 | -0.9 | 0.9 | -0.6 | 1.0 |
|  |  |  |  | 4 | -0.2 | 0.6 | -0.2 | 0.9 | 0.2 | 1.3 | -0.5 | 0.5 | -1.1 | 0.9 | -0.7 | 1.0 | -0.4 | 0.5 | -0.8 | 0.8 | -0.5 | 0.9 |
|  |  | 4 | 2 | 1 | -1.1 | 1.5 | -0.4 | 2.2 | -0.7 | 2.3 | -1.6 | 1.4 | -1.7 | 2.2 | -2.3 | 2.0 | -1.4 | 1.4 | -1.8 | 2.1 | -2.2 | 2.0 |
|  |  |  |  | 2 | -0.7 | 1.1 | 0.1 | 1.9 | -0.1 | 2.0 | -1.1 | 1.0 | -1.2 | 1.8 | -1.7 | 1.6 | -1.0 | 1.0 | -1.1 | 1.7 | -1.5 | 1.5 |
|  |  |  |  | 3 | -0.5 | 1.0 | 0.3 | 1.7 | 0.1 | 1.8 | -0.9 | 0.9 | -1.0 | 1.6 | -1.5 | 1.4 | -0.8 | 0.8 | -0.9 | 1.5 | -1.2 | 1.3 |
|  |  |  |  | 4 | -0.4 | 0.9 | 0.4 | 1.6 | 0.2 | 1.7 | -0.8 | 0.8 | -0.8 | 1.5 | -1.3 | 1.3 | -0.7 | 0.7 | -0.8 | 1.3 | -1.0 | 1.1 |

Table 10 5%-ile and 95%-ile of the SL-PRS RSRP errors with Es/Iot (0,-6,-6)

| SL-PRS based RSRP [dB] (0, -6, -6) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  |  |  |  | TDL-A |  |  |  |  |  | TDL-B |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| 15 | 48 | 4 | 4 | 1 | -0.3 | 0.5 | -0.7 | 0.9 | -0.9 | 0.8 | -0.5 | 0.4 | -1.1 | 1.1 | -2.1 | 0.6 | -0.4 | 0.4 | -1.1 | 1.1 | -1.7 | 0.7 |
|  |  |  |  | 2 | -0.2 | 0.3 | -0.4 | 0.7 | -0.6 | 0.6 | -0.3 | 0.3 | -0.8 | 0.9 | -1.7 | 0.4 | -0.3 | 0.3 | -0.7 | 0.8 | -1.2 | 0.4 |
|  |  |  |  | 3 | -0.2 | 0.3 | -0.3 | 0.6 | -0.5 | 0.5 | -0.3 | 0.3 | -0.7 | 0.8 | -1.5 | 0.3 | -0.3 | 0.2 | -0.6 | 0.7 | -1.0 | 0.3 |
|  |  |  |  | 4 | -0.1 | 0.3 | -0.3 | 0.6 | -0.4 | 0.4 | -0.3 | 0.2 | -0.6 | 0.7 | -1.3 | 0.2 | -0.2 | 0.2 | -0.5 | 0.6 | -1.0 | 0.3 |
|  |  | 4 | 2 | 1 | -0.5 | 0.7 | -0.4 | 1.5 | -2.6 | 0.3 | -0.7 | 0.7 | -1.1 | 2.0 | -6.1 | 0.7 | -0.7 | 0.7 | -1.0 | 1.8 | -4.1 | 0.7 |
|  |  |  |  | 2 | -0.4 | 0.5 | -0.1 | 1.2 | -2.1 | -0.1 | -0.5 | 0.5 | -0.7 | 1.6 | -4.3 | 0.5 | -0.5 | 0.5 | -0.6 | 1.4 | -3.3 | 0.3 |
|  |  |  |  | 3 | -0.3 | 0.4 | 0.1 | 1.1 | -1.8 | -0.2 | -0.5 | 0.4 | -0.5 | 1.5 | -3.5 | 0.3 | -0.4 | 0.4 | -0.4 | 1.2 | -2.7 | 0.2 |
|  |  |  |  | 4 | -0.2 | 0.4 | 0.1 | 1.0 | -1.7 | -0.4 | -0.4 | 0.4 | -0.4 | 1.4 | -3.1 | 0.2 | -0.3 | 0.4 | -0.3 | 1.1 | -2.4 | 0.1 |
|  | 96 | 4 | 4 | 1 | -0.2 | 0.3 | -0.2 | 0.9 | -0.5 | 0.7 | -0.3 | 0.3 | -0.5 | 1.1 | -0.9 | 0.8 | -0.3 | 0.3 | -0.5 | 1.0 | -0.8 | 0.7 |
|  |  |  |  | 2 | -0.2 | 0.2 | 0.0 | 0.7 | -0.3 | 0.5 | -0.2 | 0.2 | -0.3 | 0.9 | -0.7 | 0.6 | -0.2 | 0.2 | -0.3 | 0.8 | -0.6 | 0.5 |
|  |  |  |  | 3 | -0.1 | 0.2 | 0.0 | 0.7 | -0.2 | 0.5 | -0.2 | 0.2 | -0.2 | 0.8 | -0.6 | 0.5 | -0.2 | 0.2 | -0.2 | 0.7 | -0.4 | 0.5 |
|  |  |  |  | 4 | -0.1 | 0.2 | 0.1 | 0.6 | -0.2 | 0.4 | -0.2 | 0.2 | -0.2 | 0.7 | -0.5 | 0.4 | -0.1 | 0.1 | -0.2 | 0.6 | -0.4 | 0.4 |
|  |  | 4 | 2 | 1 | -0.4 | 0.5 | -0.5 | 0.9 | -1.1 | 0.5 | -0.5 | 0.5 | -0.6 | 1.4 | -2.2 | 0.9 | -0.5 | 0.5 | -0.6 | 1.4 | -1.9 | 0.9 |
|  |  |  |  | 2 | -0.3 | 0.4 | -0.3 | 0.7 | -0.8 | 0.3 | -0.4 | 0.3 | -0.4 | 1.2 | -1.9 | 0.7 | -0.3 | 0.3 | -0.4 | 1.0 | -1.4 | 0.6 |
|  |  |  |  | 3 | -0.2 | 0.3 | -0.2 | 0.6 | -0.7 | 0.2 | -0.3 | 0.3 | -0.3 | 1.0 | -1.5 | 0.5 | -0.3 | 0.3 | -0.3 | 0.9 | -1.2 | 0.5 |
|  |  |  |  | 4 | -0.2 | 0.3 | -0.1 | 0.6 | -0.7 | 0.2 | -0.2 | 0.2 | -0.2 | 1.0 | -1.4 | 0.5 | -0.2 | 0.2 | -0.2 | 0.8 | -1.1 | 0.4 |
| 30 | 24 | 4 | 4 | 1 | -0.5 | 0.6 | -1.3 | 1.1 | -0.6 | 1.6 | -0.7 | 0.6 | -2.2 | 1.4 | -1.5 | 1.8 | -0.6 | 0.6 | -2.0 | 1.5 | -1.3 | 1.6 |
|  |  |  |  | 2 | -0.3 | 0.5 | -0.9 | 0.8 | -0.2 | 1.3 | -0.5 | 0.4 | -1.8 | 1.0 | -1.0 | 1.4 | -0.4 | 0.4 | -1.4 | 1.1 | -0.9 | 1.2 |
|  |  |  |  | 3 | -0.2 | 0.4 | -0.7 | 0.7 | 0.0 | 1.2 | -0.4 | 0.4 | -1.4 | 0.9 | -0.9 | 1.3 | -0.3 | 0.3 | -1.2 | 0.9 | -0.7 | 1.0 |
|  |  |  |  | 4 | -0.2 | 0.4 | -0.6 | 0.6 | 0.0 | 1.1 | -0.4 | 0.3 | -1.4 | 0.8 | -0.8 | 1.1 | -0.3 | 0.3 | -1.1 | 0.8 | -0.6 | 0.9 |
|  |  | 4 | 2 | 1 | -0.8 | 1.0 | -0.4 | 2.0 | -1.8 | 1.6 | -1.1 | 0.9 | -1.4 | 2.6 | -3.4 | 2.5 | -0.9 | 0.9 | -1.5 | 2.3 | -2.8 | 2.3 |
|  |  |  |  | 2 | -0.5 | 0.7 | 0.0 | 1.7 | -1.2 | 1.2 | -0.8 | 0.7 | -0.9 | 2.3 | -2.5 | 2.2 | -0.7 | 0.6 | -0.9 | 1.8 | -2.1 | 1.8 |
|  |  |  |  | 3 | -0.4 | 0.6 | 0.2 | 1.5 | -0.9 | 1.0 | -0.6 | 0.6 | -0.7 | 2.1 | -1.9 | 1.9 | -0.5 | 0.5 | -0.7 | 1.6 | -1.7 | 1.5 |
|  |  |  |  | 4 | -0.3 | 0.6 | 0.3 | 1.4 | -0.8 | 0.9 | -0.6 | 0.5 | -0.6 | 2.0 | -1.7 | 1.7 | -0.5 | 0.5 | -0.6 | 1.5 | -1.4 | 1.3 |
|  | 48 | 4 | 4 | 1 | -0.3 | 0.5 | -0.7 | 0.9 | -0.8 | 1.0 | -0.4 | 0.4 | -1.1 | 1.1 | -1.2 | 1.0 | -0.4 | 0.4 | -1.1 | 1.1 | -1.1 | 1.0 |
|  |  |  |  | 2 | -0.2 | 0.4 | -0.4 | 0.7 | -0.5 | 0.8 | -0.3 | 0.3 | -0.8 | 0.9 | -0.9 | 0.8 | -0.3 | 0.3 | -0.7 | 0.8 | -0.8 | 0.7 |
|  |  |  |  | 3 | -0.1 | 0.3 | -0.3 | 0.6 | -0.3 | 0.7 | -0.3 | 0.3 | -0.7 | 0.7 | -0.8 | 0.6 | -0.2 | 0.2 | -0.6 | 0.7 | -0.7 | 0.6 |
|  |  |  |  | 4 | -0.1 | 0.3 | -0.2 | 0.6 | -0.3 | 0.6 | -0.2 | 0.2 | -0.6 | 0.6 | -0.7 | 0.6 | -0.2 | 0.2 | -0.5 | 0.6 | -0.6 | 0.5 |
|  |  | 4 | 2 | 1 | -0.6 | 0.7 | -0.4 | 1.4 | -1.9 | 0.7 | -0.7 | 0.7 | -1.0 | 1.9 | -2.4 | 1.3 | -0.6 | 0.6 | -1.1 | 1.8 | -2.2 | 1.3 |
|  |  |  |  | 2 | -0.3 | 0.5 | -0.1 | 1.2 | -1.5 | 0.4 | -0.4 | 0.5 | -0.7 | 1.6 | -2.0 | 1.0 | -0.5 | 0.5 | -0.7 | 1.4 | -1.6 | 0.9 |
|  |  |  |  | 3 | -0.3 | 0.4 | 0.0 | 1.1 | -1.3 | 0.2 | -0.4 | 0.4 | -0.5 | 1.5 | -1.8 | 0.9 | -0.4 | 0.4 | -0.5 | 1.2 | -1.3 | 0.7 |
|  |  |  |  | 4 | -0.2 | 0.4 | 0.1 | 1.0 | -1.2 | 0.1 | -0.3 | 0.4 | -0.5 | 1.4 | -1.6 | 0.8 | -0.3 | 0.3 | -0.4 | 1.1 | -1.1 | 0.6 |
|  | 96 | 4 | 4 | 1 | -0.2 | 0.3 | -0.2 | 0.9 | -0.7 | 0.5 | -0.3 | 0.3 | -0.4 | 1.1 | -0.9 | 0.7 | -0.3 | 0.3 | -0.6 | 1.0 | -0.8 | 0.7 |
|  |  |  |  | 2 | -0.1 | 0.2 | 0.0 | 0.7 | -0.5 | 0.4 | -0.2 | 0.2 | -0.3 | 1.0 | -0.7 | 0.5 | -0.2 | 0.2 | -0.4 | 0.8 | -0.6 | 0.5 |
|  |  |  |  | 3 | -0.1 | 0.2 | 0.1 | 0.7 | -0.4 | 0.3 | -0.2 | 0.2 | -0.2 | 0.9 | -0.6 | 0.4 | -0.2 | 0.2 | -0.3 | 0.7 | -0.5 | 0.4 |
|  |  |  |  | 4 | -0.1 | 0.2 | 0.1 | 0.6 | -0.4 | 0.2 | -0.1 | 0.2 | -0.2 | 0.8 | -0.5 | 0.4 | -0.1 | 0.1 | -0.2 | 0.6 | -0.4 | 0.3 |
|  |  | 4 | 2 | 1 | -0.4 | 0.5 | -0.5 | 0.9 | -0.8 | 0.8 | -0.5 | 0.5 | -0.6 | 1.5 | -1.6 | 0.8 | -0.4 | 0.4 | -0.7 | 1.3 | -1.3 | 0.9 |
|  |  |  |  | 2 | -0.2 | 0.4 | -0.3 | 0.7 | -0.5 | 0.6 | -0.3 | 0.4 | -0.4 | 1.3 | -1.3 | 0.6 | -0.3 | 0.3 | -0.5 | 1.0 | -0.9 | 0.6 |
|  |  |  |  | 3 | -0.2 | 0.3 | -0.2 | 0.6 | -0.4 | 0.5 | -0.3 | 0.3 | -0.3 | 1.2 | -1.2 | 0.6 | -0.3 | 0.3 | -0.3 | 0.9 | -0.8 | 0.5 |
|  |  |  |  | 4 | -0.2 | 0.3 | -0.1 | 0.6 | -0.4 | 0.5 | -0.2 | 0.3 | -0.3 | 1.1 | -1.1 | 0.5 | -0.2 | 0.2 | -0.2 | 0.8 | -0.7 | 0.4 |
| 60 | 24 | 4 | 4 | 1 | -0.5 | 0.6 | -1.1 | 1.3 | -0.5 | 1.8 | -0.6 | 0.6 | -2.0 | 1.5 | -1.2 | 1.8 | -0.6 | 0.6 | -1.8 | 1.5 | -1.1 | 1.8 |
|  |  |  |  | 2 | -0.3 | 0.5 | -0.7 | 1.0 | -0.1 | 1.5 | -0.5 | 0.4 | -1.6 | 1.2 | -0.9 | 1.5 | -0.4 | 0.4 | -1.3 | 1.1 | -0.7 | 1.4 |
|  |  |  |  | 3 | -0.3 | 0.4 | -0.5 | 0.8 | 0.1 | 1.4 | -0.4 | 0.4 | -1.4 | 1.1 | -0.7 | 1.3 | -0.3 | 0.4 | -1.0 | 1.0 | -0.5 | 1.2 |
|  |  |  |  | 4 | -0.2 | 0.4 | -0.4 | 0.8 | 0.2 | 1.3 | -0.3 | 0.3 | -1.3 | 1.1 | -0.6 | 1.2 | -0.3 | 0.3 | -0.9 | 0.8 | -0.4 | 1.1 |
|  |  | 4 | 2 | 1 | -0.8 | 1.0 | -0.2 | 2.2 | -0.8 | 2.3 | -1.0 | 0.9 | -1.4 | 2.4 | -2.4 | 2.1 | -0.9 | 0.9 | -1.7 | 2.4 | -2.4 | 2.1 |
|  |  |  |  | 2 | -0.5 | 0.7 | 0.2 | 1.8 | -0.3 | 1.9 | -0.7 | 0.7 | -1.0 | 2.1 | -1.8 | 1.7 | -0.6 | 0.7 | -1.1 | 1.8 | -1.6 | 1.6 |
|  |  |  |  | 3 | -0.4 | 0.6 | 0.4 | 1.7 | 0.0 | 1.7 | -0.6 | 0.6 | -0.8 | 2.0 | -1.5 | 1.5 | -0.5 | 0.6 | -0.8 | 1.6 | -1.3 | 1.4 |
|  |  |  |  | 4 | -0.3 | 0.6 | 0.5 | 1.6 | 0.1 | 1.6 | -0.5 | 0.5 | -0.7 | 1.9 | -1.4 | 1.4 | -0.4 | 0.5 | -0.6 | 1.4 | -1.1 | 1.2 |

Table 11 5%-ile and 95%-ile of the SL-PRS RSRP errors with Es/Iot (0,-3,-3)

| SL-PRS based RSRP [dB] (0, -3, -3) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  |  |  |  | TDL-A |  |  |  |  |  | TDL-B |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| 15 | 48 | 4 | 4 | 1 | -0.5 | 0.5 | -0.5 | 0.6 | -0.6 | 0.6 | -0.6 | 0.6 | -0.7 | 0.8 | -1.2 | 0.4 | -0.6 | 0.5 | -0.7 | 0.7 | -1.0 | 0.4 |
|  |  |  |  | 2 | -0.3 | 0.4 | -0.3 | 0.5 | -0.4 | 0.4 | -0.4 | 0.4 | -0.5 | 0.6 | -1.0 | 0.3 | -0.4 | 0.4 | -0.5 | 0.5 | -0.7 | 0.3 |
|  |  |  |  | 3 | -0.3 | 0.3 | -0.2 | 0.4 | -0.3 | 0.4 | -0.4 | 0.3 | -0.5 | 0.5 | -0.9 | 0.2 | -0.3 | 0.3 | -0.4 | 0.5 | -0.6 | 0.2 |
|  |  |  |  | 4 | -0.2 | 0.3 | -0.2 | 0.4 | -0.3 | 0.3 | -0.3 | 0.3 | -0.4 | 0.5 | -0.8 | 0.1 | -0.3 | 0.3 | -0.3 | 0.4 | -0.6 | 0.2 |
|  |  | 4 | 2 | 1 | -0.8 | 0.8 | -0.3 | 0.9 | -1.5 | 0.2 | -1.0 | 0.9 | -0.7 | 1.3 | -3.4 | 0.6 | -0.9 | 0.9 | -0.6 | 1.2 | -2.3 | 0.5 |
|  |  |  |  | 2 | -0.5 | 0.6 | -0.1 | 0.8 | -1.3 | -0.1 | -0.7 | 0.7 | -0.5 | 1.1 | -2.9 | 0.4 | -0.6 | 0.6 | -0.4 | 1.0 | -1.8 | 0.3 |
|  |  |  |  | 3 | -0.4 | 0.5 | 0.0 | 0.7 | -1.1 | -0.2 | -0.6 | 0.6 | -0.4 | 1.0 | -2.4 | 0.3 | -0.5 | 0.5 | -0.3 | 0.8 | -1.5 | 0.2 |
|  |  |  |  | 4 | -0.3 | 0.5 | 0.0 | 0.6 | -1.1 | -0.2 | -0.6 | 0.5 | -0.3 | 0.9 | -2.1 | 0.2 | -0.5 | 0.4 | -0.3 | 0.7 | -1.4 | 0.1 |
|  | 96 | 4 | 4 | 1 | -0.3 | 0.4 | -0.2 | 0.6 | -0.3 | 0.5 | -0.4 | 0.4 | -0.3 | 0.7 | -0.6 | 0.5 | -0.4 | 0.4 | -0.3 | 0.7 | -0.5 | 0.5 |
|  |  |  |  | 2 | -0.2 | 0.3 | -0.1 | 0.5 | -0.2 | 0.4 | -0.3 | 0.3 | -0.2 | 0.6 | -0.4 | 0.4 | -0.3 | 0.3 | -0.2 | 0.5 | -0.4 | 0.4 |
|  |  |  |  | 3 | -0.2 | 0.2 | 0.0 | 0.4 | -0.2 | 0.3 | -0.2 | 0.2 | -0.2 | 0.5 | -0.4 | 0.3 | -0.2 | 0.2 | -0.2 | 0.4 | -0.3 | 0.3 |
|  |  |  |  | 4 | -0.1 | 0.2 | 0.0 | 0.4 | -0.1 | 0.3 | -0.2 | 0.2 | -0.1 | 0.4 | -0.3 | 0.3 | -0.2 | 0.2 | -0.1 | 0.4 | -0.3 | 0.3 |
|  |  | 4 | 2 | 1 | -0.6 | 0.6 | -0.4 | 0.5 | -0.7 | 0.3 | -0.6 | 0.6 | -0.4 | 0.9 | -1.3 | 0.7 | -0.6 | 0.6 | -0.5 | 0.9 | -1.2 | 0.6 |
|  |  |  |  | 2 | -0.4 | 0.5 | -0.3 | 0.4 | -0.6 | 0.2 | -0.4 | 0.5 | -0.3 | 0.8 | -1.1 | 0.5 | -0.4 | 0.4 | -0.3 | 0.7 | -0.8 | 0.4 |
|  |  |  |  | 3 | -0.3 | 0.4 | -0.2 | 0.3 | -0.5 | 0.1 | -0.4 | 0.4 | -0.2 | 0.7 | -0.9 | 0.4 | -0.4 | 0.3 | -0.2 | 0.6 | -0.7 | 0.3 |
|  |  |  |  | 4 | -0.3 | 0.3 | -0.2 | 0.3 | -0.4 | 0.1 | -0.3 | 0.3 | -0.2 | 0.6 | -0.8 | 0.4 | -0.3 | 0.3 | -0.2 | 0.5 | -0.7 | 0.3 |
| 30 | 24 | 4 | 4 | 1 | -0.6 | 0.7 | -0.9 | 0.7 | -0.4 | 1.1 | -0.8 | 0.7 | -1.3 | 0.9 | -0.9 | 1.2 | -0.7 | 0.7 | -1.2 | 1.0 | -0.9 | 1.1 |
|  |  |  |  | 2 | -0.4 | 0.5 | -0.6 | 0.5 | -0.2 | 0.9 | -0.6 | 0.6 | -1.1 | 0.7 | -0.6 | 1.0 | -0.5 | 0.5 | -0.9 | 0.7 | -0.6 | 0.8 |
|  |  |  |  | 3 | -0.3 | 0.5 | -0.5 | 0.4 | -0.1 | 0.8 | -0.5 | 0.5 | -0.9 | 0.6 | -0.6 | 0.8 | -0.4 | 0.4 | -0.7 | 0.6 | -0.5 | 0.7 |
|  |  |  |  | 4 | -0.3 | 0.4 | -0.4 | 0.3 | 0.0 | 0.7 | -0.5 | 0.4 | -0.9 | 0.5 | -0.5 | 0.7 | -0.4 | 0.4 | -0.7 | 0.5 | -0.4 | 0.6 |
|  |  | 4 | 2 | 1 | -1.1 | 1.1 | -0.3 | 1.3 | -1.3 | 1.0 | -1.4 | 1.2 | -0.9 | 1.7 | -1.7 | 1.8 | -1.2 | 1.1 | -1.0 | 1.5 | -1.4 | 1.7 |
|  |  |  |  | 2 | -0.7 | 0.8 | -0.1 | 1.1 | -0.9 | 0.7 | -1.0 | 0.9 | -0.7 | 1.5 | -1.5 | 1.5 | -0.8 | 0.8 | -0.6 | 1.2 | -1.1 | 1.2 |
|  |  |  |  | 3 | -0.6 | 0.7 | 0.0 | 1.0 | -0.7 | 0.6 | -0.9 | 0.7 | -0.5 | 1.3 | -1.3 | 1.3 | -0.7 | 0.7 | -0.5 | 1.0 | -0.9 | 1.1 |
|  |  |  |  | 4 | -0.5 | 0.6 | 0.1 | 0.9 | -0.6 | 0.5 | -0.8 | 0.6 | -0.5 | 1.3 | -1.2 | 1.2 | -0.6 | 0.6 | -0.4 | 0.9 | -0.8 | 1.0 |
|  | 48 | 4 | 4 | 1 | -0.4 | 0.5 | -0.5 | 0.6 | -0.5 | 0.7 | -0.5 | 0.5 | -0.7 | 0.7 | -0.8 | 0.7 | -0.5 | 0.5 | -0.7 | 0.7 | -0.7 | 0.7 |
|  |  |  |  | 2 | -0.3 | 0.4 | -0.3 | 0.5 | -0.4 | 0.5 | -0.4 | 0.4 | -0.5 | 0.6 | -0.6 | 0.5 | -0.4 | 0.4 | -0.5 | 0.6 | -0.5 | 0.5 |
|  |  |  |  | 3 | -0.2 | 0.3 | -0.2 | 0.4 | -0.3 | 0.4 | -0.3 | 0.3 | -0.5 | 0.5 | -0.5 | 0.4 | -0.3 | 0.3 | -0.4 | 0.5 | -0.4 | 0.4 |
|  |  |  |  | 4 | -0.2 | 0.3 | -0.2 | 0.4 | -0.2 | 0.4 | -0.3 | 0.3 | -0.4 | 0.4 | -0.4 | 0.4 | -0.2 | 0.3 | -0.3 | 0.4 | -0.4 | 0.4 |
|  |  | 4 | 2 | 1 | -0.8 | 0.8 | -0.3 | 0.9 | -1.3 | 0.4 | -0.8 | 0.8 | -0.7 | 1.3 | -1.4 | 0.9 | -0.8 | 0.8 | -0.7 | 1.2 | -1.3 | 0.9 |
|  |  |  |  | 2 | -0.5 | 0.6 | -0.1 | 0.7 | -1.0 | 0.2 | -0.6 | 0.6 | -0.5 | 1.1 | -1.2 | 0.8 | -0.6 | 0.6 | -0.5 | 1.0 | -1.0 | 0.6 |
|  |  |  |  | 3 | -0.4 | 0.5 | -0.1 | 0.7 | -0.9 | 0.1 | -0.4 | 0.5 | -0.4 | 1.0 | -1.1 | 0.7 | -0.5 | 0.5 | -0.4 | 0.8 | -0.8 | 0.5 |
|  |  |  |  | 4 | -0.3 | 0.5 | 0.0 | 0.6 | -0.8 | 0.0 | -0.4 | 0.5 | -0.4 | 0.9 | -1.0 | 0.6 | -0.4 | 0.4 | -0.3 | 0.7 | -0.7 | 0.4 |
|  | 96 | 4 | 4 | 1 | -0.3 | 0.4 | -0.1 | 0.6 | -0.5 | 0.3 | -0.4 | 0.4 | -0.3 | 0.7 | -0.6 | 0.4 | -0.3 | 0.3 | -0.4 | 0.6 | -0.5 | 0.5 |
|  |  |  |  | 2 | -0.2 | 0.3 | 0.0 | 0.5 | -0.4 | 0.2 | -0.2 | 0.3 | -0.2 | 0.6 | -0.4 | 0.3 | -0.2 | 0.3 | -0.3 | 0.5 | -0.4 | 0.3 |
|  |  |  |  | 3 | -0.2 | 0.2 | 0.0 | 0.4 | -0.3 | 0.2 | -0.2 | 0.2 | -0.2 | 0.6 | -0.4 | 0.3 | -0.2 | 0.2 | -0.2 | 0.4 | -0.3 | 0.3 |
|  |  |  |  | 4 | -0.1 | 0.2 | 0.0 | 0.4 | -0.3 | 0.1 | -0.2 | 0.2 | -0.1 | 0.5 | -0.3 | 0.2 | -0.2 | 0.2 | -0.1 | 0.4 | -0.3 | 0.2 |
|  |  | 4 | 2 | 1 | -0.5 | 0.6 | -0.4 | 0.5 | -0.5 | 0.6 | -0.6 | 0.6 | -0.4 | 1.0 | -0.9 | 0.6 | -0.6 | 0.6 | -0.5 | 0.9 | -0.9 | 0.6 |
|  |  |  |  | 2 | -0.4 | 0.4 | -0.2 | 0.4 | -0.4 | 0.4 | -0.4 | 0.5 | -0.3 | 0.9 | -0.7 | 0.5 | -0.4 | 0.4 | -0.3 | 0.7 | -0.6 | 0.5 |
|  |  |  |  | 3 | -0.3 | 0.4 | -0.2 | 0.3 | -0.3 | 0.4 | -0.3 | 0.4 | -0.2 | 0.8 | -0.7 | 0.4 | -0.3 | 0.3 | -0.2 | 0.6 | -0.5 | 0.4 |
|  |  |  |  | 4 | -0.2 | 0.3 | -0.2 | 0.3 | -0.2 | 0.3 | -0.3 | 0.3 | -0.2 | 0.7 | -0.6 | 0.4 | -0.3 | 0.3 | -0.2 | 0.5 | -0.4 | 0.3 |
| 60 | 24 | 4 | 4 | 1 | -0.7 | 0.7 | -0.7 | 0.9 | -0.4 | 1.3 | -0.8 | 0.7 | -1.2 | 1.0 | -0.8 | 1.2 | -0.7 | 0.7 | -1.2 | 1.1 | -0.8 | 1.2 |
|  |  |  |  | 2 | -0.5 | 0.5 | -0.5 | 0.7 | -0.1 | 1.1 | -0.6 | 0.5 | -1.0 | 0.8 | -0.6 | 1.0 | -0.5 | 0.5 | -0.8 | 0.8 | -0.5 | 0.9 |
|  |  |  |  | 3 | -0.4 | 0.5 | -0.3 | 0.6 | 0.0 | 1.0 | -0.5 | 0.5 | -0.9 | 0.8 | -0.5 | 0.9 | -0.4 | 0.4 | -0.7 | 0.7 | -0.4 | 0.8 |
|  |  |  |  | 4 | -0.3 | 0.4 | -0.3 | 0.5 | 0.1 | 0.9 | -0.4 | 0.4 | -0.9 | 0.7 | -0.5 | 0.8 | -0.4 | 0.4 | -0.6 | 0.6 | -0.3 | 0.7 |
|  |  | 4 | 2 | 1 | -1.2 | 1.1 | -0.2 | 1.5 | -0.5 | 1.7 | -1.3 | 1.1 | -0.9 | 1.6 | -1.6 | 1.4 | -1.1 | 1.1 | -1.1 | 1.6 | -1.5 | 1.5 |
|  |  |  |  | 2 | -0.8 | 0.8 | 0.1 | 1.3 | -0.1 | 1.4 | -0.9 | 0.9 | -0.7 | 1.4 | -1.3 | 1.2 | -0.8 | 0.8 | -0.7 | 1.3 | -1.1 | 1.1 |
|  |  |  |  | 3 | -0.6 | 0.7 | 0.2 | 1.2 | 0.0 | 1.3 | -0.7 | 0.7 | -0.6 | 1.3 | -1.2 | 1.1 | -0.6 | 0.7 | -0.5 | 1.1 | -0.8 | 0.9 |
|  |  |  |  | 4 | -0.5 | 0.6 | 0.3 | 1.1 | 0.1 | 1.2 | -0.6 | 0.6 | -0.5 | 1.3 | -1.1 | 1.0 | -0.5 | 0.6 | -0.4 | 1.0 | -0.7 | 0.8 |

Table 12 5%-ile and 95%-ile of the SL-PRS RSRP errors with Es/Iot (0,0,0)

| SL-PRS based RSRP [dB] (0, 0, 0) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  |  |  |  | TDL-A |  |  |  |  |  | TDL-B |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| 15 | 48 | 4 | 4 | 1 | -0.6 | 0.7 | -0.4 | 0.4 | -0.4 | 0.4 | -0.8 | 0.7 | -0.5 | 0.6 | -0.8 | 0.3 | -0.7 | 0.7 | -0.5 | 0.5 | -0.7 | 0.3 |
|  |  |  |  | 2 | -0.4 | 0.5 | -0.3 | 0.3 | -0.3 | 0.3 | -0.6 | 0.5 | -0.4 | 0.4 | -0.7 | 0.2 | -0.5 | 0.5 | -0.4 | 0.4 | -0.5 | 0.2 |
|  |  |  |  | 3 | -0.4 | 0.4 | -0.2 | 0.3 | -0.2 | 0.3 | -0.5 | 0.5 | -0.3 | 0.4 | -0.6 | 0.2 | -0.4 | 0.4 | -0.3 | 0.4 | -0.4 | 0.2 |
|  |  |  |  | 4 | -0.3 | 0.4 | -0.2 | 0.2 | -0.2 | 0.2 | -0.4 | 0.4 | -0.3 | 0.3 | -0.6 | 0.1 | -0.4 | 0.3 | -0.3 | 0.3 | -0.4 | 0.1 |
|  |  | 4 | 2 | 1 | -1.1 | 1.0 | -0.3 | 0.7 | -1.1 | 0.1 | -1.4 | 1.1 | -0.5 | 1.0 | -2.2 | 0.5 | -1.1 | 1.1 | -0.5 | 0.9 | -1.6 | 0.5 |
|  |  |  |  | 2 | -0.7 | 0.7 | -0.1 | 0.5 | -0.9 | 0.0 | -1.0 | 0.9 | -0.4 | 0.8 | -1.9 | 0.4 | -0.8 | 0.8 | -0.3 | 0.7 | -1.2 | 0.3 |
|  |  |  |  | 3 | -0.6 | 0.6 | -0.1 | 0.5 | -0.8 | -0.1 | -0.9 | 0.7 | -0.3 | 0.7 | -1.6 | 0.3 | -0.7 | 0.6 | -0.3 | 0.6 | -1.1 | 0.2 |
|  |  |  |  | 4 | -0.5 | 0.5 | 0.0 | 0.4 | -0.8 | -0.2 | -0.8 | 0.6 | -0.2 | 0.6 | -1.5 | 0.2 | -0.6 | 0.6 | -0.2 | 0.5 | -0.9 | 0.1 |
|  | 96 | 4 | 4 | 1 | -0.5 | 0.5 | -0.2 | 0.4 | -0.3 | 0.3 | -0.5 | 0.5 | -0.3 | 0.5 | -0.4 | 0.4 | -0.5 | 0.5 | -0.3 | 0.5 | -0.4 | 0.4 |
|  |  |  |  | 2 | -0.3 | 0.3 | -0.1 | 0.3 | -0.2 | 0.3 | -0.4 | 0.4 | -0.2 | 0.4 | -0.3 | 0.3 | -0.4 | 0.3 | -0.2 | 0.4 | -0.3 | 0.3 |
|  |  |  |  | 3 | -0.3 | 0.3 | 0.0 | 0.3 | -0.1 | 0.2 | -0.3 | 0.3 | -0.1 | 0.3 | -0.3 | 0.2 | -0.3 | 0.3 | -0.1 | 0.3 | -0.2 | 0.2 |
|  |  |  |  | 4 | -0.2 | 0.2 | 0.0 | 0.3 | -0.1 | 0.2 | -0.3 | 0.3 | -0.1 | 0.3 | -0.2 | 0.2 | -0.3 | 0.2 | -0.1 | 0.3 | -0.2 | 0.2 |
|  |  | 4 | 2 | 1 | -0.8 | 0.7 | -0.3 | 0.4 | -0.5 | 0.3 | -0.9 | 0.8 | -0.3 | 0.7 | -0.9 | 0.5 | -0.8 | 0.8 | -0.4 | 0.7 | -0.8 | 0.5 |
|  |  |  |  | 2 | -0.5 | 0.5 | -0.2 | 0.3 | -0.4 | 0.2 | -0.6 | 0.6 | -0.2 | 0.5 | -0.8 | 0.4 | -0.6 | 0.5 | -0.2 | 0.5 | -0.6 | 0.3 |
|  |  |  |  | 3 | -0.4 | 0.5 | -0.2 | 0.2 | -0.4 | 0.1 | -0.5 | 0.5 | -0.2 | 0.5 | -0.6 | 0.4 | -0.5 | 0.4 | -0.2 | 0.4 | -0.5 | 0.3 |
|  |  |  |  | 4 | -0.4 | 0.4 | -0.2 | 0.2 | -0.3 | 0.1 | -0.4 | 0.4 | -0.1 | 0.4 | -0.6 | 0.3 | -0.4 | 0.4 | -0.1 | 0.4 | -0.5 | 0.2 |
| 30 | 24 | 4 | 4 | 1 | -0.8 | 0.9 | -0.7 | 0.5 | -0.4 | 0.8 | -1.1 | 1.0 | -0.9 | 0.7 | -0.7 | 0.9 | -0.9 | 0.9 | -0.9 | 0.7 | -0.7 | 0.8 |
|  |  |  |  | 2 | -0.6 | 0.6 | -0.5 | 0.3 | -0.2 | 0.7 | -0.8 | 0.7 | -0.7 | 0.5 | -0.5 | 0.7 | -0.7 | 0.6 | -0.7 | 0.5 | -0.5 | 0.6 |
|  |  |  |  | 3 | -0.4 | 0.5 | -0.4 | 0.3 | -0.1 | 0.6 | -0.7 | 0.6 | -0.6 | 0.4 | -0.4 | 0.6 | -0.5 | 0.5 | -0.5 | 0.4 | -0.4 | 0.5 |
|  |  |  |  | 4 | -0.4 | 0.5 | -0.4 | 0.2 | -0.1 | 0.5 | -0.6 | 0.5 | -0.6 | 0.4 | -0.4 | 0.6 | -0.5 | 0.5 | -0.5 | 0.4 | -0.3 | 0.5 |
|  |  | 4 | 2 | 1 | -1.5 | 1.3 | -0.3 | 1.0 | -1.0 | 0.7 | -1.9 | 1.5 | -0.7 | 1.2 | -1.2 | 1.4 | -1.5 | 1.3 | -0.8 | 1.1 | -1.0 | 1.3 |
|  |  |  |  | 2 | -1.0 | 1.0 | -0.1 | 0.8 | -0.7 | 0.5 | -1.3 | 1.1 | -0.5 | 1.0 | -1.1 | 1.2 | -1.1 | 1.0 | -0.5 | 0.9 | -0.7 | 1.0 |
|  |  |  |  | 3 | -0.8 | 0.8 | 0.0 | 0.7 | -0.6 | 0.4 | -1.2 | 0.9 | -0.4 | 0.9 | -1.0 | 1.0 | -0.9 | 0.8 | -0.4 | 0.8 | -0.6 | 0.8 |
|  |  |  |  | 4 | -0.7 | 0.7 | 0.0 | 0.7 | -0.5 | 0.3 | -1.0 | 0.8 | -0.4 | 0.9 | -0.9 | 0.9 | -0.8 | 0.8 | -0.3 | 0.7 | -0.5 | 0.7 |
|  | 48 | 4 | 4 | 1 | -0.6 | 0.6 | -0.4 | 0.4 | -0.4 | 0.5 | -0.7 | 0.6 | -0.5 | 0.6 | -0.6 | 0.5 | -0.6 | 0.6 | -0.5 | 0.6 | -0.6 | 0.5 |
|  |  |  |  | 2 | -0.4 | 0.5 | -0.3 | 0.3 | -0.3 | 0.4 | -0.5 | 0.5 | -0.4 | 0.4 | -0.4 | 0.4 | -0.4 | 0.4 | -0.4 | 0.4 | -0.4 | 0.4 |
|  |  |  |  | 3 | -0.3 | 0.4 | -0.2 | 0.3 | -0.2 | 0.3 | -0.4 | 0.4 | -0.4 | 0.4 | -0.4 | 0.3 | -0.4 | 0.4 | -0.3 | 0.3 | -0.3 | 0.3 |
|  |  |  |  | 4 | -0.3 | 0.3 | -0.2 | 0.2 | -0.2 | 0.3 | -0.3 | 0.3 | -0.3 | 0.3 | -0.3 | 0.3 | -0.3 | 0.3 | -0.3 | 0.3 | -0.3 | 0.3 |
|  |  | 4 | 2 | 1 | -1.0 | 1.0 | -0.3 | 0.7 | -1.0 | 0.3 | -1.1 | 1.0 | -0.5 | 0.9 | -1.1 | 0.7 | -1.0 | 1.0 | -0.6 | 0.9 | -1.0 | 0.7 |
|  |  |  |  | 2 | -0.7 | 0.7 | -0.1 | 0.5 | -0.8 | 0.1 | -0.7 | 0.8 | -0.4 | 0.8 | -0.9 | 0.6 | -0.7 | 0.7 | -0.4 | 0.7 | -0.7 | 0.5 |
|  |  |  |  | 3 | -0.6 | 0.6 | -0.1 | 0.5 | -0.7 | 0.1 | -0.6 | 0.6 | -0.3 | 0.7 | -0.8 | 0.5 | -0.6 | 0.6 | -0.3 | 0.6 | -0.6 | 0.4 |
|  |  |  |  | 4 | -0.5 | 0.5 | 0.0 | 0.4 | -0.6 | 0.0 | -0.5 | 0.6 | -0.3 | 0.6 | -0.7 | 0.5 | -0.5 | 0.5 | -0.3 | 0.5 | -0.5 | 0.4 |
|  | 96 | 4 | 4 | 1 | -0.4 | 0.4 | -0.1 | 0.4 | -0.4 | 0.2 | -0.5 | 0.5 | -0.2 | 0.5 | -0.4 | 0.3 | -0.4 | 0.4 | -0.3 | 0.5 | -0.4 | 0.3 |
|  |  |  |  | 2 | -0.3 | 0.3 | 0.0 | 0.4 | -0.3 | 0.2 | -0.3 | 0.4 | -0.2 | 0.5 | -0.3 | 0.3 | -0.3 | 0.3 | -0.2 | 0.4 | -0.3 | 0.2 |
|  |  |  |  | 3 | -0.2 | 0.3 | 0.0 | 0.3 | -0.3 | 0.1 | -0.3 | 0.3 | -0.1 | 0.4 | -0.3 | 0.2 | -0.2 | 0.3 | -0.1 | 0.3 | -0.2 | 0.2 |
|  |  |  |  | 4 | -0.2 | 0.2 | 0.0 | 0.3 | -0.2 | 0.1 | -0.2 | 0.3 | -0.1 | 0.4 | -0.2 | 0.2 | -0.2 | 0.2 | -0.1 | 0.3 | -0.2 | 0.2 |
|  |  | 4 | 2 | 1 | -0.7 | 0.7 | -0.3 | 0.4 | -0.4 | 0.5 | -0.8 | 0.8 | -0.3 | 0.7 | -0.6 | 0.5 | -0.7 | 0.7 | -0.4 | 0.7 | -0.6 | 0.5 |
|  |  |  |  | 2 | -0.5 | 0.5 | -0.2 | 0.3 | -0.3 | 0.3 | -0.6 | 0.6 | -0.2 | 0.6 | -0.5 | 0.4 | -0.5 | 0.5 | -0.3 | 0.5 | -0.4 | 0.4 |
|  |  |  |  | 3 | -0.4 | 0.4 | -0.2 | 0.2 | -0.2 | 0.3 | -0.4 | 0.5 | -0.2 | 0.6 | -0.4 | 0.3 | -0.4 | 0.4 | -0.2 | 0.4 | -0.4 | 0.3 |
|  |  |  |  | 4 | -0.3 | 0.4 | -0.2 | 0.2 | -0.2 | 0.3 | -0.4 | 0.4 | -0.2 | 0.5 | -0.4 | 0.3 | -0.3 | 0.4 | -0.2 | 0.4 | -0.3 | 0.3 |
| 60 | 24 | 4 | 4 | 1 | -1.0 | 0.9 | -0.5 | 0.7 | -0.3 | 1.0 | -1.0 | 0.9 | -0.9 | 0.8 | -0.7 | 0.9 | -0.9 | 0.9 | -0.9 | 0.8 | -0.6 | 0.9 |
|  |  |  |  | 2 | -0.7 | 0.6 | -0.3 | 0.5 | -0.1 | 0.8 | -0.7 | 0.7 | -0.7 | 0.6 | -0.5 | 0.7 | -0.7 | 0.7 | -0.6 | 0.6 | -0.4 | 0.7 |
|  |  |  |  | 3 | -0.5 | 0.5 | -0.3 | 0.4 | 0.0 | 0.7 | -0.6 | 0.6 | -0.7 | 0.6 | -0.4 | 0.6 | -0.5 | 0.5 | -0.5 | 0.5 | -0.3 | 0.6 |
|  |  |  |  | 4 | -0.4 | 0.4 | -0.2 | 0.4 | 0.0 | 0.7 | -0.5 | 0.5 | -0.6 | 0.5 | -0.4 | 0.6 | -0.5 | 0.5 | -0.4 | 0.4 | -0.3 | 0.5 |
|  |  | 4 | 2 | 1 | -1.6 | 1.3 | -0.2 | 1.2 | -0.3 | 1.4 | -1.7 | 1.5 | -0.7 | 1.2 | -1.2 | 1.1 | -1.5 | 1.4 | -0.8 | 1.2 | -1.1 | 1.2 |
|  |  |  |  | 2 | -1.1 | 1.0 | 0.0 | 1.0 | -0.1 | 1.2 | -1.2 | 1.1 | -0.6 | 1.0 | -1.0 | 1.0 | -1.0 | 1.1 | -0.5 | 0.9 | -0.8 | 0.9 |
|  |  |  |  | 3 | -0.9 | 0.8 | 0.1 | 0.9 | 0.1 | 1.1 | -1.0 | 0.9 | -0.5 | 1.0 | -0.9 | 0.9 | -0.8 | 0.9 | -0.4 | 0.8 | -0.6 | 0.7 |
|  |  |  |  | 4 | -0.7 | 0.7 | 0.2 | 0.8 | 0.1 | 1.0 | -0.9 | 0.8 | -0.4 | 0.9 | -0.8 | 0.8 | -0.7 | 0.8 | -0.3 | 0.7 | -0.5 | 0.6 |

Table 13 absolute and relative accuracy of the SL-PRS RSRP with Es/Iot (-3,-6,-6)

| SL-PRS based RSRP [dB] (-3, -6, -6) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  |  |  |  | TDL-A |  |  |  |  |  | TDL-B |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel |
| 15 | 48 | 4 | 4 | 1 | 0.7 | 1.2 | 1.0 | 1.6 | 0.9 | 1.7 | 0.7 | 1.4 | 1.1 | 2.2 | 1.8 | 2.5 | 0.7 | 1.3 | 1.0 | 2.0 | 1.3 | 2.1 |
|  |  |  |  | 2 | 0.6 | 0.9 | 0.8 | 1.1 | 0.7 | 1.1 | 0.5 | 1.1 | 0.8 | 1.6 | 1.4 | 1.9 | 0.5 | 1.0 | 0.8 | 1.5 | 1.0 | 1.5 |
|  |  |  |  | 3 | 0.5 | 0.7 | 0.7 | 0.9 | 0.6 | 0.9 | 0.4 | 0.9 | 0.7 | 1.4 | 1.2 | 1.6 | 0.4 | 0.8 | 0.7 | 1.3 | 0.8 | 1.2 |
|  |  |  |  | 4 | 0.5 | 0.6 | 0.7 | 0.8 | 0.6 | 0.8 | 0.4 | 0.8 | 0.6 | 1.3 | 1.1 | 1.5 | 0.4 | 0.7 | 0.6 | 1.1 | 0.8 | 1.1 |
|  |  | 4 | 2 | 1 | 1.1 | 1.8 | 1.5 | 2.0 | 1.8 | 2.6 | 1.2 | 2.3 | 1.8 | 3.0 | 4.4 | 5.4 | 1.1 | 2.1 | 1.6 | 2.7 | 3.1 | 4.0 |
|  |  |  |  | 2 | 0.8 | 1.3 | 1.3 | 1.4 | 1.4 | 1.8 | 0.9 | 1.7 | 1.4 | 2.3 | 3.5 | 4.1 | 0.8 | 1.5 | 1.2 | 2.0 | 2.5 | 3.0 |
|  |  |  |  | 3 | 0.7 | 1.0 | 1.1 | 1.1 | 1.2 | 1.5 | 0.8 | 1.5 | 1.3 | 2.0 | 2.8 | 3.3 | 0.6 | 1.2 | 1.0 | 1.6 | 2.1 | 2.4 |
|  |  |  |  | 4 | 0.7 | 0.9 | 1.1 | 1.0 | 1.1 | 1.3 | 0.7 | 1.3 | 1.2 | 1.7 | 2.4 | 2.8 | 0.5 | 1.1 | 0.9 | 1.4 | 1.7 | 2.0 |
|  | 96 | 4 | 4 | 1 | 0.5 | 0.8 | 0.9 | 1.1 | 0.8 | 1.2 | 0.5 | 1.0 | 0.9 | 1.5 | 0.8 | 1.5 | 0.5 | 0.9 | 0.9 | 1.5 | 0.7 | 1.4 |
|  |  |  |  | 2 | 0.4 | 0.6 | 0.7 | 0.8 | 0.6 | 0.8 | 0.4 | 0.7 | 0.7 | 1.1 | 0.6 | 1.1 | 0.3 | 0.7 | 0.7 | 1.0 | 0.5 | 1.0 |
|  |  |  |  | 3 | 0.4 | 0.5 | 0.6 | 0.6 | 0.5 | 0.7 | 0.3 | 0.6 | 0.6 | 0.9 | 0.5 | 1.0 | 0.3 | 0.5 | 0.6 | 0.8 | 0.4 | 0.9 |
|  |  |  |  | 4 | 0.3 | 0.4 | 0.6 | 0.6 | 0.5 | 0.6 | 0.3 | 0.5 | 0.6 | 0.8 | 0.5 | 0.9 | 0.2 | 0.5 | 0.5 | 0.7 | 0.4 | 0.7 |
|  |  | 4 | 2 | 1 | 0.8 | 1.3 | 1.0 | 1.5 | 0.9 | 1.6 | 0.8 | 1.5 | 1.2 | 2.0 | 1.7 | 2.7 | 0.7 | 1.5 | 1.2 | 1.9 | 1.5 | 2.3 |
|  |  |  |  | 2 | 0.6 | 0.9 | 0.8 | 1.1 | 0.6 | 1.2 | 0.6 | 1.1 | 1.0 | 1.5 | 1.4 | 2.1 | 0.5 | 1.0 | 0.9 | 1.4 | 1.1 | 1.7 |
|  |  |  |  | 3 | 0.5 | 0.8 | 0.7 | 0.9 | 0.5 | 0.9 | 0.5 | 0.9 | 0.9 | 1.3 | 1.2 | 1.7 | 0.4 | 0.8 | 0.8 | 1.1 | 0.9 | 1.4 |
|  |  |  |  | 4 | 0.5 | 0.7 | 0.6 | 0.8 | 0.4 | 0.8 | 0.4 | 0.8 | 0.8 | 1.1 | 1.1 | 1.5 | 0.4 | 0.7 | 0.7 | 1.0 | 0.8 | 1.2 |
| 30 | 24 | 4 | 4 | 1 | 1.0 | 1.6 | 1.2 | 2.3 | 1.6 | 2.2 | 1.1 | 2.1 | 1.9 | 3.3 | 1.6 | 3.2 | 0.9 | 1.8 | 1.8 | 3.2 | 1.5 | 2.8 |
|  |  |  |  | 2 | 0.8 | 1.1 | 1.0 | 1.6 | 1.3 | 1.5 | 0.8 | 1.5 | 1.5 | 2.6 | 1.3 | 2.5 | 0.7 | 1.3 | 1.3 | 2.3 | 1.1 | 2.1 |
|  |  |  |  | 3 | 0.7 | 0.9 | 0.8 | 1.3 | 1.2 | 1.3 | 0.7 | 1.3 | 1.3 | 2.2 | 1.1 | 2.1 | 0.5 | 1.1 | 1.0 | 1.8 | 1.0 | 1.7 |
|  |  |  |  | 4 | 0.6 | 0.8 | 0.7 | 1.2 | 1.1 | 1.0 | 0.6 | 1.2 | 1.2 | 2.0 | 1.0 | 1.9 | 0.5 | 0.9 | 0.9 | 1.6 | 0.9 | 1.5 |
|  |  | 4 | 2 | 1 | 1.4 | 2.5 | 2.1 | 2.6 | 1.7 | 3.3 | 1.9 | 3.3 | 2.4 | 4.2 | 3.1 | 5.5 | 1.5 | 2.8 | 2.1 | 3.8 | 2.4 | 4.6 |
|  |  |  |  | 2 | 1.1 | 1.8 | 1.7 | 1.8 | 1.3 | 2.3 | 1.3 | 2.4 | 1.9 | 3.1 | 2.3 | 4.2 | 1.0 | 2.0 | 1.7 | 2.8 | 1.8 | 3.4 |
|  |  |  |  | 3 | 1.0 | 1.4 | 1.6 | 1.5 | 1.1 | 1.9 | 1.2 | 2.1 | 1.8 | 2.7 | 1.8 | 3.5 | 0.8 | 1.6 | 1.4 | 2.3 | 1.4 | 2.8 |
|  |  |  |  | 4 | 0.9 | 1.2 | 1.5 | 1.3 | 1.1 | 1.6 | 1.0 | 1.8 | 1.6 | 2.5 | 1.5 | 3.1 | 0.7 | 1.4 | 1.3 | 2.0 | 1.2 | 2.4 |
|  | 48 | 4 | 4 | 1 | 0.7 | 1.1 | 1.0 | 1.6 | 1.0 | 1.7 | 0.7 | 1.3 | 1.0 | 2.1 | 1.1 | 2.1 | 0.6 | 1.3 | 1.0 | 2.0 | 1.0 | 2.0 |
|  |  |  |  | 2 | 0.6 | 0.8 | 0.8 | 1.1 | 0.8 | 1.2 | 0.5 | 1.0 | 0.8 | 1.5 | 0.8 | 1.5 | 0.5 | 0.9 | 0.8 | 1.4 | 0.7 | 1.4 |
|  |  |  |  | 3 | 0.5 | 0.7 | 0.7 | 0.9 | 0.7 | 1.0 | 0.4 | 0.8 | 0.7 | 1.3 | 0.7 | 1.3 | 0.4 | 0.8 | 0.6 | 1.2 | 0.6 | 1.2 |
|  |  |  |  | 4 | 0.5 | 0.6 | 0.6 | 0.8 | 0.7 | 0.8 | 0.4 | 0.7 | 0.6 | 1.1 | 0.6 | 1.1 | 0.3 | 0.6 | 0.6 | 1.0 | 0.5 | 1.1 |
|  |  | 4 | 2 | 1 | 1.1 | 1.8 | 1.5 | 2.0 | 1.5 | 2.5 | 1.0 | 2.0 | 1.7 | 2.8 | 2.0 | 3.3 | 1.0 | 1.9 | 1.6 | 2.8 | 1.8 | 3.1 |
|  |  |  |  | 2 | 0.8 | 1.3 | 1.2 | 1.4 | 1.1 | 1.7 | 0.7 | 1.5 | 1.4 | 2.2 | 1.6 | 2.7 | 0.7 | 1.4 | 1.3 | 2.0 | 1.3 | 2.2 |
|  |  |  |  | 3 | 0.7 | 1.0 | 1.1 | 1.1 | 0.9 | 1.5 | 0.6 | 1.2 | 1.2 | 1.9 | 1.4 | 2.3 | 0.6 | 1.1 | 1.1 | 1.7 | 1.1 | 1.8 |
|  |  |  |  | 4 | 0.7 | 0.9 | 1.0 | 1.0 | 0.8 | 1.3 | 0.5 | 1.0 | 1.1 | 1.6 | 1.3 | 2.1 | 0.5 | 1.0 | 1.0 | 1.5 | 0.9 | 1.6 |
|  | 96 | 4 | 4 | 1 | 0.5 | 0.8 | 0.9 | 1.1 | 0.6 | 1.2 | 0.5 | 1.0 | 1.0 | 1.5 | 0.8 | 1.5 | 0.4 | 0.9 | 0.9 | 1.4 | 0.7 | 1.4 |
|  |  |  |  | 2 | 0.4 | 0.6 | 0.7 | 0.8 | 0.5 | 0.9 | 0.4 | 0.7 | 0.8 | 1.1 | 0.6 | 1.1 | 0.3 | 0.6 | 0.7 | 1.0 | 0.5 | 1.0 |
|  |  |  |  | 3 | 0.4 | 0.5 | 0.7 | 0.6 | 0.4 | 0.7 | 0.3 | 0.6 | 0.7 | 1.0 | 0.5 | 0.9 | 0.3 | 0.5 | 0.6 | 0.8 | 0.4 | 0.8 |
|  |  |  |  | 4 | 0.3 | 0.4 | 0.6 | 0.5 | 0.3 | 0.6 | 0.3 | 0.5 | 0.6 | 0.9 | 0.4 | 0.8 | 0.2 | 0.4 | 0.5 | 0.7 | 0.4 | 0.7 |
|  |  | 4 | 2 | 1 | 0.8 | 1.3 | 1.0 | 1.5 | 1.0 | 1.6 | 0.8 | 1.5 | 1.3 | 2.1 | 1.3 | 2.2 | 0.7 | 1.4 | 1.2 | 2.0 | 1.2 | 2.1 |
|  |  |  |  | 2 | 0.6 | 0.9 | 0.8 | 1.1 | 0.8 | 1.2 | 0.6 | 1.1 | 1.1 | 1.6 | 1.0 | 1.7 | 0.5 | 1.0 | 0.9 | 1.4 | 0.8 | 1.5 |
|  |  |  |  | 3 | 0.5 | 0.7 | 0.7 | 0.9 | 0.7 | 0.9 | 0.5 | 0.9 | 1.0 | 1.4 | 0.9 | 1.5 | 0.4 | 0.8 | 0.8 | 1.1 | 0.7 | 1.2 |
|  |  |  |  | 4 | 0.5 | 0.6 | 0.6 | 0.7 | 0.6 | 0.8 | 0.4 | 0.8 | 0.9 | 1.2 | 0.8 | 1.3 | 0.4 | 0.7 | 0.7 | 1.0 | 0.6 | 1.0 |
| 60 | 24 | 4 | 4 | 1 | 1.0 | 1.7 | 1.4 | 2.3 | 1.7 | 2.2 | 1.0 | 1.9 | 1.7 | 3.1 | 1.6 | 2.9 | 0.9 | 1.8 | 1.6 | 3.0 | 1.6 | 2.7 |
|  |  |  |  | 2 | 0.8 | 1.2 | 1.1 | 1.6 | 1.5 | 1.5 | 0.7 | 1.4 | 1.4 | 2.5 | 1.3 | 2.2 | 0.7 | 1.3 | 1.1 | 2.2 | 1.2 | 2.0 |
|  |  |  |  | 3 | 0.7 | 1.0 | 1.0 | 1.3 | 1.3 | 1.3 | 0.6 | 1.2 | 1.2 | 2.2 | 1.1 | 1.9 | 0.6 | 1.1 | 0.9 | 1.8 | 1.0 | 1.6 |
|  |  |  |  | 4 | 0.6 | 0.8 | 0.9 | 1.1 | 1.3 | 1.1 | 0.5 | 1.0 | 1.1 | 2.0 | 1.0 | 1.7 | 0.5 | 0.9 | 0.8 | 1.6 | 0.9 | 1.4 |
|  |  | 4 | 2 | 1 | 1.5 | 2.6 | 2.2 | 2.6 | 2.3 | 3.0 | 1.6 | 3.0 | 2.2 | 3.9 | 2.3 | 4.3 | 1.4 | 2.8 | 2.1 | 3.9 | 2.2 | 4.2 |
|  |  |  |  | 2 | 1.1 | 1.8 | 1.9 | 1.8 | 2.0 | 2.1 | 1.1 | 2.2 | 1.8 | 3.0 | 1.7 | 3.3 | 1.0 | 2.0 | 1.7 | 2.8 | 1.5 | 3.0 |
|  |  |  |  | 3 | 1.0 | 1.5 | 1.7 | 1.5 | 1.8 | 1.7 | 0.9 | 1.8 | 1.6 | 2.6 | 1.5 | 2.9 | 0.8 | 1.6 | 1.5 | 2.4 | 1.3 | 2.5 |
|  |  |  |  | 4 | 0.9 | 1.3 | 1.6 | 1.3 | 1.7 | 1.5 | 0.8 | 1.6 | 1.5 | 2.4 | 1.3 | 2.6 | 0.7 | 1.4 | 1.3 | 2.1 | 1.1 | 2.1 |

Table 14 absolute and relative accuracy of the SL-PRS RSRP with Es/Iot (0,-6,-6)

| SL-PRS based RSRP [dB] (0, -6, -6) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  |  |  |  | TDL-A |  |  |  |  |  | TDL-B |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel |
| 15 | 48 | 4 | 4 | 1 | 0.5 | 0.8 | 0.9 | 1.6 | 0.9 | 1.7 | 0.5 | 0.9 | 1.1 | 2.2 | 2.1 | 2.7 | 0.4 | 0.8 | 1.1 | 2.2 | 1.7 | 2.3 |
|  |  |  |  | 2 | 0.3 | 0.6 | 0.7 | 1.1 | 0.6 | 1.2 | 0.3 | 0.6 | 0.9 | 1.7 | 1.7 | 2.1 | 0.3 | 0.6 | 0.8 | 1.6 | 1.2 | 1.7 |
|  |  |  |  | 3 | 0.3 | 0.5 | 0.6 | 0.9 | 0.5 | 1.0 | 0.3 | 0.5 | 0.8 | 1.5 | 1.5 | 1.8 | 0.3 | 0.5 | 0.7 | 1.3 | 1.0 | 1.3 |
|  |  |  |  | 4 | 0.3 | 0.4 | 0.6 | 0.8 | 0.4 | 0.8 | 0.3 | 0.5 | 0.7 | 1.3 | 1.3 | 1.5 | 0.2 | 0.4 | 0.6 | 1.2 | 1.0 | 1.2 |
|  |  | 4 | 2 | 1 | 0.7 | 1.2 | 1.5 | 1.8 | 2.6 | 2.8 | 0.7 | 1.4 | 2.0 | 3.1 | 6.1 | 6.8 | 0.7 | 1.4 | 1.8 | 2.7 | 4.1 | 4.8 |
|  |  |  |  | 2 | 0.5 | 0.9 | 1.2 | 1.3 | 2.1 | 2.0 | 0.5 | 1.1 | 1.6 | 2.4 | 4.3 | 4.8 | 0.5 | 1.0 | 1.4 | 2.0 | 3.3 | 3.6 |
|  |  |  |  | 3 | 0.4 | 0.7 | 1.1 | 1.0 | 1.8 | 1.6 | 0.5 | 0.9 | 1.5 | 2.0 | 3.5 | 3.8 | 0.4 | 0.8 | 1.2 | 1.6 | 2.7 | 2.9 |
|  |  |  |  | 4 | 0.4 | 0.6 | 1.0 | 0.9 | 1.7 | 1.4 | 0.4 | 0.8 | 1.4 | 1.8 | 3.1 | 3.3 | 0.4 | 0.7 | 1.1 | 1.4 | 2.4 | 2.4 |
|  | 96 | 4 | 4 | 1 | 0.3 | 0.6 | 0.9 | 1.1 | 0.7 | 1.2 | 0.3 | 0.6 | 1.1 | 1.5 | 0.9 | 1.7 | 0.3 | 0.6 | 1.0 | 1.5 | 0.8 | 1.5 |
|  |  |  |  | 2 | 0.2 | 0.4 | 0.7 | 0.8 | 0.5 | 0.8 | 0.2 | 0.4 | 0.9 | 1.2 | 0.7 | 1.2 | 0.2 | 0.4 | 0.8 | 1.1 | 0.6 | 1.1 |
|  |  |  |  | 3 | 0.2 | 0.3 | 0.7 | 0.6 | 0.5 | 0.7 | 0.2 | 0.4 | 0.8 | 1.0 | 0.6 | 1.1 | 0.2 | 0.3 | 0.7 | 0.9 | 0.5 | 0.9 |
|  |  |  |  | 4 | 0.2 | 0.3 | 0.6 | 0.5 | 0.4 | 0.6 | 0.2 | 0.3 | 0.7 | 0.9 | 0.5 | 1.0 | 0.1 | 0.3 | 0.6 | 0.8 | 0.4 | 0.8 |
|  |  | 4 | 2 | 1 | 0.5 | 0.9 | 0.9 | 1.4 | 1.1 | 1.6 | 0.5 | 1.0 | 1.4 | 2.0 | 2.2 | 3.0 | 0.5 | 1.0 | 1.4 | 2.0 | 1.9 | 2.7 |
|  |  |  |  | 2 | 0.4 | 0.6 | 0.7 | 1.0 | 0.8 | 1.2 | 0.4 | 0.7 | 1.2 | 1.5 | 1.9 | 2.5 | 0.3 | 0.7 | 1.0 | 1.4 | 1.4 | 1.9 |
|  |  |  |  | 3 | 0.3 | 0.5 | 0.6 | 0.8 | 0.7 | 1.0 | 0.3 | 0.6 | 1.0 | 1.3 | 1.5 | 2.0 | 0.3 | 0.5 | 0.9 | 1.2 | 1.2 | 1.6 |
|  |  |  |  | 4 | 0.3 | 0.5 | 0.6 | 0.7 | 0.7 | 0.9 | 0.2 | 0.5 | 1.0 | 1.2 | 1.4 | 1.8 | 0.2 | 0.5 | 0.8 | 1.0 | 1.1 | 1.5 |
| 30 | 24 | 4 | 4 | 1 | 0.6 | 1.1 | 1.3 | 2.4 | 1.6 | 2.2 | 0.7 | 1.3 | 2.2 | 3.5 | 1.8 | 3.3 | 0.6 | 1.2 | 2.0 | 3.4 | 1.6 | 2.9 |
|  |  |  |  | 2 | 0.5 | 0.8 | 0.9 | 1.7 | 1.3 | 1.5 | 0.5 | 0.9 | 1.8 | 2.8 | 1.4 | 2.5 | 0.4 | 0.8 | 1.4 | 2.5 | 1.2 | 2.1 |
|  |  |  |  | 3 | 0.4 | 0.6 | 0.7 | 1.4 | 1.2 | 1.2 | 0.4 | 0.8 | 1.4 | 2.3 | 1.3 | 2.1 | 0.3 | 0.7 | 1.2 | 2.0 | 1.0 | 1.8 |
|  |  |  |  | 4 | 0.4 | 0.5 | 0.6 | 1.2 | 1.1 | 1.1 | 0.4 | 0.7 | 1.4 | 2.1 | 1.1 | 1.9 | 0.3 | 0.6 | 1.1 | 1.9 | 0.9 | 1.6 |
|  |  | 4 | 2 | 1 | 1.0 | 1.7 | 2.0 | 2.4 | 1.8 | 3.4 | 1.1 | 2.0 | 2.6 | 4.0 | 3.4 | 5.9 | 0.9 | 1.8 | 2.3 | 3.8 | 2.8 | 5.1 |
|  |  |  |  | 2 | 0.7 | 1.2 | 1.7 | 1.7 | 1.2 | 2.4 | 0.8 | 1.5 | 2.3 | 3.2 | 2.5 | 4.6 | 0.7 | 1.3 | 1.8 | 2.8 | 2.1 | 3.8 |
|  |  |  |  | 3 | 0.6 | 1.0 | 1.5 | 1.4 | 1.0 | 1.9 | 0.6 | 1.2 | 2.1 | 2.8 | 1.9 | 3.9 | 0.5 | 1.1 | 1.6 | 2.3 | 1.7 | 3.2 |
|  |  |  |  | 4 | 0.6 | 0.9 | 1.4 | 1.2 | 0.9 | 1.7 | 0.6 | 1.1 | 2.0 | 2.6 | 1.7 | 3.4 | 0.5 | 0.9 | 1.5 | 2.0 | 1.4 | 2.7 |
|  | 48 | 4 | 4 | 1 | 0.5 | 0.8 | 0.9 | 1.6 | 1.0 | 1.8 | 0.4 | 0.8 | 1.1 | 2.2 | 1.2 | 2.2 | 0.4 | 0.8 | 1.1 | 2.2 | 1.1 | 2.1 |
|  |  |  |  | 2 | 0.4 | 0.5 | 0.7 | 1.2 | 0.8 | 1.2 | 0.3 | 0.6 | 0.9 | 1.7 | 0.9 | 1.6 | 0.3 | 0.6 | 0.8 | 1.6 | 0.8 | 1.5 |
|  |  |  |  | 3 | 0.3 | 0.4 | 0.6 | 0.9 | 0.7 | 1.0 | 0.3 | 0.5 | 0.7 | 1.4 | 0.8 | 1.4 | 0.2 | 0.5 | 0.7 | 1.3 | 0.7 | 1.3 |
|  |  |  |  | 4 | 0.3 | 0.4 | 0.6 | 0.8 | 0.6 | 0.9 | 0.2 | 0.4 | 0.6 | 1.3 | 0.7 | 1.2 | 0.2 | 0.4 | 0.6 | 1.1 | 0.6 | 1.2 |
|  |  | 4 | 2 | 1 | 0.7 | 1.2 | 1.4 | 1.8 | 1.9 | 2.6 | 0.7 | 1.3 | 1.9 | 2.9 | 2.4 | 3.6 | 0.6 | 1.3 | 1.8 | 2.9 | 2.2 | 3.4 |
|  |  |  |  | 2 | 0.5 | 0.9 | 1.2 | 1.3 | 1.5 | 1.9 | 0.5 | 0.9 | 1.6 | 2.3 | 2.0 | 3.0 | 0.5 | 0.9 | 1.4 | 2.1 | 1.6 | 2.5 |
|  |  |  |  | 3 | 0.4 | 0.7 | 1.1 | 1.0 | 1.3 | 1.5 | 0.4 | 0.8 | 1.5 | 2.0 | 1.8 | 2.8 | 0.4 | 0.7 | 1.2 | 1.8 | 1.3 | 2.1 |
|  |  |  |  | 4 | 0.4 | 0.6 | 1.0 | 0.9 | 1.2 | 1.3 | 0.4 | 0.7 | 1.4 | 1.8 | 1.6 | 2.4 | 0.3 | 0.6 | 1.1 | 1.6 | 1.1 | 1.8 |
|  | 96 | 4 | 4 | 1 | 0.3 | 0.6 | 0.9 | 1.1 | 0.7 | 1.3 | 0.3 | 0.6 | 1.1 | 1.6 | 0.9 | 1.6 | 0.3 | 0.6 | 1.0 | 1.5 | 0.8 | 1.5 |
|  |  |  |  | 2 | 0.2 | 0.4 | 0.7 | 0.8 | 0.5 | 0.9 | 0.2 | 0.4 | 1.0 | 1.3 | 0.7 | 1.2 | 0.2 | 0.4 | 0.8 | 1.1 | 0.6 | 1.0 |
|  |  |  |  | 3 | 0.2 | 0.3 | 0.7 | 0.6 | 0.4 | 0.7 | 0.2 | 0.4 | 0.9 | 1.1 | 0.6 | 1.0 | 0.2 | 0.3 | 0.7 | 0.9 | 0.5 | 0.9 |
|  |  |  |  | 4 | 0.2 | 0.3 | 0.6 | 0.5 | 0.4 | 0.6 | 0.2 | 0.3 | 0.8 | 1.0 | 0.5 | 0.9 | 0.1 | 0.3 | 0.6 | 0.8 | 0.4 | 0.7 |
|  |  | 4 | 2 | 1 | 0.5 | 0.9 | 0.9 | 1.4 | 0.8 | 1.7 | 0.5 | 1.0 | 1.5 | 2.1 | 1.6 | 2.4 | 0.4 | 0.9 | 1.3 | 2.1 | 1.3 | 2.2 |
|  |  |  |  | 2 | 0.4 | 0.6 | 0.7 | 1.0 | 0.6 | 1.2 | 0.4 | 0.7 | 1.3 | 1.7 | 1.3 | 2.0 | 0.3 | 0.6 | 1.0 | 1.5 | 0.9 | 1.6 |
|  |  |  |  | 3 | 0.3 | 0.5 | 0.6 | 0.8 | 0.5 | 1.0 | 0.3 | 0.6 | 1.2 | 1.5 | 1.2 | 1.7 | 0.3 | 0.5 | 0.9 | 1.2 | 0.8 | 1.3 |
|  |  |  |  | 4 | 0.3 | 0.4 | 0.6 | 0.7 | 0.5 | 0.8 | 0.3 | 0.5 | 1.1 | 1.4 | 1.1 | 1.6 | 0.2 | 0.4 | 0.8 | 1.1 | 0.7 | 1.1 |
| 60 | 24 | 4 | 4 | 1 | 0.6 | 1.1 | 1.3 | 2.3 | 1.8 | 2.3 | 0.6 | 1.2 | 2.0 | 3.5 | 1.8 | 3.0 | 0.6 | 1.2 | 1.8 | 3.3 | 1.8 | 2.9 |
|  |  |  |  | 2 | 0.5 | 0.8 | 1.0 | 1.6 | 1.5 | 1.6 | 0.5 | 0.9 | 1.6 | 2.8 | 1.5 | 2.4 | 0.4 | 0.8 | 1.3 | 2.4 | 1.4 | 2.1 |
|  |  |  |  | 3 | 0.4 | 0.7 | 0.8 | 1.3 | 1.4 | 1.3 | 0.4 | 0.7 | 1.4 | 2.6 | 1.3 | 2.0 | 0.4 | 0.7 | 1.0 | 2.0 | 1.2 | 1.7 |
|  |  |  |  | 4 | 0.4 | 0.6 | 0.8 | 1.2 | 1.3 | 1.1 | 0.3 | 0.6 | 1.3 | 2.3 | 1.2 | 1.8 | 0.3 | 0.6 | 0.9 | 1.7 | 1.1 | 1.5 |
|  |  | 4 | 2 | 1 | 1.0 | 1.8 | 2.2 | 2.4 | 2.3 | 3.1 | 1.0 | 1.9 | 2.4 | 3.9 | 2.4 | 4.5 | 0.9 | 1.8 | 2.4 | 4.0 | 2.4 | 4.5 |
|  |  |  |  | 2 | 0.7 | 1.3 | 1.8 | 1.6 | 1.9 | 2.2 | 0.7 | 1.4 | 2.1 | 3.1 | 1.8 | 3.5 | 0.7 | 1.3 | 1.8 | 2.9 | 1.6 | 3.2 |
|  |  |  |  | 3 | 0.6 | 1.0 | 1.7 | 1.3 | 1.7 | 1.7 | 0.6 | 1.2 | 2.0 | 2.8 | 1.5 | 3.1 | 0.6 | 1.0 | 1.6 | 2.4 | 1.4 | 2.6 |
|  |  |  |  | 4 | 0.6 | 0.9 | 1.6 | 1.2 | 1.6 | 1.5 | 0.5 | 1.0 | 1.9 | 2.6 | 1.4 | 2.8 | 0.5 | 0.9 | 1.4 | 2.1 | 1.2 | 2.3 |

Table 15 absolute and relative accuracy of the SL-PRS RSRP with Es/Iot (0,-3,-3)

| SL-PRS based RSRP [dB] (0, -3, -3) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  |  |  |  | TDL-A |  |  |  |  |  | TDL-B |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel |
| 15 | 48 | 4 | 4 | 1 | 0.5 | 1.0 | 0.6 | 1.1 | 0.6 | 1.1 | 0.6 | 1.1 | 0.8 | 1.5 | 1.2 | 1.6 | 0.6 | 1.1 | 0.7 | 1.4 | 1.0 | 1.4 |
|  |  |  |  | 2 | 0.4 | 0.7 | 0.5 | 0.8 | 0.4 | 0.8 | 0.4 | 0.8 | 0.6 | 1.1 | 1.0 | 1.3 | 0.4 | 0.8 | 0.5 | 1.0 | 0.7 | 1.0 |
|  |  |  |  | 3 | 0.3 | 0.6 | 0.4 | 0.6 | 0.4 | 0.7 | 0.4 | 0.7 | 0.5 | 1.0 | 0.9 | 1.1 | 0.3 | 0.6 | 0.5 | 0.9 | 0.6 | 0.8 |
|  |  |  |  | 4 | 0.3 | 0.5 | 0.4 | 0.6 | 0.3 | 0.6 | 0.3 | 0.6 | 0.5 | 0.8 | 0.8 | 0.9 | 0.3 | 0.5 | 0.4 | 0.8 | 0.6 | 0.8 |
|  |  | 4 | 2 | 1 | 0.8 | 1.6 | 0.9 | 1.2 | 1.5 | 1.7 | 1.0 | 1.9 | 1.3 | 2.0 | 3.4 | 4.0 | 0.9 | 1.7 | 1.2 | 1.8 | 2.3 | 2.9 |
|  |  |  |  | 2 | 0.6 | 1.1 | 0.8 | 0.9 | 1.3 | 1.2 | 0.7 | 1.4 | 1.1 | 1.5 | 2.9 | 3.4 | 0.6 | 1.3 | 1.0 | 1.3 | 1.8 | 2.1 |
|  |  |  |  | 3 | 0.5 | 0.9 | 0.7 | 0.7 | 1.1 | 1.0 | 0.6 | 1.2 | 1.0 | 1.3 | 2.4 | 2.7 | 0.5 | 1.0 | 0.8 | 1.1 | 1.5 | 1.7 |
|  |  |  |  | 4 | 0.5 | 0.8 | 0.6 | 0.6 | 1.1 | 0.8 | 0.6 | 1.1 | 0.9 | 1.2 | 2.1 | 2.4 | 0.5 | 0.9 | 0.7 | 1.0 | 1.4 | 1.5 |
|  | 96 | 4 | 4 | 1 | 0.4 | 0.7 | 0.6 | 0.7 | 0.5 | 0.8 | 0.4 | 0.8 | 0.7 | 1.0 | 0.6 | 1.1 | 0.4 | 0.7 | 0.7 | 1.0 | 0.5 | 1.0 |
|  |  |  |  | 2 | 0.3 | 0.5 | 0.5 | 0.5 | 0.4 | 0.6 | 0.3 | 0.6 | 0.6 | 0.8 | 0.4 | 0.8 | 0.3 | 0.5 | 0.5 | 0.7 | 0.4 | 0.7 |
|  |  |  |  | 3 | 0.2 | 0.4 | 0.4 | 0.4 | 0.3 | 0.5 | 0.2 | 0.5 | 0.5 | 0.6 | 0.4 | 0.7 | 0.2 | 0.4 | 0.4 | 0.6 | 0.3 | 0.6 |
|  |  |  |  | 4 | 0.2 | 0.4 | 0.4 | 0.4 | 0.3 | 0.4 | 0.2 | 0.4 | 0.4 | 0.6 | 0.3 | 0.6 | 0.2 | 0.4 | 0.4 | 0.5 | 0.3 | 0.5 |
|  |  | 4 | 2 | 1 | 0.6 | 1.2 | 0.5 | 0.9 | 0.7 | 1.1 | 0.6 | 1.3 | 0.9 | 1.4 | 1.3 | 2.0 | 0.6 | 1.2 | 0.9 | 1.4 | 1.2 | 1.8 |
|  |  |  |  | 2 | 0.5 | 0.8 | 0.4 | 0.7 | 0.6 | 0.8 | 0.5 | 0.9 | 0.8 | 1.0 | 1.1 | 1.6 | 0.4 | 0.9 | 0.7 | 1.0 | 0.8 | 1.3 |
|  |  |  |  | 3 | 0.4 | 0.7 | 0.3 | 0.5 | 0.5 | 0.6 | 0.4 | 0.7 | 0.7 | 0.9 | 0.9 | 1.3 | 0.4 | 0.7 | 0.6 | 0.8 | 0.7 | 1.0 |
|  |  |  |  | 4 | 0.3 | 0.6 | 0.3 | 0.5 | 0.4 | 0.5 | 0.3 | 0.6 | 0.6 | 0.8 | 0.8 | 1.2 | 0.3 | 0.6 | 0.5 | 0.7 | 0.7 | 0.9 |
| 30 | 24 | 4 | 4 | 1 | 0.7 | 1.4 | 0.9 | 1.6 | 1.1 | 1.6 | 0.8 | 1.6 | 1.3 | 2.2 | 1.2 | 2.1 | 0.7 | 1.4 | 1.2 | 2.2 | 1.1 | 1.9 |
|  |  |  |  | 2 | 0.5 | 1.0 | 0.6 | 1.1 | 0.9 | 1.1 | 0.6 | 1.2 | 1.1 | 1.8 | 1.0 | 1.6 | 0.5 | 1.0 | 0.9 | 1.6 | 0.8 | 1.4 |
|  |  |  |  | 3 | 0.5 | 0.8 | 0.5 | 0.9 | 0.8 | 0.9 | 0.5 | 1.0 | 0.9 | 1.5 | 0.8 | 1.4 | 0.4 | 0.8 | 0.7 | 1.3 | 0.7 | 1.1 |
|  |  |  |  | 4 | 0.4 | 0.7 | 0.4 | 0.8 | 0.7 | 0.8 | 0.5 | 0.9 | 0.9 | 1.4 | 0.7 | 1.2 | 0.4 | 0.7 | 0.7 | 1.2 | 0.6 | 1.0 |
|  |  | 4 | 2 | 1 | 1.1 | 2.2 | 1.3 | 1.7 | 1.3 | 2.3 | 1.4 | 2.6 | 1.7 | 2.6 | 1.8 | 3.5 | 1.2 | 2.3 | 1.5 | 2.5 | 1.7 | 3.1 |
|  |  |  |  | 2 | 0.8 | 1.6 | 1.1 | 1.1 | 0.9 | 1.6 | 1.0 | 1.9 | 1.5 | 2.1 | 1.5 | 3.0 | 0.8 | 1.6 | 1.2 | 1.8 | 1.2 | 2.3 |
|  |  |  |  | 3 | 0.7 | 1.3 | 1.0 | 1.0 | 0.7 | 1.3 | 0.9 | 1.6 | 1.3 | 1.8 | 1.3 | 2.7 | 0.7 | 1.3 | 1.0 | 1.5 | 1.1 | 2.0 |
|  |  |  |  | 4 | 0.6 | 1.1 | 0.9 | 0.8 | 0.6 | 1.1 | 0.8 | 1.4 | 1.3 | 1.7 | 1.2 | 2.4 | 0.6 | 1.2 | 0.9 | 1.3 | 1.0 | 1.8 |
|  | 48 | 4 | 4 | 1 | 0.5 | 1.0 | 0.6 | 1.1 | 0.7 | 1.2 | 0.5 | 1.0 | 0.7 | 1.4 | 0.8 | 1.4 | 0.5 | 1.0 | 0.7 | 1.4 | 0.7 | 1.4 |
|  |  |  |  | 2 | 0.4 | 0.7 | 0.5 | 0.8 | 0.5 | 0.9 | 0.4 | 0.7 | 0.6 | 1.1 | 0.6 | 1.1 | 0.4 | 0.7 | 0.6 | 1.0 | 0.5 | 1.0 |
|  |  |  |  | 3 | 0.3 | 0.5 | 0.4 | 0.6 | 0.4 | 0.7 | 0.3 | 0.6 | 0.5 | 1.0 | 0.5 | 0.9 | 0.3 | 0.6 | 0.5 | 0.9 | 0.4 | 0.8 |
|  |  |  |  | 4 | 0.3 | 0.5 | 0.4 | 0.6 | 0.4 | 0.6 | 0.3 | 0.5 | 0.4 | 0.8 | 0.4 | 0.8 | 0.3 | 0.5 | 0.4 | 0.7 | 0.4 | 0.7 |
|  |  | 4 | 2 | 1 | 0.8 | 1.6 | 0.9 | 1.3 | 1.3 | 1.7 | 0.8 | 1.7 | 1.3 | 1.9 | 1.4 | 2.4 | 0.8 | 1.6 | 1.2 | 2.0 | 1.3 | 2.3 |
|  |  |  |  | 2 | 0.6 | 1.1 | 0.7 | 0.9 | 1.0 | 1.2 | 0.6 | 1.2 | 1.1 | 1.6 | 1.2 | 2.0 | 0.6 | 1.1 | 1.0 | 1.5 | 1.0 | 1.6 |
|  |  |  |  | 3 | 0.5 | 0.9 | 0.7 | 0.7 | 0.9 | 1.0 | 0.5 | 0.9 | 1.0 | 1.4 | 1.1 | 1.8 | 0.5 | 0.9 | 0.8 | 1.2 | 0.8 | 1.3 |
|  |  |  |  | 4 | 0.5 | 0.8 | 0.6 | 0.6 | 0.8 | 0.9 | 0.5 | 0.9 | 0.9 | 1.2 | 1.0 | 1.6 | 0.4 | 0.8 | 0.7 | 1.0 | 0.7 | 1.1 |
|  | 96 | 4 | 4 | 1 | 0.4 | 0.7 | 0.6 | 0.7 | 0.5 | 0.9 | 0.4 | 0.8 | 0.7 | 1.0 | 0.6 | 1.0 | 0.3 | 0.7 | 0.6 | 1.0 | 0.5 | 1.0 |
|  |  |  |  | 2 | 0.3 | 0.5 | 0.5 | 0.5 | 0.4 | 0.6 | 0.3 | 0.5 | 0.6 | 0.8 | 0.4 | 0.7 | 0.3 | 0.5 | 0.5 | 0.7 | 0.4 | 0.7 |
|  |  |  |  | 3 | 0.2 | 0.4 | 0.4 | 0.4 | 0.3 | 0.5 | 0.2 | 0.5 | 0.6 | 0.7 | 0.4 | 0.6 | 0.2 | 0.4 | 0.4 | 0.6 | 0.3 | 0.6 |
|  |  |  |  | 4 | 0.2 | 0.3 | 0.4 | 0.4 | 0.3 | 0.4 | 0.2 | 0.4 | 0.5 | 0.7 | 0.3 | 0.6 | 0.2 | 0.4 | 0.4 | 0.5 | 0.3 | 0.5 |
|  |  | 4 | 2 | 1 | 0.6 | 1.1 | 0.5 | 0.9 | 0.6 | 1.1 | 0.6 | 1.2 | 1.0 | 1.4 | 0.9 | 1.5 | 0.6 | 1.1 | 0.9 | 1.4 | 0.9 | 1.5 |
|  |  |  |  | 2 | 0.4 | 0.8 | 0.4 | 0.7 | 0.4 | 0.8 | 0.5 | 0.9 | 0.9 | 1.2 | 0.7 | 1.2 | 0.4 | 0.8 | 0.7 | 1.0 | 0.6 | 1.1 |
|  |  |  |  | 3 | 0.4 | 0.6 | 0.3 | 0.5 | 0.4 | 0.6 | 0.4 | 0.7 | 0.8 | 1.0 | 0.7 | 1.1 | 0.3 | 0.6 | 0.6 | 0.8 | 0.5 | 0.9 |
|  |  |  |  | 4 | 0.3 | 0.6 | 0.3 | 0.5 | 0.3 | 0.6 | 0.3 | 0.6 | 0.7 | 0.9 | 0.6 | 1.0 | 0.3 | 0.6 | 0.5 | 0.7 | 0.4 | 0.7 |
| 60 | 24 | 4 | 4 | 1 | 0.7 | 1.4 | 0.9 | 1.6 | 1.3 | 1.6 | 0.8 | 1.5 | 1.2 | 2.3 | 1.2 | 2.0 | 0.7 | 1.4 | 1.2 | 2.2 | 1.2 | 2.0 |
|  |  |  |  | 2 | 0.5 | 1.0 | 0.7 | 1.1 | 1.1 | 1.2 | 0.6 | 1.1 | 1.0 | 1.9 | 1.0 | 1.6 | 0.5 | 1.0 | 0.8 | 1.6 | 0.9 | 1.4 |
|  |  |  |  | 3 | 0.5 | 0.8 | 0.6 | 0.9 | 1.0 | 0.9 | 0.5 | 0.9 | 0.9 | 1.7 | 0.9 | 1.4 | 0.4 | 0.8 | 0.7 | 1.3 | 0.8 | 1.2 |
|  |  |  |  | 4 | 0.4 | 0.7 | 0.5 | 0.8 | 0.9 | 0.8 | 0.4 | 0.8 | 0.9 | 1.6 | 0.8 | 1.3 | 0.4 | 0.7 | 0.6 | 1.2 | 0.7 | 1.0 |
|  |  | 4 | 2 | 1 | 1.2 | 2.3 | 1.5 | 1.7 | 1.7 | 2.2 | 1.3 | 2.4 | 1.6 | 2.6 | 1.6 | 3.1 | 1.1 | 2.3 | 1.6 | 2.7 | 1.5 | 3.0 |
|  |  |  |  | 2 | 0.8 | 1.6 | 1.3 | 1.2 | 1.4 | 1.5 | 0.9 | 1.8 | 1.4 | 2.1 | 1.3 | 2.5 | 0.8 | 1.6 | 1.3 | 2.0 | 1.1 | 2.2 |
|  |  |  |  | 3 | 0.7 | 1.3 | 1.2 | 1.0 | 1.3 | 1.2 | 0.7 | 1.5 | 1.3 | 2.0 | 1.2 | 2.2 | 0.7 | 1.3 | 1.1 | 1.6 | 0.9 | 1.8 |
|  |  |  |  | 4 | 0.6 | 1.1 | 1.1 | 0.8 | 1.2 | 1.1 | 0.6 | 1.3 | 1.3 | 1.8 | 1.1 | 2.0 | 0.6 | 1.1 | 1.0 | 1.4 | 0.8 | 1.5 |

Table 16 absolute and relative accuracy of the SL-PRS RSRP with Es/Iot (0,0,0)

| SL-PRS based RSRP [dB] (0, 0, 0) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  |  |  |  | TDL-A |  |  |  |  |  | TDL-B |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  | UE 1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel | abs | rel |
| 15 | 48 | 4 | 4 | 1 | 0.7 | 1.3 | 0.4 | 0.8 | 0.4 | 0.9 | 0.8 | 1.5 | 0.6 | 1.1 | 0.8 | 1.1 | 0.7 | 1.4 | 0.5 | 1.1 | 0.7 | 1.0 |
|  |  |  |  | 2 | 0.5 | 0.9 | 0.3 | 0.6 | 0.3 | 0.6 | 0.6 | 1.1 | 0.4 | 0.8 | 0.7 | 0.9 | 0.5 | 1.0 | 0.4 | 0.8 | 0.5 | 0.7 |
|  |  |  |  | 3 | 0.4 | 0.8 | 0.3 | 0.5 | 0.3 | 0.5 | 0.5 | 0.9 | 0.4 | 0.7 | 0.6 | 0.8 | 0.4 | 0.8 | 0.4 | 0.6 | 0.4 | 0.6 |
|  |  |  |  | 4 | 0.4 | 0.7 | 0.2 | 0.4 | 0.2 | 0.4 | 0.4 | 0.8 | 0.3 | 0.6 | 0.6 | 0.7 | 0.4 | 0.7 | 0.3 | 0.6 | 0.4 | 0.6 |
|  |  | 4 | 2 | 1 | 1.1 | 2.1 | 0.7 | 0.9 | 1.1 | 1.2 | 1.4 | 2.5 | 1.0 | 1.5 | 2.2 | 2.7 | 1.1 | 2.2 | 0.9 | 1.4 | 1.6 | 2.1 |
|  |  |  |  | 2 | 0.7 | 1.5 | 0.5 | 0.7 | 0.9 | 0.9 | 1.0 | 1.9 | 0.8 | 1.1 | 1.9 | 2.3 | 0.8 | 1.6 | 0.7 | 1.0 | 1.2 | 1.5 |
|  |  |  |  | 3 | 0.6 | 1.2 | 0.5 | 0.5 | 0.8 | 0.7 | 0.9 | 1.6 | 0.7 | 1.0 | 1.6 | 1.9 | 0.7 | 1.3 | 0.6 | 0.8 | 1.1 | 1.3 |
|  |  |  |  | 4 | 0.5 | 1.0 | 0.4 | 0.5 | 0.8 | 0.6 | 0.8 | 1.4 | 0.6 | 0.9 | 1.5 | 1.7 | 0.6 | 1.2 | 0.5 | 0.7 | 0.9 | 1.1 |
|  | 96 | 4 | 4 | 1 | 0.5 | 0.9 | 0.4 | 0.6 | 0.3 | 0.6 | 0.5 | 1.0 | 0.5 | 0.8 | 0.4 | 0.8 | 0.5 | 1.0 | 0.5 | 0.8 | 0.4 | 0.8 |
|  |  |  |  | 2 | 0.3 | 0.7 | 0.3 | 0.4 | 0.3 | 0.4 | 0.4 | 0.7 | 0.4 | 0.6 | 0.3 | 0.6 | 0.4 | 0.7 | 0.4 | 0.5 | 0.3 | 0.5 |
|  |  |  |  | 3 | 0.3 | 0.5 | 0.3 | 0.3 | 0.2 | 0.4 | 0.3 | 0.6 | 0.3 | 0.5 | 0.3 | 0.5 | 0.3 | 0.6 | 0.3 | 0.4 | 0.2 | 0.5 |
|  |  |  |  | 4 | 0.2 | 0.5 | 0.3 | 0.3 | 0.2 | 0.3 | 0.3 | 0.5 | 0.3 | 0.4 | 0.2 | 0.5 | 0.3 | 0.5 | 0.3 | 0.4 | 0.2 | 0.4 |
|  |  | 4 | 2 | 1 | 0.8 | 1.5 | 0.4 | 0.7 | 0.5 | 0.8 | 0.9 | 1.7 | 0.7 | 1.0 | 0.9 | 1.5 | 0.8 | 1.6 | 0.7 | 1.0 | 0.8 | 1.3 |
|  |  |  |  | 2 | 0.5 | 1.1 | 0.3 | 0.5 | 0.4 | 0.6 | 0.6 | 1.2 | 0.5 | 0.8 | 0.8 | 1.2 | 0.6 | 1.1 | 0.5 | 0.7 | 0.6 | 0.9 |
|  |  |  |  | 3 | 0.5 | 0.9 | 0.2 | 0.4 | 0.4 | 0.5 | 0.5 | 1.0 | 0.5 | 0.7 | 0.6 | 1.0 | 0.5 | 0.9 | 0.4 | 0.6 | 0.5 | 0.8 |
|  |  |  |  | 4 | 0.4 | 0.8 | 0.2 | 0.4 | 0.3 | 0.4 | 0.4 | 0.8 | 0.4 | 0.6 | 0.6 | 0.9 | 0.4 | 0.8 | 0.4 | 0.5 | 0.5 | 0.7 |
| 30 | 24 | 4 | 4 | 1 | 0.9 | 1.7 | 0.7 | 1.2 | 0.8 | 1.2 | 1.1 | 2.0 | 0.9 | 1.6 | 0.9 | 1.6 | 0.9 | 1.8 | 0.9 | 1.7 | 0.8 | 1.5 |
|  |  |  |  | 2 | 0.6 | 1.2 | 0.5 | 0.9 | 0.7 | 0.9 | 0.8 | 1.5 | 0.7 | 1.3 | 0.7 | 1.2 | 0.7 | 1.3 | 0.7 | 1.2 | 0.6 | 1.1 |
|  |  |  |  | 3 | 0.5 | 1.0 | 0.4 | 0.7 | 0.6 | 0.7 | 0.7 | 1.3 | 0.6 | 1.1 | 0.6 | 1.0 | 0.5 | 1.1 | 0.5 | 1.0 | 0.5 | 0.9 |
|  |  |  |  | 4 | 0.5 | 0.8 | 0.4 | 0.6 | 0.5 | 0.6 | 0.6 | 1.1 | 0.6 | 1.0 | 0.6 | 0.9 | 0.5 | 0.9 | 0.5 | 0.9 | 0.5 | 0.7 |
|  |  | 4 | 2 | 1 | 1.5 | 2.8 | 1.0 | 1.3 | 1.0 | 1.8 | 1.9 | 3.4 | 1.2 | 1.9 | 1.4 | 2.6 | 1.5 | 2.9 | 1.1 | 1.9 | 1.3 | 2.3 |
|  |  |  |  | 2 | 1.0 | 2.0 | 0.8 | 0.9 | 0.7 | 1.2 | 1.3 | 2.4 | 1.0 | 1.6 | 1.2 | 2.2 | 1.1 | 2.1 | 0.9 | 1.4 | 1.0 | 1.7 |
|  |  |  |  | 3 | 0.8 | 1.6 | 0.7 | 0.7 | 0.6 | 1.0 | 1.2 | 2.1 | 0.9 | 1.4 | 1.0 | 2.0 | 0.9 | 1.7 | 0.8 | 1.2 | 0.8 | 1.4 |
|  |  |  |  | 4 | 0.7 | 1.4 | 0.7 | 0.6 | 0.5 | 0.9 | 1.0 | 1.8 | 0.9 | 1.2 | 0.9 | 1.8 | 0.8 | 1.5 | 0.7 | 1.0 | 0.7 | 1.2 |
|  | 48 | 4 | 4 | 1 | 0.6 | 1.2 | 0.4 | 0.8 | 0.5 | 0.9 | 0.7 | 1.3 | 0.6 | 1.1 | 0.6 | 1.1 | 0.6 | 1.3 | 0.6 | 1.1 | 0.6 | 1.1 |
|  |  |  |  | 2 | 0.5 | 0.9 | 0.3 | 0.6 | 0.4 | 0.7 | 0.5 | 0.9 | 0.4 | 0.8 | 0.4 | 0.8 | 0.4 | 0.9 | 0.4 | 0.8 | 0.4 | 0.8 |
|  |  |  |  | 3 | 0.4 | 0.7 | 0.3 | 0.5 | 0.3 | 0.5 | 0.4 | 0.8 | 0.4 | 0.7 | 0.4 | 0.7 | 0.4 | 0.7 | 0.3 | 0.6 | 0.3 | 0.6 |
|  |  |  |  | 4 | 0.3 | 0.6 | 0.2 | 0.4 | 0.3 | 0.5 | 0.3 | 0.7 | 0.3 | 0.6 | 0.3 | 0.6 | 0.3 | 0.6 | 0.3 | 0.5 | 0.3 | 0.6 |
|  |  | 4 | 2 | 1 | 1.0 | 2.0 | 0.7 | 0.9 | 1.0 | 1.3 | 1.1 | 2.1 | 0.9 | 1.4 | 1.1 | 1.8 | 1.0 | 2.0 | 0.9 | 1.5 | 1.0 | 1.7 |
|  |  |  |  | 2 | 0.7 | 1.4 | 0.5 | 0.7 | 0.8 | 0.9 | 0.8 | 1.5 | 0.8 | 1.2 | 0.9 | 1.5 | 0.7 | 1.4 | 0.7 | 1.1 | 0.7 | 1.2 |
|  |  |  |  | 3 | 0.6 | 1.2 | 0.5 | 0.5 | 0.7 | 0.7 | 0.6 | 1.2 | 0.7 | 1.0 | 0.8 | 1.3 | 0.6 | 1.2 | 0.6 | 0.9 | 0.6 | 1.0 |
|  |  |  |  | 4 | 0.5 | 1.0 | 0.4 | 0.5 | 0.6 | 0.6 | 0.6 | 1.1 | 0.6 | 0.9 | 0.7 | 1.2 | 0.5 | 1.0 | 0.5 | 0.8 | 0.5 | 0.9 |
|  | 96 | 4 | 4 | 1 | 0.4 | 0.9 | 0.4 | 0.6 | 0.4 | 0.7 | 0.5 | 1.0 | 0.5 | 0.8 | 0.4 | 0.7 | 0.4 | 0.9 | 0.5 | 0.8 | 0.4 | 0.7 |
|  |  |  |  | 2 | 0.3 | 0.6 | 0.4 | 0.4 | 0.3 | 0.5 | 0.4 | 0.7 | 0.5 | 0.6 | 0.3 | 0.6 | 0.3 | 0.6 | 0.4 | 0.6 | 0.3 | 0.5 |
|  |  |  |  | 3 | 0.3 | 0.5 | 0.3 | 0.3 | 0.3 | 0.4 | 0.3 | 0.6 | 0.4 | 0.5 | 0.3 | 0.5 | 0.3 | 0.5 | 0.3 | 0.5 | 0.2 | 0.4 |
|  |  |  |  | 4 | 0.2 | 0.4 | 0.3 | 0.3 | 0.2 | 0.3 | 0.3 | 0.5 | 0.4 | 0.5 | 0.2 | 0.4 | 0.2 | 0.5 | 0.3 | 0.4 | 0.2 | 0.4 |
|  |  | 4 | 2 | 1 | 0.7 | 1.4 | 0.4 | 0.7 | 0.5 | 0.9 | 0.8 | 1.6 | 0.7 | 1.0 | 0.6 | 1.1 | 0.7 | 1.4 | 0.7 | 1.1 | 0.6 | 1.1 |
|  |  |  |  | 2 | 0.5 | 1.0 | 0.3 | 0.5 | 0.3 | 0.6 | 0.6 | 1.1 | 0.6 | 0.9 | 0.5 | 0.9 | 0.5 | 1.0 | 0.5 | 0.8 | 0.4 | 0.8 |
|  |  |  |  | 3 | 0.4 | 0.8 | 0.2 | 0.4 | 0.3 | 0.5 | 0.5 | 0.9 | 0.6 | 0.8 | 0.4 | 0.8 | 0.4 | 0.8 | 0.4 | 0.6 | 0.4 | 0.6 |
|  |  |  |  | 4 | 0.4 | 0.7 | 0.2 | 0.4 | 0.3 | 0.4 | 0.4 | 0.8 | 0.5 | 0.7 | 0.4 | 0.7 | 0.4 | 0.7 | 0.4 | 0.6 | 0.3 | 0.6 |
| 60 | 24 | 4 | 4 | 1 | 1.0 | 1.8 | 0.7 | 1.2 | 1.0 | 1.3 | 1.0 | 2.0 | 0.9 | 1.7 | 0.9 | 1.6 | 0.9 | 1.8 | 0.9 | 1.7 | 0.9 | 1.5 |
|  |  |  |  | 2 | 0.7 | 1.3 | 0.5 | 0.8 | 0.8 | 0.9 | 0.7 | 1.4 | 0.7 | 1.4 | 0.7 | 1.2 | 0.7 | 1.3 | 0.6 | 1.2 | 0.7 | 1.1 |
|  |  |  |  | 3 | 0.5 | 1.1 | 0.4 | 0.7 | 0.7 | 0.7 | 0.6 | 1.2 | 0.7 | 1.2 | 0.6 | 1.1 | 0.5 | 1.1 | 0.5 | 1.0 | 0.6 | 0.9 |
|  |  |  |  | 4 | 0.4 | 0.9 | 0.4 | 0.6 | 0.7 | 0.6 | 0.5 | 1.0 | 0.6 | 1.2 | 0.6 | 1.0 | 0.5 | 0.9 | 0.4 | 0.9 | 0.5 | 0.8 |
|  |  | 4 | 2 | 1 | 1.6 | 3.0 | 1.2 | 1.3 | 1.4 | 1.7 | 1.7 | 3.2 | 1.2 | 2.0 | 1.2 | 2.4 | 1.5 | 2.9 | 1.2 | 2.1 | 1.2 | 2.3 |
|  |  |  |  | 2 | 1.1 | 2.1 | 1.0 | 0.9 | 1.2 | 1.2 | 1.2 | 2.3 | 1.0 | 1.6 | 1.0 | 2.0 | 1.1 | 2.1 | 0.9 | 1.5 | 0.9 | 1.6 |
|  |  |  |  | 3 | 0.9 | 1.7 | 0.9 | 0.8 | 1.1 | 1.0 | 1.0 | 2.0 | 1.0 | 1.5 | 0.9 | 1.8 | 0.9 | 1.7 | 0.8 | 1.2 | 0.7 | 1.4 |
|  |  |  |  | 4 | 0.7 | 1.4 | 0.8 | 0.6 | 1.0 | 0.9 | 0.9 | 1.7 | 0.9 | 1.4 | 0.8 | 1.6 | 0.8 | 1.5 | 0.7 | 1.1 | 0.6 | 1.2 |

SL-PRS RSRPP simulation results

Table 17-20 shows the results for 5%-ile and 95%-ile of the SL-PRS RSRPP errors with Es/Iot (-3,-6,-6), (0,-6,-6), (0,-3,-3) and (0,0,0), respectively. For convenience, we provide the absolute and relative accuracy simulation results, Table 21-24 shows the results for the absolute and relative accuracy of the SL-PRS RSRPP based on absolute accuracy = max (abs (5%-ile error), abs (95%-ile error)) and relative accuracy = abs(95%-ile error-5%-ile error) with Es/Iot (-3,-6,-6), (0,-6,-6), (0,-3,-3) and (0,0,0), respectively.

Table 17 5%-ile and 95%-ile of the SL-PRS RSRPP errors with Es/Iot (-3,-6,-6)

| SL-PRS based RSRPP [dB] (-3, -6, -6) dB |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |
| (kHz) |  |  |  |  | Two-taps |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% |
| 15 | 48 | 4 | 4 | 1 | -0.2 | 0.3 | -1.4 | -0.7 | -1.4 | -0.7 |
|  |  |  |  | 2 | -0.2 | 0.2 | -1.3 | -0.8 | -1.3 | -0.8 |
|  |  |  |  | 3 | -0.1 | 0.2 | -1.3 | -0.9 | -1.2 | -0.9 |
|  |  |  |  | 4 | -0.1 | 0.1 | -1.2 | -0.9 | -1.2 | -0.9 |
|  |  | 4 | 2 | 1 | -0.4 | 0.4 | -1.5 | -0.6 | -2.1 | -1.1 |
|  |  |  |  | 2 | -0.3 | 0.3 | -1.4 | -0.7 | -1.9 | -1.2 |
|  |  |  |  | 3 | -0.2 | 0.2 | -1.3 | -0.8 | -1.9 | -1.2 |
|  |  |  |  | 4 | -0.2 | 0.2 | -1.3 | -0.8 | -1.8 | -1.3 |
|  | 96 | 4 | 4 | 1 | -0.2 | 0.2 | -1.3 | -0.8 | -1.3 | -0.8 |
|  |  |  |  | 2 | -0.1 | 0.1 | -1.3 | -0.9 | -1.2 | -0.9 |
|  |  |  |  | 3 | -0.1 | 0.1 | -1.2 | -0.9 | -1.2 | -0.9 |
|  |  |  |  | 4 | -0.1 | 0.1 | -1.2 | -0.9 | -1.1 | -0.9 |
|  |  | 4 | 2 | 1 | -0.3 | 0.3 | -1.5 | -0.7 | -1.8 | -1.1 |
|  |  |  |  | 2 | -0.2 | 0.2 | -1.4 | -0.8 | -1.6 | -1.2 |
|  |  |  |  | 3 | -0.1 | 0.2 | -1.3 | -0.9 | -1.6 | -1.2 |
|  |  |  |  | 4 | -0.1 | 0.2 | -1.3 | -0.9 | -1.6 | -1.2 |
| 30 | 24 | 4 | 4 | 1 | -0.3 | 0.4 | -1.5 | -0.6 | -1.5 | -0.6 |
|  |  |  |  | 2 | -0.2 | 0.3 | -1.4 | -0.7 | -1.3 | -0.7 |
|  |  |  |  | 3 | -0.2 | 0.2 | -1.3 | -0.8 | -1.3 | -0.8 |
|  |  |  |  | 4 | -0.1 | 0.2 | -1.3 | -0.8 | -1.3 | -0.8 |
|  |  | 4 | 2 | 1 | -0.5 | 0.6 | -1.6 | -0.5 | -1.9 | -0.7 |
|  |  |  |  | 2 | -0.3 | 0.4 | -1.4 | -0.6 | -1.7 | -0.9 |
|  |  |  |  | 3 | -0.3 | 0.3 | -1.4 | -0.7 | -1.6 | -1.0 |
|  |  |  |  | 4 | -0.2 | 0.3 | -1.3 | -0.7 | -1.6 | -1.0 |
|  | 48 | 4 | 4 | 1 | -0.2 | 0.3 | -1.4 | -0.7 | -1.5 | -0.9 |
|  |  |  |  | 2 | -0.2 | 0.2 | -1.3 | -0.8 | -1.4 | -1.0 |
|  |  |  |  | 3 | -0.1 | 0.2 | -1.3 | -0.9 | -1.4 | -1.0 |
|  |  |  |  | 4 | -0.1 | 0.1 | -1.2 | -0.9 | -1.3 | -1.0 |
|  |  | 4 | 2 | 1 | -0.4 | 0.4 | -1.5 | -0.6 | -1.7 | -0.8 |
|  |  |  |  | 2 | -0.2 | 0.3 | -1.4 | -0.7 | -1.6 | -1.0 |
|  |  |  |  | 3 | -0.2 | 0.2 | -1.3 | -0.8 | -1.5 | -1.0 |
|  |  |  |  | 4 | -0.2 | 0.2 | -1.3 | -0.8 | -1.5 | -1.0 |
|  | 96 | 4 | 4 | 1 | -0.2 | 0.2 | -1.3 | -0.8 | -1.3 | -0.9 |
|  |  |  |  | 2 | -0.1 | 0.1 | -1.3 | -0.9 | -1.3 | -1.0 |
|  |  |  |  | 3 | -0.1 | 0.1 | -1.2 | -0.9 | -1.2 | -1.0 |
|  |  |  |  | 4 | -0.1 | 0.1 | -1.2 | -1.0 | -1.2 | -1.0 |
|  |  | 4 | 2 | 1 | -0.3 | 0.3 | -1.5 | -0.7 | -1.4 | -0.8 |
|  |  |  |  | 2 | -0.2 | 0.2 | -1.3 | -0.8 | -1.3 | -0.9 |
|  |  |  |  | 3 | -0.1 | 0.2 | -1.3 | -0.8 | -1.3 | -1.0 |
|  |  |  |  | 4 | -0.1 | 0.2 | -1.3 | -0.9 | -1.3 | -1.0 |
| 60 | 24 | 4 | 4 | 1 | -0.3 | 0.4 | -1.5 | -0.6 | -1.5 | -0.6 |
|  |  |  |  | 2 | -0.2 | 0.3 | -1.4 | -0.7 | -1.4 | -0.7 |
|  |  |  |  | 3 | -0.2 | 0.2 | -1.3 | -0.8 | -1.3 | -0.8 |
|  |  |  |  | 4 | -0.1 | 0.2 | -1.3 | -0.8 | -1.3 | -0.8 |
|  |  | 4 | 2 | 1 | -0.5 | 0.6 | -1.6 | -0.5 | -2.1 | -0.8 |
|  |  |  |  | 2 | -0.3 | 0.4 | -1.5 | -0.6 | -1.9 | -1.0 |
|  |  |  |  | 3 | -0.3 | 0.4 | -1.4 | -0.7 | -1.8 | -1.0 |
|  |  |  |  | 4 | -0.2 | 0.3 | -1.4 | -0.8 | -1.8 | -1.1 |

Table 18 5%-ile and 95%-ile of the SL-PRS RSRPP errors with Es/Iot (0,-6,-6)

| SL-PRS based RSRPP [dB] (0, -6, -6) dB |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |
| (kHz) |  |  |  |  | Two-taps |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% |
| 15 | 48 | 4 | 4 | 1 | -0.2 | 0.2 | -2.3 | -1.6 | -2.2 | -1.6 |
|  |  |  |  | 2 | -0.1 | 0.1 | -2.2 | -1.7 | -2.1 | -1.7 |
|  |  |  |  | 3 | -0.1 | 0.1 | -2.1 | -1.7 | -2.1 | -1.7 |
|  |  |  |  | 4 | -0.1 | 0.1 | -2.1 | -1.8 | -2.1 | -1.8 |
|  |  | 4 | 2 | 1 | -0.3 | 0.3 | -2.4 | -1.5 | -3.1 | -2.1 |
|  |  |  |  | 2 | -0.2 | 0.2 | -2.3 | -1.6 | -3.0 | -2.2 |
|  |  |  |  | 3 | -0.2 | 0.2 | -2.2 | -1.7 | -2.9 | -2.2 |
|  |  |  |  | 4 | -0.1 | 0.1 | -2.2 | -1.7 | -2.9 | -2.3 |
|  | 96 | 4 | 4 | 1 | -0.1 | 0.1 | -2.2 | -1.7 | -2.1 | -1.7 |
|  |  |  |  | 2 | -0.1 | 0.1 | -2.1 | -1.8 | -2.1 | -1.7 |
|  |  |  |  | 3 | -0.1 | 0.1 | -2.1 | -1.8 | -2.0 | -1.8 |
|  |  |  |  | 4 | -0.1 | 0.1 | -2.1 | -1.8 | -2.0 | -1.8 |
|  |  | 4 | 2 | 1 | -0.2 | 0.2 | -2.4 | -1.6 | -2.7 | -2.0 |
|  |  |  |  | 2 | -0.1 | 0.1 | -2.2 | -1.7 | -2.6 | -2.1 |
|  |  |  |  | 3 | -0.1 | 0.1 | -2.2 | -1.7 | -2.6 | -2.2 |
|  |  |  |  | 4 | -0.1 | 0.1 | -2.2 | -1.7 | -2.5 | -2.2 |
| 30 | 24 | 4 | 4 | 1 | -0.2 | 0.3 | -2.3 | -1.4 | -2.3 | -1.5 |
|  |  |  |  | 2 | -0.2 | 0.2 | -2.2 | -1.6 | -2.2 | -1.6 |
|  |  |  |  | 3 | -0.1 | 0.2 | -2.2 | -1.6 | -2.2 | -1.7 |
|  |  |  |  | 4 | -0.1 | 0.1 | -2.1 | -1.7 | -2.1 | -1.7 |
|  |  | 4 | 2 | 1 | -0.4 | 0.4 | -2.4 | -1.4 | -2.8 | -1.7 |
|  |  |  |  | 2 | -0.2 | 0.3 | -2.3 | -1.5 | -2.7 | -1.8 |
|  |  |  |  | 3 | -0.2 | 0.2 | -2.2 | -1.6 | -2.6 | -1.9 |
|  |  |  |  | 4 | -0.2 | 0.2 | -2.2 | -1.6 | -2.5 | -1.9 |
|  | 48 | 4 | 4 | 1 | -0.2 | 0.2 | -2.3 | -1.6 | -2.4 | -1.8 |
|  |  |  |  | 2 | -0.1 | 0.1 | -2.2 | -1.7 | -2.3 | -1.9 |
|  |  |  |  | 3 | -0.1 | 0.1 | -2.1 | -1.7 | -2.3 | -1.9 |
|  |  |  |  | 4 | -0.1 | 0.1 | -2.1 | -1.8 | -2.2 | -1.9 |
|  |  | 4 | 2 | 1 | -0.3 | 0.3 | -2.4 | -1.5 | -2.6 | -1.8 |
|  |  |  |  | 2 | -0.2 | 0.2 | -2.3 | -1.6 | -2.5 | -1.9 |
|  |  |  |  | 3 | -0.1 | 0.2 | -2.2 | -1.6 | -2.5 | -1.9 |
|  |  |  |  | 4 | -0.1 | 0.2 | -2.2 | -1.7 | -2.4 | -2.0 |
|  | 96 | 4 | 4 | 1 | -0.1 | 0.1 | -2.2 | -1.7 | -2.2 | -1.8 |
|  |  |  |  | 2 | -0.1 | 0.1 | -2.1 | -1.8 | -2.1 | -1.8 |
|  |  |  |  | 3 | -0.1 | 0.1 | -2.1 | -1.8 | -2.1 | -1.9 |
|  |  |  |  | 4 | -0.1 | 0.1 | -2.1 | -1.8 | -2.1 | -1.9 |
|  |  | 4 | 2 | 1 | -0.2 | 0.2 | -2.3 | -1.5 | -2.3 | -1.7 |
|  |  |  |  | 2 | -0.1 | 0.2 | -2.2 | -1.6 | -2.2 | -1.8 |
|  |  |  |  | 3 | -0.1 | 0.1 | -2.2 | -1.7 | -2.2 | -1.9 |
|  |  |  |  | 4 | -0.1 | 0.1 | -2.1 | -1.7 | -2.1 | -1.9 |
| 60 | 24 | 4 | 4 | 1 | -0.2 | 0.3 | -2.3 | -1.5 | -2.4 | -1.5 |
|  |  |  |  | 2 | -0.2 | 0.2 | -2.2 | -1.6 | -2.2 | -1.6 |
|  |  |  |  | 3 | -0.1 | 0.2 | -2.2 | -1.7 | -2.2 | -1.7 |
|  |  |  |  | 4 | -0.1 | 0.1 | -2.1 | -1.7 | -2.2 | -1.7 |
|  |  | 4 | 2 | 1 | -0.4 | 0.4 | -2.4 | -1.4 | -3.0 | -1.8 |
|  |  |  |  | 2 | -0.2 | 0.3 | -2.3 | -1.5 | -2.8 | -2.0 |
|  |  |  |  | 3 | -0.2 | 0.2 | -2.2 | -1.6 | -2.8 | -2.0 |
|  |  |  |  | 4 | -0.2 | 0.2 | -2.2 | -1.6 | -2.7 | -2.1 |

Table 19 5%-ile and 95%-ile of the SL-PRS RSRPP errors with Es/Iot (0,-3,-3)

| SL-PRS based RSRPP [dB] (0, -3, -3) dB |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |
| (kHz) |  |  |  |  | Two-taps |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% |
| 15 | 48 | 4 | 4 | 1 | -0.2 | 0.2 | -1.1 | -0.6 | -1.1 | -0.6 |
|  |  |  |  | 2 | -0.1 | 0.2 | -1.0 | -0.7 | -1.0 | -0.7 |
|  |  |  |  | 3 | -0.1 | 0.1 | -1.0 | -0.7 | -1.0 | -0.7 |
|  |  |  |  | 4 | -0.1 | 0.1 | -1.0 | -0.7 | -0.9 | -0.7 |
|  |  | 4 | 2 | 1 | -0.3 | 0.4 | -1.2 | -0.5 | -1.7 | -1.0 |
|  |  |  |  | 2 | -0.2 | 0.3 | -1.1 | -0.6 | -1.6 | -1.0 |
|  |  |  |  | 3 | -0.2 | 0.2 | -1.1 | -0.6 | -1.5 | -1.1 |
|  |  |  |  | 4 | -0.2 | 0.2 | -1.1 | -0.7 | -1.5 | -1.1 |
|  | 96 | 4 | 4 | 1 | -0.1 | 0.2 | -1.1 | -0.6 | -1.0 | -0.7 |
|  |  |  |  | 2 | -0.1 | 0.1 | -1.0 | -0.7 | -0.9 | -0.7 |
|  |  |  |  | 3 | -0.1 | 0.1 | -1.0 | -0.7 | -0.9 | -0.7 |
|  |  |  |  | 4 | -0.1 | 0.1 | -1.0 | -0.7 | -0.9 | -0.7 |
|  |  | 4 | 2 | 1 | -0.2 | 0.2 | -1.2 | -0.6 | -1.4 | -0.9 |
|  |  |  |  | 2 | -0.2 | 0.2 | -1.1 | -0.6 | -1.4 | -1.0 |
|  |  |  |  | 3 | -0.1 | 0.1 | -1.1 | -0.7 | -1.3 | -1.0 |
|  |  |  |  | 4 | -0.1 | 0.1 | -1.0 | -0.7 | -1.3 | -1.0 |
| 30 | 24 | 4 | 4 | 1 | -0.3 | 0.3 | -1.2 | -0.5 | -1.1 | -0.5 |
|  |  |  |  | 2 | -0.2 | 0.2 | -1.1 | -0.6 | -1.1 | -0.6 |
|  |  |  |  | 3 | -0.1 | 0.2 | -1.0 | -0.6 | -1.0 | -0.6 |
|  |  |  |  | 4 | -0.1 | 0.2 | -1.0 | -0.6 | -1.0 | -0.7 |
|  |  | 4 | 2 | 1 | -0.4 | 0.5 | -1.2 | -0.4 | -1.5 | -0.7 |
|  |  |  |  | 2 | -0.3 | 0.4 | -1.1 | -0.5 | -1.4 | -0.8 |
|  |  |  |  | 3 | -0.2 | 0.3 | -1.1 | -0.6 | -1.3 | -0.8 |
|  |  |  |  | 4 | -0.2 | 0.3 | -1.0 | -0.6 | -1.3 | -0.9 |
|  | 48 | 4 | 4 | 1 | -0.2 | 0.2 | -1.1 | -0.6 | -1.2 | -0.7 |
|  |  |  |  | 2 | -0.1 | 0.2 | -1.0 | -0.7 | -1.1 | -0.8 |
|  |  |  |  | 3 | -0.1 | 0.1 | -1.0 | -0.7 | -1.1 | -0.8 |
|  |  |  |  | 4 | -0.1 | 0.1 | -1.0 | -0.7 | -1.1 | -0.8 |
|  |  | 4 | 2 | 1 | -0.3 | 0.4 | -1.2 | -0.5 | -1.4 | -0.7 |
|  |  |  |  | 2 | -0.2 | 0.3 | -1.1 | -0.6 | -1.3 | -0.8 |
|  |  |  |  | 3 | -0.2 | 0.2 | -1.1 | -0.6 | -1.2 | -0.9 |
|  |  |  |  | 4 | -0.1 | 0.2 | -1.0 | -0.7 | -1.2 | -0.9 |
|  | 96 | 4 | 4 | 1 | -0.1 | 0.2 | -1.0 | -0.7 | -1.0 | -0.7 |
|  |  |  |  | 2 | -0.1 | 0.1 | -1.0 | -0.7 | -1.0 | -0.8 |
|  |  |  |  | 3 | -0.1 | 0.1 | -1.0 | -0.8 | -1.0 | -0.8 |
|  |  |  |  | 4 | -0.1 | 0.1 | -1.0 | -0.8 | -1.0 | -0.8 |
|  |  | 4 | 2 | 1 | -0.2 | 0.2 | -1.2 | -0.5 | -1.1 | -0.7 |
|  |  |  |  | 2 | -0.2 | 0.2 | -1.1 | -0.6 | -1.0 | -0.8 |
|  |  |  |  | 3 | -0.1 | 0.1 | -1.0 | -0.7 | -1.0 | -0.8 |
|  |  |  |  | 4 | -0.1 | 0.1 | -1.0 | -0.7 | -1.0 | -0.8 |
| 60 | 24 | 4 | 4 | 1 | -0.3 | 0.3 | -1.1 | -0.5 | -1.2 | -0.5 |
|  |  |  |  | 2 | -0.2 | 0.2 | -1.1 | -0.6 | -1.1 | -0.6 |
|  |  |  |  | 3 | -0.1 | 0.2 | -1.0 | -0.6 | -1.0 | -0.7 |
|  |  |  |  | 4 | -0.1 | 0.2 | -1.0 | -0.7 | -1.0 | -0.7 |
|  |  | 4 | 2 | 1 | -0.4 | 0.5 | -1.2 | -0.4 | -1.7 | -0.8 |
|  |  |  |  | 2 | -0.3 | 0.4 | -1.1 | -0.6 | -1.5 | -0.9 |
|  |  |  |  | 3 | -0.2 | 0.3 | -1.1 | -0.6 | -1.5 | -0.9 |
|  |  |  |  | 4 | -0.2 | 0.3 | -1.1 | -0.6 | -1.4 | -1.0 |

Table 20 5%-ile and 95%-ile of the SL-PRS RSRPP errors with Es/Iot (0,0,0)

| SL-PRS based RSRPP [dB] (0, 0, 0) dB |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |
| (kHz) |  |  |  |  | Two-taps |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% |
| 15 | 48 | 4 | 4 | 1 | -0.2 | 0.3 | -0.2 | 0.2 | -0.1 | 0.2 |
|  |  |  |  | 2 | -0.2 | 0.2 | -0.1 | 0.2 | -0.1 | 0.2 |
|  |  |  |  | 3 | -0.1 | 0.2 | -0.1 | 0.1 | -0.1 | 0.1 |
|  |  |  |  | 4 | -0.1 | 0.1 | -0.1 | 0.1 | -0.1 | 0.1 |
|  |  | 4 | 2 | 1 | -0.4 | 0.4 | -0.3 | 0.3 | -0.6 | -0.1 |
|  |  |  |  | 2 | -0.3 | 0.3 | -0.2 | 0.2 | -0.5 | -0.1 |
|  |  |  |  | 3 | -0.2 | 0.3 | -0.2 | 0.2 | -0.5 | -0.2 |
|  |  |  |  | 4 | -0.2 | 0.2 | -0.2 | 0.2 | -0.5 | -0.2 |
|  | 96 | 4 | 4 | 1 | -0.2 | 0.2 | -0.1 | 0.2 | -0.1 | 0.2 |
|  |  |  |  | 2 | -0.1 | 0.1 | -0.1 | 0.1 | 0.0 | 0.1 |
|  |  |  |  | 3 | -0.1 | 0.1 | -0.1 | 0.1 | 0.0 | 0.1 |
|  |  |  |  | 4 | -0.1 | 0.1 | -0.1 | 0.1 | 0.0 | 0.1 |
|  |  | 4 | 2 | 1 | -0.3 | 0.3 | -0.2 | 0.3 | -0.4 | -0.1 |
|  |  |  |  | 2 | -0.2 | 0.2 | -0.2 | 0.2 | -0.4 | -0.1 |
|  |  |  |  | 3 | -0.2 | 0.2 | -0.2 | 0.2 | -0.4 | -0.1 |
|  |  |  |  | 4 | -0.1 | 0.2 | -0.1 | 0.1 | -0.3 | -0.1 |
| 30 | 24 | 4 | 4 | 1 | -0.3 | 0.4 | -0.2 | 0.3 | -0.2 | 0.3 |
|  |  |  |  | 2 | -0.2 | 0.3 | -0.1 | 0.3 | -0.1 | 0.2 |
|  |  |  |  | 3 | -0.2 | 0.2 | -0.1 | 0.2 | -0.1 | 0.2 |
|  |  |  |  | 4 | -0.1 | 0.2 | -0.1 | 0.2 | -0.1 | 0.2 |
|  |  | 4 | 2 | 1 | -0.5 | 0.6 | -0.3 | 0.3 | -0.5 | 0.2 |
|  |  |  |  | 2 | -0.3 | 0.4 | -0.2 | 0.3 | -0.4 | 0.1 |
|  |  |  |  | 3 | -0.3 | 0.4 | -0.2 | 0.2 | -0.4 | 0.0 |
|  |  |  |  | 4 | -0.2 | 0.3 | -0.1 | 0.2 | -0.3 | 0.0 |
|  | 48 | 4 | 4 | 1 | -0.2 | 0.3 | -0.2 | 0.2 | -0.2 | 0.1 |
|  |  |  |  | 2 | -0.2 | 0.2 | -0.1 | 0.2 | -0.2 | 0.1 |
|  |  |  |  | 3 | -0.1 | 0.2 | -0.1 | 0.1 | -0.2 | 0.1 |
|  |  |  |  | 4 | -0.1 | 0.1 | -0.1 | 0.1 | -0.1 | 0.0 |
|  |  | 4 | 2 | 1 | -0.4 | 0.4 | -0.3 | 0.3 | -0.4 | 0.1 |
|  |  |  |  | 2 | -0.2 | 0.3 | -0.2 | 0.2 | -0.3 | 0.0 |
|  |  |  |  | 3 | -0.2 | 0.3 | -0.2 | 0.2 | -0.3 | 0.0 |
|  |  |  |  | 4 | -0.2 | 0.2 | -0.1 | 0.2 | -0.3 | 0.0 |
|  | 96 | 4 | 4 | 1 | -0.2 | 0.2 | -0.1 | 0.2 | -0.1 | 0.1 |
|  |  |  |  | 2 | -0.1 | 0.1 | -0.1 | 0.1 | -0.1 | 0.1 |
|  |  |  |  | 3 | -0.1 | 0.1 | -0.1 | 0.1 | -0.1 | 0.1 |
|  |  |  |  | 4 | -0.1 | 0.1 | -0.1 | 0.1 | -0.1 | 0.1 |
|  |  | 4 | 2 | 1 | -0.3 | 0.3 | -0.3 | 0.3 | -0.2 | 0.1 |
|  |  |  |  | 2 | -0.2 | 0.2 | -0.2 | 0.2 | -0.1 | 0.1 |
|  |  |  |  | 3 | -0.2 | 0.2 | -0.1 | 0.2 | -0.1 | 0.1 |
|  |  |  |  | 4 | -0.1 | 0.2 | -0.1 | 0.2 | -0.1 | 0.1 |
| 60 | 24 | 4 | 4 | 1 | -0.3 | 0.4 | -0.2 | 0.3 | -0.2 | 0.3 |
|  |  |  |  | 2 | -0.2 | 0.3 | -0.2 | 0.2 | -0.2 | 0.2 |
|  |  |  |  | 3 | -0.2 | 0.2 | -0.1 | 0.2 | -0.1 | 0.2 |
|  |  |  |  | 4 | -0.1 | 0.2 | -0.1 | 0.2 | -0.1 | 0.2 |
|  |  | 4 | 2 | 1 | -0.5 | 0.6 | -0.3 | 0.4 | -0.7 | 0.1 |
|  |  |  |  | 2 | -0.3 | 0.4 | -0.2 | 0.3 | -0.5 | 0.0 |
|  |  |  |  | 3 | -0.3 | 0.4 | -0.2 | 0.2 | -0.5 | 0.0 |
|  |  |  |  | 4 | -0.2 | 0.3 | -0.2 | 0.2 | -0.5 | -0.1 |

Table 21 absolute and relative accuracy of the SL-PRS RSRPP with Es/Iot (-3,-6,-6)

| SL-PRS based RSRPP [dB] (-3, -6, -6) dB |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |
| (kHz) |  |  |  |  | Two-taps |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | abs | rel | abs | rel | abs | rel |
| 15 | 48 | 4 | 4 | 1 | 0.3 | 0.5 | 1.4 | 0.7 | 1.4 | 0.6 |
|  |  |  |  | 2 | 0.2 | 0.4 | 1.3 | 0.5 | 1.3 | 0.4 |
|  |  |  |  | 3 | 0.2 | 0.3 | 1.3 | 0.4 | 1.2 | 0.4 |
|  |  |  |  | 4 | 0.1 | 0.3 | 1.2 | 0.3 | 1.2 | 0.3 |
|  |  | 4 | 2 | 1 | 0.4 | 0.8 | 1.5 | 0.9 | 2.1 | 1.0 |
|  |  |  |  | 2 | 0.3 | 0.5 | 1.4 | 0.7 | 1.9 | 0.7 |
|  |  |  |  | 3 | 0.2 | 0.4 | 1.3 | 0.6 | 1.9 | 0.6 |
|  |  |  |  | 4 | 0.2 | 0.4 | 1.3 | 0.5 | 1.8 | 0.5 |
|  | 96 | 4 | 4 | 1 | 0.2 | 0.4 | 1.3 | 0.5 | 1.3 | 0.4 |
|  |  |  |  | 2 | 0.1 | 0.3 | 1.3 | 0.4 | 1.2 | 0.3 |
|  |  |  |  | 3 | 0.1 | 0.2 | 1.2 | 0.3 | 1.2 | 0.3 |
|  |  |  |  | 4 | 0.1 | 0.2 | 1.2 | 0.3 | 1.1 | 0.2 |
|  |  | 4 | 2 | 1 | 0.3 | 0.5 | 1.5 | 0.8 | 1.8 | 0.7 |
|  |  |  |  | 2 | 0.2 | 0.4 | 1.4 | 0.6 | 1.6 | 0.5 |
|  |  |  |  | 3 | 0.2 | 0.3 | 1.3 | 0.5 | 1.6 | 0.4 |
|  |  |  |  | 4 | 0.2 | 0.3 | 1.3 | 0.4 | 1.6 | 0.4 |
| 30 | 24 | 4 | 4 | 1 | 0.4 | 0.7 | 1.5 | 0.9 | 1.5 | 0.9 |
|  |  |  |  | 2 | 0.3 | 0.5 | 1.4 | 0.7 | 1.3 | 0.6 |
|  |  |  |  | 3 | 0.2 | 0.4 | 1.3 | 0.6 | 1.3 | 0.5 |
|  |  |  |  | 4 | 0.2 | 0.4 | 1.3 | 0.5 | 1.3 | 0.5 |
|  |  | 4 | 2 | 1 | 0.6 | 1.1 | 1.6 | 1.1 | 1.9 | 1.2 |
|  |  |  |  | 2 | 0.4 | 0.8 | 1.4 | 0.8 | 1.7 | 0.9 |
|  |  |  |  | 3 | 0.3 | 0.6 | 1.4 | 0.7 | 1.6 | 0.7 |
|  |  |  |  | 4 | 0.3 | 0.5 | 1.3 | 0.6 | 1.6 | 0.6 |
|  | 48 | 4 | 4 | 1 | 0.3 | 0.5 | 1.4 | 0.7 | 1.5 | 0.6 |
|  |  |  |  | 2 | 0.2 | 0.4 | 1.3 | 0.5 | 1.4 | 0.4 |
|  |  |  |  | 3 | 0.2 | 0.3 | 1.3 | 0.4 | 1.4 | 0.4 |
|  |  |  |  | 4 | 0.1 | 0.3 | 1.2 | 0.3 | 1.3 | 0.3 |
|  |  | 4 | 2 | 1 | 0.4 | 0.8 | 1.5 | 0.9 | 1.7 | 0.9 |
|  |  |  |  | 2 | 0.3 | 0.5 | 1.4 | 0.6 | 1.6 | 0.6 |
|  |  |  |  | 3 | 0.2 | 0.4 | 1.3 | 0.5 | 1.5 | 0.5 |
|  |  |  |  | 4 | 0.2 | 0.4 | 1.3 | 0.5 | 1.5 | 0.5 |
|  | 96 | 4 | 4 | 1 | 0.2 | 0.4 | 1.3 | 0.5 | 1.3 | 0.5 |
|  |  |  |  | 2 | 0.1 | 0.3 | 1.3 | 0.3 | 1.3 | 0.3 |
|  |  |  |  | 3 | 0.1 | 0.2 | 1.2 | 0.3 | 1.2 | 0.3 |
|  |  |  |  | 4 | 0.1 | 0.2 | 1.2 | 0.2 | 1.2 | 0.2 |
|  |  | 4 | 2 | 1 | 0.3 | 0.5 | 1.5 | 0.8 | 1.4 | 0.6 |
|  |  |  |  | 2 | 0.2 | 0.4 | 1.3 | 0.5 | 1.3 | 0.4 |
|  |  |  |  | 3 | 0.2 | 0.3 | 1.3 | 0.5 | 1.3 | 0.3 |
|  |  |  |  | 4 | 0.2 | 0.3 | 1.3 | 0.4 | 1.3 | 0.3 |
| 60 | 24 | 4 | 4 | 1 | 0.4 | 0.7 | 1.5 | 0.9 | 1.5 | 0.9 |
|  |  |  |  | 2 | 0.3 | 0.5 | 1.4 | 0.6 | 1.4 | 0.6 |
|  |  |  |  | 3 | 0.2 | 0.4 | 1.3 | 0.5 | 1.3 | 0.5 |
|  |  |  |  | 4 | 0.2 | 0.4 | 1.3 | 0.4 | 1.3 | 0.5 |
|  |  | 4 | 2 | 1 | 0.6 | 1.1 | 1.6 | 1.2 | 2.1 | 1.3 |
|  |  |  |  | 2 | 0.4 | 0.7 | 1.5 | 0.8 | 1.9 | 0.9 |
|  |  |  |  | 3 | 0.4 | 0.6 | 1.4 | 0.7 | 1.8 | 0.8 |
|  |  |  |  | 4 | 0.3 | 0.5 | 1.4 | 0.6 | 1.8 | 0.7 |

Table 22 absolute and relative accuracy of the SL-PRS RSRPP with Es/Iot (0,-6,-6)

| SL-PRS based RSRPP [dB] (0, -6, -6) dB |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |
| (kHz) |  |  |  |  | Two-taps |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | abs | rel | abs | rel | abs | rel |
| 15 | 48 | 4 | 4 | 1 | 0.2 | 0.4 | 2.3 | 0.7 | 2.2 | 0.6 |
|  |  |  |  | 2 | 0.1 | 0.3 | 2.2 | 0.5 | 2.1 | 0.4 |
|  |  |  |  | 3 | 0.1 | 0.2 | 2.1 | 0.4 | 2.1 | 0.4 |
|  |  |  |  | 4 | 0.1 | 0.2 | 2.1 | 0.3 | 2.1 | 0.3 |
|  |  | 4 | 2 | 1 | 0.3 | 0.6 | 2.4 | 1.0 | 3.1 | 1.1 |
|  |  |  |  | 2 | 0.2 | 0.4 | 2.3 | 0.7 | 3.0 | 0.8 |
|  |  |  |  | 3 | 0.2 | 0.3 | 2.2 | 0.6 | 2.9 | 0.7 |
|  |  |  |  | 4 | 0.1 | 0.3 | 2.2 | 0.5 | 2.9 | 0.6 |
|  | 96 | 4 | 4 | 1 | 0.1 | 0.3 | 2.2 | 0.5 | 2.1 | 0.4 |
|  |  |  |  | 2 | 0.1 | 0.2 | 2.1 | 0.4 | 2.1 | 0.3 |
|  |  |  |  | 3 | 0.1 | 0.1 | 2.1 | 0.3 | 2.0 | 0.3 |
|  |  |  |  | 4 | 0.1 | 0.1 | 2.1 | 0.3 | 2.0 | 0.2 |
|  |  | 4 | 2 | 1 | 0.2 | 0.4 | 2.4 | 0.8 | 2.7 | 0.7 |
|  |  |  |  | 2 | 0.1 | 0.3 | 2.2 | 0.6 | 2.6 | 0.5 |
|  |  |  |  | 3 | 0.1 | 0.2 | 2.2 | 0.5 | 2.6 | 0.4 |
|  |  |  |  | 4 | 0.1 | 0.2 | 2.2 | 0.4 | 2.5 | 0.3 |
| 30 | 24 | 4 | 4 | 1 | 0.3 | 0.5 | 2.3 | 0.9 | 2.3 | 0.9 |
|  |  |  |  | 2 | 0.2 | 0.3 | 2.2 | 0.7 | 2.2 | 0.6 |
|  |  |  |  | 3 | 0.2 | 0.3 | 2.2 | 0.6 | 2.2 | 0.5 |
|  |  |  |  | 4 | 0.1 | 0.2 | 2.1 | 0.5 | 2.1 | 0.4 |
|  |  | 4 | 2 | 1 | 0.4 | 0.8 | 2.4 | 1.0 | 2.8 | 1.1 |
|  |  |  |  | 2 | 0.3 | 0.5 | 2.3 | 0.7 | 2.7 | 0.8 |
|  |  |  |  | 3 | 0.2 | 0.4 | 2.2 | 0.6 | 2.6 | 0.7 |
|  |  |  |  | 4 | 0.2 | 0.4 | 2.2 | 0.5 | 2.5 | 0.6 |
|  | 48 | 4 | 4 | 1 | 0.2 | 0.4 | 2.3 | 0.7 | 2.4 | 0.6 |
|  |  |  |  | 2 | 0.1 | 0.3 | 2.2 | 0.5 | 2.3 | 0.4 |
|  |  |  |  | 3 | 0.1 | 0.2 | 2.1 | 0.4 | 2.3 | 0.4 |
|  |  |  |  | 4 | 0.1 | 0.2 | 2.1 | 0.3 | 2.2 | 0.3 |
|  |  | 4 | 2 | 1 | 0.3 | 0.6 | 2.4 | 0.9 | 2.6 | 0.8 |
|  |  |  |  | 2 | 0.2 | 0.4 | 2.3 | 0.7 | 2.5 | 0.6 |
|  |  |  |  | 3 | 0.2 | 0.3 | 2.2 | 0.5 | 2.5 | 0.5 |
|  |  |  |  | 4 | 0.2 | 0.3 | 2.2 | 0.5 | 2.4 | 0.4 |
|  | 96 | 4 | 4 | 1 | 0.1 | 0.3 | 2.2 | 0.5 | 2.2 | 0.4 |
|  |  |  |  | 2 | 0.1 | 0.2 | 2.1 | 0.3 | 2.1 | 0.3 |
|  |  |  |  | 3 | 0.1 | 0.1 | 2.1 | 0.3 | 2.1 | 0.3 |
|  |  |  |  | 4 | 0.1 | 0.1 | 2.1 | 0.2 | 2.1 | 0.2 |
|  |  | 4 | 2 | 1 | 0.2 | 0.4 | 2.3 | 0.8 | 2.3 | 0.5 |
|  |  |  |  | 2 | 0.2 | 0.3 | 2.2 | 0.6 | 2.2 | 0.4 |
|  |  |  |  | 3 | 0.1 | 0.2 | 2.2 | 0.5 | 2.2 | 0.3 |
|  |  |  |  | 4 | 0.1 | 0.2 | 2.1 | 0.4 | 2.1 | 0.3 |
| 60 | 24 | 4 | 4 | 1 | 0.3 | 0.5 | 2.3 | 0.8 | 2.4 | 0.9 |
|  |  |  |  | 2 | 0.2 | 0.4 | 2.2 | 0.6 | 2.2 | 0.6 |
|  |  |  |  | 3 | 0.2 | 0.3 | 2.2 | 0.5 | 2.2 | 0.5 |
|  |  |  |  | 4 | 0.1 | 0.2 | 2.1 | 0.4 | 2.2 | 0.4 |
|  |  | 4 | 2 | 1 | 0.4 | 0.8 | 2.4 | 1.0 | 3.0 | 1.2 |
|  |  |  |  | 2 | 0.3 | 0.5 | 2.3 | 0.8 | 2.8 | 0.9 |
|  |  |  |  | 3 | 0.2 | 0.4 | 2.2 | 0.6 | 2.8 | 0.7 |
|  |  |  |  | 4 | 0.2 | 0.4 | 2.2 | 0.6 | 2.7 | 0.7 |

Table 23 absolute and relative accuracy of the SL-PRS RSRPP with Es/Iot (0,-3,-3)

| SL-PRS based RSRPP [dB] (0, -3, -3) dB |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |
| (kHz) |  |  |  |  | Two-taps |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | abs | rel | abs | rel | abs | rel |
| 15 | 48 | 4 | 4 | 1 | 0.2 | 0.4 | 1.1 | 0.5 | 1.1 | 0.5 |
|  |  |  |  | 2 | 0.2 | 0.3 | 1.0 | 0.4 | 1.0 | 0.3 |
|  |  |  |  | 3 | 0.1 | 0.3 | 1.0 | 0.3 | 1.0 | 0.3 |
|  |  |  |  | 4 | 0.1 | 0.2 | 1.0 | 0.3 | 0.9 | 0.2 |
|  |  | 4 | 2 | 1 | 0.4 | 0.7 | 1.2 | 0.7 | 1.7 | 0.7 |
|  |  |  |  | 2 | 0.3 | 0.5 | 1.1 | 0.5 | 1.6 | 0.5 |
|  |  |  |  | 3 | 0.2 | 0.4 | 1.1 | 0.4 | 1.5 | 0.4 |
|  |  |  |  | 4 | 0.2 | 0.3 | 1.1 | 0.4 | 1.5 | 0.4 |
|  | 96 | 4 | 4 | 1 | 0.2 | 0.3 | 1.1 | 0.4 | 1.0 | 0.3 |
|  |  |  |  | 2 | 0.1 | 0.2 | 1.0 | 0.3 | 0.9 | 0.2 |
|  |  |  |  | 3 | 0.1 | 0.2 | 1.0 | 0.2 | 0.9 | 0.2 |
|  |  |  |  | 4 | 0.1 | 0.2 | 1.0 | 0.2 | 0.9 | 0.2 |
|  |  | 4 | 2 | 1 | 0.2 | 0.5 | 1.2 | 0.6 | 1.4 | 0.5 |
|  |  |  |  | 2 | 0.2 | 0.3 | 1.1 | 0.5 | 1.4 | 0.3 |
|  |  |  |  | 3 | 0.1 | 0.3 | 1.1 | 0.4 | 1.3 | 0.3 |
|  |  |  |  | 4 | 0.1 | 0.2 | 1.0 | 0.3 | 1.3 | 0.3 |
| 30 | 24 | 4 | 4 | 1 | 0.3 | 0.6 | 1.2 | 0.7 | 1.1 | 0.6 |
|  |  |  |  | 2 | 0.2 | 0.4 | 1.1 | 0.5 | 1.1 | 0.5 |
|  |  |  |  | 3 | 0.2 | 0.3 | 1.0 | 0.4 | 1.0 | 0.4 |
|  |  |  |  | 4 | 0.2 | 0.3 | 1.0 | 0.4 | 1.0 | 0.3 |
|  |  | 4 | 2 | 1 | 0.5 | 0.9 | 1.2 | 0.8 | 1.5 | 0.8 |
|  |  |  |  | 2 | 0.4 | 0.7 | 1.1 | 0.6 | 1.4 | 0.6 |
|  |  |  |  | 3 | 0.3 | 0.5 | 1.1 | 0.5 | 1.3 | 0.5 |
|  |  |  |  | 4 | 0.3 | 0.5 | 1.0 | 0.4 | 1.3 | 0.4 |
|  | 48 | 4 | 4 | 1 | 0.2 | 0.4 | 1.1 | 0.5 | 1.2 | 0.5 |
|  |  |  |  | 2 | 0.2 | 0.3 | 1.0 | 0.4 | 1.1 | 0.3 |
|  |  |  |  | 3 | 0.1 | 0.2 | 1.0 | 0.3 | 1.1 | 0.3 |
|  |  |  |  | 4 | 0.1 | 0.2 | 1.0 | 0.3 | 1.1 | 0.2 |
|  |  | 4 | 2 | 1 | 0.4 | 0.7 | 1.2 | 0.7 | 1.4 | 0.6 |
|  |  |  |  | 2 | 0.3 | 0.5 | 1.1 | 0.5 | 1.3 | 0.5 |
|  |  |  |  | 3 | 0.2 | 0.4 | 1.1 | 0.4 | 1.2 | 0.4 |
|  |  |  |  | 4 | 0.2 | 0.3 | 1.0 | 0.4 | 1.2 | 0.3 |
|  | 96 | 4 | 4 | 1 | 0.2 | 0.3 | 1.0 | 0.4 | 1.0 | 0.3 |
|  |  |  |  | 2 | 0.1 | 0.2 | 1.0 | 0.3 | 1.0 | 0.2 |
|  |  |  |  | 3 | 0.1 | 0.2 | 1.0 | 0.2 | 1.0 | 0.2 |
|  |  |  |  | 4 | 0.1 | 0.1 | 1.0 | 0.2 | 1.0 | 0.2 |
|  |  | 4 | 2 | 1 | 0.2 | 0.5 | 1.2 | 0.6 | 1.1 | 0.4 |
|  |  |  |  | 2 | 0.2 | 0.3 | 1.1 | 0.5 | 1.0 | 0.3 |
|  |  |  |  | 3 | 0.1 | 0.3 | 1.0 | 0.4 | 1.0 | 0.2 |
|  |  |  |  | 4 | 0.1 | 0.2 | 1.0 | 0.3 | 1.0 | 0.2 |
| 60 | 24 | 4 | 4 | 1 | 0.3 | 0.6 | 1.1 | 0.6 | 1.2 | 0.7 |
|  |  |  |  | 2 | 0.2 | 0.4 | 1.1 | 0.5 | 1.1 | 0.5 |
|  |  |  |  | 3 | 0.2 | 0.3 | 1.0 | 0.4 | 1.0 | 0.4 |
|  |  |  |  | 4 | 0.2 | 0.3 | 1.0 | 0.3 | 1.0 | 0.3 |
|  |  | 4 | 2 | 1 | 0.5 | 0.9 | 1.2 | 0.8 | 1.7 | 0.9 |
|  |  |  |  | 2 | 0.4 | 0.6 | 1.1 | 0.6 | 1.5 | 0.7 |
|  |  |  |  | 3 | 0.3 | 0.5 | 1.1 | 0.5 | 1.5 | 0.6 |
|  |  |  |  | 4 | 0.3 | 0.5 | 1.1 | 0.4 | 1.4 | 0.5 |

Table 24 absolute and relative accuracy of the SL-PRS RSRPP with Es/Iot (0,0,0)

| SL-PRS based RSRPP [dB] (0, 0, 0) dB |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |
| (kHz) |  |  |  |  | Two-taps |  |  |  |  |  |
|  |  |  |  |  | UE1 |  | UE 2 |  | UE 3 |  |
|  |  |  |  |  | abs | rel | abs | rel | abs | rel |
| 15 | 48 | 4 | 4 | 1 | 0.3 | 0.5 | 0.2 | 0.4 | 0.2 | 0.4 |
|  |  |  |  | 2 | 0.2 | 0.4 | 0.2 | 0.3 | 0.2 | 0.3 |
|  |  |  |  | 3 | 0.2 | 0.3 | 0.1 | 0.2 | 0.1 | 0.2 |
|  |  |  |  | 4 | 0.1 | 0.3 | 0.1 | 0.2 | 0.1 | 0.2 |
|  |  | 4 | 2 | 1 | 0.4 | 0.8 | 0.3 | 0.6 | 0.6 | 0.6 |
|  |  |  |  | 2 | 0.3 | 0.6 | 0.2 | 0.4 | 0.5 | 0.4 |
|  |  |  |  | 3 | 0.3 | 0.5 | 0.2 | 0.4 | 0.5 | 0.3 |
|  |  |  |  | 4 | 0.2 | 0.4 | 0.2 | 0.3 | 0.5 | 0.3 |
|  | 96 | 4 | 4 | 1 | 0.2 | 0.4 | 0.2 | 0.3 | 0.2 | 0.3 |
|  |  |  |  | 2 | 0.1 | 0.3 | 0.1 | 0.2 | 0.1 | 0.2 |
|  |  |  |  | 3 | 0.1 | 0.2 | 0.1 | 0.2 | 0.1 | 0.2 |
|  |  |  |  | 4 | 0.1 | 0.2 | 0.1 | 0.2 | 0.1 | 0.1 |
|  |  | 4 | 2 | 1 | 0.3 | 0.6 | 0.3 | 0.5 | 0.4 | 0.4 |
|  |  |  |  | 2 | 0.2 | 0.4 | 0.2 | 0.4 | 0.4 | 0.3 |
|  |  |  |  | 3 | 0.2 | 0.3 | 0.2 | 0.3 | 0.4 | 0.2 |
|  |  |  |  | 4 | 0.2 | 0.3 | 0.1 | 0.3 | 0.3 | 0.2 |
| 30 | 24 | 4 | 4 | 1 | 0.4 | 0.7 | 0.3 | 0.6 | 0.3 | 0.5 |
|  |  |  |  | 2 | 0.3 | 0.5 | 0.3 | 0.4 | 0.2 | 0.4 |
|  |  |  |  | 3 | 0.2 | 0.4 | 0.2 | 0.3 | 0.2 | 0.3 |
|  |  |  |  | 4 | 0.2 | 0.3 | 0.2 | 0.3 | 0.2 | 0.3 |
|  |  | 4 | 2 | 1 | 0.6 | 1.1 | 0.3 | 0.6 | 0.5 | 0.7 |
|  |  |  |  | 2 | 0.4 | 0.8 | 0.3 | 0.5 | 0.4 | 0.5 |
|  |  |  |  | 3 | 0.4 | 0.6 | 0.2 | 0.4 | 0.4 | 0.4 |
|  |  |  |  | 4 | 0.3 | 0.5 | 0.2 | 0.3 | 0.3 | 0.3 |
|  | 48 | 4 | 4 | 1 | 0.3 | 0.5 | 0.2 | 0.4 | 0.2 | 0.4 |
|  |  |  |  | 2 | 0.2 | 0.4 | 0.2 | 0.3 | 0.2 | 0.3 |
|  |  |  |  | 3 | 0.2 | 0.3 | 0.1 | 0.3 | 0.2 | 0.2 |
|  |  |  |  | 4 | 0.1 | 0.3 | 0.1 | 0.2 | 0.1 | 0.2 |
|  |  | 4 | 2 | 1 | 0.4 | 0.8 | 0.3 | 0.6 | 0.4 | 0.5 |
|  |  |  |  | 2 | 0.3 | 0.6 | 0.2 | 0.4 | 0.3 | 0.4 |
|  |  |  |  | 3 | 0.3 | 0.5 | 0.2 | 0.4 | 0.3 | 0.3 |
|  |  |  |  | 4 | 0.2 | 0.4 | 0.2 | 0.3 | 0.3 | 0.3 |
|  | 96 | 4 | 4 | 1 | 0.2 | 0.3 | 0.2 | 0.3 | 0.1 | 0.3 |
|  |  |  |  | 2 | 0.1 | 0.2 | 0.1 | 0.2 | 0.1 | 0.2 |
|  |  |  |  | 3 | 0.1 | 0.2 | 0.1 | 0.2 | 0.1 | 0.1 |
|  |  |  |  | 4 | 0.1 | 0.2 | 0.1 | 0.2 | 0.1 | 0.1 |
|  |  | 4 | 2 | 1 | 0.3 | 0.6 | 0.3 | 0.5 | 0.2 | 0.3 |
|  |  |  |  | 2 | 0.2 | 0.4 | 0.2 | 0.4 | 0.1 | 0.2 |
|  |  |  |  | 3 | 0.2 | 0.3 | 0.2 | 0.3 | 0.1 | 0.2 |
|  |  |  |  | 4 | 0.2 | 0.3 | 0.2 | 0.3 | 0.1 | 0.2 |
| 60 | 24 | 4 | 4 | 1 | 0.4 | 0.7 | 0.3 | 0.5 | 0.3 | 0.5 |
|  |  |  |  | 2 | 0.3 | 0.5 | 0.2 | 0.4 | 0.2 | 0.4 |
|  |  |  |  | 3 | 0.2 | 0.4 | 0.2 | 0.3 | 0.2 | 0.3 |
|  |  |  |  | 4 | 0.2 | 0.3 | 0.2 | 0.3 | 0.2 | 0.3 |
|  |  | 4 | 2 | 1 | 0.6 | 1.1 | 0.4 | 0.7 | 0.7 | 0.7 |
|  |  |  |  | 2 | 0.4 | 0.8 | 0.3 | 0.5 | 0.5 | 0.5 |
|  |  |  |  | 3 | 0.4 | 0.6 | 0.2 | 0.4 | 0.5 | 0.5 |
|  |  |  |  | 4 | 0.3 | 0.5 | 0.2 | 0.4 | 0.5 | 0.4 |

SL-PRS Rx-Tx simulation results

Table 25-28 shows the results for 90%-ile of the SL-PRS Rx-Tx errors with Es/Iot (-3,-6,-6), (0,-6,-6), (0,-3,-3) and (0,0,0) respectively.

Table 25 90%-ile of the SL-PRS Rx-Tx errors with Es/Iot (-3,-6,-6)

| SL-PRS based Rx-Tx [Tc] (-3, -6, -6) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  | TDL-A |  |  | TDL-B |  |  |
|  |  |  |  |  | UE1 | UE2 | UE3 | UE1 | UE2 | UE3 | UE1 | UE2 | UE3 |
|  |  |  |  |  | 90% | 90% | 90% | 90% | 90% | 90% | 90% | 90% | 90% |
| 15 | 48 | 4 | 4 | 1 | 64.7 | 66.4 | 66.4 | 194.9 | 191.2 | 198.4 | 349.1 | 347.4 | 347.0 |
|  |  |  |  | 2 | 64.6 | 66.3 | 66.5 | 129.2 | 136.7 | 132.8 | 325.5 | 321.3 | 325.1 |
|  |  |  |  | 3 | 63.9 | 64.9 | 64.9 | 118.1 | 125.2 | 125.2 | 260.1 | 252.7 | 261.6 |
|  |  |  |  | 4 | 62.9 | 63.8 | 64.1 | 117.0 | 122.9 | 112.6 | 236.9 | 241.7 | 230.6 |
|  |  | 4 | 2 | 1 | 69.0 | 69.1 | 77.0 | 194.0 | 194.6 | 195.0 | 354.1 | 350.9 | 353.1 |
|  |  |  |  | 2 | 68.6 | 68.8 | 76.8 | 128.1 | 137.2 | 161.3 | 332.2 | 331.9 | 327.6 |
|  |  |  |  | 3 | 67.2 | 67.4 | 75.7 | 121.4 | 126.3 | 126.9 | 280.4 | 259.5 | 269.8 |
|  |  |  |  | 4 | 65.9 | 66.2 | 74.0 | 118.5 | 123.8 | 120.3 | 240.3 | 233.4 | 234.1 |
|  | 96 | 4 | 4 | 1 | 31.4 | 32.9 | 33.7 | 128.7 | 129.5 | 131.5 | 221.5 | 234.1 | 248.3 |
|  |  |  |  | 2 | 31.3 | 32.9 | 33.5 | 118.0 | 120.8 | 122.0 | 165.0 | 169.0 | 167.9 |
|  |  |  |  | 3 | 31.0 | 32.5 | 33.1 | 109.1 | 114.6 | 114.7 | 152.1 | 163.2 | 165.5 |
|  |  |  |  | 4 | 30.8 | 32.0 | 32.9 | 102.2 | 105.1 | 102.0 | 157.6 | 166.1 | 160.1 |
|  |  | 4 | 2 | 1 | 32.9 | 35.0 | 36.0 | 129.5 | 132.4 | 128.6 | 235.2 | 242.3 | 256.9 |
|  |  |  |  | 2 | 32.8 | 34.9 | 35.9 | 121.1 | 123.4 | 121.1 | 167.4 | 175.8 | 173.1 |
|  |  |  |  | 3 | 32.4 | 34.3 | 35.3 | 108.8 | 115.2 | 114.0 | 161.3 | 171.3 | 174.5 |
|  |  |  |  | 4 | 31.8 | 33.9 | 34.9 | 105.9 | 106.4 | 104.6 | 154.9 | 168.8 | 175.2 |
| 30 | 24 | 4 | 4 | 1 | 68.0 | 77.2 | 71.5 | 197.0 | 194.5 | 205.3 | 355.0 | 353.7 | 354.3 |
|  |  |  |  | 2 | 67.9 | 76.4 | 71.0 | 184.1 | 185.7 | 195.3 | 335.3 | 337.7 | 336.3 |
|  |  |  |  | 3 | 66.5 | 74.4 | 69.4 | 155.6 | 157.2 | 180.3 | 282.6 | 280.6 | 275.1 |
|  |  |  |  | 4 | 65.6 | 73.2 | 68.5 | 138.4 | 142.9 | 149.6 | 242.0 | 252.2 | 244.3 |
|  |  | 4 | 2 | 1 | 73.3 | 81.3 | 79.2 | 196.2 | 229.0 | 1038.7 | 358.4 | 376.2 | 362.5 |
|  |  |  |  | 2 | 72.9 | 77.4 | 73.3 | 188.4 | 212.4 | 201.0 | 337.7 | 349.9 | 337.7 |
|  |  |  |  | 3 | 70.8 | 76.0 | 71.4 | 163.3 | 190.9 | 180.2 | 293.8 | 310.2 | 298.4 |
|  |  |  |  | 4 | 69.2 | 74.2 | 69.8 | 144.1 | 167.3 | 152.7 | 254.0 | 260.5 | 255.4 |
|  | 48 | 4 | 4 | 1 | 32.5 | 33.3 | 35.6 | 128.6 | 132.4 | 132.2 | 242.6 | 247.5 | 240.9 |
|  |  |  |  | 2 | 32.4 | 33.2 | 35.5 | 129.1 | 146.3 | 142.3 | 173.3 | 177.2 | 177.6 |
|  |  |  |  | 3 | 32.0 | 32.7 | 34.8 | 120.0 | 124.4 | 124.9 | 164.6 | 182.0 | 171.9 |
|  |  |  |  | 4 | 31.6 | 32.2 | 34.4 | 114.1 | 118.1 | 116.8 | 156.2 | 180.1 | 152.6 |
|  |  | 4 | 2 | 1 | 34.4 | 34.9 | 36.0 | 131.2 | 141.6 | 146.6 | 254.9 | 296.8 | 252.5 |
|  |  |  |  | 2 | 34.3 | 34.8 | 35.8 | 139.7 | 142.5 | 147.1 | 178.1 | 183.9 | 178.8 |
|  |  |  |  | 3 | 33.6 | 34.0 | 35.2 | 119.8 | 125.4 | 130.4 | 171.4 | 173.3 | 179.7 |
|  |  |  |  | 4 | 33.0 | 33.3 | 34.3 | 114.6 | 116.7 | 114.1 | 166.5 | 188.2 | 167.3 |
|  | 96 | 4 | 4 | 1 | 15.7 | 16.3 | 16.4 | 95.8 | 103.6 | 101.6 | 107.1 | 109.9 | 107.3 |
|  |  |  |  | 2 | 15.7 | 16.3 | 16.4 | 62.3 | 80.0 | 63.4 | 91.3 | 92.1 | 92.9 |
|  |  |  |  | 3 | 15.5 | 16.0 | 16.2 | 57.8 | 60.9 | 57.0 | 88.8 | 89.3 | 89.1 |
|  |  |  |  | 4 | 15.4 | 15.9 | 16.0 | 55.8 | 56.5 | 55.4 | 87.4 | 87.7 | 87.8 |
|  |  | 4 | 2 | 1 | 16.4 | 17.5 | 16.5 | 94.5 | 106.5 | 109.7 | 109.8 | 113.2 | 111.4 |
|  |  |  |  | 2 | 16.4 | 17.4 | 16.5 | 73.7 | 79.3 | 85.8 | 92.4 | 93.5 | 94.3 |
|  |  |  |  | 3 | 16.1 | 17.1 | 16.3 | 59.4 | 59.2 | 59.2 | 89.3 | 91.1 | 92.6 |
|  |  |  |  | 4 | 15.9 | 16.9 | 16.0 | 56.6 | 56.9 | 56.9 | 88.2 | 90.7 | 88.9 |
| 60 | 24 | 4 | 4 | 1 | 34.0 | 40.9 | 36.4 | 130.0 | 139.9 | 129.8 | 247.3 | 280.6 | 253.4 |
|  |  |  |  | 2 | 33.9 | 39.6 | 36.0 | 148.3 | 150.4 | 151.7 | 179.1 | 207.8 | 179.7 |
|  |  |  |  | 3 | 33.4 | 38.9 | 35.2 | 144.2 | 145.8 | 144.6 | 171.7 | 176.4 | 166.2 |
|  |  |  |  | 4 | 32.7 | 38.2 | 34.6 | 133.3 | 135.2 | 132.7 | 168.1 | 173.2 | 162.3 |
|  |  | 4 | 2 | 1 | 36.6 | 40.1 | 41.6 | 138.9 | 156.1 | 978.1 | 277.5 | 318.0 | 737.8 |
|  |  |  |  | 2 | 36.4 | 38.0 | 41.3 | 151.6 | 162.1 | 150.6 | 194.0 | 294.5 | 220.2 |
|  |  |  |  | 3 | 35.6 | 37.1 | 40.3 | 144.6 | 158.0 | 146.9 | 175.9 | 201.7 | 182.0 |
|  |  |  |  | 4 | 34.5 | 36.4 | 39.4 | 134.4 | 152.6 | 134.8 | 173.4 | 189.3 | 181.6 |

Table 26 90%-ile of the SL-PRS Rx-Tx errors with Es/Iot (0,-6,-6)

| SL-PRS based Rx-Tx [Tc] (0, -6, -6) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  | TDL-A |  |  | TDL-B |  |  |
|  |  |  |  |  | UE1 | UE2 | UE3 | UE1 | UE2 | UE3 | UE1 | UE2 | UE3 |
|  |  |  |  |  | 90% | 90% | 90% | 90% | 90% | 90% | 90% | 90% | 90% |
| 15 | 48 | 4 | 4 | 1 | 63.0 | 67.0 | 66.9 | 195.6 | 191.8 | 196.7 | 347.2 | 346.2 | 346.0 |
|  |  |  |  | 2 | 62.9 | 66.9 | 66.8 | 128.6 | 136.2 | 135.6 | 327.3 | 327.2 | 326.2 |
|  |  |  |  | 3 | 62.2 | 65.4 | 65.4 | 122.7 | 126.2 | 133.5 | 261.8 | 254.3 | 263.5 |
|  |  |  |  | 4 | 61.6 | 64.6 | 64.6 | 121.2 | 124.3 | 118.2 | 236.6 | 243.0 | 239.2 |
|  |  | 4 | 2 | 1 | 66.2 | 69.2 | 79.1 | 194.6 | 700.6 | 3247.5 | 352.5 | 351.1 | 352.8 |
|  |  |  |  | 2 | 66.0 | 68.8 | 78.9 | 130.6 | 138.0 | 138.7 | 333.7 | 334.8 | 331.5 |
|  |  |  |  | 3 | 64.8 | 67.8 | 77.5 | 124.7 | 129.8 | 129.1 | 277.7 | 269.6 | 276.3 |
|  |  |  |  | 4 | 64.1 | 66.1 | 76.6 | 118.6 | 129.4 | 120.2 | 244.2 | 235.2 | 247.4 |
|  | 96 | 4 | 4 | 1 | 30.7 | 33.3 | 34.3 | 128.8 | 129.5 | 131.5 | 213.7 | 238.4 | 237.8 |
|  |  |  |  | 2 | 30.7 | 33.3 | 34.2 | 118.6 | 121.8 | 123.6 | 166.1 | 165.6 | 168.1 |
|  |  |  |  | 3 | 30.4 | 32.8 | 33.7 | 109.1 | 114.9 | 114.1 | 154.9 | 169.7 | 168.2 |
|  |  |  |  | 4 | 30.2 | 32.4 | 33.4 | 104.0 | 107.3 | 102.6 | 152.2 | 166.0 | 158.4 |
|  |  | 4 | 2 | 1 | 31.9 | 35.5 | 36.9 | 128.3 | 138.0 | 129.6 | 225.5 | 248.5 | 255.1 |
|  |  |  |  | 2 | 31.8 | 35.4 | 36.8 | 119.9 | 124.1 | 120.9 | 164.9 | 175.3 | 179.4 |
|  |  |  |  | 3 | 31.4 | 34.9 | 36.2 | 108.7 | 117.3 | 114.6 | 155.3 | 179.8 | 172.0 |
|  |  |  |  | 4 | 31.1 | 34.4 | 35.8 | 104.8 | 107.6 | 104.4 | 159.4 | 184.6 | 176.6 |
| 30 | 24 | 4 | 4 | 1 | 65.3 | 80.2 | 72.1 | 197.7 | 310.4 | 209.8 | 353.4 | 353.1 | 357.7 |
|  |  |  |  | 2 | 65.2 | 78.5 | 71.7 | 183.2 | 187.8 | 197.4 | 335.6 | 336.8 | 340.8 |
|  |  |  |  | 3 | 64.4 | 76.8 | 70.0 | 146.3 | 161.4 | 181.9 | 293.2 | 301.6 | 292.0 |
|  |  |  |  | 4 | 63.4 | 75.4 | 68.7 | 135.5 | 139.5 | 159.1 | 246.0 | 259.8 | 248.4 |
|  |  | 4 | 2 | 1 | 69.5 | 101.2 | 106.4 | 197.4 | 261.5 | 1028.7 | 355.4 | 412.0 | 365.6 |
|  |  |  |  | 2 | 69.4 | 78.6 | 73.6 | 183.3 | 233.0 | 204.7 | 334.5 | 350.1 | 338.9 |
|  |  |  |  | 3 | 67.7 | 76.9 | 72.0 | 152.1 | 206.0 | 183.2 | 293.4 | 321.6 | 302.9 |
|  |  |  |  | 4 | 66.4 | 75.3 | 70.2 | 143.2 | 188.2 | 152.4 | 250.4 | 265.5 | 265.2 |
|  | 48 | 4 | 4 | 1 | 31.5 | 33.6 | 36.1 | 127.8 | 141.8 | 132.6 | 241.3 | 246.0 | 241.1 |
|  |  |  |  | 2 | 31.5 | 33.4 | 36.0 | 128.4 | 148.2 | 144.1 | 172.6 | 177.9 | 181.1 |
|  |  |  |  | 3 | 31.2 | 32.9 | 35.3 | 120.3 | 126.1 | 125.9 | 166.3 | 174.3 | 177.3 |
|  |  |  |  | 4 | 31.0 | 32.3 | 34.8 | 114.6 | 118.6 | 116.3 | 159.2 | 176.8 | 162.6 |
|  |  | 4 | 2 | 1 | 33.0 | 35.0 | 36.1 | 129.6 | 144.1 | 149.2 | 244.3 | 300.5 | 248.3 |
|  |  |  |  | 2 | 32.9 | 34.8 | 36.0 | 137.5 | 144.9 | 148.8 | 177.7 | 221.4 | 182.6 |
|  |  |  |  | 3 | 32.4 | 34.0 | 35.3 | 121.9 | 126.4 | 129.1 | 171.2 | 179.3 | 183.7 |
|  |  |  |  | 4 | 32.0 | 33.4 | 34.6 | 115.4 | 118.3 | 116.8 | 160.5 | 188.5 | 168.2 |
|  | 96 | 4 | 4 | 1 | 15.3 | 16.5 | 16.6 | 94.6 | 98.0 | 97.3 | 106.6 | 110.2 | 108.3 |
|  |  |  |  | 2 | 15.3 | 16.5 | 16.6 | 61.5 | 81.5 | 64.2 | 90.9 | 92.3 | 93.2 |
|  |  |  |  | 3 | 15.2 | 16.2 | 16.3 | 57.7 | 60.2 | 57.6 | 89.0 | 90.6 | 89.8 |
|  |  |  |  | 4 | 15.1 | 16.1 | 16.2 | 56.2 | 56.9 | 56.3 | 88.1 | 89.9 | 87.9 |
|  |  | 4 | 2 | 1 | 15.9 | 17.7 | 16.5 | 92.8 | 107.3 | 110.1 | 108.8 | 112.8 | 112.0 |
|  |  |  |  | 2 | 15.9 | 17.6 | 16.5 | 62.9 | 80.9 | 87.3 | 91.6 | 94.7 | 94.3 |
|  |  |  |  | 3 | 15.7 | 17.3 | 16.3 | 58.6 | 59.6 | 62.4 | 89.5 | 91.9 | 92.8 |
|  |  |  |  | 4 | 15.5 | 17.2 | 16.1 | 56.2 | 57.5 | 57.4 | 88.5 | 91.1 | 90.5 |
| 60 | 24 | 4 | 4 | 1 | 32.6 | 51.5 | 37.4 | 129.4 | 133.2 | 129.8 | 246.3 | 259.2 | 253.6 |
|  |  |  |  | 2 | 32.6 | 40.5 | 36.7 | 146.1 | 150.7 | 150.0 | 174.9 | 217.9 | 181.3 |
|  |  |  |  | 3 | 32.2 | 39.7 | 35.9 | 141.6 | 146.4 | 144.4 | 168.8 | 180.0 | 171.2 |
|  |  |  |  | 4 | 31.7 | 39.0 | 35.2 | 129.7 | 135.7 | 133.9 | 167.8 | 172.3 | 169.6 |
|  |  | 4 | 2 | 1 | 34.8 | 51.0 | 42.7 | 130.9 | 163.5 | 1360.0 | 255.2 | 340.4 | 937.1 |
|  |  |  |  | 2 | 34.7 | 38.6 | 42.4 | 148.5 | 168.9 | 782.7 | 184.7 | 300.4 | 217.7 |
|  |  |  |  | 3 | 34.0 | 37.5 | 41.3 | 142.8 | 165.3 | 312.0 | 174.7 | 213.6 | 181.3 |
|  |  |  |  | 4 | 33.3 | 36.9 | 40.4 | 128.8 | 161.5 | 130.8 | 170.1 | 197.3 | 180.4 |

Table 27 90%-ile of the SL-PRS Rx-Tx errors with Es/Iot (0,-3,-3)

| SL-PRS based Rx-Tx [Tc] (0, -3, -3) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  | TDL-A |  |  | TDL-B |  |  |
|  |  |  |  |  | UE1 | UE2 | UE3 | UE1 | UE2 | UE3 | UE1 | UE2 | UE3 |
|  |  |  |  |  | 90% | 90% | 90% | 90% | 90% | 90% | 90% | 90% | 90% |
| 15 | 48 | 4 | 4 | 1 | 64.3 | 64.7 | 64.7 | 193.8 | 191.3 | 194.7 | 347.3 | 347.0 | 346.0 |
|  |  |  |  | 2 | 64.2 | 64.7 | 64.6 | 129.8 | 135.6 | 134.5 | 332.5 | 329.6 | 327.7 |
|  |  |  |  | 3 | 63.3 | 63.7 | 63.8 | 123.2 | 127.2 | 134.2 | 262.4 | 251.9 | 262.1 |
|  |  |  |  | 4 | 62.6 | 63.1 | 63.0 | 120.9 | 125.8 | 119.1 | 240.0 | 242.9 | 234.5 |
|  |  | 4 | 2 | 1 | 68.3 | 66.4 | 74.0 | 196.8 | 192.8 | 195.9 | 353.3 | 347.1 | 353.3 |
|  |  |  |  | 2 | 68.0 | 66.3 | 73.9 | 130.5 | 137.2 | 136.2 | 336.9 | 330.3 | 330.2 |
|  |  |  |  | 3 | 66.5 | 65.5 | 72.8 | 125.5 | 129.0 | 130.1 | 283.3 | 262.2 | 273.8 |
|  |  |  |  | 4 | 65.5 | 64.3 | 72.0 | 120.7 | 127.9 | 118.7 | 245.9 | 238.5 | 247.0 |
|  | 96 | 4 | 4 | 1 | 31.1 | 32.3 | 33.0 | 128.5 | 129.2 | 129.3 | 216.8 | 233.8 | 241.4 |
|  |  |  |  | 2 | 31.1 | 32.3 | 32.9 | 118.9 | 121.4 | 123.1 | 167.7 | 163.2 | 166.8 |
|  |  |  |  | 3 | 30.8 | 31.9 | 32.6 | 109.0 | 114.6 | 112.5 | 154.2 | 164.1 | 168.6 |
|  |  |  |  | 4 | 30.5 | 31.5 | 32.3 | 102.5 | 108.0 | 102.5 | 155.5 | 158.5 | 161.5 |
|  |  | 4 | 2 | 1 | 32.6 | 33.9 | 34.9 | 128.5 | 130.9 | 128.8 | 231.1 | 238.6 | 247.8 |
|  |  |  |  | 2 | 32.6 | 33.9 | 34.8 | 121.1 | 123.2 | 120.4 | 165.9 | 172.2 | 175.5 |
|  |  |  |  | 3 | 32.1 | 33.5 | 34.4 | 109.5 | 115.4 | 113.1 | 156.1 | 172.6 | 171.9 |
|  |  |  |  | 4 | 31.8 | 33.1 | 34.1 | 105.0 | 107.4 | 104.9 | 163.6 | 185.1 | 180.9 |
| 30 | 24 | 4 | 4 | 1 | 67.1 | 74.6 | 69.3 | 195.6 | 194.1 | 206.6 | 352.8 | 352.3 | 352.8 |
|  |  |  |  | 2 | 67.0 | 74.1 | 69.2 | 182.2 | 187.9 | 194.7 | 334.6 | 334.3 | 336.6 |
|  |  |  |  | 3 | 65.9 | 72.6 | 67.9 | 148.6 | 156.4 | 183.8 | 297.0 | 295.8 | 279.8 |
|  |  |  |  | 4 | 64.8 | 71.3 | 66.7 | 134.9 | 140.5 | 154.8 | 244.5 | 260.0 | 243.8 |
|  |  | 4 | 2 | 1 | 72.3 | 76.3 | 72.4 | 197.8 | 221.0 | 207.7 | 356.5 | 364.5 | 358.2 |
|  |  |  |  | 2 | 72.2 | 75.4 | 71.4 | 186.0 | 202.2 | 194.9 | 339.0 | 343.3 | 336.1 |
|  |  |  |  | 3 | 70.0 | 73.9 | 69.9 | 155.8 | 181.5 | 180.8 | 301.7 | 311.1 | 291.3 |
|  |  |  |  | 4 | 68.4 | 72.9 | 68.3 | 140.3 | 156.1 | 154.1 | 248.2 | 263.3 | 257.6 |
|  | 48 | 4 | 4 | 1 | 32.1 | 32.5 | 34.6 | 127.8 | 132.6 | 131.5 | 240.4 | 244.2 | 238.0 |
|  |  |  |  | 2 | 32.1 | 32.5 | 34.5 | 127.3 | 145.1 | 144.0 | 171.9 | 175.6 | 175.2 |
|  |  |  |  | 3 | 31.8 | 32.1 | 34.1 | 120.6 | 125.9 | 125.9 | 166.6 | 180.2 | 173.2 |
|  |  |  |  | 4 | 31.4 | 31.6 | 33.7 | 114.0 | 120.2 | 116.4 | 163.5 | 177.7 | 161.6 |
|  |  | 4 | 2 | 1 | 33.9 | 33.6 | 34.7 | 130.4 | 133.3 | 144.4 | 250.7 | 283.7 | 243.5 |
|  |  |  |  | 2 | 33.9 | 33.5 | 34.6 | 139.6 | 139.7 | 146.0 | 178.6 | 184.5 | 182.6 |
|  |  |  |  | 3 | 33.4 | 32.9 | 34.0 | 123.8 | 126.8 | 123.5 | 169.0 | 174.4 | 182.4 |
|  |  |  |  | 4 | 32.8 | 32.5 | 33.4 | 115.2 | 116.9 | 116.9 | 157.1 | 184.3 | 168.8 |
|  | 96 | 4 | 4 | 1 | 15.6 | 16.0 | 16.1 | 92.4 | 94.8 | 95.5 | 106.0 | 108.6 | 106.7 |
|  |  |  |  | 2 | 15.6 | 16.0 | 16.1 | 62.1 | 76.0 | 63.1 | 91.1 | 92.4 | 93.2 |
|  |  |  |  | 3 | 15.4 | 15.8 | 15.9 | 59.5 | 58.4 | 58.4 | 89.0 | 90.6 | 90.1 |
|  |  |  |  | 4 | 15.2 | 15.7 | 15.8 | 56.3 | 56.6 | 56.6 | 88.4 | 88.6 | 87.9 |
|  |  | 4 | 2 | 1 | 16.3 | 16.9 | 16.1 | 94.3 | 104.5 | 107.2 | 108.1 | 110.6 | 108.9 |
|  |  |  |  | 2 | 16.2 | 16.9 | 16.1 | 63.7 | 64.1 | 83.2 | 92.4 | 93.7 | 93.8 |
|  |  |  |  | 3 | 16.0 | 16.7 | 15.9 | 59.5 | 58.9 | 59.9 | 89.4 | 91.6 | 91.8 |
|  |  |  |  | 4 | 15.8 | 16.5 | 15.6 | 58.0 | 57.3 | 56.8 | 88.8 | 89.5 | 89.0 |
| 60 | 24 | 4 | 4 | 1 | 33.5 | 39.3 | 35.1 | 129.6 | 131.0 | 130.1 | 247.2 | 251.2 | 248.0 |
|  |  |  |  | 2 | 33.5 | 38.8 | 35.0 | 146.5 | 149.0 | 149.9 | 179.8 | 206.4 | 176.2 |
|  |  |  |  | 3 | 32.9 | 38.2 | 34.3 | 140.7 | 144.2 | 144.1 | 169.5 | 174.5 | 164.6 |
|  |  |  |  | 4 | 32.3 | 37.6 | 33.8 | 129.2 | 132.4 | 132.6 | 167.0 | 174.4 | 168.6 |
|  |  | 4 | 2 | 1 | 36.2 | 37.5 | 39.8 | 132.9 | 152.2 | 129.3 | 264.5 | 309.9 | 253.5 |
|  |  |  |  | 2 | 36.0 | 37.0 | 39.6 | 149.7 | 157.2 | 147.8 | 192.6 | 244.9 | 180.9 |
|  |  |  |  | 3 | 35.1 | 36.3 | 38.8 | 142.3 | 154.9 | 143.4 | 180.0 | 198.2 | 171.7 |
|  |  |  |  | 4 | 34.3 | 35.6 | 38.1 | 131.1 | 146.0 | 132.5 | 172.5 | 187.0 | 169.7 |

Table 28 90%-ile of the SL-PRS Rx-Tx errors with Es/Iot (0,0,0)

| SL-PRS based Rx-Tx [Tc] (0, 0, 0) dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | RB | Comb | Symbol | Sample | Channel |  |  |  |  |  |  |  |  |
| (kHz) |  |  |  |  | AWGN |  |  | TDL-A |  |  | TDL-B |  |  |
|  |  |  |  |  | UE1 | UE2 | UE3 | UE1 | UE2 | UE3 | UE1 | UE2 | UE3 |
|  |  |  |  |  | 90% | 90% | 90% | 90% | 90% | 90% | 90% | 90% | 90% |
| 15 | 48 | 4 | 4 | 1 | 65.7 | 63.4 | 63.4 | 194.8 | 192.1 | 196.4 | 348.7 | 345.4 | 345.3 |
|  |  |  |  | 2 | 65.7 | 63.4 | 63.5 | 129.8 | 134.8 | 135.3 | 332.3 | 328.5 | 327.4 |
|  |  |  |  | 3 | 64.7 | 62.6 | 62.7 | 124.9 | 127.2 | 133.5 | 265.3 | 251.7 | 263.9 |
|  |  |  |  | 4 | 63.5 | 62.0 | 62.1 | 120.7 | 125.1 | 120.1 | 242.1 | 240.1 | 235.1 |
|  |  | 4 | 2 | 1 | 70.4 | 64.7 | 70.9 | 192.4 | 193.1 | 199.0 | 354.0 | 344.8 | 350.7 |
|  |  |  |  | 2 | 70.0 | 64.6 | 70.9 | 131.7 | 138.1 | 136.2 | 338.0 | 328.1 | 326.4 |
|  |  |  |  | 3 | 68.2 | 64.1 | 69.9 | 124.9 | 127.1 | 128.4 | 287.4 | 255.5 | 270.2 |
|  |  |  |  | 4 | 67.1 | 63.1 | 69.3 | 120.8 | 124.5 | 118.5 | 252.3 | 237.8 | 247.6 |
|  | 96 | 4 | 4 | 1 | 31.6 | 31.6 | 32.2 | 128.4 | 129.6 | 129.2 | 219.8 | 237.7 | 243.5 |
|  |  |  |  | 2 | 31.6 | 31.6 | 32.2 | 118.6 | 121.7 | 123.2 | 169.4 | 164.0 | 166.0 |
|  |  |  |  | 3 | 31.1 | 31.3 | 31.9 | 109.7 | 114.3 | 112.6 | 154.5 | 170.2 | 168.5 |
|  |  |  |  | 4 | 30.8 | 31.0 | 31.7 | 105.0 | 108.0 | 101.6 | 159.3 | 168.8 | 160.7 |
|  |  | 4 | 2 | 1 | 33.5 | 33.0 | 33.7 | 128.7 | 130.5 | 129.1 | 233.3 | 234.5 | 245.4 |
|  |  |  |  | 2 | 33.5 | 32.9 | 33.6 | 120.5 | 122.7 | 121.5 | 170.6 | 171.4 | 174.5 |
|  |  |  |  | 3 | 32.8 | 32.6 | 33.3 | 109.5 | 115.1 | 112.1 | 159.3 | 171.2 | 173.8 |
|  |  |  |  | 4 | 32.4 | 32.3 | 33.0 | 105.0 | 107.4 | 105.5 | 164.1 | 178.1 | 180.2 |
| 30 | 24 | 4 | 4 | 1 | 68.9 | 71.0 | 67.6 | 196.3 | 193.6 | 202.9 | 353.8 | 350.8 | 351.2 |
|  |  |  |  | 2 | 68.7 | 70.7 | 67.5 | 184.6 | 185.2 | 193.1 | 337.1 | 333.4 | 335.7 |
|  |  |  |  | 3 | 67.6 | 69.7 | 66.3 | 151.5 | 154.1 | 179.7 | 297.0 | 297.9 | 275.4 |
|  |  |  |  | 4 | 66.2 | 68.6 | 65.5 | 137.1 | 138.3 | 150.8 | 247.4 | 255.6 | 243.0 |
|  |  | 4 | 2 | 1 | 75.4 | 72.8 | 69.4 | 197.7 | 209.4 | 206.2 | 360.3 | 358.2 | 354.5 |
|  |  |  |  | 2 | 74.9 | 72.6 | 69.3 | 186.9 | 195.8 | 194.0 | 339.9 | 341.9 | 336.4 |
|  |  |  |  | 3 | 72.6 | 71.3 | 67.7 | 159.4 | 173.1 | 174.6 | 301.7 | 308.1 | 290.3 |
|  |  |  |  | 4 | 70.5 | 70.5 | 66.7 | 144.5 | 154.1 | 155.0 | 249.2 | 262.9 | 255.0 |
|  | 48 | 4 | 4 | 1 | 32.8 | 31.9 | 33.7 | 127.8 | 131.1 | 130.4 | 240.1 | 243.0 | 234.0 |
|  |  |  |  | 2 | 32.8 | 31.9 | 33.7 | 128.0 | 143.3 | 141.9 | 172.4 | 175.7 | 172.3 |
|  |  |  |  | 3 | 32.4 | 31.6 | 33.3 | 121.4 | 124.1 | 125.1 | 165.3 | 178.9 | 169.4 |
|  |  |  |  | 4 | 31.9 | 31.2 | 33.0 | 113.6 | 117.6 | 116.4 | 161.0 | 178.4 | 162.3 |
|  |  | 4 | 2 | 1 | 35.0 | 32.8 | 33.8 | 131.9 | 131.1 | 137.9 | 251.1 | 272.4 | 239.3 |
|  |  |  |  | 2 | 35.0 | 32.8 | 33.7 | 143.6 | 138.4 | 144.2 | 178.5 | 183.6 | 180.3 |
|  |  |  |  | 3 | 34.3 | 32.3 | 33.3 | 123.2 | 125.7 | 123.4 | 167.8 | 171.4 | 181.1 |
|  |  |  |  | 4 | 33.6 | 31.9 | 32.8 | 115.1 | 118.1 | 115.5 | 157.4 | 184.3 | 165.7 |
|  | 96 | 4 | 4 | 1 | 15.8 | 15.7 | 15.8 | 94.0 | 94.4 | 94.0 | 106.5 | 108.5 | 107.2 |
|  |  |  |  | 2 | 15.8 | 15.6 | 15.8 | 62.3 | 62.9 | 62.8 | 91.2 | 92.4 | 93.1 |
|  |  |  |  | 3 | 15.6 | 15.5 | 15.6 | 59.5 | 58.4 | 58.1 | 89.1 | 90.2 | 90.1 |
|  |  |  |  | 4 | 15.4 | 15.4 | 15.5 | 57.3 | 56.5 | 56.5 | 88.5 | 89.1 | 88.3 |
|  |  | 4 | 2 | 1 | 16.7 | 16.4 | 15.8 | 95.8 | 102.9 | 105.8 | 108.5 | 109.3 | 107.7 |
|  |  |  |  | 2 | 16.6 | 16.4 | 15.8 | 68.2 | 62.7 | 75.8 | 92.6 | 93.2 | 93.2 |
|  |  |  |  | 3 | 16.3 | 16.2 | 15.6 | 60.2 | 59.0 | 58.6 | 89.5 | 91.0 | 91.7 |
|  |  |  |  | 4 | 16.0 | 16.1 | 15.5 | 58.8 | 57.0 | 56.7 | 88.8 | 89.3 | 89.2 |
| 60 | 24 | 4 | 4 | 1 | 34.5 | 37.5 | 33.7 | 129.5 | 131.3 | 130.2 | 251.5 | 248.4 | 242.9 |
|  |  |  |  | 2 | 34.4 | 37.4 | 33.7 | 147.8 | 148.8 | 148.7 | 186.5 | 196.1 | 175.2 |
|  |  |  |  | 3 | 33.7 | 36.9 | 33.2 | 141.3 | 142.8 | 143.0 | 176.2 | 172.2 | 165.6 |
|  |  |  |  | 4 | 33.0 | 36.4 | 32.7 | 129.6 | 130.8 | 129.9 | 169.3 | 175.2 | 164.4 |
|  |  | 4 | 2 | 1 | 37.7 | 35.8 | 38.0 | 138.8 | 147.1 | 130.2 | 283.7 | 300.2 | 249.8 |
|  |  |  |  | 2 | 37.4 | 35.6 | 37.8 | 151.5 | 153.0 | 149.1 | 207.1 | 230.1 | 176.9 |
|  |  |  |  | 3 | 36.4 | 35.0 | 37.1 | 142.1 | 148.2 | 143.3 | 181.9 | 182.5 | 167.1 |
|  |  |  |  | 4 | 35.3 | 34.6 | 36.5 | 132.9 | 136.4 | 131.1 | 176.5 | 175.1 | 170.0 |

Conclusion

In this contribution, we present simulation results for SL-PRS RSTD, SL-PRS RSRP, SL-PRS RSRPP and SL-PRS Rx-Tx. Based on analysis following observations and proposals are present.

Observation 1: Better Es/Iot condition shows little improvements in SL-PRS measurements.

Observation 2: SL-PRS measurement accuracy becomes better as the number of used samples increase, especially in fading channel TDL-A and TDL-B.

Observation 3: Wider SL-PRS BW yield higher accuracy results for SL-PRS RSTD and SL-PRS Rx-Tx.

Observation 4: The performance of partially staggered SL-PRS are worse than fully staggered SL-PRS.

Proposal 1: At least support number of SL-PRS sample equals 4 for SL positioning measurements.

References

- R4-2317381, Updated simulation assumptions for sidelink positioning, CATT