3GPP TSG-RAN WG4 Meeting #109	R4-2319148

Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Pre-MG and ConMGs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some latest agreements in RAN4 #108b about Pre-MG are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce the latest RAN4 #108b agreement of dynamic collision rule for concurrent Pre-MG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for concurrent pre-configured gaps is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.x, 9.1.x.1, 9.1.x.2, 9.1.x.3, 9.1.x.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------ START OF CHANGE 12: 9.1.x [R4-2317294] --------------

### 9.1.x	Concurrent measurement gaps with Pre-MG

#### 9.1.x.1	Introduction

When UE supports [ConMGs with Pre-MG] capability, network can provide multiple measurement gap patterms with at least one of the measurement gaps is pre-configured measurement gap (Pre-MG) pattern configured by RRC message(s) as specified in TS 38.331 [2].

Requirements in this section apply when the UE is in SA operation mode.

#### 9.1.x.2	Requirements

If the UE requires measurement gaps and/or Pre-MGs to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports [ConMGs with Pre-MG] but does not support independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [18, 19, 20], in order for the requirements in the following clauses to apply, the network can provide the UE with not more than two per-UE measurement gap patterns for monitoring all the frequency layers.

Editor Notes: FFS additional capability for UE to support Pre-MG+Pre-MG

Two Pre-MGs may be configured by the network only if the UE supports [Pre-MG+Pre-MG].

If the UE supports both [ConMGs with Pre-MG] and independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [18, 19, 20], in order for the requirements defined for concurrent measurement gaps with Pre-MG to apply, the network can provide the measurement gap patterns’combinations specified in Table 9.1.x.1 for monitoring of all frequency layers.

Table 9.1.x-1: The number of Gap Combination Configurations by UE supporting both [ConMGs with Pre-MG] and independent measurement gap patterns

| Gap Combination Configuration Id | The number of simultaneous configured measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 measurement gap | Per-FR2 measurement gap |  | Per-UE measurement gap |  |
| 0 | 2 | 1 |  | 0 |  |
| 1 | 1 | 2 |  | 0 |  |
| 2 | 0 | 0 |  | 2 |  |
| 3Note 1 | 1 | 0 |  | 1 |  |
| 4Note 1 | 0 | 1 |  | 1 |  |
| 5Note 1 | 1 | 1 |  | 1 |  |
| 6 | 2 | 0 | 0 |  |  |
| 7 | 0 | 2 | 0 |  |  |
| Note 1: Gap Combination Configuration Id #3, #4, #5 are only applicable when the per-UE measurement gap is associated to measure PRS for any RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement defined in TS 38.215 [4]. [Note 2]: For UE capable of [Concurrent Pre-MG], up to 2 measurement gap patterns can be configured as Pre-MG in one FR, regardless of whether they are per-UE or per-FR configuration and the configuration in the other FR. Otherwise, the gaps can only be configured as Type-1/2 MG. Note 3: In Gap Combination Configuration Id #0, #1, #6, #7, one per-FR measurement gap in an FR can be associated to measure PRS for any RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement defined in TS 38.215 [4] provided that UE supports independentGapConfigPRS-r17. |  |  |  |

When UE supports [Concurrent Pre-MG], the gap association for a frequency layer is configured by the network via [gapAssociation] in [ToBeMeasureConfig]. In this case the gap association rules in clause 9.1.8.2 shall also apply to either measurement gap or Pre-MG.

When autonomous mechanism [1] is used for activation/deactivation of Pre-MG pattern, the UE shall autonomously determine the Pre-MG status only based on the measurement objects associated with the concerned Pre-MG. The related Pre-MG autonomous activation/deactivation mechanism is specified in clause 9.1.7.3.1.

When network-controlled mechanism [1] is used for activation/deactivation, the requirements specified in clause 9.1.7.3.2 apply.

When UE supports [Concurrent Pre-MG], where at least one of the concurrent gaps is Pre-MG, for a measurement gap pattern supported by the UE is listed in Table 9.1.2-1 based on the applicability specified in table 9.1.2-3.

The requirements in clause 9.1.2 are applicable for the UE capable of [Concurrent Pre-MG] and configured with multiple concurrent measurement gap patterns within each activated Pre-MG pattern.

#### 9.1.x.3	Collision for concurrent pre-configured measurement gaps

Collisions between two [gap occasions], where at least one of the gaps is a Pre-MG, may occur as specified in this clause for the following scenarios.

- between two Pre-MG occasions if both Pre-MGs are activated,

- between one Pre-MG ocassion and one measurement gap ocassion if the Pre-MG is activated,

and these two measurement gaps are

-	two per-UE Pre-MGs, or

-	two per-FR Pre-MGs in the same FR, or

-	one per-UE Pre-MG and one per-UE measurement gap, or

-	one per-UE measurement gap and one per-FR Pre-MG, or

-	one per-UE Pre-MG and one per-FR measurement gap

the gap collison rule applies if the gap collision condition specified in clause 9.1.8.3 is met.

No gap collision rule applies when one or two Pre-MG(s) are deactivated.

The requirements for [concurrent measurement gaps with Pre-MG] apply provided that the two measurement gaps(at least one of the gaps is activated Pre-MG) colliding with each other are configured with different priorities.

Editor Notes: capture the Pre-MG activation collision with measurement gap(scenario 1, 2, 3).

#### 9.1.x.4	Collision between Pre-MG activation/deactivation and measurement gap

When one measurement gap pattern and one Pre-MG pattern are configured by network, the measurement gap occasion is considered as colliding with the Pre-MG activation/deactivation procedure if the distance in time between the measurement gap occasion and the Pre-MG activation/deactivation procedure is equal to or smaller than 4ms.

The distance between the measurement gap occasion and the Pre-MG activation/deactivation procedure is defined as the time difference between the ending point of the gap occasion and the ending point of the Pre-MG activation/deactivation procedure, or the starting point of the gap occasion and the ending point of the Pre-MG activation/deactivation procedure. The starting and ending point of the Pre-MG activation/deactivation procedure are defined in clause [8.x.y].

In case of the collision with Pre-MG activation procedure, and the Pre-MG is configured with higher priority, the UE shall perform measurements in the occasion of the measurement gap during the Pre-MG activation procedure and the earliest occasion of the Pre-MG to be activated after the Pre-MG activation procedure shall be dropped if the distance in time between the occasion of Pre-MG to be activated and the measurement gap occasion is equal to or smaller than 5ms.

In case of the collision with Pre-MG deactivation procedure, and the Pre-MG is configured with higher priority, the UE shall drop measurements in the occasion of the measurement gap during the Pre-MG deactivation procedure if the distance in time between the occasion of Pre-MG to be deactivated and the measurement gap occasion is equal to or smaller than 5ms.

When the activated Pre-MG and measurement gap meets the collision rule defined in 9.1.8.3 and the Pre-MG is configured with lower priority, the UE shall perform measurements in the occasion of the measurement gap regardless of whether colliding with the Pre-MG activation procedure.

The UE is expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI in the corresponding NR serving cells in the slots of the configured Pre-MG that are dropped according to the requirements in clause 9.1.8.4.

-------------END OF CHANGE 12: 9.1.x [R4-2317294] --------------