3GPP TSG RAN WG4 Meeting #109	R4-2320779

Chicago, USA, 13 Nov. 2023 – 17 Nov. 2023

Agenda Item:	8.29.2.1

Source: 	Ericsson

Title:	Discussion on introducing TDCP requirements

Document for:	Discussion

# Introduction

In this contribution we analyze the RRM requirements for Rel-18 TDCP measurements.

# Discussion

In RAN4#108, following agreements are made for TDCP amplitude channel autocorrelation.

  - Do not define TDCP measurement delay requirements.

  - Further identify feasibility of methodology and test setup to define TDCP accuracy requirements including at least ideal TDCP definition, channel models.

  - Define TDCP accuracy requirements subject to conclusions of feasibility analysis.

  - For TDCP measurement averaging across RX ports should be left to UE implementation.

    - The reported TDCP amplitude is expected to between the minimum and maximum measured values across the Rx branches if receiver diversity is used.

In our previous contribution [5], we have described in detail on how the ideal autocorrelation values for TDCP tests can be derived when using the TDL channel models with a specific Doppler spread (fmax).

For TDL-A/B/C channel model, the autocorrelation is given by

A∆t=J02π∙fc∙vc∙∆t,

where,

- A∆t is the channel auto-correlation at a configured lag ∆t.

- J0 () is Bessel function.

Furthermore, we have described that the ideal autocorrelation is the expectation value of the instantaneous autocorrelation, and that the instantaneous autocorrelation needs to be averaged over time and frequency to obtain a TDCP report close to the ideal autocorrelation. The impact of the delay spread, noise and time-averaging on TDCP report has also been discussed in Section 2.1 of [5].

- RAN4 to agree that ideal value can be calculated for each delay value and over different doppler spread (fmax).

Extensive simulation results were provided for the TDL-A channel and 100 MHz bandwidth for different delay spreads, SNRs, lags and number of measurements used for averaging in [5]. By analyzing the probability distribution of the TDCP amplitude values for different scenarios it was noticed that it is possible to define TDCP accuracy requirements. The estimator used for the simulations is summarized in Appendix A.

In [6], the link level assumptions in Table 1 for evaluating TDCP measurements were agreed.

Table 1: Link level simulation assumptions for evaluating TRS based TDCP measurements in NR

| Parameter | Value |
| --- | --- |
| Delay (between TRS symbols) | 1slot |
| Channel model | TDL-A, delay spread=30ns |
| Doppler Spread | 10, 30, 75, 100, 200, 300 |
| SNR | 5:5:20 |
| Number of averaging samples: | one shot as baseline, 4 samples |
| Channel BW | 10MHz |
| SCS | 30KHz as baseline, 15KHz 2nd priority |
| Reference Channel estimation | LS CE for TRS as baseline, MMSE CE as 2nd priority |
| Correlation matrix and antenna configuration | 1x2 Low |

A lag of 1 slot was agreed as the baseline for defining the accuracy requirements. A TDL-A channel model with delay spread of 30 ns corresponds to an indoor environment with small frequency selectivity within the agreed bandwidth of 10 MHz. Small frequency selectivity within the bandwidth implies that the amplitude of instantaneous autocorrelation suffers from low averaging over frequency.

All simulations were performed for a carrier frequency of 3.5GHz with 30kHz subcarrier spacing. A TRS with two slot bursts and a periodicity of 40ms was used. Least squares channel estimation (LS CE) was used with noise suppression.

## Simulation results and requirements for a lag of 1 slot

In the following, we present simulation results for a few parameter combinations for the cases of single sample and average of 4 samples. Table 2 and Table 3 show the cdf of unquantized and pmf of quantized levels for the estimated autocorrelation amplitude at 1 slot lag for the cases of single shot measurement and averaging over 4 samples, respectively. The results are shown for SNRs = [5, 10, 15, 20] dB.

Table 2: CDF of unquantized and PMF of quantized TDCP for the case of no averaging.

Table 3: CDF of unquantized and PMF of quantized TDCP for the 4 samples averaging

Table 4: CDF of unquantized amplitude of estimated autocorrelation for the case of no averaging

| CDF | SNR (dB) | Amplitude for single shot | Amplitude for 4 sample average |
| --- | --- | --- | --- |
| Doppler spread = 10 Hz, Ideal value=0.99975 |  |  |  |
| 10% | [5, 10, 15, 20] | [0.9944, 1, 1, 1] | [0.99887,1, 1,1] |
| 50% | [5, 10, 15, 20] | [1, 1, 1, 1] | [1,1, 1, 1] |
| 90% | [5, 10, 15, 20] | [1, 1, 1, 1] | [1,1, 1, 1] |
| Doppler spread = 100 Hz, Ideal value=0.99778 |  |  |  |
| 10% | [5, 10, 15, 20] | [0.9517, 0.95474, 0.95483, 0.95515] | [0.96491, 0.96643, 0.96592, 0.96611] |
| 50% | [5, 10, 15, 20] | [0.9931, 0.99267, 0.99208, 0.99208] | [0.98869, 0.98738, 0.98692, 0.98674] |
| 90% | [5, 10, 15, 20] | [1, 1, 1, 1] | [1, 0.99759, 0.99675, 0.99654] |
| Doppler spread = 200 Hz, Ideal value = 0.90371 |  |  |  |
| 10% | [5, 10, 15, 20] | [0.8112, 0.81225, 0.81238, 0.81134] | [0.85943, 0.85782, 0.85696, 0.85721] |
| 50% | [5, 10, 15, 20] | [0.9518, 0.94988, 0.94937, 0.9488] | [0.93154, 0.92909, 0.92836, 0.92804] |
| 90% | [5, 10, 15, 20] | [0.9947, 0.99138, 0.99056, 0.99027] | [0.97154, 0.96778, 0.96707, 0.96689] |
| Doppler spread = 300 Hz, Ideal value = 0.78996 |  |  |  |
| 10% | [5, 10, 15, 20] | [0.6235, 0.6208, 0.62045 , 0.61932] | [0.72973, 0.72513, 0.72614, 0.72534] |
| 50% | [5, 10, 15, 20] | [0.88274, 0.88058, 0.87995, 0.87962] | [0.84625, 0.84331, 0.84215, 0.84189] |
| 90% | [5, 10, 15, 20] | [0.972978, 0.96955, 0.96879, 0.96849] | [0.92185, 0.91883, 0.9178, 0.91741] |

In the above table we summarize the TDCP CDF plot for different SNR values and doppler spreads. In some scenarios, owing to the small bandwidth and small delay spread, the instantaneous autocorrelation estimates in the case of single measurement deviate from the ideal autocorrelation to a some extent and still within the range of RAN4 requirements we generally define. The impact of SNR and therefore noise is not significant for Doppler spreads greater than 30Hz as discussed in [5].

However, in the case of 4 samples averaging, the time averaging reduces the deviation of the estimated autocorrelation amplitude from the ideal autocorrelation to a large extent despite the small bandwidth and delay spread. In the following discussion we focus on the case of 10 dB SNR and 4 samples averaging.

Figure 1: PMF of quantized levels for the estimated autocorrelation amplitude averaged over 4 samples.

Figure 1 shows the PMF of quantized levels for the estimated autocorrelation amplitude averaged over 4 samples. From the results, it can be observed that it is possible to define accuracy requirements for selected Doppler spreads given the constraint of 1 slot lag and small bandwidth agreed. At a 1 slot lag it is harder to differentiate among different smaller Doppler spreads when there isn’t enough bandwidth to benefit from frequency averaging. Therefore, one can pick a high Doppler spread, e.g., 300 Hz and a low Doppler spread 10 Hz as two scenarios that can be easily distinguished.

From observing the results, an accuracy test can be defined as that the reported values by a UE must be within a window that includes levels around the ideal value that contribute to X% of values seen in simulations. For instance, for the case of 300 Hz such a window could include levels Q8 to Q12 as shown in figure 1. We think accuracy window of ±3 quantization levels from the ideal value are acceptable from RAN4 accuracy point of view. And for the case of 10 Hz, since the values are close to 1, the window could be relaxed and could include levels Q0 to Q3. Furthermore, a UE could be required to pass the test only P% of the time.

Based on the simulation results, as we can observe, for higher doppler, accuracy is around 3 levels from the ideal value calculated.

Since the TDCP observed from simulation results are within acceptable range from the ideal value, it possible to define the accuracy requirements that can be tested.

- RAN4 to define TDCP accuracy requirements as

  - TDCP observed from simulation results are within acceptable range from the ideal value; and

  - it is possible to define the accuracy requirements that can be tested using the ideal value derived from Bessel function as reference.

In RAN4 though we define accuracy requirements based on worst case configuration, in real NW implementation configuration can be much better and actual values reported by UE can be more accurate than the values defined in RAN4 spec and defining accuracy requirements for TDCP requirements are useful for the feature usability.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 to agree that ideal value can be calculated for each delay value and over different doppler spread (fmax).

- RAN4 to define TDCP accuracy requirements as

  - TDCP observed from simulation results are within acceptable range from the ideal value; and

  - it is possible to define the accuracy requirements that can be tested using the ideal value derived from Bessel function as reference.

# References

[1] 	3GPP RP-223276, WID Update: MIMO Evolution for Downlink and Uplink. Samsung
	
[2]		3GPP R4-2314453

[3]		3GPP R1-2305715

[4]		3GPP TS 38.133 v18.3.0 	NR; Requirements for support of radio resource management

[5]       3GPP R4-2316827, Discussion on introducing TDCP requirements, Ericsson, RAN4#108bis, Xiamen, Oct. 2023.

[6]  	3GPP R4-2317371, Way forward on R18 NR MIMO RRM requirement, Samsung, RAN4#108bis, Xiamen, Oct. 2023.

# Appendix

- Appendix A, Autocorrelation estimator

Let Rn be the received signal for the TRS subcarrier n

Rn=hn∙Sn+ηn

where ηn is AWGN with

Eηn2=σ2

and Sn is the known transmitted signal (i.e. the TRS) on the TRS subcarrier n and Sn=1.

We also define

Xn=Rn∙Sn*

(which may be viewed as a first rough estimate of the channel hn) and

Yk=ifftXn

The autocorrelation estimator we used in the evaluations can be written as

At,∆t=k∈ΓYkt+∆t∙Yk*tk∈ΓYkt+∆t2-Γ∙σt+∆t 2k∈ΓYkt2-Γ∙σt2

Here Γ is an interval in the time domain containing the part of the channel which is above noise. Identifying this interval is a standard part of channel estimation and should already be performed by UEs as a part of channel estimation. Γ is the length of the interval in samples and σ is an estimate of the noise. The negative term -Γ∙σ2 in the denominator corrects for the noise bias in Yk2. Note that there is no corresponding bias in the numerator since the noise at the instances t and t+∆t is uncorrelated.

The estimator may alternatively be calculated in the frequency domain in order to avoid the FFT as

At,∆t=n=0N-1Xnt+∆t∙Xn*tnXnt+∆t2-Γ∙σt+∆t 2nXnt2-Γ∙σt 2

where Xn is Xn filtered with a sinc-filter corresponding to the interval Γ.

Averaging over multiple symbols within a TRS burst was done coherently. Averaging over multiple measurement occasions was, however, done incoherently taking the absolute value of At,∆t for each measurement occasion before averaging. This makes the estimator robust against phase jumps.

## For normalization (i.e. in the denominator) we used the geometric average over the time instances t and t+∆t in order to make the estimator robust towards AGC as requested by UE manufacturers.