3GPP TSG-RAN WG4 Meeting #109	R4-2321199

Chicago, USA, 13 – 17 Nov. 2023

Title: 	WF for UE 8Rx performance requirements

Agenda Item:	8.3.4

Source: 	Huawei, HiSilicon, MediaTek

Document for:	Approval

1 General

Issue 1-1: Test applicability rules

- Agreement

If a 8Rx capable UE supports both hybrid Rx band combination (4Rx+8Rx, 2Rx+8Rx) with Rank 2 and 8Rx+8Rx band combination with Rank 8, CA test for 8Rx+8Rx with Rank 8 should be selected for testing, otherwise Rank 2 CA test should be applied

Issue 1-2: Applicability rules for CA configurations selection

- Agreement

Create new section 5.1.1.7.5 Applicability and test rules for different CA configurations and bandwidth combination sets for 8Rx UE

Table 5.1.1.7.5-1: Applicability and test rules for CA UE demodulation tests for 8Rx UE

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test 1 in Clause 5.2A.2.1, 5.2A.3.1 and 5.2A.4.1 | CA_C, CA_N, CA_AX | Table 5.1.1.7.5-2 | Largest aggregated CA bandwidth combination | Any of CCs |
| Test 2 in Clause 5.2A.2.1, 5.2A.3.1 and 5.2A.4.1 | CA_C, CA_N, CA_AX | Table 5.1.1.7.5-2 | Largest aggregated CA bandwidth combination | Any of CCs |
| Test 3 in Clause 5.2A.2.1, 5.2A.3.1 and 5.2A.4.1 | CA_AX | Table 5.1.1.7.5-2 | Largest aggregated CA bandwidth combination | TDD CC if supported, otherwise FDD CC |
| NOTE 1: In case CA_AX with different number of X is supported then one or two CA configurations are selected based on procedure from Table 5.1.1.7.2-2. |  |  |  |  |

Table 5.1.1.7.5-2: Selection of CA configurations for CA tests with 8 Rx

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C or CA_N | If the UE support CA configuration that for each CC, supported maximum number of Rx and maximum number of MIMO layers is 8: - Select the CA configurations with the maximum number of CCs, for which the supported maximum number of Rx and MIMO layers is 8. Otherwise: - Select the CA configurations with the maximum number of CCs, conditioned that at least for one CC the supported maximum number of Rx is 8 and for each CC the supported maximum number of MIMO layers is not lower than 2 | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | If the UE support CA configuration that for each CC, supported maximum number of Rx and maximum number of MIMO layers is 8: - Select the CA configurations with the maximum number of CCs, for which the supported maximum number of Rx and MIMO layers is 8. Otherwise: - Select the CA configurations with the maximum number of CCs, conditioned that at least for one CC the supported maximum number of Rx is 8 and for each CC the supported maximum number of MIMO layers is not lower than 2 | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | If the tested UE support CA configuration that for each CC, supported maximum number of Rx and maximum number of MIMO layers is 8: - Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of Rx and MIMO layers is 8. Otherwise: - Select the CA configurations with the largest number of bands and with the maximum number of CCs, conditioned that at least for one CC the supported maximum number of Rx is 8 and for each CC the supported maximum number of MIMO layers is not lower than 2 | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |
| NOTE 1: For CA_AX capability, if CA configuration from step 2 is CA configuration with the largest number of bands then Step 3 and Step 4 are skipped. Otherwise, the two CA configurations selected from Step 2 and Step 4 are used for testing. |  |  |  |  |

Issue 1-3: Applicability rules for different number of RX antenna ports for CA demodulation requirements

- Agreement

Add new section: 5.1.1.7.6 Applicability rule and antenna connection for CA tests with 8 RX

  - Within the CA configuration if any of the PCell and/or the SCells is a 2Rx supported RF band, 2 out of the 8Rx should be connected with data source from system simulator, depending on UE’s declaration and AP configuration. Requirements from Clause 5.2A.2.1 are applied.

  - Within the CA configuration if any of the PCell and/or the SCells is a 4Rx supported RF band, 4 out of the 8Rx should be connected with data source from system simulator, depending on UE’s declaration and AP configuration. Requirements from Clause 5.2A.3.1 are applied.

  - Within the CA configuration if any of the PCell and/or the SCells is a 8Rx supported RF band, all 8Rx should be connected with data source from system simulator. Requirements from Clause5.2A.4.1 are applied.

  - For 8Rx capable UEs, the 2Rx supported RF bands, 4Rx supported RF bands and 8Rx supported RF bands are up to UE’s declaration.

Issue 1-4: Specification structure to capture 8Rx CA requirements

- Agreement

  - Create a new clause 5.2A.4 in TS 38.101-4

  - Include requirements for Rank 2 and Rank 8 in separate tables for FDD and TDD respectively

  - Add the following notes in the new tables for FDD and TDD 8Rx CA requirements

    - NOTE X: For CA combinations between 8Rx and 4Rx or 2Rx, Rank 2 requirements in Tables 5.2A.4.1-1 and 5.2A.4.1-2 shall be applied for both CCs.

    - NOTE Y: For CA Combinations with two 8Rx CCs, Rank 8 requirements in Tables 5.2A.4.1-1 and 5.2A.4.1-2 shall be applied for both CCs.

2 Single carrier requirements

Issue 2-1: Antenna correlation for 8Rx Rank 4 and Rank 8

- Agreement

  - No further discussion on the Medium antenna correlation selection for Rank 4 and Rank 8 for 8Rx tests

Issue 2-2: How to align the ideal results alignment

- Proposals

  - Option 1: Remove the farthest outlier from the average results (the methodology used from NR BS Rel-15)

  - Option 2: Set the max allowed span to 3dB

Issue 2-3: Margin to be added on top of the averaged impairment results for requirements derivation

- Proposal

  - Rank2 and Rank8

    - Add 0.8dB margin for 64QAM (the methodology used from NR UE Rel-15)

  - Rank4

    - Option 1: Add 0.8dB margin for 64QAM (the methodology used from NR UE Rel-15)

    - Option 2: Add 1.5dB margin for 64QAM

    - Option 2: Define two sets of requirements applicable to UE support of maxNumberMIMO-LayersPDSCH = 8 and maxNumberMIMO-LayersPDSCH = 4 respectively

      - Set A (maxNumberMIMO-LayersPDSCH=8): Add 0.8dB margin (the methodology used from NR UE Rel-15)

      - Set B (maxNumberMIMO-LayersPDSCH=4): Add an additional margin on top of Set A results (e.g., 1.2dB); i.e., When UE supports maxNumberMIMO-LayersPDSCH=4, an additional x dB margin is added to the SNR requirement

3 CA requirements

Issue 3-1: Antenna correlation for carrier with Rank 2 in 8Rx CA test

- Proposal

  - Option 1: Revisit Rank 2 to TDLA30-10 Low

  - Option 2: Keep TDLC300-100 ULA Medium B ( = 0.3,  = 0.005154) that is same as Rank 2 single carrier test

  - Note: Interested companies can provide simulation results with both channel conditions TDLA30-10 Low and TDLC300-100 ULA Medium B ( = 0.3,  = 0.005154) for the next RAN4#110 meeting

Issue 3-2: Number of HARQ process for 8Rx CA test

- Proposal

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 8 |
|  | TDD PCell | Option 1: 8 Option 2: For CC with Rank 2: 10 For CC with Rank 8: 8 Other options not precluded | 8 |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | 4 | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | Option 1: 8 Option 2: For CC with Rank 2: 10 For CC with Rank 8: 8 Other options not precluded | N/A |

Issue 3-3: K1 value for 8Rx CA test

- Proposal

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | Option 1: For CC with Rank 2: {8,7,6,5,5,4,3,2} For CC with Rank 8: {8,7,6,5,5,4,3} Option 2: For CC with Rank 2: {8,7,6,5,5,4,3,11} For CC with Rank 8: {8,7,6,5,5,4,3} Other options not precluded | {7,5,4,11,9} |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | {2} | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | Option 1: For CC with Rank 2: {8,7,6,5,5,4,3,2} For CC with Rank 8: {8,7,6,5,5,4,3} Option 2: For CC with Rank2: {8,7,6,5,5,4,3,11} For CC with Rank 8: {8,7,6,5,5,4,3} Other options not precluded | N/A |

4 SDR and CQI requirements

Issue 4-1: Agenda items for SDR and CQI

- Agreement

  - Confirm no more open issues for 8Rx SDR and CQI tests.

  - No further contributions are expected for these AI’s

Annex A: Simulation assumptions (for information)

A.1 PDSCH

Other common parameters, refer to TS 38.101-4: Table 5.2-1: Common test parameters for PDSCH

Table 1: Simulation assumptions for PDSCH performance

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD, FDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  |  | TDD: 30/40 FDD: 15/10 |
| MIMO layer and Antenna configuration |  |  | Rank 2: 2x8 Rank 4: 4x8 Rank 8: 8x8 |
| Propagation condition and antenna correlation |  |  | Rank 2: TDLC300-100 ULA Medium B ( = 0.3, = 0.005154) Rank 4: TDLA30-10 Low Rank 8: TDLA30-10 Low |
| MCS |  |  | Rank2: MCS19 Rank 4: MCS17 Rank 8: MCS17 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) for DL |  | Rank2/4: Single-symbol Rank 8: Double-symbol |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| NZP-CSI-RS configuration |  |  | Rank 8: row 6 |
| Overhead |  |  | 0 |
| N1 and N2 configurations for 8Tx cases |  |  | Use (N1,N2) = (4,1), (O1, O2) = (4,1) |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 8 layers cases |
| Number of HARQ Processes |  |  | TDD: 8 FDD: 4 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Tx EVM (Explicitly modeled in the simulation) |  |  | 64QAM:6% 256QAM:3% |
| Test metric |  |  | SNR@70% max TP |

A.2 SDR

Other common parameters, refer to TS 38.101-4:

- Table 5.5A-1: Common test parameters for FDD and TDD component carriers

- Table 5.5A-2: Additional test parameters for FDD CC

- Table 5.5A-3: Additional test parameters for TDD CC

Table 2: Simulation assumptions for SDR performance

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband |
|  | Resource allocation type |  | Type 0 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) DL |  | Rank2/4: Single-symbol Rank 8: Double-symbol |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs {1000 – 1007} for 8 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 2 layers CCs 2 for others |
| PTRS configuration |  |  | PTRS is not configured |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity chosen avoid zeros on Rx |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Tx EVM |  |  | Tx EVM = 6% for up to 64QAM Tx EVM = 3% for 256QAM Tx EVM = 2.5% for 1024QAM |
| MIMO layers |  |  | 64QAM and 256QAM: 8 1024QAM: 2 |
| Antenna configuration | 2 layers |  | 2x8 |
|  | 4 layers |  | 4x8 |
|  | 8 layers |  | 8x8 |

A.3 CQI test

Other common parameters, refer to TS 38.101-4:

- Table 6.2.2.1.1.1-1: CQI reporting definition test for FDD

- Table 6.2.2.2.1.1-1: CQI reporting definition test for TDD

Table 3: Simulation assumptions for CQI test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Duplex Mode |  |  |  | TDD, FDD |  |  |  |
| Bandwidth/SCS |  |  | MHz | TDD: 40MHz/30kHz FDD: 10MHz/15kHz |  |  |  |
| TDD UL-DL pattern |  |  |  | 7D1S2U S=6D+4G+4U |  |  |  |
| SNR |  |  | dB | 4 | 5 | 10 | 11 |
| Propagation channel |  |  |  | static channel model with 4T8R |  |  |  |
| Antenna configuration |  |  |  | XP 4×8 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 in 38.101-4 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | TDD: 10/1 FDD: 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,(0) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | TDD: 10/1 FDD: 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | TDD: 10/1 FDD: 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | TDD: 16 FDD: 8 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | TDD: 10/9 FDD: 5/0 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | two-one-TypeI-SinglePanel-Restriction |  | 00000001 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | TDD: 9.5 FDD: 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Number of HARQ Processes |  |  |  | FDD: 4 TDD: 8 |  |  |  |
| Measurement channel |  |  |  | TDD: As specified in Table A.4-3, TBS.3-4 FDD: As specified in Table A.4-1, TBS.1-3 |  |  |  |

Annex B: CR split

B.1 CR work split for 8Rx single carrier test in

| Section | Test cases | Companies |
| --- | --- | --- |
| 5.1.1.2 Applicability of requirements for different number of RX antenna ports | Table PDSCH/PDCCH/PBCH test applicability rules for 8Rx | Nokia |
| 5.2.4 8RX requirements (New) | 5.2.4.1 FDD PDSCH performance requirements | Samsung |
|  | 5.2.4.2 TDD PDSCH performance requirements |  |
| 5.5A Sustained downlink data rate provided by lower layers | SDR tests | Ericsson |
| 6.1.1.2 Applicability of requirements for different number of RX antenna ports | Table 6.1.1.2-1: Requirements applicability for CSI applicabaility rules | Huawei |
| 6.2.4 8RX requirements (New) | 6.2.4.1 FDD CQI requirements | CTC |
|  | 6.2.4.2 TDD CQI requirements |  |
| A.3.2 Reference measurement channels for PDSCH performance requirements | A.3.2.1.1 Reference measurement channels for SCS 15 kHz FR1 | MediaTek |
|  | A.3.2.2.2 Reference measurement channels for SCS 30 kHz FR1 |  |
| B.1 Static propagation condition | B.1.2 UE Receiver with 8Rx (New) | ZTE |
| B.2.3 MIMO Channel Correlation Matrices | MIMO Correlation Matrices using Uniform Linear Array (ULA) | Apple |
| 5.2A.4 8Rx requirements | Demodulation requirements for 8Rx PDSCH CA | Huawei |

B.2 CR work split for 8Rx CA test

| Section | Test case | Company |
| --- | --- | --- |
| 5.1.1.7 | CA applicability rules | Huawei |
| 5.2A.4 (New) | 8Rx CA performance requirements | Nokia, Nokia Shanghai Bell |
| A.3.2.1.1 Reference measurement channels for SCS 15 kHz FR1 | FRC for FDD 15kHz SCS 2 layers (5MHz,10MHz,15MHz,20MHz,25MHz) : MCS19 | Ericsson |
|  | FRC for FDD 15kHz SCS 2 layers (30MHz,35MHz,40MHz,45MHz,50MHz): MCS 19 | China Telecom |
| A.3.2.2.2 Reference measurement channels for SCS 30 kHz FR1 | FRC for TDD 30kHz SCS 2 layers (5MHz,10MHz,15MHz,20MHz,25MHz,30MHz): MCS 19 | Apple |
|  | FRC for TDD 30kHz SCS 2 layers (40MHz,50MHz,60MHz,80MHz,90MHz,100MHz): MCS 19 | Apple |
| A.3.2.1.1 Reference measurement channels for SCS 15 kHz FR1 | FRC for FDD 15kHz SCS 8 layers (5MHz,10MHz,15MHz,20MHz,25MHz) : MCS17 | MTK |
|  | FRC for FDD 15kHz SCS 8 layers (30MHz,35MHz,40MHz,45MHz,50MHz): MCS 17 | Ericsson |
| A.3.2.2.2 Reference measurement channels for SCS 30 kHz FR1 | FRC for TDD 30kHz SCS 8 layers (5MHz,10MHz,15MHz,20MHz,25MHz,30MHz): MCS 17 | Ericsson |
|  | FRC for TDD 30kHz SCS 8 layers (40MHz,50MHz,60MHz,80MHz,90MHz,100MHz): MCS 17 | Samsung |

Annex C: Reference

[1] R4-2220613, WF on 8RX UE demodulation and CSI requirements, RAN4#105, Huawei, HiSilicon

[2] R4-2302942, WF for 8Rx UE performance requirements, RAN4#106, Huawei, HiSilicon

[3] R4-2305888, WF for 8Rx UE performance requirements, RAN4#106bis-e, Huawei, HiSilicon

[4] R4-2309806, WF for 8Rx UE performance requirements final, RAN4#107, Huawei, HiSilicon

[5] R4-2313877, WF for 8Rx UE performance requirements final, RAN4#108, Huawei, HiSilicon

[6] R4-2316914, WF for 8Rx UE performance requirements final, RAN4#108bis, Huawei, HiSilicon