3GPP TSG-RAN WG4 Meeting #112				                                                                            R4-2411071

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	CATT

Title: 	Discussion on the AIoT LLS for passive devices in RAN4

Agenda item:	8.20.1

Document for:	Approval

- Introduction

The AIoT co-existence simulation is on-going. SINR is agreed to be used for the performance metric for calibration purpose. However, SINR threshold is not decided yet. Usually SINR threshold relies on LLS, but at current stage, LLS may need more inputs and discussion to conclude. It should be discussed how to handle this issue in the limited timeline. As the co-existence simulation focuses on passive devices, this contribution only discusses RAN4 LLS for passive devices, i.e. Device 1 and Device 2a.

- Discussion

  - RAN1 LLS status

LLS discussion is still ongoing in RAN1. The progress can be reviewed to see if RAN4 can refer or borrow some agreements from RAN1. The following is the summary for RAN1 LLS status until RAN1#117.

========================================

For R2D link in the coverage evaluation,

- Budget-Alt1 is used if receiver architecture is RF ED.

  - Note: RF ED is used by device 1, device 2a and device 2b with RF ED receiver.

- Budget-Alt2 is used if receiver architecture is IF/ZIF ED (Only device 2b).

For D2R link in the coverage evaluation,

- Budget-Alt2 is used.

========================================

So for R2D, no LLS is conducted in RAN1 for passive devices at current stage. For D2R, LLS is conducted for both passive devices and active devices. Based on the RAN1 progress, the R2D and D2R LLS consideration in RAN4 should be considered separately.

Observation 1: Regarding RAN1 LLS status, no R2D LLS for passive devices is conducted, D2R LLS for both passive devices and active devices is planned.

  - RAN4 R2D LLS consideration for passive devices

For the R2D LLS, the most difficult part is to model the ED architecture. Figure 1 shows a typical ED + comparator architecture.

Figure 1: Typical ED + comparator architecture

There’re some digital models for ED in simulation. One of the models for envelope detector is to use squaring and low pass filtering as show in the Figure 2.

abs()^2	LPF

Figure 2: Squiring + LPF model for ED

This may be sufficient for the case of the ASK signal like (1+sin(x))*sin(y) if the frequency sin(y) is much higher than sin(x). However, if the AIoT waveform is different than the ASK, this model may not work well. Hilbert Transform is another digital ED model, but it seems it’s also not perfect for some waveform. Figure 3 is copied from [2]. The waveform in Figure 3 was simulated in Cadence, it’s more realistic compared with the RF component performance.

Figure 3: A simulated waveform for ED and comparator using Cadence, copied from [2]

As shown in Figure 3, if the waveform is OOK with single tone, ED can’t detect the envelope perfectly especially for for the OOK ‘0’ when negative clipping happens. ED has some limitations such as ripples and negative clipping. The time constant τ = RC determines the discharging time of the capacitor. The comparator output is also impacted by the ED limitations. This performance is not modelled in the ED model in Figure 2, especially for the capacitor discharging behavior which impacts the comparator’s output. When the waveform is OFDM based OOK waveform, the envelope detector behavior may be different compared with the single tone OOK due to the OFDM random power. One candidate model can be that the average power is supposed to be detected by the ED because detecting the peak power is not correct. However, the negative clipping is not modeled in this method either. There’s another drawback for the average method that averaging also make the noise power averaged. Then SNR is incorrectly improved because the noise power value is closer to the variance if more sampling points are averaged. However, Figure 1 clearly shows that the SNR may not be improved because of the ED limitations especially at the time of the negative clipping. So a more reasonable way is to do some measurement using the AIoT waveform. Similarly the comparator may also not be so ideal like a “if else” decision, the detection threshold is also need to be discussed and decided. So we have the following proposal.

Proposal 1: RAN4 should discuss and decide the ED and comparator model when the R2D LLS is started. The detection threshold is also need to be discussed and decided.

Observation 2: Some measurement for the ED and comparator using AIoT waveform brings benefit for the simulation model.

RAN1 is still discussing the details of the OFDM-based OOK waveform generation. From figure 3, the ED output may be related to the waveform design even the average power is the same value. So it’s better to begin the R2D LLS after the waveform design in RAN1 is fixed.

Observation 3: It’s better to start the R2D LLS after the waveform design in RAN1 is fixed.

The R2D LLS may not be that urgent for defining RF requirements. There may be some issue for co-existence simulation if SINR threshold is not decided. It can’t be decided if the devices is outage or not. Considering there’s only 3 meetings left for SI, even LLS is started in Oct. meeting, the conclusion for LLS may be a little late to meet the timeline for co-existence simulation. There could be another choice for the AIoT R2D performance metric that the SINR loss is used for the performance metric without considering the outage of the devices. If this approach can be used should be discussed and decided in RAN4.

Proposal 2: SINR loss can be considered as a candidate for the AIoT R2D performance metric for RAN4 co-existence simulation before the SINR threshold is decided.

How to use the SINR loss can be discussed further, LTE NB-IoT co-existence simulation [3] uses 5%, 50%, 95% and 99% SNR loss. The similar approach can be a candidate.

  - RAN4 D2R LLS consideration for passive devices

In order to have good REFSENS in reader side, it’s assumed that the Rx path for reader has higher performance compared with ED architecture. It can be assumed that BPF, ADC with sufficient sampling rate, bit width, digital filter, etc are used at reader side. So from reader Rx performance aspect, reader requirements can be analysed similar with NR BS except the BB OOK modulation part. All of the noise outside the BW of wanted signal can use the similar analysis approach with NR Rx path. Then RAN1 D2R LLS conclusion can be reused by RAN4.

Observation 4: RAN4 can reuse RAN1 D2R LLS conclusion for the SINR performance.

There may be one issue that there’re only 3 meetings left for SI, RAN1 LLS conclusion may be a little late to be used by RAN4 co-existence simulation. So there can be another choice like R2D link that SINR loss is used.

Proposal 3: If the timeline is allowed, RAN1 D2R LLS SINR conclusion is used by RAN4 D2R link. If it’s difficult to meet the timeline, SINR loss can be considered to be used as the performance metric.

There’s another issue needs to be noticed that Tx leakage performance for backscattering devices is different with the NR/LTE UE. Theoretically, Fourier transform of rectangular function in spatial domain is sinc function in frequency domain. A typical sinc function in logarithmic scale is plotted in Figure 3,

Figure 4: Fourier transform of rectangular function in spatial domain

The backscattering devices can’t use pulse shaping filter to improve spectrum emission outside the signal BW. The exact Tx leakage performance may be related to other aspects, such as antenna performance, impedance matching, etc. Some measurement for the passive devices Tx leakage brings benefit to the analysis.

Observation 5: Some measurement for the passive devices Tx leakage brings benefit to the analysis.

- Conclusion

This contribution provides our consideration for RAN4 LLS for the SINR threshold. We have the following proposals and observation.

Observation 1: Regarding RAN1 LLS status, no R2D LLS for passive devices is conducted. D2R LLS for both passive devices and active devices is planned in RAN1.

Proposal 1: RAN4 should discuss and decide the ED and comparator model when the R2D LLS is started. The OOK detection threshold is also need to be discussed and decided.

Observation 2: Some measurement for the ED and comparator using AIoT waveform brings benefit for the simulation model.

Observation 3: It’s better to start the R2D LLS after the waveform design in RAN1 is fixed.

Proposal 2: SINR loss can be considered as a candidate for the AIoT R2D performance metric for RAN4 co-existence simulation before the SINR threshold is decided.

Observation 4: RAN4 can reuse RAN1 D2R LLS conclusion for the SINR performance.

Proposal 3: If the timeline is allowed, RAN1 D2R LLS SINR conclusion is used by RAN4 D2R link. If it’s difficult to meet the timeline, SINR loss can be considered to be used as the performance metric.

Observation 5: Some measurement for the passive devices Tx leakage brings benefit to the analysis.

- References

[1] R4-2410567, “WF on co-existence study for ambient IoT and NR/LTE”, CMCC, RAN4#111

[2] A. Trigui, M. Ali, A. C. Ammari, Y. Savaria and M. Sawan, "Energy Efficient Generic Demodulator for High Data Transmission Rate Over an Inductive Link for Implantable Devices," in IEEE Access, vol. 7, pp. 159379-159389, 2019, doi: 10.1109/ACCESS.2019.2950622.

[3] TR 36.802, E-UTRA NB-IOT; Technical Report for BS and UE radio transmission and reception