3GPP TSG-RAN WG4 Meeting #112                                                                                                      R4-2411122

Maastricht, Netherlands, 19th –23th August, 2024

Source: 	CATT

Title: 	Discussion on SAN RF core requirments for Ku-band for NR NTN

Agenda item:	8.9.5

Document for:	Approval

- Introduction

In this contribution, we provide our views on SAN RF core requirements for Ku-band for NR NTN.

- Discussion

For objective of introduction of Ku Band for NR NTN in WID, we discuss SAN RF core requirements for Ku-band for NR-NTN as below:

SAN type and RF core requirements

As per the WID [1], the objectives concerning SAN RF core requirements for Ku-band is shown as below:

- Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band.

As discussion in our contribution [2], in TS 38.108, the NTN frequency ranges were defined as FR1-NTN and FR2-NTN, frequency range of FR1-NTN and FR2-NTN is 410-7125MHz and 17300-30000MHz respectively.  The frequency range of Ku-band is 10.70-14.5GHz, which is not in FR1-NTN and FR2-NTN. This Ku-band is located between FR1-NTN and FR2-NTN, so it is reasonable to extend FR1-NTN upwards or extend FR2-NTN downwards. In TS38.104, for FR2-NTN, 60,120 kHz SCS were defined, for FR1-NTN, 15,30,60kHz SCS were defined, from numerology point of view, large subcarrier spacing (SCS) can ensure that large Doppler frequency shift do not cause large inter-carrier interference (ICI), for satellite for Ku-band, the  large  subcarrier spacing (SCS) is needed. So we prefer to extend FR2-NTN downwards for Ku-band, for extending FR2-NTN downwards for Ku-band, it is reasonable to define SAN type 2-O for Ku-band.

Proposal 1: Define SAN type 2-O RF core requirements for Ku-band.

After checking the SAN type 2-O requirement in TS 38.108, for OTA reference sensitivity level, it is need to check existing EISREFSENS_50M declared range is applicable for Ku-band, we think that existing EISREFSENS_50M declared range is also applicable for Ku-band. If no other channel bandwidth and SCS will be introduced for Ku-band, the existing SAN RF core requirement for SAN type-2 excluding ACLR, ACS requirement can be applicable for SAN type 2-O for Ku-band.

Observation 1: Existing SAN RF core requirement for SAN type-2 excluding ACLR, ACS requirement can be applicable for SAN type 2-O for Ku-band.

Generally, for ACLR, ACS requirement are dependent on adjacent channel coexistence simulation, for Ku-band, some system parameter, such as Satellite EIRP density, SAN antenna parameter, SAN NF, VSAT antenna, VSAT NF will be different from Ka-band, so adjacent channel coexistence simulation is needed to check whether existing ACLR, ACS for Ka-band is applicable for Ku-band.

Observation 2: Whether existing ACLR ACS for Ka-band is applicable for Ku-band is dependent on adjacent channel coexistence simulation for Ku-band.

- Conclusion

This contribution provides analysis on SAN RF core requirements for Ku-band for NR NTN, and the following proposals and observations are concluded as follows:

Proposal 1: Define SAN type 2-O RF core requirements for Ku-band.

Observation 1: Existing SAN RF core requirement for SAN type-2 excluding ACLR, ACS requirement can be applicable for SAN type 2-O for Ku-band.

Observation 2: Whether existing ACLR ACS for Ka-band is applicable for Ku-band is dependent on adjacent channel coexistence simulation for Ku-band.

- References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104

- R4-2411121, Discussion on system parameters for Ku-band for NR NTN, CATT, RAN4#112