3GPP TSG-RAN4 Meeting #112	R4-2419771

Maastricht, NL, 19th – 23rd August, 2024

Agenda item:	8.12.2.2

Source:	Keysight Technologies

Title:	NTN OTA Test Metrics

Document for:	Approval

# Introduction

This contribution addresses NTN OTA test metrics based on the Rel-19 OTA WI in [1].

# Discussion

A number of suitable performance metrics were proposed by various companies in RAN4#100-bis as captured in the WF [2]:

Issue 3-2-2: Proper performance metric for NR-NTN Way forward: Consider the following initial input as a starting point for further discussions: - Option 1: consider the following aspects - Option 1: integrated power/sensitivity within declared half sphere - Option 2: peak EIRP/EIS only - Option 3: peak EIRP/EIS + X%-tile spherical coverage within declared half sphere - Option 4: peak EIRP/EIS + Y%-tile spherical coverage from whole sphere - Option 2: consider the following aspects - Consider a general framework for each UE type, to specify performance metric for different use cases / power classes, e.g., a set of metric for Handheld, and other set for FR1 VSAT-like UE. - Consider the assumption: UE elevation angles for NTN are typically >20°, in majority of cases. - Consider TRP, TRS, EIRP, and EIS as starting point. Further discuss other performance metric based on NTN usage scenarios, e.g., directivity requirements, Antenna Gain. - Option 3: consider the following aspects - consider WRP and WRS defined in R4-2404278 as performance metrics for NTN devices - consider EIRP and EIS CDF percentile thresholds as performance metrics for NTN devices - Option 4: Conical TRP/EIRP for Tx and conical TRS/EIS for Rx could be considered as the starting point for the metric of NTN device OTA testing. FFS on the range of q and f. - Option 5: Consider the following aspects for NTN performance metric - Adopting a directional antenna pattern as the starting point for the antenna characteristics of an FR1 NTN device. - Take UIRP and UHIS as the as the starting point for the performance metric of devices using directional antenna. - Further discuss whether a smaller angle of test scan can be used or whether a EIRP/EIS CDF-like performance metric and be considered. - Option 6: other aspects are not precluded

In the last meeting, additional discussions on the metrics were held with additional down selections made in the WF [3], i.e.,

Issue 3-2-1: Categorized performance metric for NR-NTN handheld Agreements - RAN4 should not adopt direction 1, other directions are FFS - Direction 1: Only single point performance metric, i.e., Peak EIRP/EIS - Direction 2: Single point + measured full sphere (integrated or CDF), e.g., - D2a, Peak EIRP/EIS+TRP/TRS, - D2b, Peak EIRP/EIS+ full Spherical coverage CDF - Direction 3: Single point + measured partial sphere (integrated or CDF), e.g., - D3a, Peak EIRP/EIS+ partial TRP/TRS (e.g., within selected 30, 60, 90, 180 degree range of angles), - D3b, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles) - Direction 4: Only Full sphere (integrated TRP/TRS or Spherical coverage CDF) with/without weighting, - D4a, Spherical Coverage with EIRP-CDF and EIS-CCDF at [>50%]-ile - D4b, TRP/TRS or Weighted metric, e.g., weighted TRP/TRS (with larger weight for declared hemisphere) - Direction 5: Only Partial sphere (integrated or CDF) (e.g., within selected 30, 60, 90, 180 degree range of angles),

This contribution intends to analyse the select test metrics for simulated and synthesized antenna patterns.

# NTN Antenna Patterns

This contribution is investigating sample antenna patterns for smartphone UEs, i.e., no VSAT-like devices. The first set of patterns were simulated in CST Studio Suite. Here, a full smartphone model including most components was investigated first with the device held in the hand and is shown in Figure 1

Figure 1: CST Studio Suite smartphone model in hand right (HR) talk condition

Two different antenna locations for NTN antennas were investigated, i.e., one at the top (with frequency ~1.6 GHz, n255) and one on the side (with frequency range ~2.1 GHz, n256) as further illustrated in Figure 2.

Figure 2: NTN antennas investigated on top (left) and on side (right)

The simulated antenna patterns are shown in Figure 3.

Figure 3: NTN antennas patterns for antenna location on top (left) and on side (right)

From the patterns shown in Figure 3, it can be observed that the simulated antennas yield fairly omnidirectional patterns even in the HR talk condition. In the remainder of the contribution, the simulated antenna patterns had to be adjusted/rotated to comply with the conventional smartphone UE, e.g., Figure N.1-2 of [4].

Figure N.1-2: DUT default alignment of example smartphone UE to coordinate system

In order to analyse more directive patterns, two additional patterns with higher directivity were synthesized, generally using the sinc(theta) pattern shape. One has a 120° HPBW and the other a 60° HPBW, both with a 15 dB FTBR. All 4 analysed patterns are visualised in Figure 4.

Figure 4: Simulated and synthesized NTN antennas patterns. Top left: simulated smartphone antenna on top, top right: simulated smartphone antenna on right, bottom left: directive antenna with 120° HPBW, bottom right: directive antenna with 60° HPBW

# Analysis of Metrics

This section is investigating the following performance metrics discussed in the last couple meetings:

- Peak EIRP: Maximum total component of EIRP measured on the respective measurement grid

- TRP: Total radiated power, i.e., spatially weighted surface integral of the EIRPs measured on the respective measurement grid

- UHRP: Upper hemisphere radiated power, i.e., spatially weighted surface integral of the EIRPs measured on the respective measurement grid in the upper hemisphere (θ≤°90)

- WRP: weighted radiated power per [6][7], definition in Annex. Parameters used here: Wi=0.5 and threshold of 15dB.

- Weighted TRP: weighted TRP based on the following equation 2/3*UHRP + 1/3*LHRP

- CDF85%: 85th-percentile CDF of EIRPs measured on the respective measurement grid points with spatially normalized PDFs.

The following investigations take into account various device rotations and thus antenna pattern rotations around the y-axis of the smartphone. Select device rotations of the smartphone in the hand right talk condition are visualized in Figure 5.

Figure 5: Visualization of various device orientations/rotations around the y-axis.

It is clearly understood that for certification testing, just a single orientation is selected, i.e., any variability w.r.t. device orientation is irrelevant. OEMs commented in the last meeting that “NTN communication is important for emergency and life critical scenarios” and specifically that there is a “software assistant function to guide user how to hold the UE” [8] or “for NTN devices it could be feasible to assume that a connection manager assists the user with positioning the device to obtain a higher quality signal” [10]. In other words, OEMs suggest that in emergency situations, the optimal device orientation could be suggested to the user using device prompts which supports the idea that any performance metric should be independent on device orientation as the optimal device orientation could be declared by the OEMs.

Observation 1: OEMs suggest that in emergency situations, the optimal device orientation could be suggested to the user using device prompts

On the other hand, satellite operators suggested that any device orientation could be possible [11], i.e.,

- For handheld/smartphone UE – UE may be upside-down when a call is initiated (e.g. in pocket): - If we assume only synchronization and paging typically occur when UE is upside down, we can still sacrifice some antenna performance on the bottom hemisphere, due to lower SNR requirements for the physical channels associated with these procedures. - When data transfer is occurring– e.g. PUSCH/PDSCH and the UE is pointing upwards, it is advantageous to have a higher antenna performance in the upper hemisphere to allow for a higher link margin. - This would also reduce the antenna noise temperature, meaning the system noise is less significant, which increases G/T, increasing the DL link margin, while increasing the UL margin due to higher gain towards the satellite orbital arc. - Assessing the directivity of the antenna can be done with a similar effort level to obtaining the TRP, TRS and EIS values, and would allow for more detailed link level analysis.

Observation 2: Satellite operators suggest that in emergency situations, the device orientation could be arbitrary

Given these contrary views, the intent of the following performance metric analyses is to determine the variability of these metrics as a function of various device/pattern rotations and two different measurement grids, i.e., a constant-step size grid with Dq=Df=5° (fine) and 15° (coarse, default). Ideally, the selected metric should show rather limited dependence on the device orientations for different NTN antenna patterns so that the metric demonstrates general applicability.

Observation 3: Ideally, the selected metric should show rather limited dependence on the device orientations

The results for the different metrics as a function of pattern rotation angles and measurement grids are tabulated in Table 1 through Table 6.

Table 1: Deviation of Peak EIRP for various NTN antenna patterns as a function of device orientations and measurement grids

| Pattern → | NTN (Top) |  | NTN (Side) |  | Directive 120° HPBW |  | Directive 60° HPBW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grid [°] Rotation [°] | 5 | 15 | 5 | 15 | 5 | 15 | 5 | 15 |
| 0 | 0.00 | -0.02 | 0.00 | -0.07 | 0.00 | 0.00 | 0.00 | 0.00 |
| 15 | -0.01 | -0.02 | -0.01 | -0.10 | 0.00 | 0.00 | 0.00 | 0.00 |
| 30 | 0.00 | -0.01 | 0.00 | -0.07 | 0.00 | 0.00 | 0.00 | 0.00 |
| 45 | 0.00 | -0.03 | -0.01 | -0.06 | 0.00 | 0.00 | 0.00 | 0.00 |
| 90 | 0.00 | -0.02 | -0.01 | -0.04 | 0.00 | 0.00 | 0.00 | 0.00 |
| 135 | 0.00 | -0.07 | -0.01 | -0.01 | 0.00 | 0.00 | 0.00 | 0.00 |
| 180 | 0.00 | -0.02 | 0.00 | -0.07 | 0.00 | 0.00 | 0.00 | 0.00 |

Table 2: Deviation of TRP for various NTN antenna patterns as a function of device orientations and measurement grids

| Pattern → | NTN (Top) |  | NTN (Side) |  | Directive 120° HPBW |  | Directive 60° HPBW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grid [°] Rotation [°] | 5 | 15 | 5 | 15 | 5 | 15 | 5 | 15 |
| 0 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | 0.00 |
| 15 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 30 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 45 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 90 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 135 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 180 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | 0.00 |

Table 3: Deviation of UHRP for various NTN antenna patterns as a function of device orientations and measurement grids

| Pattern → | NTN (Top) |  | NTN (Side) |  | Directive 120° HPBW |  | Directive 60° HPBW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grid [°] Rotation [°] | 5 | 15 | 5 | 15 | 5 | 15 | 5 | 15 |
| 0 | 0.00 | 0.01 | 0.00 | -0.01 | 0.00 | -0.02 | 0.00 | 0.01 |
| 15 | -0.05 | -0.05 | -0.30 | -0.30 | -0.07 | -0.09 | 0.03 | 0.04 |
| 30 | -0.09 | -0.09 | -0.65 | -0.66 | -0.26 | -0.28 | 0.09 | 0.08 |
| 45 | -0.16 | -0.16 | -1.08 | -1.08 | -0.60 | -0.62 | 0.05 | 0.03 |
| 90 | -0.51 | -0.50 | -2.50 | -2.48 | -2.53 | -2.53 | -2.37 | -2.37 |
| 135 | -0.29 | -0.29 | -3.39 | -3.35 | -6.08 | -6.02 | -8.33 | -8.18 |
| 180 | 0.08 | 0.07 | -2.50 | -2.49 | -9.29 | -9.15 | -8.00 | -8.09 |

Table 4: Deviation of WRP for various NTN antenna patterns as a function of device orientations and measurement grids

| Pattern → | NTN (Top) |  | NTN (Side) |  | Directive 120° HPBW |  | Directive 60° HPBW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grid [°] Rotation [°] | 5 | 15 | 5 | 15 | 5 | 15 | 5 | 15 |
| 0 | 0.00 | 0.00 | 0.00 | 0.00 | -0.03 | -0.17 | -0.04 | 0.05 |
| 15 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | -0.04 | 0.00 | 0.01 |
| 30 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | -0.03 | 0.01 | 0.01 |
| 45 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | -0.04 | 0.00 | 0.03 |
| 90 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | -0.05 | -0.01 | 0.01 |
| 135 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | -0.04 | 0.00 | 0.03 |
| 180 | 0.00 | 0.00 | 0.00 | 0.00 | -0.03 | -0.17 | -0.04 | 0.05 |

Table 5: Deviation of Weighted TRP for various NTN antenna patterns as a function of device orientations and measurement grids

| Pattern → | NTN (Top) |  | NTN (Side) |  | Directive 120° HPBW |  | Directive 60° HPBW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grid [°] Rotation [°] | 5 | 15 | 5 | 15 | 5 | 15 | 5 | 15 |
| 0 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.03 | 0.00 | -0.03 |
| 15 | -0.02 | -0.02 | -0.04 | -0.04 | 0.13 | 0.16 | -0.05 | -0.07 |
| 30 | -0.03 | -0.03 | -0.11 | -0.12 | 0.41 | 0.43 | -0.15 | -0.14 |
| 45 | -0.06 | -0.06 | -0.24 | -0.24 | 0.67 | 0.68 | -0.08 | -0.05 |
| 90 | -0.19 | -0.19 | -0.83 | -0.82 | 0.57 | 0.57 | 0.29 | 0.29 |
| 135 | -0.11 | -0.11 | -1.28 | -1.26 | -1.16 | -1.12 | -2.87 | -2.78 |
| 180 | 0.03 | 0.02 | -0.83 | -0.83 | -3.09 | -3.01 | -2.67 | -2.73 |

Table 6: Deviation of 85th-percentile CDF for various NTN antenna patterns as a function of device orientations and measurement grids

| Pattern → | NTN (Top) |  | NTN (Side) |  | Directive 120° HPBW |  | Directive 60° HPBW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grid [°] Rotation [°] | 5 | 15 | 5 | 15 | 5 | 15 | 5 | 15 |
| 0 | 0.01 | 0.03 | 0.06 | 0.11 | 0.06 | 0.07 | 0.31 | 0.41 |
| 15 | 0.00 | 0.07 | 0.06 | 0.18 | 0.06 | 0.06 | 0.35 | 0.32 |
| 30 | 0.00 | 0.01 | 0.06 | 0.28 | 0.07 | 0.08 | 0.41 | 0.41 |
| 45 | 0.00 | 0.06 | 0.05 | 0.19 | 0.08 | 0.08 | 0.44 | 0.43 |
| 90 | -0.01 | 0.04 | 0.02 | 0.13 | 0.09 | 0.06 | 0.48 | 0.35 |
| 135 | -0.01 | 0.03 | -0.02 | -0.04 | 0.08 | 0.08 | 0.44 | 0.43 |
| 180 | 0.00 | 0.01 | -0.04 | -0.09 | 0.05 | 0.06 | 0.29 | 0.36 |

From these results, the following observations can be made:

- The peak EIRP and TRP, as expected, have very little variability in terms of pattern rotations and measurement grids

- Similarly, the WRP metric shows very little variability regardless of orientations, grids, and patterns

- The UHRP and weighted TRP metric generally shows strong variability as a function of device orientation and pattern, especially for directive patterns

- The CDF metric shows relatively small variability for realistic (simulated) NTN antenna patterns with different

Observation 4: While the peak EIRP, TRP, and WRP metrics generally show no variability in terms of device orientations and NTN antenna patterns, the CDF metric shows generally small variability and the UHRP and weighted TRP metrics show strong variability.

For metrics not to be too dependent on device orientations, it is proposed to no longer focus on partial radiated powers and/or weighted TRPs.

Proposal 1: No longer consider partial radiated power and/or weighted TRPs but focus on TRP, WRP, or 85th %-ile CDF metrics with optional peak EIRP. Consider similar metrics for the RX measurements.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: OEMs suggest that in emergency situations, the optimal device orientation could be suggested to the user using device prompts

Observation 2: Satellite operators suggest that in emergency situations, the device orientation could be arbitrary

Observation 3: Ideally, the selected metric should show rather limited dependence on the device orientations

Observation 4: While the peak EIRP, TRP, and WRP metrics generally show no variability in terms of device orientations and NTN antenna patterns, the CDF metric shows generally small variability and the UHRP and weighted TRP metrics show strong variability.

Proposal 1: No longer consider partial radiated power and/or weighted TRPs but focus on TRP, WRP, or CDF metrics with optional peak EIRP. Consider similar metrics for the RX measurements.

# References

- RP‑240841, New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, vivo, CAICT, RAN meeting #103, Mar 2024

- R4-2406086, Way Forward for [110bis][338] TRP_TRS_MIMO_OTA, vivo, CAICT, 3GPP TSG-RAN WG4 Meeting #110bis, April 2024

- R4-2409931, WF for [111][338] TRP_TRS_MIMO_OTA, vivo, 3GPP TSG-RAN WG4 Meeting #111, May 2024

- TS 38.101-2, User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone

- TS 38.101-5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements

- R4-2402875, WF for [110][331] NR_FR1_TRP_TRS_enh, vivo, 3GPP TSG-RAN WG4 Meeting #110, March 2024

- R4-2404278, on test methodology and radiated performance metric for FR1 NTN devices, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting # 110bis, April 2024

- R4-2409771, NTN OTA Testability Aspects, Keysight Technologies, 3GPP TSG-RAN4 Meeting #111, May 2024

- R4-2407897, Discussion on NTN UE OTA test, Samsung, 3GPP TSG-RAN WG4 Meeting #111, May 2024

- R4-2407060, On NTN TRP TRS methodology, Apple, 3GPP RAN4 Meeting #111, May 2024

- RP-240615, Satellite Operator views on OTA requirements for NTN, Inmarsat, Viasat, RAN#103

# Annex: Definition of WRP

- Measure radiated power for all 4 TPMI values, namely TPMI 2, 3, 4 and 5 at all grid points. The measurement can be done either for a single TPMI for all grid points or repeat all 4 TPMI per grid point. The aim is to make available the radiated power values at all grid points for all 4 TPMI indexes.

- Define an EIRP threshold for the total component EIRPtotal,threshold, above which the weighting value Wi is assigned to a default value of Wi=1.

EIRPtotal,threshold is defined to be [-10 to -15] dB below the peak of the respective radiation pattern (total component)

- Calculate Weighted Radiated Power (WRP) using equation (1) and (2) with the above weights Wi

WRP= 1Kθ=0πϕ=02πWiEIRPθθ,ϕ+EIRPϕθ,ϕsinθdϕdθ

where Wi denotes weight at each grid point and K is given below

K= θ=0πϕ=02πWisinθdϕdθ