3GPP TSG-RAN WG4 Meeting #112                           R4-2411411

Maastricht, NL, 19th August- 23th August, 2024

Agenda Item:	8.17.4

Source:	Apple

Title:	Discussion on testability and interoperability issues for CSI Compression and Prediction

Release:       Rel-19

Document for:	Discussion

# Introduction

Rel-19 Work Item (WI) was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (WID in [1]). The application of AI/ML techniques to NR air interface has been studied in FS_NR_AIML_Air.

This work item provides normative support for the general framework of AI/ML concerning air interfaces. It also enables the implementation of recommended use cases outlined in the previous study. Furthermore, several study objectives within this project aim to address outstanding issues identified during the study, with the goal of enhancing understanding in preparation for future normative effort.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [2]

In this contribution, we provide our viewpoints on some of the interoperability and testability aspects for AI/ML for NR air interface for CSI compression and prediction.

# Discussion

# 2.1 Testability and interoperability issues for CSI compression

In R4-2410570 it was agreed to study the feasibility of the test decoder for Option 3.

2.1.1 Agreements about the parameters used in the simulation for checking the feasibility of testing options for CSI compression

The following agreement was reached: Parameters agreed are just for the feasibility study of testing options.

If/when RAN4 discusses requirement definition, RAN4 will define a new test decoder which may or may not reuse any of the parameters agreed in the feasibility study.

2.1.2 Agreements regarding the process for checking the feasibility of deriving a test decoder for Option 3

Test decoder derivation procedure:

Option 1

Companies bring encoder + decoder set based on agreed parameters. RAN4 chooses one of the decoders and interested companies further check if an encoder can be trained with this decoder to obtain similar performance/complexity (or other evaluation criteria)

Option 2

Companies bring training data for encoder + decoder pair, interested companies train an encoder + decoder pair based on the aggregated dataset from all companies to check decoder derivation feasibility and performance/complexity (or other evaluation criteria)

RAN4 agrees to work on option 1.

Interested companies are invited to bring further feasibility analysis for Option 2

2.1.3 Simulation parameters for verifying the feasibility of aligning model among companies

The following simulation parameters have been agreed:

System-level simulation parameters (from TR):

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

Based on the two rounds of discussions, the following parameters were agreed for the AI models:

Fig 1. Detailed model diagrams of encoder.

Fig 2. Detailed model diagrams of decoder.

System parameters:

| Frequency range | 2GHz (4GHz optional) |
| --- | --- |
| MIMO Layers | 1 layer (2 layers optional, companies to |
| Sub-bands | 13 |
| Channel estimation | ideal |
| Metrics | SGCS of AI CSI feedback and eTypeII CSI feedback, both relative to ideal CSI. |

2.1.4 Simulation results for the agreed test decoder for Option 3

We implemented the test decoder in Pytorch. The training parameters (and channel type) are shown in the table below:

| Training parameters | Encoder/Decoder |
| --- | --- |
| Optimizer | Adam |
| Batch size | 256 |
| Loss function | 1-SGCS |
| Stopping Criterion | No change in metric over a given number of epochs |
| Max Epochs | 100 |
| Learning Rate | 0.0025 |
| Dataset size | 600K |
| Quantization of Training Data | Not quantized (Float 32) |
| Channel Type | UMa |
| Weight Initialization | Default |

Table 1. Training parameters

The complexity and size of both Encoder and Decoder were profiled with the Pytorch model in inference mode. The results of the profiling are shown below:

| Complexity (MACs) | Complexity (FLOPS) | Number of Parameters | Memory Size | Model |
| --- | --- | --- | --- | --- |
| 47MMacs | 93MFlops | 125795 | 491.3KBytes | Encoder |
| 740.3MMacs | 1480MFlops | 1806951 | 7058.4KBytes | Decoder |

Table 2. Computational Complexity and Memory Size

We report the validation loss of the AI/ML model in terms of SGCS and we compare it with the SGCS loss of the e-typeII codebook with configuration 1. The results are shown below and captured as an observation:

Observation 1: Simulation results comparing SGCS between AI/ML and e-typeII are captured below:

| SGCS | Encoder/Decoder |
| --- | --- |
| 0.78 | AI/ML (64 bits) |
| 0.747 | e-type II (configuration 1, 62 bits) |

Table 3. Simulation results for AI/ML and e-typeII codebook

2.1.5 Updated diagram feasibility analysis for Option 3

We provide an updated flowchart diagram for the procedures to follow for testing the feasibility of the test decoder for Option 3.

Figure 3. Flowchart of procedures for feasibility analysis of Option 3 Test decoder

For aiding the performance alignment of the simulation results reported by each company, each company should list the training parameters that the encoder, decoder pair was trained with.  As a next step we could employ data sharing as shown in step 8.

Proposal 1: For aiding the performance alignment of the simulation results reported by each company, each company should list the training parameters used for the encoder-decoder pair. An example of training parameters is shown below:

- Optimizer

- Batch size

- Loss function

- Stopping Criterion

- Max Epochs

- Learning rate

- Dataset size

- Quantization of training data

- Channel type

- Weight initialization

Additionally, data sharing (Step 8) could be employed:

- Company A trains its encoder/decoder with company’s B dataset.

- Company B tests the encoder decoder pair with company’s A testing data

Proposal 2: RAN4 to employ the procedures shown in the flowchart for studying the feasibility analysis of test decoder Option 3.

We further elaborate on the steps described in the flowchart. We need to agree on the criteria to determine if feasibility is met. A test decoder that delivers marginal throughput performance compared to e-type II under ideal conditions (unquantized training data, high SNR) should be considered not feasible. A test decoder with better performance should be considered for feasibility analysis (loopback to step 1). Another aspect of feasibility concerns the alignment of all the UE preferred encoder with the chosen test decoder (step 6).

Proposal 3: The following criteria should establish the feasibility of the test decoder:

- The test decoder (along with the encoder) should deliver reasonable throughput performance gain compared to e-type II in ideal conditions. RAN4 to extend the feasibility analysis to higher ranks and different payload sizes to determine consistency of performance improvement. FFS on how to quantify the acceptable performance gain. (performance-complexity tradeoff)

- All UE preferred encoders should be aligned (work well) with the chosen test decoder

Therefore, the feasibility should entail both a performance and alignment aspect.

To avoid extensive throughput simulation results at this point it would be beneficial to derive an empirical model that maps SGCS to some metric of throughput (spectral efficiency)

Proposal 4: To determine if the performance aspect of the feasibility analysis in throughput is met, RAN4 should investigate an empirical model that maps SGCS to a throughput metric, such as spectral efficiency.

We will also need to agree on how to determine the alignment aspect.

Proposal 5: The following principles should guide the establishment of the alignment aspect of the feasibility analysis (steps 6-7):

- A well-trained UE encoder with the same or higher complexity compared to the Ref encoder that the test decoder was trained with, should have at least similar performance with the reference performance. (in throughput)

- FFS on how to define a criterion for the similarity/dissimilarity of all companies' performance results with respect to the reference performance, considering UE encoder complexity as well.

At step 5 of the flowchart, we will need to choose the test decoder (among all companies’ test decoders) to proceed with the feasibility analysis. We will need a criterion to select a test decoder

Proposal 6: RAN4 to define a criterion to select a test decoder among all the test decoders reported after performance alignment is achieved (step 5). The criterion could be based upon:

- Test decoder trained with the highest number of samples

- Each company tests its pair of {Encoder, test decoder) with other companies testing data set. Chose the pair that has the best performance

- Others

According to the agreed test decoder derivation procedure:

Option 1

Companies bring encoder + decoder set based on agreed parameters. RAN4 chooses one of the decoders and interested companies further check if an encoder can be trained with this decoder to obtain similar performance/complexity (or other evaluation criteria)

Option 2

Companies bring training data for encoder + decoder pair, interested companies train an encoder + decoder pair based on the aggregated dataset from all companies to check decoder derivation feasibility and performance/complexity (or other evaluation criteria)

We are merging the two options in one flowchart. First, we evaluate the feasibility analysis of Option 1 (steps 1-7) and if Option 1 is not feasible then we investigate Option 2 (steps 8-13).

Proposal 7: According to the flowchart, RAN4 should first evaluate feasibility analysis with Option 1 and proceed to Option 2 if Option 1 does not produce a feasible test decoder

## 2.2 Generalization/scalability aspects

The performance of generalization is of importance for a successful AI/ML feature deployment and the necessity and feasibility of defining requirements or test to verify the generalization of AI/ML has been studied. The content below from the TR discusses achieving minimum performance under certain conditions while ensuring that performance is not significantly degraded under other conditions Accordingly for the generalization verification aspects, the following contents are agreed to be captured in TR 38.843 [1]:

The goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations. The following aspects should be considered for generalization/scalability related testing: - details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality - what the minimum level performance for each identified scenario and/or configuration is - what the significant degradation for other scenarios and/or configurations is It should also be considered that generalization and/or scalability related requirements for different scenarios/ configurations can be implicitly handled in the test case definition. As for the handling of generalization tests, the following option is considered as baseline: Signaling based LCM procedures and performance monitoring are considered in dedicated test cases and are excluded in tests verifying generalization. RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability. (environment differs in each test but not changing dynamically during the test) - Specified UE configuration includes functionality and/or model ID if defined

Consider the following aspects regarding the different conditions for testing generalization for CSI AI/ML use:

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Various antenna spacings (e.g., 0.5 lambda, 0.8 lambda, etc)

- Various antenna virtualization (TxRU mapping)

- Various carrier frequencies and bands (e.g., 2GHz, 4.0GHz)

- Various outdoor/indoor UE distributions for UMa/Umi

- Various UE speeds.

Consider the following aspects regarding the scalability aspect for generalization testing for CSI AI/ML use:

- Various bandwidths (e.g., 20MHz, 50MHz) and/or frequency granularities, (e.g., size of subband), different layers

- Various sizes of CSI feedback payloads

Proposal 8:

Consider the following aspects regarding the different conditions for testing generalization for CSI AI/ML use:

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Various antenna spacings (e.g., 0.5 lambda, 0.8 lambda, etc)

- Various antenna virtualization (TxRU mapping)

- Various carrier frequencies and bands (e.g., 2GHz, 4.0GHz)

- Various outdoor/indoor UE distributions for UMa/Umi

- Various UE speeds.

Consider the following aspects regarding the scalability aspect for generalization testing for CSI AI/ML use:

- Various bandwidths (e.g., 20MHz, 50MHz) and/or frequency granularities, (e.g., size of subband), different layers

- Various sizes of CSI feedback payloads

# Conclusion

In conclusion the following observations and proposals were discussed in this contribution:

Observation 1: Simulation results comparing SGCS between AI/ML and e-typeII are captured below:

| SGCS | Encoder/Decoder |
| --- | --- |
| 0.78 | AI/ML (64 bits) |
| 0.747 | e-type II (configuration 1, 62 bits) |

Table 3. Simulation results for AI/ML and e-typeII codebook

Proposal 1: For aiding the performance alignment of the simulation results reported by each company, each company should list the training parameters used for the encoder-decoder pair. An example of training parameters is shown below:

- Optimizer

- Batch size

- Loss function

- Stopping Criterion

- Max Epochs

- Learning rate

- Dataset size

- Quantization of training data

- Channel type

- Weight initialization

Additionally, data sharing (Step 8) could be employed to aid misalignment issues:

- Company A trains its encoder/decoder with company’s B dataset.

- Company B tests the encoder decoder pair with company’s A testing data

Proposal 2: RAN4 to employ the procedures shown in the flowchart for studying the feasibility analysis of test decoder Option 3.

Proposal 3: The following criteria should establish the feasibility of the test decoder:

- The test decoder (along with the encoder) should deliver reasonable throughput performance gain compared to e-type II in ideal conditions. RAN4 to extend the feasibility analysis to higher ranks and different payload sizes to determine consistency of performance improvement. FFS on how to quantify the acceptable performance gain. (performance-complexity tradeoff)

- All UE preferred encoders should be aligned (work well) with the chosen test decoder

Proposal 4: To determine if the performance aspect of the feasibility analysis in throughput is met, RAN4 should investigate an empirical model that maps SGCS to a throughput metric, such as spectral efficiency.

Proposal 5: The following principles should guide the establishment of the alignment aspect of the feasibility analysis (steps 6-7):

- A well-trained UE encoder with the same or higher complexity compared to the Ref encoder that the test decoder was trained with, should have at least similar performance with the reference performance. (in throughput or SGCS)

- FFS on how to define a criterion for the similarity/dissimilarity of all companies' performance results (with UE preferred encoders) with respect to the reference performance, considering UE encoder complexity as well.

Proposal 6: RAN4 to define a criterion to select a test decoder among all the test decoders reported after performance alignment is achieved (step 5). The criterion could be based upon:

- Test decoder trained with the highest number of samples

- Each company tests its pair of {Encoder, test decoder) with other companies testing data set. Chose the pair with best performance

- Others

Proposal 7: According to the flowchart, RAN4 should first evaluate feasibility analysis with Option 1 and proceed to Option 2 if Option 1 does not produce a feasible test decoder

Figure 3. Flowchart of procedures for feasibility analysis of Option 3 Test decoder

Proposal 8: Consider the following aspects regarding the different conditions for testing generalization for CSI AI/ML use:

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Various antenna spacings (e.g., 0.5 lambda, 0.8 lambda, etc)

- Various antenna virtualization (TxRU mapping)

- Various carrier frequencies and bands (e.g., 2GHz, 4.0GHz)

- Various outdoor/indoor UE distributions for UMa/Umi

- Various UE speeds.

Consider the following aspects regarding the scalability aspect for generalization testing for CSI AI/ML use:

- Various bandwidths (e.g., 20MHz, 50MHz) and/or frequency granularities, (e.g., size of subband), different layers

- Various sizes of CSI feedback payloads

# Reference

[1] RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[2] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”