3GPP TSG-RAN WG4 Meeting #112			                                         R4-2411598

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.12.2.2

Source:	Xiaomi

Title:			Discussion on OTA test for FR1 NTN devices

Document for:	Discussion

- Introduction

As one of the objectives for RAN4 Rel-19 TRP, TRS and MIMO OTA testing enhancement, the test methodology and radiated performance metric for FR1 NTN devices has been discussed in previous meetings.

In the last meeting, it was agreed prioritize Handheld UE and further study VSAT-like UE. This contribution continues to provide our views on the performance metrics and Usage scenarios.

- Discussion

performance metrics

In the last meeting, several companies provided their views on performance metrics for FR1 NTN handheld UE which were summarized as 5 directions as shown in the below [1]. And it was agreed direction 1 should be abandon due to the formfactor.

Sub-topic 3-2 UE performance metric Issue 3-2-1: Categorized performance metric for NR-NTN handheld Agreements - RAN4 should not adopt direction 1, other directions are FFS - Direction 1: Only single point performance metric, i.e., Peak EIRP/EIS - Direction 2: Single point + measured full sphere (integrated or CDF), e.g., - D2a, Peak EIRP/EIS+TRP/TRS, - D2b, Peak EIRP/EIS+ full Spherical coverage CDF - Direction 3: Single point + measured partial sphere (integrated or CDF), e.g., - D3a, Peak EIRP/EIS+ partial TRP/TRS (e.g., within selected 30, 60, 90, 180 degree range of angles), - D3b, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles) - Direction 4: Only Full sphere (integrated TRP/TRS or Spherical coverage CDF) with/without weighting, - D4a, Spherical Coverage with EIRP-CDF and EIS-CCDF at [>50%]-ile - D4b, TRP/TRS or Weighted metric, e.g., weighted TRP/TRS (with larger weight for declared hemisphere) - Direction 5: Only Partial sphere (integrated or CDF) (e.g., within selected 30, 60, 90, 180 degree range of angles), Issue 3-2-3: Whether performance metric should be different for GEO and NGEO (including LEO and MEO) Agreements - FFS whether define different metrics for GSO and NGSO

As mentioned in our contribution [1], different UE types and form factors could have different performance metrics due to the different antenna gain and coverage area shown in the follow.

Table 1, the possible antenna gain and coverage area for different UE types

| UE types | Antenna Gain | Spherical coverage (Coverage area) |
| --- | --- | --- |
| Handheld UE | low | High |
| VSAT-like UE (mobile) | high | Medium |
| VSAT-like UE (fixed) | high | low |
| Vehicle UE | low/ Medium | Medium |

For handheld UE, it can be expected the spherical coverage should be large otherwise the orientation of the UE will be greatly restricted when communicating with SAN. In other words, the sufficient spherical coverage is more important for handheld UE compared with other UE types. Regarding whether Peak EIRP/EIS is also needed or not, from our perspective, as the antenna gain could not be so high due to small form factor, it doesn’t make much sense to adopt EIRP/EIS performance metric in addition to Full sphere or Only Partial sphere performance metric. In this case, direction 4 and 5 is more reasonable than other directions.

Proposal 1: the UE performance metric should be down select from direction 4 and 5

The other issue is whether performance metric should be different for GEO and NGEO (including LEO and MEO). Based on the UE capability in Ts38.306 in the follow, theoretically there will be 3 different UEs in the field, that is UE only support GSO, UE only supporting NGSO and UE supporting both. We think developing different performance metrics for GSO and NGSO is meaningful for fixed UE other than handheld UE. The reason is that the elevation angle of the fixed UE is generally fixed, so the spherical coverage can be smaller when it only supports the GSO scenario. However, the orientation of the handheld UE is more flexible, so even if the handheld UE only supports the GSO scenario, its spherical coverage is still required to be larger. In addition, from UE implementation point of view, if the supported satellite band is the same, the unified antenna design will be adopted.

Observation 1: As the spherical coverage for handheld UE should be large even it only supports GSO scenario, given the orientation of handheld UE is flexible, developing different performance metrics for GSO and NGSO is meaningful for fixed UE other than handheld UE.

| nonTerrestrialNetwork-r17 Indicates whether the UE supports NR NTN access. If the UE indicates this capability the UE shall support the following NTN essential features, e.g., timer extension in MAC/RLC/PDCP layers and RACH adaptation to handle long RTT, acquiring NTN specific SIB and more than one TAC per PLMN broadcast in one cell. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| ntn-ScenarioSupport-r17 Indicates whether the UE supports the NTN features in GSO scenario or NGSO scenario. If a UE does not include this field but includes nonTerrestrialNetwork-r17, the UE supports the NTN features for both GSO and NGSO scenarios, and also supports mobility between GSO and NGSO scenarios. | UE | No | No | No |

Usage scenarios

The discussion of the usage scenarios is about which scenarios should be tested for FR1 NTN handheld UE. The possible options are listed in the WF as below.

Issue 3-1-6: Usage scenarios for NR-NTN and IoT-NTN handheld UE Agreements - RAN4 further discuss prioritized usage scenarios. Free space only is not considered. - Option 1: Prioritize the browsing mode (with hand phantom) - Option 2: Prioritize the browsing mode (with hand phantom) and Free Space - Option 3: Prioritize browsing mode (with hand phantom) and talk mode (Head+Hand) - Option 4: Prioritize Head+Hand talk mode, hand only browsing mode and talk mode (new positioning guideline), and Free Space - Option 5: Prioritize Free Space mode only

For handheld UE, both browsing mode and talk mode shall be considered as usage scenarios like normal UE in TN network. However, if priority is needed, we agree some companies’ view that browsing mode is the more typic usage scenario. In this case both option 1 and option 3 are acceptable.

Proposal 1: Both browsing mode and talk mode shall be considered, but if prioritization is needed, browsing mode can be prioritized.

3	Conclusions

In this paper, we provide our views on developing test methodology and radiated performance metric for FR1 NTN devices, and provide the proposal as below:

Proposal 1: the UE performance metric should be down select from direction 4 and 5

Whether performance metric for should be different for GEO and NGEO?

Observation 1: As the spherical coverage for handheld UE should be large even it only supports GSO scenario, given the orientation of handheld UE is flexible, developing different performance metrics for GSO and NGSO is meaningful for fixed UE other than handheld UE.

Proposal 2: Both browsing mode and talk mode shall be considered, but if prioritization is needed, browsing mode can be prioritized.

4 References

[1] R4-2409931, Way Forward for [111] TRP_TRS_MIMO_OTA, Vivo, CAICT

[2] R4-2407809,	Discussion on OTA test for FR1 NTN devices, Xiaomi

[3] RP-240841, New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, Moderator (RAN4 VC, Qualcomm Incorporated)