3GPP TSG-RAN WG4 Meeting #112			                                         R4-2411600

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	9.15.2

Source:	Xiaomi

Title:			Discussion on RF requirements for redcap UE in FR1 NTN bands

Document for:	Discussion

- Introduction

The task for the WID on Non-Terrestrial Networks (NTN) for NR Phase 3 [1] in RAN4 is to define RF requirements for Rel-17 RedCap and Rel-18 eRedCap UEs. In previous meetings, the requirements for full-duplex FDD redcap were agreed. In the last meeting, although multiple agreements were achieved for HD-FDD, there are still some open issues. This contribution provides our views on those open issues.

- Discussion

According to the WF [2] in the last meeting, The conclusion of the discussion on HD-FDD RedCap UE REFSENS is illustrated in the below.

Issue 1-2: HD-FDD refsens for 2 Rx Agreement: [0.5] dB tightening for n254 and n256, FFS for n255. n256 to be confirmed in RAN4#112. Note: RAN4#111 maintenance agreement on n256 refsens in R4-2408787 was not considered for this agreement.

For 2Rx HD-FDD refsens, we support most company’s view that the tightening value can follow the previous agreement as summarized in the following table. The reason is that this agreement is the compromise achieved after a lot of discussion, which can avoid wasting time on too trivial discussions. It needs to emphasized that the REFSENS is the minimum requirement which never prevent the better REFSENS in the field.

Table 1：2Rx HD-FDD 5MHz REFSENS tightening relative to 2Rx FD-FDD 5MHz REFSENS [3]

| 2Rx FD-FDD 5MHz REFSENS | 2Rx HD-FDD 5MHz REFSENS Tightening |
| --- | --- |
| ≤ -100 dBm | 0 dB |
| > -100 dBm and ≤ -99 dBm | 0.5 dB |
| > -99 dBm | 0.8 dB |

Based on the above REFSENS tightening and the scaling over channel BW, the 2Rx HD-FDD REFSENS and UL configuration can be derived and specified as in Table 2.

Table 2：REFSENS requirement for 2Rx HD-FDD REFSENS

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.3 |
| n256 | 30 |  | -97.1 | -95.1 | -93.5 |
|  | 60 |  | -97.5 | -95.4 | -93.7 |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -100.0 | -96.8 | -95.0 |  |
| n254 | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

Proposal 1: 0.5 dB tightening for n254 and n256, 0 dB tightening for n255.

Proposal 2: The refsens requirements for 2Rx HD-FDD NTN UE shall be specified as table 2.

In the last meeting, there was a discussion on the possibility of considering an increase maximum output power for UEs only support HD-FDD operation. From the WF, there are still some options for further discussion on this issue as follow.

## Issue 1-5: Maximum output power options for further discussion: Option 1: Consider increase in PC3 power level for NTN RedCap UE which only supports HD-FDD operation for a band. Option 2: PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE

From the UE implementation point of view, there may be some room to improve the maximum output power due to the possible reduction of IL from RF front-end for UEs only support HD-FDD operation. However, the room would be very limited if the power class of PA is not changed and the upper tolerance +2 is considered, and not all UEs could have such room to improve the maximum output power. Moreover, if the maximum output power is increased, no matter by increasing the upper tolerance or power boosting, there need some discussion on whether it will impact some of Tx requirements, e.g., MPR, ACLR etc. In view of the above, to be safe, option 2 is our preference at current stage.

Proposal 3: it is preferred not to consider increase in current PC3 power level even for UE only supports HD-FDD operation.

3	Conclusions

In this paper, we provide our views on some open issues listed in the WF, and made the following proposals:

Proposal 1: 0.5 dB tightening for n254 and n256, 0 dB tightening for n255.

Proposal 2: The refsens requirements for 2Rx HD-FDD NTN UE shall be specified as table 2.

Proposal 3: it is preferred not to consider increase in current PC3 power level even for UE only supports HD-FDD operation.

4 References

[1] RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei (Moderator, RAN1 Vice-Chair)

[2] R4-2410574, WF on Rel-19 phase 3 NR-NTN UE RF requirements, Qualcomm

[3] R4-2203692 “RedCap UE HD-FDD REFSENS requirements”, Apple, 3GPP TSG RAN WG4 Meeting #102-e, Online, February 21st – March 3rd, 2022