3GPP TSG-RAN WG4 Meeting #112	R4-2411631

Maastricht, Netherlands, 19th -23rd August, 2024

Source:	Qualcomm Incorporated

Title:	Power boosting and MPR reduction

Agenda item:	8.1.1.2.1

Document for:	Approval

# 1.	Introduction

In RAN4#111 there was a new WF [1] to study power domain enhancements for NR single carrier. In this paper we present further views on this topic.

# 2. 	Discussion

In RAN4#111 there was a new WF [1] to study power domain enhancements for NR single carrier. Some tentative agreements were captured in this WF [1] as stated below.

## 1.2. Mechanisms for enabling MPR reduction and/or power boosting

Way forward:

- Study the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW, e.g. each side of the UE BW is equal to ½ of the UE BW, inside a larger BS CBW as starting point with consideration of the following aspects

  - The edge of the extended UE BW, i.e. the UE CBW plus the shifted frequency symmetrically at each side of the UE CBW, should be inside BS CBW or at least aligned with the BS CBW edge

    - ① FFS feasibility of case where extended UE CBW edge exceeds the BS CBW edge, i.e. gap between edges of UE CBW and BS CBW < 1/2 UE CBW

    - ② FFS whether IBE is used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW

    - ③ FFS ACLR and SEM are applicable from the edge of extended UE CBW or edge of BS CBW, i.e. the start point of ΔfOOB

    - ④ FFS integral region and boundary of OOBE is based on extended UE CBW or UE CBW

- Other mechanisms are not precluded

As stated in the WF we would like to make the UE bandwidth extension such that all RBs in the original UE BW are ‘inner’ allocations in the extended BW scenario. If the original UE bandwidth is X then to ensure that all allocations in this bandwidth are inner allocations the excess BW on either side of the original UE bandwidth has to be at least X/2. Therefore, the extended UE BW is now X/2+X+X/2=2X which is the minimum BW required from the BS to make all RB allocations of the original UE BW into inner allocations in the extended BW scenario.

Observation 1: To make all RB allocations in the original UE BW ‘inner’ allocations the minimum excess BW on either side must be at least half the original UE BW.

If the minimum excess BW criteria mentioned in observation 1 can be met then the extended UE BW can be treated as the new UE bandwidth and all performance metrics such as SEM and ACLR should be based on this new BW. IBE can be used as an evaluation metric in all unallocated regions of this new UE bandwidth.

Proposal 1: If minimum excess BW equal to half of the original UE BW can be placed on both sides of the original UE BW this extended UE BW can be treated as the new UE BW and all performance metrics can be based on this new BW.

If the minimum extended BW on both sides of UE BW cannot meet the minimum requirement, then some portion of the original UE band will be in the outer MPR region. In this case we think that it is best not to extend the UE BW and to retain the original UE BW. In this case all performance metrics will be based on the original UE BW and no MPR relaxation would be possible.

Proposal 2:  If the minimum extended BW on both sides of original UE BW is less than half the original UE BW then the original UE BW should not be extended, and all performance metrics should be based on the original UE BW.

The scenario with excess bandwidth on either side of the allocated bandwidth, though relatively easy to analyse, may not be the most realistic deployment scenario selected by operators as it requires unused BW on both sides of the allocated spectrum. We do not think that an operator would partition a band into smaller sub-bands just to create guard bands on either side of a given sub-band. Partitioning a band into smaller sub-bands reduces the total number of inner allocations. This can be seen in figure1 below where a given band has been partitioned into 2 and 4 sub-bands. Figure 1a shows a band divided into 4 sub-bands where each triangle shows the inner allocations for each sub-band. The dashed triangle shows the inner allocations for an unpartitioned band. By comparing the area of the 4 small triangles with the area of the dashed triangle it is observed that an un-partitioned band has more inner allocations compared to the partitioned band. The same is true in figure 1b. Also, comparing figure 1a with figure 1b it can be seen that the band that it divided into 2 sub-bands has more inner allocations than the one divided into 4 showing that the number of inner allocations decreases as a given band is divided into more sub-bands.

- (b)

Figure 1: Inner allocation for band divided into 2 and 4 channels

Observation 2: Dividing a given band into more sub-bands results in lesser inner allocations over the entire band

We think that a more realistic deployment scenario is one where a given band is owned by, for example, 2 operators and excess bandwidth is only available on one side of the allocated UE spectrum as illustrated in figure 2.

Figure2: Scenario where excess BW is available on one side of the allocated BW

If for example excess BW equal to at least 1/2 the allocated BW is available on the lower frequency side, indicated as region 1, in figure 3 then it can be shown that the inner allocation region of the original UE BW extends as indicated in figure 4 to region 1. Therefore, the inner region now occupies both region 1 and the original inner region. If the minimum excess bandwidth is added to region 2 instead of region 1 of figure 3 then the inner region of the MPR triangle extends into region 2 so that the new inner region comprises of both region2 and the original inner region as illustrated in figure4.

Excess BW Region1

UE bandwidth

Excess BW Region2

Figure3: UE bandwidth with excess BW band either side

Figure 4: Expansion of inner region when excess bandwidth is added onto either side of UE allocated BW

Observation 3: The addition of excess BW equal to at least ½ of the allocated UE BW to only one side of the allocated UE BW may be a more realistic scenario due to the way the sub-bands are allotted within an operator’s band compared to adding excess BW on both sides which requires not only more BW but also needs to have unused BW on both sides of the allocated BW.

Observation 4: Adding excess BW to either side of the allocated spectrum increases the number of inner allocations

Adding excess bandwidth only on one side of the allocated UE BW modifies the MPR regions as indicated in figure 4 as one side of the spectrum now has more inner allocations created that were originally outer allocations. In [2] the inner region for single CC is defined as:

Observation 5: Adding excess BW to either side of the allocated UE spectrum modifies the MPR tables

If the inner region is expanded to also include region 1 then the above equations for the inner region should be modified as follows:

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB – RBStart,Low – LCRB

RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(2NRB/3)

If the inner region is expanded to also include region 2 then the above equations for the inner region should be modified as follows:

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB – RBStart,Low – LCRB

RBStart_Low  ≤  RBStart

Proposal 3: Study the use case where excess BW is only added on one side of the allocated UE BW

Proposal 4: For the case where minimum excess BW equal to half the allocated UE BW is added only on side of the allocated UE BW use the following equations to describe the inner allocations:

Case 1: Excess BW added on lower side of allocated UE BW use:

. RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB – RBStart,Low – LCRB

RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(2NRB/3)

Case 2: Excess BW added on higher side of allocated UE BW use:

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB – RBStart,Low – LCRB

RBStart_Low  ≤  RBStart

# Conclusion

In this paper we further discuss our views on the study of power domain enhancement and make the following proposals:

Observation 1: To make all RB allocations in the original UE BW ‘inner’ allocations the minimum excess BW on either side must be at least half the original UE BW.

Proposal 1: If minimum excess BW equal to half of the original UE BW can be placed on both sides of the original UE BW this extended UE BW can be treated as the new UE BW and all performance metrics can be based on this new BW.

Proposal 2:  If the minimum extended BW on both sides of original UE BW is less than half the original UE BW then the original UE BW should not be extended, and all performance metrics should be based on the original UE BW.

Observation 2: Dividing a given band into more sub-bands results in lesser inner allocations over the entire band

Observation 3: The addition of excess BW equal to at least ½ of the allocated UE BW to only one side of the allocated UE BW may be a more realistic scenario due to the way the sub-bands are allotted within an operator’s band compared to adding excess BW on both sides which requires not only more BW but also needs to have unused BW on both sides of the allocated BW.

Observation 4: Adding excess BW to either side of the allocated spectrum increases the number of inner allocations

Observation 5: Adding excess BW to either side of the allocated UE spectrum modifies the MPR tables.

Proposal 3: Study the use case where excess BW is only added on one side of the allocated UE BW

Proposal 4: For the case where minimum excess BW equal to half the allocated UE BW is added only on side of the allocated UE BW use the following equations to describe the inner allocations:

Case 1: Excess BW added on lower side of allocated UE BW use:

. RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB – RBStart,Low – LCRB

RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(2NRB/3)

Case 2: Excess BW added on higher side of allocated UE BW use:

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB – RBStart,Low – LCRB

RBStart_Low  ≤  RBStart

# References

[1] R4-2410749, WF on power domain enhancements, RAN4#111, May 20th – May 24th , 2024

[2] TS38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V18.6.0