# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411842

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on system parameters and UE RF requirements for NR-NTN S-band

# Agenda item:	7.17.2

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of NR-NTN S-band with the following RAN4 objective. In this contribution, we want to share some initial views about NR-NTN S-band from system parameters and UE RF requirements perspective.

The objective of the core part is to: - Specify a new NR-NTN FDD band with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario - Support channel bandwidths and SCS as presented in Table 4.1-1 below; - Mandatory support of channel raster points at step of 10 kHz; - Support UE Power class 3 (+23dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. Table 4.1-1: Channel bandwidth and SCS system parameters. SCS (kHz); Channel bandwidth (MHz). 15; 5; 10; 15; 20. 30; 10; 15; 20. 60; 10; 15; 20. The specification work of this WI shall leverage the studies and requirements for NR NTN n256, n255, n254, where applicable. All UE requirements specified as part of this WI shall be Release-independent from Rel-17.

# 2 Discussion

  - 2.1 Operating band and band numbering

This WID is to introduce a new NR-NTN FDD band with a UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz. For the band number, RAN4 has defined a principle of “first come first served”. And there is another approved WID of introducing a new NR NTN bands to support the extended L-band, whose tdoc number is after this WID. Hence, we can treat this new NR-NTN S-band as the first, and IoT-NTN band has already defined band 253. The corresponding operating band and band number can be defined as Table 2.1-1.

Figure 2.1-1: New NR-NTN FDD band with UL at 2000 - 2020 MHz and DL at 2180 - 2200 MHz

Table 2.1-1: NTN satellite bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| [n252] | 2000 MHz - 2020 MHz | 2180 MHz - 2200 MHz | FDD |

Proposal 1: Operating band and band numbering for the new NR-NTN FDD band can be defined as Table 2.1-1.

  - 2.2 Channel bandwidth

According to the objective of this WID, supported channel bandwidths and SCS can be defined as Table 2.2-1.

Table 2.2-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| [n252] | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |

Proposal 2: Supported channel bandwidths and SCS for the new NR-NTN FDD band can be defined as Table 2.2-1.

  - 2.3 Asymmetric channel bandwidths

The objective of this WID does not states support asymmetric channel bandwidths. Whether to support asymmetric channel bandwidths depends on operators’ input.

Proposal 3: Whether to support asymmetric channel bandwidths depends on operators’ input.

  - 2.4 Channel raster and sync raster

The objective of this WID has specified that mandatory support of channel raster points at step of 10 kHz. The new NR-NTN FDD band needs to support 100 kHz and 10 kHz channel raster. For NR-ARFCN, we propose to use the similar descending manner to define as following:

Table 2.4-1: Applicable NR-ARFCN per operating band in FR1-NTN with 100 kHz channel rater

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n252] | 100 | 400000 – <20> – 404000 | 436000 – <20> – 440000 |

Table 2.4-2: Applicable NR-ARFCN per operating band in FR1-NTN with 10 kHz channel rater

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) | Mandatory support |
| --- | --- | --- | --- | --- |
| [n252] | 10 | 400000 – <2> – 404000 | 436000 – <2> – 440000 | Yes |

Proposal 4: NR-ARFCN for the new NR-NTN FDD band supporting 100 kHz and 10 kHz channel raster can be defined as Table 2.4-1 and Table 2.4-2, respectively.

In addition, downlink of 2180 - 2200 MHz for the new NR-NTN FDD band is a part of NR band n65 and n66. For n65, it only supports case A SSB pattern, and n66 supports both case A and case B SSB pattern, which is shown as follows.

Table 2.4-3: Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n65 | 15 kHz | Case A | 5279 – <1> – 5494 |
| n66 | 15 kHz | Case A | 5279 – <1> – 5494 |
|  | 30 kHz | Case B | 5285 – <1> – 5488 |

So, we propose that at least case A SSB pattern is supported by the new NR-NTN FDD band, and case B SSB pattern also can be supported. Sync raster and GSCN for the new NR-NTN FDD band should follow the current design of sync raster in TS 38.101-5 and defined as Table 2.4-3.

Table 2.4-4: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n252] | 15 kHz | Case A | 5456 – <1> – 5494 |
|  | 30 kHz | Case B | 5460 – <1> – 5488 |

Proposal 5: We propose that at least case A SSB pattern is supported by the new NR-NTN FDD band, and case B SSB pattern also can be supported. Sync raster and GSCN for the new NR-NTN FDD band can be defined as Table 2.4-4.

  - 2.5 TX-RX frequency separation

For TX-RX frequency separation, similar as LTE band 23, which supports default UE TX-RX frequency separation of 180 MHz, it should be defined as 180 MHz for the new NR-NTN FDD band as Table 2.5-1.

Table 2.5-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| [n252] | 180 MHz |

Proposal 6: The default TX-RX frequency separation for the new NR-NTN FDD band should be defined as Table 2.5-1.

  - 2.6 UE RF requirements

The following UE RF requirements need to be defined for the new NR-NTN FDD band.

Table 2.6-1 UE RF requirements for the new NR-NTN FDD band

| NR-NTN UE Tx/Rx requirement | Proposals for the new NR-NTN FDD band |
| --- | --- |
| 6.2.1 UE maximum output power | Band specific --> Specification impact. Support UE Power class 3 (+23dBm). |
| 6.2.2 MPR | Not band specific --> No specification impact. |
| 6.2.3 A-MPR | Band specific --> Specification impact. Considering use the same A-MPR requirement as LTE band 23 as a starting point since operating band of the new NR-NTN FDD band is same as B23. |
| 6.2.4 Configured transmitted power | Not band specific --> No specification impact. |
| 6.3.1 Minimum output power | Not band specific --> No specification impact. |
| 6.3.2 Transmit OFF power | Not band specific --> No specification impact. |
| 6.3.3 Transmit ON/OFF time mask | Not band specific --> No specification impact. |
| 6.3.4 Power control | Not band specific --> No specification impact. |
| 6.4.1 Frequency error | Not band specific --> No specification impact. |
| 6.4.2 Transmit modulation quality | Not band specific --> No specification impact. |
| 6.5.1 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.5.2.2 Spectrum emission mask | Not band specific --> No specification impact. |
| 6.5.2.3 Additional Spectrum emission mask | Specification impact. Considering define the same additional SEM requirement as LTE band 23 as a starting point since operating band of the new NR-NTN FDD band is same as B23. |
| 6.5.2.4.1 NR ACLR | Not band specific --> No specification impact. |
| 6.5.2.4.2 UTRA ACLR | Not band specific --> No specification impact. |
| 6.5.3.1 General spurious emissions | Not band specific --> No specification impact. |
| 6.5.3.2 Spurious emissions for UE co-existence | Specification impact. Band [n252] needs to be added in Table 6.5.3.2-1. |
| 6.5.3.3 Additional spurious emissions | Specification impact. Considering define the same additional spurious emissions requirement as LTE band 23 as a starting point since operating band of the new NR-NTN FDD band is same as B23. |
| 6.5.4 Transmit intermodulation | Not band specific --> No specification impact. |
| 7.3.2 Reference sensitivity | Band specific --> Specification impact. Considering use the same REFSENS requirement as LTE band 23 as a starting point since operating band of the new NR-NTN FDD band is same as B23. |
| 7.4 Max input level | Not band specific --> No specification impact. |
| 7.5 Adjacent channel selectivity | Not band specific --> No specification impact. |
| 7.6.2 In-band blocking | Specification impact --> Define in-band blocking requirement similar to band n254, n255, n256. |
| 7.6.3 Out of band blocking | Specification impact --> Considering use the same out-of-band blocking requirement for band [n252] as LTE band 23 as a starting point since operating band of the new NR-NTN FDD band is same as B23. |
| 7.6.4 Narrow band blocking | Specification impact --> Define narrow band blocking requirement similar to band n254, n255, n256. |
| 7.7 Spurious response | Not band specific --> No specification impact. |
| 7.8 Intermodulation characteristics | Not band specific --> No specification impact. |
| 7.9 Spurious emissions | Not band specific --> No specification impact. |

Proposal 7: To use the proposals in Table 2.6-1 for UE RF requirements for the new NR-NTN FDD band.

# 3 Conclusion

In this contribution, we shared some views on system parameters and UE RF requirements for the new NR-NTN FDD band and the proposals are made as follows:

Proposal 1: Operating band and band numbering for the new NR-NTN FDD band can be defined as Table 2.1-1.

Table 2.1-1: NTN satellite bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| [n252] | 2000 MHz - 2020 MHz | 2180 MHz - 2200 MHz | FDD |

Proposal 2: Supported channel bandwidths and SCS for the new NR-NTN FDD band can be defined as Table 2.2-1.

Table 2.2-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| [n252] | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |

Proposal 3: Whether to support asymmetric channel bandwidths depends on operators’ input.

Proposal 4: NR-ARFCN for the new NR-NTN FDD band supporting 100 kHz and 10 kHz channel raster can be defined as Table 2.4-1 and Table 2.4-2, respectively.

Table 2.4-1: Applicable NR-ARFCN per operating band in FR1-NTN with 100 kHz channel rater

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n252] | 100 | 400000 – <20> – 404000 | 436000 – <20> – 440000 |

Table 2.4-2: Applicable NR-ARFCN per operating band in FR1-NTN with 10 kHz channel rater

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) | Mandatory support |
| --- | --- | --- | --- | --- |
| [n252] | 10 | 400000 – <2> – 404000 | 436000 – <2> – 440000 | Yes |

Proposal 5: We propose that at least case A SSB pattern is supported by the new NR-NTN FDD band, and case B SSB pattern also can be supported. Sync raster and GSCN for the new NR-NTN FDD band can be defined as Table 2.4-4.

Table 2.4-4: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n252] | 15 kHz | Case A | 5456 – <1> – 5494 |
|  | 30 kHz | Case B | 5460 – <1> – 5488 |

Proposal 6: The default TX-RX frequency separation for the new NR-NTN FDD band should be defined as Table 2.5-1.

Table 2.5-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| [n252] | 180 MHz |

Proposal 7: To use the proposals in Table 2.6-1 for UE RF requirements for the new NR-NTN FDD band.

Table 2.6-1 UE RF requirements for the new NR-NTN FDD band

| NR-NTN UE Tx/Rx requirement | Proposals for the new NR-NTN FDD band |
| --- | --- |
| 6.2.1 UE maximum output power | Band specific --> Specification impact. Support UE Power class 3 (+23dBm). |
| 6.2.2 MPR | Not band specific --> No specification impact. |
| 6.2.3 A-MPR | Band specific --> Specification impact. Considering use the same A-MPR requirement as LTE band 23 as a starting point since operating band of the new NR-NTN FDD band is same as B23. |
| 6.2.4 Configured transmitted power | Not band specific --> No specification impact. |
| 6.3.1 Minimum output power | Not band specific --> No specification impact. |
| 6.3.2 Transmit OFF power | Not band specific --> No specification impact. |
| 6.3.3 Transmit ON/OFF time mask | Not band specific --> No specification impact. |
| 6.3.4 Power control | Not band specific --> No specification impact. |
| 6.4.1 Frequency error | Not band specific --> No specification impact. |
| 6.4.2 Transmit modulation quality | Not band specific --> No specification impact. |
| 6.5.1 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.5.2.2 Spectrum emission mask | Not band specific --> No specification impact. |
| 6.5.2.3 Additional Spectrum emission mask | Specification impact. Considering define the same additional SEM requirement as LTE band 23 as a starting point since operating band of the new NR-NTN FDD band is same as B23. |
| 6.5.2.4.1 NR ACLR | Not band specific --> No specification impact. |
| 6.5.2.4.2 UTRA ACLR | Not band specific --> No specification impact. |
| 6.5.3.1 General spurious emissions | Not band specific --> No specification impact. |
| 6.5.3.2 Spurious emissions for UE co-existence | Specification impact. Band [n252] needs to be added in Table 6.5.3.2-1. |
| 6.5.3.3 Additional spurious emissions | Specification impact. Considering define the same additional spurious emissions requirement as LTE band 23 as a starting point since operating band of the new NR-NTN FDD band is same as B23. |
| 6.5.4 Transmit intermodulation | Not band specific --> No specification impact. |
| 7.3.2 Reference sensitivity | Band specific --> Specification impact. Considering use the same REFSENS requirement as LTE band 23 as a starting point since operating band of the new NR-NTN FDD band is same as B23. |
| 7.4 Max input level | Not band specific --> No specification impact. |
| 7.5 Adjacent channel selectivity | Not band specific --> No specification impact. |
| 7.6.2 In-band blocking | Specification impact --> Define in-band blocking requirement similar to band n254, n255, n256. |
| 7.6.3 Out of band blocking | Specification impact --> Considering use the same out-of-band blocking requirement for band [n252] as LTE band 23 as a starting point since operating band of the new NR-NTN FDD band is same as B23. |
| 7.6.4 Narrow band blocking | Specification impact --> Define narrow band blocking requirement similar to band n254, n255, n256. |
| 7.7 Spurious response | Not band specific --> No specification impact. |
| 7.8 Intermodulation characteristics | Not band specific --> No specification impact. |
| 7.9 Spurious emissions | Not band specific --> No specification impact. |

# 4 Reference

- RP-241661, New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) , DISH Network et al.