3GPP TSG-RAN WG4 Meeting #112	R4-2412042

Maastricht, Netherlands, 19th – 23th Aug. 2024

Source: 	Nokia, Nokia Shanghai Bell

Title: 					n28 NS_17 and NS_18 A-MPR simulations

Agenda Item: 			7.20.2

Document for:	Discussion

# 1	Introduction

We simulate the required backoff due to additional spurious emission masks signaled through NS_17 and NS_18. For PC2 we simulate all the available CBWs while for PC3 we only simulate 40 MHz as the A-MPR for lower CBWs has already been specified.

# 2	Simulation assumptions and scenario

## 2.1	Simulation parameters

- Operating band n28 (703 MHz – 748 MHz)

- Power classes and CBWs:

  - PC2: 3 MHz, 5 MHz, 10 MHz, 15 MHz, 20 MHz, 25 MHz, 30 MHz, and 40 MHz

  - PC3: 40 MHz

- 40 MHz is simulated at fixed center frequency of 723 MHz; channel positions for other CBWs are allowed to vary within the operating band

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc

- Only additional spurious emission masks from NS_17 and NS_18 are simulated. Other gating factors (ACLR, SEM, etc.) are excluded.

## 2.2	NS_17

Table 6.5.3.3.2-1: Additional requirements for "NS_17"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 3, 5, 10 |  |  |
| 470 ≤ f ≤ 710 | -26.2 | 6 MHz | 1 |
| NOTE 1: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. Applicable when the assigned NR carrier is confined within 715 MHz and 718 MHz and when the channel bandwidth used is 3 MHz. |  |  |  |

## 2.3	NS_18

Table 6.5.3.3.3-1: Additional requirements for "NS_18"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |  |
| --- | --- | --- | --- |
|  | 3, 5, 10, 15, 20, 30 |  |  |
| 692-698 | -26.2 | 6 MHz |  |

# 3	Simulation results and discussion

Here we present examples of the simulation results along with the discussion. Complete sets of A-MPR triangles for all modulations, waveforms and SCSs are presented in the appendix.

## 3.1	PC2 NS_17

According to NOTE 1 in the mask definition we only consider the ≥718 MHz region for 5 MHz and 10 MHz CBWs and fixed center frequency of 716.5 MHz for 3 MHz CBW. Only 10 MHz CBW requires A-MPR, and only for Fc below 728.18 MHz. In Figure 1 we present the A-MPR for 10 MHz CBW in the lowest possible position of Fc=723 MHz for QPSK modulation and 15 kHz SCS.

Fc=723 MHz

Figure 1: Examples of the simulated backoff to satisfy the NS_17 additional spurious emission limit for 10 MHz CBW at Fc=723 MHz.

Observation 1: For PC2 and NS_17 A-MPR is required for 10 MHz CBW.

## 3.2	PC2 NS_18

In Table 1 we present the center frequencies where A-MPR is needed for CBWs from 5 MHz to 20 MHz. 3 MHz CBW doesn’t require A-MPR at all while A-MPR cannot be completely avoided anywhere in the operating band at 25 MHz and 30 MHz CBWs.

Table 1: The center frequencies where A-MPR for NS_18 is not needed according to simulations. For higher CBWs A-MPR cannot be avoided anywhere in the operating band. 3 MHz CBW does not need A-MPR anywhere in the operating band.

| Channel Bandwidth [MHz] | Fc, noAMPR [MHz] |
| --- | --- |
| 5 | ≥ 707.34 |
| 10 | ≥ 716.49 |
| 15 | ≥ 725.47 |
| 20 | ≥ 733.22 |

In Figure 2 we present A-MPR triangles for 20 MHz CBW at the lower edge of the operating band for QPSK modulation and 15 kHz SCS. Note that for CBW≤30 MHz we have performed the simulations so that also the spectrum below the mask segment is compared to the mask limit. In this way it is easier to use the triangles to form A-MPR specifications that are valid in a frequency range instead of a fixed center frequency.

Fc=713 MHz

Figure 2: Examples of the simulated backoff to satisfy the NS_18 additional spurious emission limit for 20 MHz CBW at Fc=713 MHz.

For 40 MHz CBW we only simulate the fixed position of Fc=723 MHz according to the WID [1]. In Figure 3 we present the A-MPR triangles for QPSK modulation and 15 kHz SCS. At the lower left corner of the triangles there is no A-MPR needed because the IMD components fall below the mask that is only 6 MHz wide. The two ”legs” seen at small LCRB are due to allocation-carrier and allocation-image IMDs.

Fc=723 MHz

Figure 3: Examples of the simulated backoff to satisfy the NS_18 additional spurious emission limit for 40 MHz CBW at Fc=723 MHz. Power class is PC2.

Observation 2: For PC2 and NS_18 A-MPR is required for CBWs of 5 MHz and larger.

## 3.2	PC3 NS_18

For PC3 we only simulate the 40 MHz CBW since the A-MPR for lower CBWs has already been specified. In Figure 4 we present the A-MPR triangles for QPSK modulation and 15 kHz SCS at Fc =723 MHz. The A-MPR regions are very similar to PC2, but the backoff values are smaller.

Fc=723 MHz

Figure 4: Examples of the simulated backoff to satisfy the NS_18 additional spurious emission limit for 40 MHz CBW at Fc=723 MHz. Power class is PC3.

Observation 3: For PC3 and NS_18 A-MPR is required for 40 MHz CBW.

# 4	Conclusions

We have simulated the A-MPR for band n28 taking into account the additional emission requirements signaled through NS_17 and NS_18. Based on our simulations we have made the following observations.

Observation 1: For PC2 and NS_17 A-MPR is required for 10 MHz CBW.

Observation 2: For PC2 and NS_18 A-MPR is required for CBWs of 5 MHz and larger.

Observation 3: For PC3 and NS_18 A-MPR is required for 40 MHz CBW.

# 5	References

[1] RP-241660, New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28

# 6	Appendix

Here we present complete sets of A-MPR triangles for the center frequencies where A-MPR is needed at the interval of 10 MHz. Note that for CBW≤30 MHz we have performed the simulations so that also the spectrum below the mask segment is compared to the mask limit. In this way it is easier to use the triangles to form A-MPR specifications that are valid in a frequency range instead of a fixed center frequency.

## 6.1	PC2 NS_17

## 6.2	PC2 NS_18

## 6.3	PC3 NS_18