3GPP TSG-RAN WG4 Meeting #112	R4-2412126

Maastricht, Netherlands, August 19-23, 2024

Agenda Item:	8.2.4.1

Source:	Ericsson

Title:	On Co-existence simulation assumptions for 14800 to 15350 MHz frequency range

Document for:	Discussion

# 1	Introduction

ITU-R WP5D sent a LS [1] to 3GPP RAN seeking information on terrestrial component IMT-2030 parameters to be used for sharing and compatibility studies in preparation for WRC-27, for frequency ranges 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 1530 MHz.

In 3GPP RAN4#110-bis meeting, the Work Plan [2] for delivery of information to ITU-R LS for those respective frequency ranges was agreed, following a phase approach to send information in 3 phases as below -

- Track 1: 4400 to 4800 MHz, the estimated date for completion is latest by May 2024 (RAN4#111).

- Track 2: 7125 to 8400 MHz, the estimated date for completion is latest by August 2024 (RAN4#112).

- Track 3: 14800 to 15350 MHz, the estimated date for completion is latest by November 2024 (RAN4#113).

WF [3] captured agreements on co-existence simulation assumptions for companies to present results in August meeting.

This contribution triggers discussion on open issues related to co-existence simulation assumptions.

# 2	Discussion

In the previous 3GPP RAN4#111 meeting, agreements were made on the co-existence study to prioritize Macro Urban scenarios, followed by Indoor Hotspot and then Dense Urban. Most of the assumptions for Urban scenarios have been agreed and captured in the TR 38.922 v0.1.0 [4]. However, some issues remain open and are discussed in the following sub-sections.

## Urban Macro Scenario

### Inter-site distance (ISD)

As discussed in our companion contribution [5] where preliminary results are presented, for Urban macro scenario where ISD is 450m, the DL coverage seems sufficient for Outdoor users but for the case where indoor users are also considered, the UL coverage is reduced and 5%tile throughput is 0. In this case, it would be interesting to study scenarios with lower ISD of 350 m or even further lower ISD to get adequate coverage for indoor users.

UL coverage is challenged in Urban Macro scenarios with the assumption of 20% Indoor users.

As larger antenna arrays are considered for co-existence simulations leading to sufficient coverage in DL assuming 0% Indoor Users, a possibility of higher ISD than 450m can also be considered. However, UL coverage with 20% Indoor Users might still remain a challenge.

ISD higher than 450m can be considered as sufficient DL coverage is achieved assuming 0% indoor users. However, UL coverage still remains a challenge to be addressed with 20% Indoor users.

RAN4 should further discuss on how to achieve sufficient UL coverage in Urban Macro scenarios, with a reduced ISD and/ or higher maximum transmit power.

### Indoor UE ratio

As per the last WF [3], it has been agreed to keep both options – 0% Indoor and 20% Indoor probability users for Urban Macro simulation assumptions.

As discussed in previous section and pre-liminary results [5] presented in our companion contribution, both the indoor probability options present different behaviours. The case with 20% Indoor probability presents UL outage and hence remains a challenge to be addressed with either reducing the ISD or increasing UE power, improving the architecture of the antenna.

0% and 20% Indoor probability options present different behaviors, with the latter presenting UL coverage challenges with ISD as 450m.

RAN4 to consider keeping only 0% Indoor users for Urban Macro scenario.

In WF [4], it has been agreed that the BS output power is 43 dBm. However, it should be clarified that this power is defined per polarization, and per 100 MHz. If channel bandwidth is increased, the Power Spectral Density (PSD) will decrease.

RAN4 to clarify for Urban Macro scenario that BS output power 43 dBm is defined per polarization. It is given for 100 MHz channel bandwidth.

### UE maximum TX power

As per the last WF, both options assuming 23 dBm and 26 dBm UE maximum Tx power is to be considered for FR1 UE. Clarify if both options apply to both UE antenna architectures as mentioned in Table 6.1.2.4-1 of TR 38.922 v0.1.0

The UE maximum Tx power should be clarified in the Table 6.1.2.4-1 of TR 38.922 for the two UE antenna architecture options.

## Indoor Hotspot Scenario

### Network layout – 1 sector or 3 sector per node

The details on Indoor network layout model is listed in Table 2-1 and depicted in Figure 2-1, as discussed in the TR 38.922 v0.1.0 [4], Table 6.1.2.1.3-1. However, different options are defined for the indoor office deployment, as detailed in TR 38.802, so it would be useful to clarify the option to consider and number of sectors, if 1 or 3. We have provided detailed simulation results in our companion contribution considering, both 1 sector and 3 sectors [5].

Table 2-1: Single operator layout for indoor

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | 50m x 120m, 12BSs |  |
| Inter-site distance |  | 20m |  |
| BS antenna height |  | 3 m | ceiling |
| UE location | Outdoor/indoor | Indoor |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | 1 m |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 0 m |  |
| Channel model |  | Indoor Office |  |
| Shadowing correlation |  | NA |  |

Figure 2-1: Network layout for indoor

RAN4 to consider Indoor office deployment with 1 sector, i.e. option 1 from TR38.802, section A.2.1 and capture it in TR 38.922.

### BS output power

.In the latest TR in Table 6.1.2.4-1, it has been captured that the BS output power is 23 dBm. However, it should be clarified that this power is dual polarized, and per 100 MHz. If channel bandwidth is increased, the Power Spectral Density (PSD) will decrease.

RAN4 to clarify for Indoor scenario that BS output power 23 dBm is dual polarized. It is given for 100 MHz channel bandwidth.

## Common assumptions

### BS antenna characteristics

In Table 2.4.1-1 BS antenna parameters for different deployment scenarios are listed.

Table 2.3.1-1: Antenna array parameters

| Parameter | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 5 | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º |
| Antenna sub-array configuration (M × N) (Note 4) | 4 x 4 | 1536 (8x16) / 2048 (8x32) elements | 1536 (8x16) / 2048 (8x32) elements |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 0.5 of wavelength for V | 0.5 of wavelength for H, 4.2/2.8 of wavelength for V | 0.5 of wavelength for H, 4.2/2.8 of wavelength for V |
| Number of element rows in sub-array | N/A | (6x1)/(4x1) | (6x1)/(4x1) |
| Vertical element separation in sub-array (dv,sub) | 0.5 of wavelength of V | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | N/A | 3 | 3 |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array/element (dBm) (Note 3) | 8 | 21.9/18.9 | 21.9/18.9 |
| Base station horizontal coverage range (degrees) | +/-90 | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 0-180 | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | 90 | 6 | 6 |
| Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180°. Note 2: The element gain includes the loss and is per polarization. Note 3: The conducted power per sub-array/element assumes 2xMxN subarrays/elements (i.e., power per H/V polarized element). Note 4: M and N represents rows and columns of dual polarized elements of the array or subarrays in the array Note 5: For the case of 4/6 elements per sub array, dv will be 2.8/4.2 wavelengths. |  |  |  |

The template is taken from the agreements in the last meeting and as captured in TR. However, we found that certain parameters need to be updated/ revised as per the WF.

- For Urban Macro Scenario,

  - Vertical element separation in sub-array should be 0.7 of wavelength V

- For Indoor Scenario,

  - Pre-set sub-array down-tilt should be NA

- Note 3, 4 and 5 should be updated to maintain parity for all the scenarios.

RAN4 to revise the table in TR with the correct antenna parameters for (dv,sub) and Pre-set sub-array down-tilt and modify Note 3,4 and 5 wordings to maintain parity between the scenarios.

RAN4 to adopt the proposed Antenna configuration parameters for Antenna sub-array configuration and update the Horizontal/Vertical radiating sub-array spacing, Number of element rows in sub-array and Conducted power (before Ohmic loss) per sub-array/element (dBm) parameters for the specified scenarios.

### Channel Bandwidth

Keep 100 MHz as baseline. If channel bandwidth is increased, to other options like 200 MHz or 400 MHz, the PSD decreases.

RAN4 to clarify for Urban Macro and Indoor scenario that BS output power is given per 100 MHz, so if wider channel bandwidth is considered, the PSD reduces.

### Transmission power control

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in TR 36.942, subclause 9.1 is applied with following parameters.

-	CLx-ile = –SNR_target + UE_max_eirp– ThermalNoise – BS_NoiseFigure - 10*log10(BW)

-	γ = 1

Where, SNR_target for FR1 and FR2-1 is 15 dB.

For UL transmission power control, RAN4 to consider subclause 9.1 from TR 36.921 with SNR target as 15 dB.

# Conclusion

In the previous sections we made the following observations:

Observation 1	UL coverage is challenged in Urban Macro scenarios with the assumption of 20% Indoor users.

Observation 2	ISD higher than 450m can be considered as sufficient DL coverage is achieved assuming 0% indoor users. However, UL coverage still remains a challenge to be addressed with 20% Indoor users.

Observation 3	0% and 20% Indoor probability options present different behaviors, with the latter presenting UL coverage challenges with ISD as 450m.

Observation 4	The UE maximum Tx power should be clarified in the Table 6.1.2.4-1 of TR 38.922 for the two UE antenna architecture options.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 should further discuss on how to achieve sufficient UL coverage in Urban Macro scenarios, with a reduced ISD and/ or higher maximum transmit power.

Proposal 2	RAN4 to consider keeping only 0% Indoor users for Urban Macro scenario.

Proposal 3	RAN4 to clarify for Urban Macro scenario that BS output power 43 dBm is defined per polarization. It is given for 100 MHz channel bandwidth.

Proposal 4	RAN4 to consider Indoor office deployment with 1 sector, i.e. option 1 from TR38.802, section A.2.1 and capture it in TR 38.922.

Proposal 5	RAN4 to clarify for Indoor scenario that BS output power 23 dBm is dual polarized. It is given for 100 MHz channel bandwidth.

Proposal 6	RAN4 to revise the table in TR with the correct antenna parameters for (dv,sub) and Pre-set sub-array down-tilt and modify Note 3,4 and 5 wordings to maintain parity between the scenarios.

Proposal 7	RAN4 to adopt the proposed Antenna configuration parameters for Antenna sub-array configuration and update the Horizontal/Vertical radiating sub-array spacing, Number of element rows in sub-array and Conducted power (before Ohmic loss) per sub-array/element (dBm) parameters for the specified scenarios.

Proposal 8	RAN4 to clarify for Urban Macro and Indoor scenario that BS output power is given per 100 MHz, so if wider channel bandwidth is considered, the PSD reduces.

Proposal 9	For UL transmission power control, RAN4 to consider subclause 9.1 from TR 36.921 with SNR target as 15 dB.

# References

- R4-2400333, “ITU-R WP5D LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz, and 14.8-15.35 GHz”.

- R4-2405874, “Work Plan on IMT parameters relevant for sharing and compatibility for 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 1530 MHz frequency ranges”, 3GPP RAN4#110-bis, Ericsson

- R4-2410741, “WF on IMT parameters study”, 3GPP RAN4#111, Ericsson

- R4-2410763, “3GPP TR 38.922 v01.0: Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz, and 14800-15350 MHz”, 3GPP RAN4#111

- R4-2412127, “On Urban Macro and Indoor Co-existence simulation results for 14800 to 15350 MHz frequency range”, 3GPP RAN4#112, Ericsson

R4-2412128, “On Antenna feasibility and RF parameters for 14800 to 15350 MHz frequency range”, 3GPP RAN4#112, Ericsson