3GPP TSG-RAN4 Meeting #112	R4-2412410

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4822 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on test cases for Rel-18 NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add applicability rule of the tests. Fix some mistakes of the test setup |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove editor’s note Add applicability rule of the tests. Fix some mistakes of the test setup |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The configuration can not be set correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.14.3.1.2 & A.14.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------Start Change #1-------------------------

A.14.3.1.2	NR UE Transmit Timing Test for FR2-NTN

A.14.3.1.2.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the reference cell and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1C.2. Supported test configurations are shown in Table A.14.3.1.2.1-1.

Table A.14.3.1.2.1-1: Supported test configurations for FR2-NTN PCell

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 120 kHz, data SCS 120 kHz, BW 100 MHz |
| 2 | NGSO, NR FDD, SSB SCS 120 kHz, data SCS 120 kHz, BW 100 MHz |
| 3 | GSO, NR FDD, SSB SCS 120 kHz, data SCS 120 kHz, BW 100 MHz |
| Note: For mobile VSAT UE type, it is only requied to pass test config 3. |  |

The test consists a single NR cell (PCell). Table A.14.3.1.2.1-2 and A.14.3.1.2.1-2A defines the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.14.3.1.2.1-3.

Table A.14.3.1.2.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 | Test2 |
| --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1,2,3 | Freq1 | Freq1 |
| Serving satellite configuration |  | 1 | SSC.1. |  |
|  |  | 2 | SSC.2 |  |
|  |  | 3 | SSC.1 |  |
| BWchannel | MHz | 1,2,3 | 100: NRB,c = 66 |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |
| DRX Cycle | ms | 1,2,3 | N/A | DRX.8Note5 |
| PDSCH Reference measurement channel |  | 1,2,3 | SR.X.X FDD |  |
| RMSI CORESET Reference Channel |  | 1,2,3 | CR.X.X FDD |  |
| Dedicated CORESET Reference Channel |  | 1,2,3 | CCR.X.X FDD |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |
| SSB configuration |  | 1,2,3 | SSB.3 FR2 |  |
| SMTC Configuration |  | 1,2,3 | SMTC.1 |  |
| TRS configuration |  | 1,2,3 | TRS.2.1 FDD |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Propagation condition |  | 1,2, | AWGN |  |
|  |  | 3 | AWGN with constant Doppler [TBD]Hz |  |
| SRS Config |  | 1,2,3 | SRSConf.1Note6 | SRSConf.2Note6 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: DRX related parameters are given in Table A.3.3.8-1 Note 6: SRS configs are given in Table A.14.3.X.1.1-3 |  |  |  |  |

Table A.14.3.1.2.1-2A: OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  |
| Assumption for UE beamsNote 6 |  | Fine (For electronic steering antenna type) RX beam of RX beam peak direction (For mechanical steering antenna type) |  |
| Note1 | dBm/15kHzNote4 | -112 |  |
| Note1 | dBm/SCSNote3 | -103 |  |
|  | dB | 4 |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -96 |  |
|  | dB | 4 |  |
| IoNote2 | dBm/95.04 MHz Note4 | -68.5 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS B_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Void Note 5: Void Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |

Table A.14.3.1.2.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 | SRSConf.2 | Comments |
| --- | --- | --- | --- | --- |
| SRS- | srs-ResourceSetId | 0 | 0 |  |
| ResourceSet | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | 17 | 17 | Matches NRB,c |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1, 0 | Sl2560, 4 | Offset to align with DRX periodicity |
|  | sequenceId | 0 | 0 | Any 10 bit number |

A.14.3.1.2.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1)	Set up PCell according to parameters given in Table A.14.3.1.2.1-2.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±(Te_NTN –TGNSS_margin)  of the first detected path of DL SSB.

a.	The NTA_offset value (in Tc units) is 0

b.	The NTA,adjcommon value is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation.

c.	The NTA,adjUE value is computed by the UE based on UE position and serving-satellite-ephemeris-related higher-layers parameters.

d.	The Te_NTN values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1C.2-2 and 7.1C.2-3

e.	The TGNSS_margin counts for the margin for the GNSS position definition error considered in the core requirement, which needs to be subtracted for the test requirement, due to the usage of AT commands in the test. TGNSS_margin=327,68×Tc

3)	If the NTN parameters are configured as GSO scenario, the test system shall adjust the timing of the DL path by values given in Table A.14.3.1.2.2-1. If the NTN parameters are configured as NGSO scenario, the test system shall adjust the timing of the DL path according to the serving-satellite-ephemeris-related higher-layers parameters.

Table A.14.3.1.2.2-1: Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 120 | +[8]*64Tc | +[4]*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in Clause 7.1C.2 Table 7.1C.2.1-1 until the UE transmit timing offset is within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±(Te_NTN –TGNSS_margin) respective to the first detected path (in time) of DL SSB. Skip this step for test 2 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±(Te_NTN –TGNSS_margin) of the first detected path of DL SSB. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

----------------------------------------End Change #1-------------------------

----------------------------------------Start Change #2-------------------------

A.14.3.2.3	SA FR2-NTN timing advance adjustment accuracy

A.14.3.2.3.1	Test Purpose and Environment

The purpose of the test is to verify UE Timing Advance adjustment delay and accuracy requirement defined in clause 7.3C.

A.14.3.2.3.2	Test Parameters

Supported test configurations are shown in table A.14.3.2.3.2-1. Both timing advance adjustment delay and accuracy are tested by using the parameters in table A.14.3.2.3.2-2, A.14.3.2.3.2-3 and A.14.3.2.3.2-4.

In all test cases, single cell served by SAN is used. Each test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.14.3.2.3.2-4, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

The UE shall be provided with the valid information about the SAN serving cell before the test. During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.4 in TS 38.321 [7]. The Timing Advance Command value shall be set to 31, which according to Clause 4.2 in TS 38.213 [3] results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.14.3.2.3.2-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in Clause 7.3C.2.1, the UE adjusts its uplink timing at slot n+k+1+2µ∙Koffset for a timing advance command received in slot n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 38.321 [7], shall be configured so that it does not expire in the duration of the test.

Table A.14.3.2.3.2-1: Timing advance supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 120 kHz, data SCS 120 kHz, BW 100 MHz |
| 2 | NGSO, NR FDD, SSB SCS 120 kHz, data SCS 120 kHz, BW 100 MHz |
| 3 | GSO, NR FDD, SSB SCS 120 kHz, data SCS 120 kHz, BW 100 MHz |
| Note: For mobile VSAT UE type, it is only requied to pass test config 3. |  |

Table A.14.3.2.3.2-2: General test parameters for timing advance

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF channel number |  | 1 |  |
| Initial DL BWP |  | DLBWP.0.1 | As specified in Table A.3.9.2.1-1 |
| Dedicated DL BWP |  | DLBWP.1.1 | As specified in Table A.3.9.2.2-1 |
| Initial UL BWP |  | ULBWP.0.1 | As specified in Table A.3.9.3.1-1 |
| Dedicated UL BWP |  | ULBWP.1.1 | As specified in Table A.3.9.3.2-1 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | For 120 kHz SCS NTA_new = NTA_old + 1024*Tc (based on equation in clause 4.2 of TS 38.213 [3]) |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.14.3.2.3.2-3: Cell specific test parameters for timing advance

| Parameter |  |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| Duplex mode | Config 1,2,3 |  |  | FDD |  |
| Satellite information | Config 1 |  |  | SSC.1 |  |
|  | Config 2 |  |  | SSC.2 |  |
|  | Config 3 |  |  | SSC.1 |  |
| BWchannel | Config 1,2,3 |  | MHz | 100: NRB,c = 66 |  |
| BWP BW | Config 1,2,3 |  | MHz | 100: NRB,c = 66 |  |
| DRX Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel | Config 1,2,3 |  |  | SR.X.X FDD |  |
| RMSI CORESET Reference Channel | Config 1,2,3 |  |  | CR.X.X FDD |  |
| Dedicated CORESET Reference Channel | Config 1,2,3 |  |  | CCR.X.X FDD |  |
| TRS configuration | Config 1,2,3 |  |  | TRS.2.1 FDD |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |
| SMTC configuration | Config 1,2,3 |  |  | SMTC.1 |  |
| SSB configuration | Config 1,2,3 |  |  | SSB.3 FR2 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,3 |  | kHz | 120 kHz |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,3 |  | kHz | 120 kHz |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Propagation condition |  | Config 1, 2 | - | AWGN |  |
|  |  | Config 3 |  | AWGN with constant Doppler [TBD]Hz |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void |  |  |  |  |  |

Table A.14.3.2.3.2-3A: OTA related test parameters

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  |
| Assumption for UE beamsNote 6 |  | Fine (For electronic steering antenna type) RX beam of RX beam peak direction (For mechanical steering antenna type) |  |
| Note1 | dBm/15kHzNote4 | -112 |  |
| Note1 | dBm/SCSNote3 | -103 |  |
|  | dB | 4 |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -99 |  |
|  | dB | 4 |  |
| IoNote2 | dBm/95.04 MHz Note4 | -68.5 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |

Table A.14.3.2.3.2-4: Sounding Reference Symbol Configuration for timing advance

| Field | Value | Comment |
| --- | --- | --- |
| c-SRS | 16 | Frequency hopping is disabled |
| b-SRS | 0 |  |
| b-hop | 0 |  |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset | sl5=4 for SCS 120kHz | Once every 5 slots |
| pathlossReferenceRS | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage | Codebook | Codebook based UL transmission |
| startPosition | 0 | resourceMapping setting. SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols | n1 |  |
| repetitionFactor | n1 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

A.14.3.2.1.3	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. k+1+2µ∙Koffset slots after the reception of the timing advance command, where k=11.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3C.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

----------------------------------------End Change #2-------------------------