3GPP TSG-RAN WG4 Meeting # 112	R4- 2412432

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	China Telecom

Title: 	Views on SAR Solution

Agenda Item:	8.1.1.1	High power UE (HPUE) for CA in terrestrial network (TN)

Document for:	Discussion

Introduction

In the last RAN4 meeting, about SAR solution we reached the following agreements for CA and increasing UE transmission power feature and we shall start the technical discussion.

So in this contribution we reiterate our point and provide some analysis.

Discussion

- 2.1 PC2 two band EN-DC with 2Tx and/or 3Tx

Duty-cycle solution is related to power class which determined by frequency band rather than Tx number, hence the general SAR solution framework for PC1.5 HPUE for intra-band CA and inter-band CA/DC with 3Tx is same as scenarios with 2Tx. So for PC2 two band EN-DC with 2Tx and/or 3Tx defined in WID, the legacy duty-cycle solution in 38.101-3 and corresponding capability and signalling shall be reused.

Proposal1: For PC2 two band EN-DC with 2Tx and/or 3Tx, the legacy duty-cycle solution in 38.101-3 and corresponding capability and signalling shall be reused.

- 2.2 PC1.5 HPUE for intra/inter-band CA

For PC1.5 HPUE for intra/inter-band CA, since only the MOP requirements are changed, the general SAR solution framework above can be kept using. The maximum UplinkDutyCycle threshold should have two values: maxUplinkDutyCycle-interBandCA-PC2 and 0.5* maxUplinkDutyCycle-interBandCA-PC2, referring to the PC2 UE for CA. When the total average percentage of uplink symbols is between maxUplinkDutyCycle-interBandCA-PC2 and 0.5* maxUplinkDutyCycle-interBandCA-PC2, ΔPPowerClass,CA =3dBm; When the total average percentage of uplink symbols is lager than maxUplinkDutyCycle-interBandCA-PC2, ΔPPowerClass,CA =6dBm.

Proposal2: For PC1.5 HPUE for intra/inter-band CA, the general SAR solution framework and the threshold of average percentage of uplink symbols should both refer to PC2 UE for CA and PC1.5 UE for single CC.

The average percentage of uplink symbols should also be defined as 50%×( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y) since the power class is per band rather than per Tx. The MOP restriction of UE for two band with 2Tx is same as UE for two band with 3Tx. And the default value of maxDutyNR,x/y should be 25% according to the baseline of SAR requirements.

Proposal3: For PC1.5 HPUE for intra/inter-band CA, if power class of one or both of the bands within the band combination is power class 1.5, the default value of maxDutyNR,x/y should be 25%.

- 2.2 PC1.5 HPUE for inter-band EN-DC

For inter band EN-DC, the legacy duty-cycle solution has two modes: FDD+TDD and TDD+TDD, and each of them has individual mechanism and threshold values. In Rel-19 in this WI we extend the scenarios to include PC1.5 and FDD+FDD, so total SAR solution need to be updated.

For legacy modes FDD+TDD and TDD+TDD with PC1.5, since only the MOP elevated, the mechanism and UE capability can be reused but we should further study the default duty-cycle threshold and trigger condition. As a more acceptable way, we can start with taking half of PC2 default threshold and 0.5*PC2capability(e.g. maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16/ maxUplinkDutyCycle-FDD-TDD-EN-DC1) as new default duty-cycle threshold and trigger condition of SAR solution for PC1.5 of FDD+TDD/TDD+TDD.

Proposal4: Start with taking half of PC2 default duty-cycle threshold and 0.5*PC2capability as new trigger condition of SAR solution for PC1.5 of FDD+TDD/TDD+TDD.

For PC1.5 FDD+FDD band combination mode in Rel-19, the design of duty-cycle solution cannot settle down until the agreement of general configurations and requirements. On the other hand, since the FDD+FDD duty-cycle solution is a new part, i.e. a new feature, the UE capability and BS indication should be considered.

Obsveration1: The design of duty-cycle solution should wait until the agreement of general configurations and requirements, and the UE capability and BS indication should also be considered.

Conclusion

Obsveration1: The design of duty-cycle solution should wait until the agreement of general configurations and requirements, and the UE capability and BS indication should also be considered.

Proposal1: For PC2 two band EN-DC with 2Tx and/or 3Tx, the legacy duty-cycle solution in 38.101-3 and corresponding capability and signalling shall be reused.

Proposal2: For PC1.5 HPUE for intra/inter-band CA, the general SAR solution framework and the threshold of average percentage of uplink symbols should both refer to PC2 UE for CA and PC1.5 UE for single CC.

Proposal3: For PC1.5 HPUE for intra/inter-band CA, if power class of one or both of the bands within the band combination is power class 1.5, the default value of maxDutyNR,x/y should be 25%.

Proposal4: Start with taking half of PC2 default duty-cycle threshold and 0.5*PC2capability as new trigger condition of SAR solution for PC1.5 of FDD+TDD/TDD+TDD.

References

- R4-2410750, WF on HPUE, RAN4#111