3GPP TSG-RAN WG4 Meeting #112	R4-2412495

Maastricht, Netherlands, Aug 19 – 23, 2024

Source:	Nokia

Title:	Discussion on L3 Rx beam sweeping factor reduction

Agenda item:	8.15.2.1

Document for:	Discussion

Introduction

The discussion about RRM phase 5 enhancements for L3 measurement delay reduction started in the RAN4#110bis meeting. In this contribution we continue the discussion based on the agreed way forward from RAN4#111 meeting [1].

Conditions for BSF reduction

Issue from last RAN4 meeting regarding BSF reduction conditions [1]:

#### Issue 2-1-2: Conditions to apply L3 measurement delay reduction by optimizing Rx BSF [Moderator note]: The condition here means in which case/condition/use-case/mode UE can apply the L3 measurement delay reduction by optimizing Rx BSF. Conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF: - FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature - Option 1 (Apple, CATT, ZTE, vivo, CTC, Intel): multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature. - Option 1a (ZTE): The UE is in multi-Rx operation if following condition is met: UE is configured with group-based beam reporting (GBBR) report - Option 1b (Intel): The UE is considered activated in multi-Rx simultaneous reception mode when the UE is configured with group-based beam reporting. The UE is considered activated for L3 reporting when the GBBR is configured not long prior to the expected L3 reporting. - Option 2 (LGE): The L3 measurement delay enhancement requirements for UE supporting multi-Rx simultaneous reception can apply if multiple panels are activate and SSBs in a SMTC window can be measured with multiple beams. - Option 3 (CMCC): the conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF is that multi-Rx simultaneous reception of UE is in active mode, as for whether the condition is same as that for Rel-18 multi-Rx simultaneous reception can be further discussed. - Option 4 (HW, Nokia, Ericsson): Do not reuse the same applicable conditions specified in Rel-18 multi-Rx - Option 4a (QC) : R19 Rx BSF is independent to R18 multi-Rx simultaneous reception feature includes conditions and requirement. - Option 5 (MTK): The conditions in R18 multi-Rx WI do not limit the discussion on having further conditions in R19 for UE supporting multi-Rx to enhance FR2-1 SSB based L3 measurement delay

Figure 1 shows one example of how the Rel-18 and Rel-19 multi-Rx enhancements can be distinguished.

Figure 1 Scenarios for multi-Rx L3 fast beam sweeping

During the last RAN4 meeting there was some discussion about how to reuse Rel-18 multi Rx conditions for BSF (beam sweeping factor) reduction for L3 measurements in the Rel-19 context. In our view this would be a very limiting strategy, since the benefits of L3 fast beam sweeping are much less evident in the conditions where the UE would have a good coverage and receiving data from multiple TRPs as in the Rel-18 scenario.

The Rel-18 features include receiving data from 2 TRPs with different QCL Type-D, L1 scheduling and measurement restrictions enhancements using multiple receive chains, and L1 fast beam sweeping. All of those features are enabled by configuration of Rel-17 group based beam reporting. Additionally, in Rel-18 some conditions were defined for considering multi Rx reception, including RRM, RF, and demodulation requirements, which are:

- The received power level between the 2 TRPs should not be large

- The receive time difference between the 2 TRPs is within CP length

- The angle of arrival between the 2 TRPs is larger than 30 degrees

For that reason, the Rel 18 multi-Rx enhancements can only be enabled in a limited area of a cell containing multiple TRPs that satisfy all of those conditions. In the example of Figure 1, the area satisfying the Rel-18 conditions is the green area between the two TRPs.

Rel-18 multi-Rx enhancements are designed for a limited area of the cell center.

For Rel-19, the RRM_ph5 establishes as an objective to develop BSF reduction for L3 measurements. In our view L3 measurements enhancements would be more useful in providing the UE with faster and more accurate mobility decisions. Therefore, BSF reduction enhancements are better suited for cell edge scenario in Rel-19. Figure 1 shows a blue area where L3 BSF reduction can achieve the best benefit.

L3 BSF reduction could also be used in the cell center, as in the white/green area of Figure 1. In that case, one manner that BSF reduction can be used for better system performance is if the network combines BSF reduction with a SMTC configuration that decreases the number of measurements, while maintaining the same measurement delay. If that is applied, the UE can benefit from a reduction in the measurement occasions with scheduling restrictions, which would result in some throughput enhancement, when the UE is in good radio conditions.

L3 fast beam sweeping due to multi-Rx operation can provide good improvement to mobility decisions at cell edge.

L3 fast beam sweeping due to multi-Rx operation can be combined with longer SMTC periodicity, providing the same measurement delay as legacy, but with less scheduling restrictions, when the UE is in good radio conditions.

RAN4 to consider L3 measurement delay reduction targeting cell edge scenarios as the first priority.

Do not reuse Rel-18 multi-Rx BSF reduction activation conditions for Rel 19 multi Rx BSF reduction.

Power consumption

Issue from last RAN4 meeting the issue was open for discussion regarding power consumption[1]:

- FFS: Power consumption issue - Option1 (Apple): considering power consumption, BSF reduction of L3 measurement will not trigger UE to activate multi-Rx. - Option 2 (LGE): For power consumption of multi-Rx operation, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. - Option 3 (Ericsson): To avoid unnecessary power consumption and computation load, enabling/disabling the parallel L3 measurement on multiple panels (if it is one of solutions addressing L3 measurement delay) may be determined by at the least one of the below options: - Option 3-1: NW indicates UE enabling parallel L3 measurement on multiple panels for serving L3 measurement delay reduction through L3 or lower layers signalings. - Option 3-2: UE determines to apply parallel L3 measurement on multiple panels for serving L3 measurement delay reduction, and enable it after acknowledged by NW. - Option 3-3: UE determines to apply parallel L3 measurement on multiple panels for serving L3 measurement delay reduction if a condition is fulfilled, e.g., at cell edge. NW may be aware of it by sending a ‘allowance’ signalling or not aware of it. - Option 4 (Nokia): Discuss FBS triggering conditions among the following options: - Option 4-1: Network configuration of FBS (similar as option 3-1) - Option 4-2: Mobility Event triggering FBS - Option 4-3: Conditional Handover configuration (…) - FFS: cell-centre UE or cell-edge UE - Option 1 (Nokia, MTK): RAN4 to consider L3 FBS targeting cell edge scenarios

It has been brought up by some companies that BSF reduction may increase battery consumption, and therefore it is desirable to limit the time when the UE is required to use BSF reduction. With that in mind, RAN4 can consider few options where L3 BSF reduction is enabled to provide better performance on cell edge. In the simplest case, the network might configure the UE to use BSF reduction. The network can configure that based on the reported UE measurements and UE multi-Rx preference indication. This is the option that gives more flexibility, since BSF reduction could also be used in combination to configure a longer SMTC period, which would result in keeping the same measurement delay while increasing the scheduling opportunities for the UE.

In a second option, BSF reduction may be enabled based on mobility events. A mobility event can be used to determine when the UE is at cell edge, and therefore should be using reduced BSF. In a third option, BSF reduction can be enabled when the network configures conditional HO. In that situation, the network proactively configures the UE with a potential target cell before the HO is needed, and by applying BSF reduction the UE would be quicker in reacting and making the CHO decision.

Lastly, BSF reduction may be always enabled, but used in combination with adaptation of the SMTC periodicity. In that case, the benefits of BSF reduction can be used also in cell center, providing additional throughput for the UE by removing some of the scheduling restrictions. In the cell edge, SMTC periodicity could be reduced in order to improve neighbor cell measurements and improve mobility decisions.

Discuss BSF reduction triggering conditions among the following options:

  - Option 1: Network configuration of BSF reduction

  - Option 2: Mobility Event triggering BSF reduction

  - Option 3: Conditional Handover configuration

  - Option 4: BSF reduction is always enabled, but used for reduced measurement delay in cell edge and used for reduced scheduling restrictions in cell center (e.g. by extending T_SMTC).

SSB processing delay

Issue from last RAN4 meeting the issue was open for discussion regarding SSB processing delay [1]:

- FFS: SSB processing delay/time for processing multiple beams received in a SMTC - Option 1 (vivo, QC, Intel, Apple): needs SSB processing delay/time for processing multiple beams received in a SMTC

It has been brought up by some companies that a UE receiving multiple beams in a SMTC window will, due to UE hardware design, need additional time for processing the received SSBs. The SSB processing time is assumed to be 2 ms as in multiple legacy requirements. If UE receives 2 different beams in a SMTC, then it can process one SSB during/after the SMTC window (while receiving), and another SSB can be post-processed in the subsequent time period. Hence, when looking at the total cell identification delay, the additional SSB processing time would only need to be added once to the total delay to cover the processing of the last measurement.

If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as

  - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms

  - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms

  - where TSSB_processing = 2 ms

Simultaneous L1 and L3 BSF reduction

The following issue is open for the discussion of simultaneous operation between L3 and L1 (Rel-18) measurement [1]:

- FFS: Simultaneous operation between L3 and L1 measurements - Option 1 (ZTE): Simultaneous operation between L3 and L1 measurements by optimizing Rx BSF, simultaneous operation between L3 measurement and data reception by optimizing Rx BSF - Option 2 (Ericsson): Scenario where L3 measurement is reduced using reduced beam sweeping and scenario where L1 measurement is reduced using multiple-reception from multi-TRP for DL measurement/data are different scenarios and not expected to operate simultaneously. - L3 measurement delay reduction may be influenced by L1 measurement/data transmission scheme, wherein L1 measurement may be: - Legacy requirements or, - Enhanced requirements for Multi-Rx in Rel-18 - We prefer legacy requirements between them. Once L1 measurement is chosen, we shall further check L1/L3 sharing scheme.

The issue above discusses whether L3 and L1 BSF reduction may be used by a UE simultaneously. In existing specification, one SSB will be either used for L1 measurements and procedures or by L3 procedures. This is guaranteed by the application of the P factor (see 9.5.4 for L1-RSRP requirements). Therefore, considering that one SSB burst will be either used for L1 or L3 measurements, there is no reason to assume that a UE would be unable to perform L3 BSF reduction if it is also activated to use Rel-18 multi-Rx BSF reduction. The only impact that this WID could foresee in this scenario would be that the P factor is optimized, but we understand that this is not within the scope agreed in the plenary.

L1 and L3 measurements are not performed in the same SSB occasions.

Rel-18 L1 BSF reduction operates independently of Rel-19 L3 BSF reduction.

Capability

According to capability considerations, it is recommended to introduce a new capability to reduce the beam sweeping factor for L3. The specific details of this capability should be discussed once the feature becomes clearer. As a starting point, it is suggested that RAN4 discusses the Rel-19 enhancement as an independent capability that does not necessarily have to be linked to the existing multi-Rx/multi-TRP capabilities.

RAN4 to introduce a new individual capability for L3 beam sweeping factor reduction due to multi-Rx operation.

Scenarios for L3 BSF reduction

In the last meeting, the scenarios for BSF reduction on RAN4 requirements was discussed in the company contributions, and the following options were listed in the WF  [1]:

#### Issue 2-1-3: Scenarios to use L3 measurement delay reduction by optimizing Rx BSF [Moderator note]: The scenarios here means which UE behavior/activity/procedure(s) would be improved with this feature or which corresponding requirements in the existing RRM spec will be enhanced to accommodate this feature. FFS on the justifications to decide which of the following scenarios shall be considered. - Encourage companies to discuss the expected benefit to considering BSF reduction for the following scenarios. FFS on following scenarios to decide which ones are considered to use L3 measurement delay reduction by optimizing Rx BSF: - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Option 1(Apple, NTT DCM, CATT, Xiaomi, OPPO, LGE, CMCC, ZTE, CTC, HW, Ericsson): Yes - Option 1a (OPPO): For deactivated SCell and PSCell in FR2-1, the enhancement of TPSS/SSS_sync and TSSB_measurement_period can also apply. - Option 2: No - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Option 1(Apple, NTT DCM, CATT, Xiaomi, OPPO, LGE, CMCC, ZTE, CTC, HW, Ericsson): Yes - Option 2: No - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Option 1(Apple, NTT DCM, CATT, Xiaomi, OPPO, LGE, CMCC, ZTE, CTC, HW, Ericsson): Yes - Option 1a (OPPO): For deactivated SCell and PSCell in FR2-1, the enhancement of TPSS/SSS_sync and TSSB_measurement_period can also apply. - Option 2: No - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Option 1(Apple, NTT DCM, CATT, Xiaomi, OPPO, LGE, CMCC, ZTE, CTC, HW, Ericsson): Yes - Option 2: No

As a starting point, RAN4 should at least agree that Rel-19 L3 BSF reduction applies to the measurement requirements in section 9.2 and 9.3.

Rel-19 BSF reduction applies for

  - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

  - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

Additionally, there were proposals to extend the BSF reduction also to other requirements:

- Scenario 5: Handover - Option 1(NTT DCM, CATT, CMCC, ZTE, CTC, Ericsson, MTK): Yes - Option 2(vivo): No - Option 3 (Apple, Xiaomi, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario - Scenario 6: PSCell addition - Option 1(NTT DCM, ZTE, MTK): Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection - Option 1(NTT DCM, ZTE): Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario - Scenario 8: SCell activation - Option 1(MTK): Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario - Scenario 9: SCG activation - Option 1(ZTE, MTK): Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario - Scenario 10: CGI identification - Option 1 (NTT DCM, ZTE): Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario. - Scenario 11: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB. The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded. - Option 1: Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario

In our view, RAN4 should consider at least some additional scenarios for L3 BSF reduction. In one example BSF reduction could be activated during handover. That would benefit the UE by reducing the handover interruption duration. In another example, if BSF reduction is enabled in combination to conditional handover, the UE can realize the condition for CHO to be met and switch to a new gNB faster if needed.

Another potential scenario is SCell addition in FR2. As an example, in Rel-18 RAN4 defined requirements for fast SCell activation using reduced beam sweeping. However, this optimization did not include AGC. As a result, multi-Rx L3 enhancement could be used as well to enhance the SCell activation delay. This type of optimization can reduce the SCell activation delay by 160 ms by using 2 Rx chains and assuming a SSB periodicity of 20 ms.

After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to these scenarios:

  - Scenario 5: Handover

  - Scenario 6: PSCell addition

  - Scenario 8: SCell activation

  - Scenario 9: SCG activation

Not to consider the following scenarios for FBS:

  - Scenario 10: CGI identification

  - Scenario 11: CSI-RS based intra-/inter-frequency measurements

UE power class

In the last meeting the following was discussed regarding power class for L3 BSF reduction:

#### Issue 2-1-1: Applicability requirement of L3 measurement delay reduction by optimizing Rx BSF (…) UE Power class: FFS the following options and moderator option: - Option 1 (Apple, OPPO, LGE, vivo, HW, Samsung): - RAN4 to consider UE supporting FR2-1 power class 3 as first priority. - Option 1a (LGE): - but RAN4 should consider if other power classes could apply the outcome of the WI discussion - Option 2 (CATT): - It is proposed not to restrict the number of configured carriers and applied power class. - Option 3 (Ericsson): - We can have generic requirements for all power classes, maybe PC6 can be precluded, if no specific use cases for some power classes are pursued. - Option 4 (Moderator) - Baseline: RAN4 to consider UE supporting FR2-1 power class 3 as first priority. - Note: whether other power classes could apply the outcome of the WI discussion can be FFS after concluding on PC3. These extra FFS parts will NOT delay the WI completion. (…) #### Issue 2-1-2: Conditions to apply L3 measurement delay reduction by optimizing Rx BSF (…) - FFS: UE’s mobility status, e.g., whether HST is precluded or not - Option 1 (Apple, ZTE(preclude HST), vivo, CTC, Samsung(preclude HST)): low mobility status, i.e., preclude HST - Option 2 (CATT, CMCC): enhanced BSF can also be used for HST - Option 3 (Ericsson): no scenario restriction (e.g., low mobility) is needed - Option 4 (MTK): For UE with high mobility

Some companies want to have some prioritization regarding power classes which can use the Rel 19 fast beam sweeping feature. In Rel-17, HST was defined for L1 measurements using an assumption that reduced beam sweeping scaling factor. It is unclear why to preclude certain power classes, since the extended measurement delay is a general FR2 problem that can be applicable to many types of UEs.

L3 measurement enhancements are not restricted by power class.

UE prior knowledge of the cell to be measured

Some companies proposed in the last RAN4 meeting that BSF reduction should be possible based on prior knowledge of the cell to be measured.

- FFS: UE has prior knowledge on the cell to be measured - Option 1 (CATT): on top of the UE capability of supporting Multi-Rx, no additional conditions of prior knowledge for target cell is needed - Option 2 (vivo, CTC, MTK): UE needs prior knowledge on the cell to be measured - Option 2a (CTC): It’s proposed to consider conditions of prior knowledge on the cell to be measured and discuss whether the conditions of prior knowledge are applicable - Option 3 (Ericsson): UE may only measure part of spatial directions with one panel out of multiple panels. It reduces L3 measurement delay as well, upon acquiring prior knowledge on the cell to be measured, e.g., - The UE has measured the cell before in a time period. - The UE has knowledge on the absolute/relative location of the cell to be measured. - The UE has knowledge on its moving state (including rotation).

We understand that those proposals mean that the UE has some idea on which direction a cell is, and only performs beam sweeping based on few beams around that position.

This approach presents few problems that we want to highlight here. Firstly, the UE is required to measure more than one cell. As an example, intra-frequency requirements related to number of cells and SSBs state that a UE shall be capable of measuring at least 6 identified cells and 24 SSBs with different SSB index and/or PCI.

Legacy intra-frequency requirements state the UEs are required to be capable of measuring at least 6 identified cells and 24 SSBs with different SSB index and/or PCI.

Figure 2 shows one example where the UE is performing measurements of 6 cells. If Option 2 and 3 discussed in the previous meeting is agreed, that would mean that the UE would not be able to monitor all of the 6 cells shown in the figure. If the UE performs BSF reduction only with beams 1 to 4, the UE will only be able to measure cells 1, 2, and 3, but not cells 4, 5, and 6.

Figure 2 Example of the scenario where the UE is performing measurements of 6 cells

BSF reduction based on prior knowledge of cell to be measured means that the UE may not be able to fulfill the measurement requirements regarding number of monitored cells and SSB indexes.

The BSF reduction on this WID is based on multi-Rx, as agreed in the WID objectives. Therefore, BSF reduction is enabled by the UE monitoring multiple Rx beams simultaneously, and not by limiting the coverage of the measurements. For this reason, we think that option 1 discussed in the last meeting leads to a better technical solution which is well aligned with the WID objectives.

Prior knowledge of target cells to be measured is not to be used as a condition for L3 BSF reduction.

Conclusion

The paper the following Observations and Proposals were made:

Observation 1: Rel-18 multi-Rx enhancements are designed for a limited area of the cell center.

Observation 2: L3 fast beam sweeping due to multi-Rx operation can provide good improvement to mobility decisions at cell edge.

Observation 3: L3 fast beam sweeping due to multi-Rx operation can be combined with longer SMTC periodicity, providing the same measurement delay as legacy, but with less scheduling restrictions, when the UE is in good radio conditions.

Proposal 1: RAN4 to consider L3 measurement delay reduction targeting cell edge scenarios as the first priority.

Proposal 2: Do not reuse Rel-18 multi-Rx BSF reduction activation conditions for Rel 19 multi Rx BSF reduction.

Proposal 3: Discuss BSF reduction triggering conditions among the following options:

a.	Option 1: Network configuration of BSF reduction

b.	Option 2: Mobility Event triggering BSF reduction

c.	Option 3: Conditional Handover configuration

d.	Option 4: BSF reduction is always enabled, but used for reduced measurement delay in cell edge and used for reduced scheduling restrictions in cell center (e.g. by extending T_SMTC).

Proposal 4: If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as

a.	Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms

b.	Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms

c.	where TSSB_processing = 2 ms

Observation 4: L1 and L3 measurements are not performed in the same SSB occasions.

Proposal 5: Rel-18 L1 BSF reduction operates independently of Rel-19 L3 BSF reduction.

Proposal 6: RAN4 to introduce a new individual capability for L3 beam sweeping factor reduction due to multi-Rx operation.

Proposal 7: Rel-19 BSF reduction applies for

a.	Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

b.	Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

c.	Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

d.	Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

Proposal 8: After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to these scenarios:

a.	Scenario 5: Handover

b.	Scenario 6: PSCell addition

c.	Scenario 8: SCell activation

d.	Scenario 9: SCG activation

Proposal 9: Not to consider the following scenarios for FBS:

a.	Scenario 10: CGI identification

b.	Scenario 11: CSI-RS based intra-/inter-frequency measurements

Proposal 10: L3 measurement enhancements are not restricted by power class.

Proposal 11: Prior knowledge of target cells to be measured is not to be used as a condition for L3 BSF reduction.

- References

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5

- R4-2404761, Discussion on L3 measurement delay reduction, Nokia, Nokia Shanghai Bell.