3GPP TSG-RAN WG4 Meeting # 112	R4-2412589

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	8.2.4.1

Source: 	Nokia

Title: 	Urban macro coexistence simulation results for 14800 to 15350 MHz frequency range

Document for:	Discussion

# 1.	Introduction

The SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved at TSG RAN#103 [1]. One of the objectives of this SI is to study the IMT parameters relevant for sharing and compatibility for 14800 to 15350 MHz frequency range.

This study item aims as answering requests from ITU-R WP5D regarding NR in these frequencies for IMT [2]. Two set of parameters requested by ITU-R WP5D are the ACLR and ACS of BS and UE for 14800 to 15350 MHz frequency range. Currently, different sets of ACLR and ACS are specified in RAN4 specifications [3, 4, 5] for NR BS and UE in FR1 and FR2 based on coexistence studies as recorded in TR 38.803 [6]. Therefore, coexistence study will need to be carried out to provide answers to ITU-R WP5D on these parameters with sound technical justifications from coexistence perspective.

The text proposal on the simulation assumptions for the coexistence study was agreed in RAN4#111 [7]. This contribution provides the urban macro coexistence simulation results for 14800 to 15350 MHz frequency range according to the agreed assumptions.

# 2.	Discussion

2.1	Downlink Results

The urban macro simulation results of the downlink SINR of the victim UE at 15 GHz carrier frequency with 450 m inter-site distance, 20% indoor UE ratio, 400 MHz channel bandwidth (previous coexistence studies have shown simulation results are similar with different channel bandwidths) and 1 UE per slot are provided in Figure 1 below using different BS and UE antenna configurations. Here the downlink ACIR is obtained using 45dB BS ACLR and 33dB UE ACS (currently defined for BS and UE in FR1). It can be seen from the figures that in all simulated cases enough downlink SINR (up to 65dB and 70dB for FR1 and FR2 like UE, respectively) can be provided using 43dBm BS conducted output power (as agreed in [7]) together with AAS BS beamforming.

(a) 64x24 BS, FR1 like UE					(b) 64x24 BS, FR2 like UE

(c) 64x32 BS, FR1 like UE					(d) 64x32 BS, FR2 like UE

Figure 1: Urban Macro DL SINR of victim UE

The urban macro simulation results of the downlink throughput loss of the victim UE with different ACIR offsets (compared to the ACIR using 45dB BS ACLR and 33dB UE ACS) with coordinated and un-coordinated operations are provided, respectively, in Tables 1 and 2 below. It can be seen from the tables that the average and 5%-tile downlink throughput losses of the victim UE in all simulated cases are below 5% with downlink ACIR offset of -15dB, except case (b) with coordinated operation and case (c) with un-coordinated operation where the 5%-tile throughput loss is higher than 5%. The higher downlink ACIR offset compared to those at 7GHz and 10 GHz carrier frequencies in TR 38.921 [8] can be explained by the fact that the larger BS antenna configurations at 15 GHz can achieve narrower beamwidths and thus lower interference from the interfering BS to the victim UE.

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (a) | 0.0012 | 0.020 | 0.0033 | 0.053 | 0.082 | 0.0122 |
| 5%-tile throughput loss (a) | 2.9364e-06 | 5.8590e-06 | 1.1690e-05 | 2.3324e-05 | 4.6537e-05 | 0.0162 |
| Average throughput loss (b) | 8.8428e-04 | 0.0016 | 0.0026 | 0.0040 | 0.0057 | 0.0078 |
| 5%-tile throughput loss (b) | 1.7440e-07 | 0.0098 | 0.0098 | 0.0098 | 0.0310 | 0.0671 |
| Average throughput loss (c) | 0.0011 | 0.0018 | 0.0030 | 0.0048 | 0.0078 | 0.0119 |
| 5%-tile throughput loss (c) | 2.4761e-08 | 4.9405e-08 | 9.8576e-08 | 1.9669e-07 | 3.9244e-07 | 0.0022 |
| Average throughput loss (d) | 6.1380e-04 | 0.0011 | 0.0019 | 0.0032 | 0.0049 | 0.0074 |
| 5%-tile throughput loss (d) | 1.1146e-08 | 2.2238e-08 | 4.4371e-08 | 0.0119 | 0.0119 | 0.0212 |

Table 1: Urban Macro DL throughput loss of victim UE (coordinated operation)

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (a) | 0.0015 | 0.0025 | 0.0041 | 0.0064 | 0.0098 | 0.0144 |
| 5%-tile throughput loss (a) | 3.3029e-08 | 6.5902e-08 | 1.3149e-07 | 0.0170 | 0.0478 | 0.0478 |
| Average throughput loss (b) | 5.0946e-04 | 8.0540e-04 | 0.0013 | 0.0020 | 0.0029 | 0.0043 |
| 5%-tile throughput loss (b) | 4.7097e-06 | 9.3971e-06 | 1.8749e-05 | 0.0128 | 0.0168 | 0.0265 |
| Average throughput loss (c) | 0.0023 | 0.0037 | 0.0057 | 0.0085 | 0.0096 | 0.0123 |
| 5%-tile throughput loss (c) | 3.9279e-09 | 7.8371e-09 | 1.5637e-08 | 3.1200e-08 | 3.9279e-08 | 0.0785 |
| Average throughput loss (d) | 6.1632e-04 | 9.6004e-04 | 0.0014 | 0.0021 | 0.0031 | 0.0045 |
| 5%-tile throughput loss (d) | 1.8081e-06 | 3.6076e-06 | 7.1981e-06 | 1.4362e-05 | 2.8655e-05 | 5.7173e-05 |

Table 2: Urban Macro DL throughput loss of victim UE (un-coordinated operation)

Comparing the results between Tables 1 and 2, the throughput loss for FR1 like UE is higher with un-coordinated operation than with coordinated operation, while the throughput loss for FR2 like UE is higher with coordinated operation than with un-coordinated operation, as the UE beam is directed towards the interfering BS that is co-located with the serving BS with coordinated operation. Comparing the results between case (a) and (b), the throughput loss for FR2 like UE is lower than that for FR1 like UE with the use of beamforming also at the victim UE.

2.2	Uplink Results

The urban macro simulation results of the victim and interfering UE transmit power at 15 GHz carrier frequency with 450 m inter-site distance, 20% indoor UE ratio, 400 MHz channel bandwidth, 1 UE per slot and 23 dBm UE maximum TX power are provided in Figure 2 below using different BS and UE antenna configurations. Here the uplink ACIR is obtained using 30dB UE ACLR and 45dB BS ACS (currently defined for BS and UE in FR1). It can be seen from the figure that around 30% of the FR1 like UE and 20% of the FR2 like UE are transmitting at maximum power, and that more UE (for both FR1 like and FR2 like UE) are transmitting at maximum power with 64x32 BS than with 64x24 BS.

(a) 64x24 BS, FR1 like UE					(b) 64x24 BS, FR2 like UE

(c) 64x32 BS, FR1 like UE					(d) 64x32 BS, FR2 like UE

Figure 2: Urban Macro UE transmit power

The urban macro simulation results of the victim UE UL SINR are provided in Figure 3 below. Here the uplink ACIR is obtained using 30dB UE ACLR and 45dB BS ACS (currently defined for BS and UE in FR1). It can be seen from the figure that the target UL SINR of 15dB is achieved by around 70% of the FR1 like UE and 80% of the FR2 like UE, and that fewer UE (for both FR1 like and FR2 like UE) achieved the target UL SINR with 64x32 BS than 64x24 BS.

(a) 64x24 BS, FR1 like UE					(b) 64x24 BS, FR2 like UE

(c) 64x32 BS, FR1 like UE					(d) 64x32 BS, FR2 like UE

Figure 3: Urban Macro UE UL SINR

The urban macro simulation results of the victim BS received blocking signal power at the TAB connector (i.e. including the antenna array gain) with coordinated and un-coordinated operations are provided, respectively, in Figures 4 and 5 below. The 99.99%-tile received blocking signal power levels are less than -41 dBm in all simulated cases.

(a) 64x24 BS, FR1 like UE					(b) 64x24 BS, FR2 like UE

(c) 64x32 BS, FR1 like UE					(d) 64x32 BS, FR2 like UE

Figure 4: Urban Macro received blocking signal power (coordinated operation)

(a) 64x24 BS, FR1 like UE					(b) 64x24 BS, FR2 like UE

(c) 64x32 BS, FR1 like UE					(d) 64x32 BS, FR2 like UE

Figure 5: Urban Macro received blocking signal power (un-coordinated operation)

The urban macro simulation results of the uplink throughput loss of the victim UE with different ACLR offsets (compared to the 30dB UE ACLR) with coordinated and un-coordinated operations are provided, respectively, in Tables 3 and 4 below. It can be seen from the tables that the average and 5%-tile uplink throughput losses in all simulated cases are below 5% with an uplink ACLR offset of -15 dB, except case (a) with coordinated operation and case (d) with un-coordinated operation where the 5%-tile throughput loss is higher than 5%.

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (a) | 0.0013 | 0.020 | 0.0031 | 0.047 | 0.069 | 0.0100 |
| 5%-tile throughput loss (a) | 0.0155 | 0.0244 | 0.0244 | 0.0244 | 0.0245 | 0.0592 |
| Average throughput loss (b) | 9.3834e-04 | 0.0014 | 0.0021 | 0.0031 | 0.0044 | 0.0061 |
| 5%-tile throughput loss (b) | 7.1089e-08 | 1.3967e-07 | 2.7652e-07 | 0.0256 | 0.0439 | 0.0452 |
| Average throughput loss (c) | 0.0013 | 0.0020 | 0.0030 | 0.0046 | 0.0068 | 0.0098 |
| 5%-tile throughput loss (c) | N/A | N/A | N/A | N/A | N/A | N/A |
| Average throughput loss (d) | 9.6386e-04 | 0.0015 | 0.0022 | 0.0032 | 0.0047 | 0.0067 |
| 5%-tile throughput loss (d) | 3.6033e-05 | 7.0794e-05 | 1.4014e-04 | 2.7848e-04 | 0.0471 | 0.0718 |

Table 3: Urban Macro UL throughput loss of victim UE (coordinated operation)

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (a) | 6.4202e-04 | 0.011 | 0.0017 | 0.027 | 0.040 | 0.0061 |
| 5%-tile throughput loss (a) | 6.2894e-06 | 1.2357e-05 | 2.4463e-05 | 4.8618e-05 | 9.6809e-05 | 1.9295e-04 |
| Average throughput loss (b) | 2.8979e-04 | 8.0540e-04 | 8.1882e-04 | 0.0013 | 0.0019 | 0.0028 |
| 5%-tile throughput loss (b) | 1.4703e-05 | 2.8886e-05 | 5.7186e-05 | 1.1365e-04 | 2.2628e-04 | 0.0092 |
| Average throughput loss (c) | 0.0011 | 0.0017 | 0.0025 | 0.0035 | 0.0050 | 0.0070 |
| 5%-tile throughput loss (c) | N/A | N/A | N/A | N/A | N/A | N/A |
| Average throughput loss (d) | 4.4003e-04 | 6.9887e-04 | 0.0011 | 0.0016 | 0.0022 | 0.0032 |
| 5%-tile throughput loss (d) | 1.2416e-06 | 2.4394e-06 | 4.8294e-06 | 9.5980e-06 | 1.9112e-05 | 3.8096e-05 |

Table 4: Urban Macro UL throughput loss of victim UE (un-coordinated operation)

Comparing the results between Tables 3 and 4, the throughput loss for FR1 like UE is higher with un-coordinated operation than with coordinated operation, while the throughput loss for FR2 like UE is higher with coordinated operation than with un-coordinated operation, as the UE beam is directed towards the interfering BS that is co-located with the serving BS with coordinated operation. Comparing the results between case (a) and (b), the throughput loss for FR2 like UE is lower than those for FR1 like UE with the use of beamforming also at the victim UE.

Note that 5%-tile throughput (i.e., cell edge coverage) cannot be achieved for case (c) with both coordinated and un-coordinated operations. This can be explained by the fact that higher sidelobes are formed in the vertical composite array pattern by the larger sub-array size for case (c) while the horizontal composite array patterns are similar for both cases, which means that there is higher interference received by the serving BS from other UEs in the neighbouring cells of the same network. Figures 6 and 7 below show the vertical composite array pattern at the azimuth angle of 0° and the vertical composite array pattern at the elevation angle of 96° with 6° mechanical downtilt.

(a) 64x24 BS									(b) 64x32 BS

Figure 6: Vertical Composite Array Pattern (0° azimuth angle)

(a) 64x24 BS									(b) 64x32 BS

Figure 7: Horizontal Composite Array Pattern (96° elevation angle)

# 3.	Conclusion

This contribution has provided the urban macro coexistence simulation results for 14800 to 15350 MHz frequency range according to the agreed assumptions.

Observations from simulation results:

1) The average and 5%-tile downlink throughput losses of the victim UE in all simulated cases are below 5% with downlink ACIR offset of -15dB, except case (b:64x24 BS, FR2 like UE) with coordinated operation and case (c:64x32 BS, FR1 like UE) with un-coordinated operation where the 5%-tile throughput loss is higher than 5%.

2) The average and 5%-tile uplink throughput losses in all simulated cases are below 5% with an uplink ACLR offset of -15 dB, except case (a:64x24 BS, FR1 like UE) with coordinated operation and case (d:64x32 BS, FR2 like UE) with un-coordinated operation where the 5%-tile throughput loss is higher than 5%.

3) 5%-tile throughput (i.e., cell edge coverage) cannot be achieved for case (c: 64x32 BS, FR1 like UE) with both coordinated and un-coordinated operations.

4) The throughput loss for FR1 like UE is higher with un-coordinated operation than with coordinated operation, while the throughput loss for FR2 like UE is higher with coordinated operation than with un-coordinated operation.

5) The throughput loss for FR2 like UE is lower than that for FR1 like UE with the use of beamforming also at the victim UE.

# References

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

[2]	R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz”, ITU-R WP5D.

[3]	3GPP TS 38.101-1 v18.5.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4]	3GPP TS 38.101-2 v18.5.0, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5]	3GPP TS 38.104 v18.5.0, “NR; Base Station (BS) radio transmission and reception”.

[6]	3GPP TR 38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.

[7]	R4-2410592, “TP to TR 38.922: System level simulation methodology and assumptions for coexistence study for 14800 - 15350 MHz frequency rang”, Nokia.

[8]	3GPP TR 38.921 v18.0.1, “NR; Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz”.