3GPP TSG-RAN WG4 Meeting # 112 	    R4-2412681

Maastricht, NL, 19 - 23 August 2024

Agenda Item:	6.1.1.4

Source: 	Ericsson

Title:	On core requirements for bandwidth aggregation for positioning measurements

Document for:	Approval

Introduction

In the last meeting remaining issues related to core requirement for PRS/SRS aggregation were discussed. The agreement reached and the open issues are listed in the WF document [1]. In this contribution Ericsson’s view on interruption delay requirement for SRS aggregation is presented.

Discussion

RAN4#111 discussed the LS response received from RAN1 and identified a need to define the interruption delay for the SRS transmission for bandwidth aggregation on CC without PUSCH/PUCCH.

Agreement from RAN4#111 [1]

- Define interruption requirements for SRS transmission with BW aggregation on CC without PUSCH/PUCCH.

- Reuse the framework of SRS carrier and/or antenna switching based interruption requirements as the starting point.

- The interruption length is FFS.

RAN4#109 in response to RAN1 LS agreed guard period values {0µs, 30µs, 100µs, 140µs, 200µs} as per band UE capability for SRS aggregation [2], [3]. These values were agreed by RAN4 RF group considering carriers configured for SRS transmission are intra-band and contiguous to each other and the CCs configured to transmitted SRS resources are transmitted using the same Tx chain. The interruption delay requirement for SRS aggregation should therefore be defined by taking the guard period needed before and after the transmission of SRS resources for aggregation and the duration of the aggregated SRS transmission. Since single Tx chain is the baseline assumption for SRS aggregation, interruption due to antenna switching is not relevant for SRS aggregation. Framework for SRS carrier switching shall therefore be only reused to define the interruption delay for SRS aggregation. In addition, interruption delay requirement should be defined separately for the UEs that are capable of supporting guard period values {0µs, 30µs, 100µs} and {140µs, 200µs} to ensure that the communication throughput of the UEs capable of faster RF switching is less impacted in comparison to the UE that require longer RF switching time.

Interruption requirements for SRS transmission with BW aggregation on CC without PUSCH/PUCCH are defined based on the existing SRS carrier switching framework only.

Interruption requirements are defined separately for UEs supporting guard period values {0µs, 30µs, 100µs} and UEs supporting guard period values {140µs, 200µs}.

Summary

In this contribution interruption delay requirement for SRS aggregation is discussed. The proposals below summarize the discussion presented in this paper.

- Interruption requirements for SRS transmission with BW aggregation on CC without PUSCH/PUCCH are defined based on the existing SRS carrier switching framework only.

Interruption requirements are defined separately for UEs supporting guard period values {0µs, 30µs, 100µs} and UEs supporting guard period values {140µs, 200µs}.

References

- R4-2410192, WF on RedCap positioning and PRS/SRS bandwidth aggregation, Ericsson.

- R4-2321741, Reply LS on guard period for SRS and PRS bandwidth aggregation for positioning, CATT.

- R1-2306216, LS to RAN4 on SRS and PRS bandwidth aggregation for positioning, ZTE.