3GPP TSG-RAN WG4 Meeting#112                              R4-2412709

Maastricht , NL, 19th – 23th Aug, 2024

Agenda item:	8.11.4

Source: 	ZTE Corporation,Sanechips

Title: 	Further discussion on Expected EIRP mask for upper 6GHz

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In last RAN4 meeting, we reached lots of consensus for U6GHz EIRP mask as captured in [5], however there are still lots of open issues left for further discussions. In this contribution, we would like to share further understandings on these remaining issues.

Issue 2-8 Expected EIRP sampling grid for average EIRP - Recommended for further discussion: - Need further discussions Issue 2-9 Other related with conformance testing declaration and RF channels - Recommended for further discussion: - The following issues to be considered: - the number of mechanical down-tilt and its corresponding angular coverage range; - For each angular coverage range, the number of beams for conformance testing and its corresponding weighting factor; - To balance the number for beams for conformance testing/complexity and test accuracy; - The impacts of potential factors (measurement sampling grid for summation error etc) on EIRP accuracy. - in conformance testing, consideration of converting coordinate system to 3GPP coordinate system, to align with other test cases - Other issues are not precluded. Issue 2-10 Other related with RF channels - Recommended for further discussion: - Need further discussions

Discussion

  - The number of mechanical downtilt and its corresponding coverage range

As mentioned in the ITU-R resolution requirements, the EEIRP mask requirement should be applicable for any mechanical down-tilt which is reasonable since NR BSs deployed in different targeted scenarios are supposed to be configured with different mechanical down-tilts to provide the corresponding coverage. If NR BS is capable to be deployed in lots of different scenarios, then NR BS should fulfill the EEIRP mask requirement at its supported mechanical down-tilt. In short, the concept proposed in [3] seems more comprehensive solutions to consist of both coverage angular range and steering ranges for each mechanical down-tilt.

Table 2.1-1: Manufacturer declarations

| CAR ID | Parameter | Description | Example values |
| --- | --- | --- | --- |
| 1 | qmin | Minimum angle along q axis | 0 degrees |
|  | qmax | Maximum angle along q axis | 6 degrees |
|  | jmin | Minimum angle along j axis | -60 degrees |
|  | jmax | Maximum angle along j axis | 60 degrees |
|  | Δk | Test beam power back-off | 0, 0, 0, 0, 0, 0, 0, 0, 0 dB |
| 2 | qmin | Minimum angle along q axis | 0 degrees |
|  | qmax | Maximum angle along q axis | 12 degrees |
|  | jmin | Minimum angle along j axis | -60 degrees |
|  | jmax | Maximum angle along j axis | 60 degrees |
|  | Δk | Test beam power back-off | 0, 0, 0, 0, 0, 0, 0, 0, 0 dB |
| ⋮ | ⋮ | ⋮ | ⋮ |
| Q | qmin | Minimum angle along q axis | 0 degrees |
|  | qmax | Maximum angle along q axis | 15 degrees |
|  | jmin | Minimum angle along j axis | -60 degrees |
|  | jmax | Maximum angle along j axis | 60 degrees |
|  | Δk | Test beam power back-off | 2, 2, 2, 2, 2, 2, 2, 2, 2 dB |

Proposal 1: to consider the manufacture declarations in Table 2.1-1 for different mechanical down-tilt and its the corresponding coverage angular range.

In addition, the direction range for each test beam within certain CAR should be declared by vendors accordingly. In addition, the number of test beams of each CAR will be further discussed to ensure the measurement EEIRP mask with sufficient confidence interval e.g. 95% in the following sections.

  - The number of beams for conformance testing and weighting factor

In the previous RAN4 meetings, there were some initial discussions on how to balance the number for beams for conformance testing/complexity and test accuracy. Technically speaking, the number of serving beams for practical NR BSs could be up to 300-400 beams considering the finer steering granularity in the vertical domain and the relative coarse granularity in the horizontal domain since its steering range is supposed to -60 degree to +60degree. If all these beams to tested within the chamber, then to be combined according to its weighting factors, then testing time would be quite long compared with Rel-15 TRP or EIRP measurement. In addition, if considering repeating the measurement among different mechanical down-tilt, then the testing time for EEIRP mask would be further longer which is not quite efficient for the BS conformance testing.

In the contribution [6], some initial EEIRP mask evaluations has been conducted to identify the appropriate number for conformance testing. In the following section, the similar EEIRP evaluation results are provided based on the following antenna geometry assumption in Table 2.2.-1.

Table 2.2-1: Array antenna geometry

| Parameter | Value |
| --- | --- |
| Frequency | 6775 MHz |
| Element model | Vertical HPBW: 65° |
|  | Horizontal HPBW: 90° |
|  | Sidelobe suppression: 30dB |
|  | Element directivity: 6.4 dBi |
| Array configuration | Number of rows: 4 |
|  | Number of columns: 8 |
|  | Sub-array: 3x1 |
|  | Sub-array tilt: 6° |
|  | Horizontal element separation: 0.5λ |
|  | Vertical element separation: 0.7λ |
| conducted power per sub-array | 28dBm per subarray |
| Peak EIRP target | 55.7 dBm/100 MHz |

According to the initial simulation results provided in Table 2.2.-2, the different weighting factors under the 9 testing beams will have relatively bigger impacts since the measured EEIRP will differ roughly around 1.5dB between Set C and Set D/E, however the different weighting factor have quite limited impacts when the number of testing beams are up to 25.

Table 2.2-2: Initial simulation results for EEIRP mask

| Vertical angle range | Beams set to be tested [R4-2409118] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Beam weight factors for Beam set | A | B | C | D | E |
| 0° - 5° | 21.38 | 21.77 | 21.69 | 20.96 | 20.08 |
| 5° - 10° | 20.47 | 20.58 | 20.94 | 20.56 | 20.23 |
| 10° - 15° | 19.73 | 19.73 | 20.13 | 20.04 | 19.74 |
| 15° - 20° | 19.15 | 18.93 | 19.73 | 19.18 | 18.58 |
| 20° - 30° | 15.05 | 14.84 | 15.84 | 15.56 | 15.37 |
| 30° - 60° | 8.62 | 7.94 | 10.28 | 9.32 | 8.41 |
| 60° - 90° | 6.47 | 5.53 | 11.10 | 9.19 | 6.58 |
| Number of measurement points | 25 | 25 | 9 | 9 | 9 |
| NOTE: the measurement step size in vertical domain is 1 degree and measurement step size in the horizontal domain is 1 degree. |  |  |  |  |  |

Observation 1: the limited number of testing beams could provide the stable measurement results compared with the finest beams set for measurements and the testing time could be reduced dramatically if the limited number of testing beams are adopted.

Observation 2: the different weighting factors under the 9 testing beams will have relatively bigger impacts since the measured EEIRP will differ roughly around 1.5dB between Set C and Set D/E, however the different weighting factor have quite limited impacts when the number of testing beams are up to 25.

Similar as NR or MSR test model design, if RAN4 could specify the testing beams with its corresponding weighting factors, this will be quite useful to have aligned EEIRP conformance testing among different vendors.  And these selected testing beams will not only provide the test coverage but also ensure the reasonable measurement uncertainty.

Proposal 2: propose to specify the testing beams to align the conformance testing among vendors and FFS for weighting factors for each testing beams since these weighting factors are more or less dependent on the traffics distribution within the serving sector.

  - Sampling grid for EEIRP mask measurement

In the previous RAN4 meeting, there were some initial discussions on sampling grid for EEIRP measurement to reduce the testing time accordingly similar as TRP measurement grid as defined in TS 38.141-2 Annex. In this section, some initial EEIRP measurement results for single beam with mechanical down-tilt as 6 degree are provided based on the antenna geometry assumption as defined in 2.2-1 similar as the contribution [7].

Table 2.3-1: Initial simulation results for EEIRP mask [updated]

| Vertical angle range | Full resolution | Vertical / Horizontal Angular Step Size |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Theta/phi | 0.1° /0. 1° | 5° / 5° | 2.5° / 5° | 2.5° / 2.5° | 1° / 15° | 1° / 10° | 1° / 5° | 1° / 1° |
| 0° - 5° | 24.09 | 27.40 | 26.05 | 26.05 | 25.24 | 24.89 | 24.89 | 24.89 |
| 5° - 10° | 21.38 | 27.83 | 24.94 | 24.94 | 23.21 | 22.93 | 22.93 | 22.93 |
| 10° - 15° | 22.30 | 23.27 | 23.19 | 23.19 | 22.99 | 22.72 | 22.72 | 22.72 |
| 15° - 20° | 21.05 | 21.58 | 21.86 | 21.86 | 21.66 | 21.43 | 21.43 | 21.43 |
| 20° - 30° | 16.00 | 18.35 | 17.05 | 17.05 | 16.48 | 16.39 | 16.39 | 16.39 |
| 30° - 60° | 10.11 | 10.55 | 10.31 | 10.31 | 10.20 | 10.18 | 10.18 | 10.18 |
| 60° - 90° | 8.75 | 10.27 | 9.52 | 9.52 | 9.07 | 9.06 | 9.05 | 9.04 |
| Number of measurement points |  | 1296 | 2592 | 5184 | 2160 | 3240 | 6480 | 32400 |

According to the initial measurement results provided in Table 2.3.1, when the EEIRP measurement results are almost the same when the spatial sampling grid of theta and phi is between 1° / 1° and 1° / 10°. When the sampling grid is up to 1° / 15°, then there will be some additional variations compared with lower sampling grid. In addition, when the sampling grid in the vertical domain is increased to 5° or 2.5° , then the measurement uncertainty will be further increased.

Observation 3: the EEIRP measurement results are almost the same when spatial sampling grid of theta and phi between 1° / 1° and 1° / 10°.

  - Measurement uncertainty

Regarding the confidence level of 95% is widely used in the RAN4 conformance testing specification as following,

the Test System shall enable the stimulus signals in the test case to be adjusted to within the specified tolerance and the equipment under test to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated. A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

From RAN4 RF requirement definition perspective, we need to define the measurement uncertainty and also test tolerance requirement on top of core requirement. From our understanding, we could still follow the legacy approach adopted for EIRP accuracy requirements as baseline, however considering the EEIRP mask requirements, some additional summation error due to the limited number of testing beams and spatial sampling grid for EEIRP mask measurement.

For the test tolerance requirement for U6GHz EEIRP mask, since this is the regulatory requirement.  If we follow the legacy requirement defined for the transmitter spurious emission requirement, then test tolerance should be set as zero.

Proposal 3: follow the legacy approach adopted for EIRP accuracy requirements as baseline, however considering the EEIRP mask requirements, some additional summation error due to the limited number of testing beams and spatial sampling grid for EEIRP mask measurement.

The detailed examples for CATR measurement could be found as following with exact value to be further discussed.

Table 9.2.3.3-1: CATR measurement uncertainty contributors for EIRP accuracy measurements, Normal test conditions, FR1

| UID / Details in annex | Uncertainty source |
| --- | --- |
| Stage 2: BS measurement |  |
| A2-1a | Misalignment and pointing error of BS (for EIRP) |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |
| A2-2a | Standing wave between BS and test range antenna |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |
| A2-4a | QZ ripple experienced by BS |
| A2-12 | Frequency flatness of test system |
| A2-13 | EEIRP summation error |
| Stage 1: Calibration measurement |  |
| C1-3 | Uncertainty of the network analyzer |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver |
| A2-6 | Insertion loss of receiver chain |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |
| A2-7 | Influence of the calibration antenna feed cable |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |
| A2-8 | Misalignment positioning system |
| A2-1b | Misalignment and pointing error of calibration antenna |
| A2-9 | Rotary joints |
| A2-2b | Standing wave between calibration antenna and test range antenna |
| A2-4b | QZ ripple experienced by calibration antenna |
| A2-11 | Switching uncertainty |

NOTE:	In the legacy technical reports for BS testability (RAN4) or UE testability (RAN5), the MU/TT derivation tables were using UID as counting numbers across multiple test chambers and requirement's clauses. In this TR a simplified approach was taken with the UID's being the annex number of the measurement uncertainty source description.

Table 9.2.3.3-2: CATR MU value derivation for EIRP accuracy measurements, Normal test conditions, FR1

| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the | Divisor based | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | f ≤ 3 GHz | 3 < f ≤ 4.2 GHz | 4.2 < f ≤ 6 GHz | probability | on distribution shape |  | f ≤ 3 GHz | 3 < f ≤ 4.2 GHz | 4.2 < f ≤ 6 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| A2-1a | Misalignment and pointing error of BS (for EIRP) | 0.00 | 0.00 | 0.00 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 | 0.00 |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | 0.26 | Gaussian | 1.00 | 1 | 0.14 | 0.26 | 0.26 |
| A2-2a | Standing wave between BS and test range antenna | 0.21 | 0.21 | 0.21 | U-shaped | 1.41 | 1 | 0.15 | 0.15 | 0.15 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | 0.00 | 0.00 | Gaussian | 1.00 | 1 | 0.00 | 0.00 | 0.00 |
| A2-4a | QZ ripple experienced by BS | 0.09 | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.09 | 0.09 | 0.09 |
| A2-12 | Frequency flatness of test system | 0.25 | 0.25 | 0.25 | Gaussian | 1.00 | 1 | 0.25 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer | 0.13 | 0.20 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 | 0.20 |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver | 0.13 | 0.33 | 0.33 | U-shaped | 1.41 | 1 | 0.09 | 0.23 | 0.23 |
| A2-6 | Insertion loss of receiver chain | 0.18 | 0.18 | 0.18 | Rectangular | 1.73 | 1 | 0.10 | 0.10 | 0.10 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | 0.00 | 0.00 | Gaussian | 1.00 | 1 | 0.00 | 0.00 | 0.00 |
| A2-7 | Influence of the calibration antenna feed cable | 0.02 | 0.02 | 0.02 | U-shaped | 1.41 | 1 | 0.02 | 0.02 | 0.02 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.50 | 0.43 | 0.43 | Rectangular | 1.73 | 1 | 0.29 | 0.25 | 0.25 |
| A2-8 | Misalignment positioning system | 0.00 | 0.00 | 0.00 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 | 0.00 |
| A2-1b | Misalignment and pointing error of calibration antenna | 0.50 | 0.50 | 0.50 | Exp. normal | 2.00 | 1 | 0.25 | 0.25 | 0.25 |
| A2-9 | Rotary joints | 0.05 | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.03 | 0.03 | 0.03 |
| A2-2b | Standing wave between calibration antenna and test range antenna | 0.09 | 0.09 | 0.09 | U-shaped | 1.41 | 1 | 0.06 | 0.06 | 0.06 |
| A2-4b | QZ ripple experienced by calibration antenna | 0.01 | 0.01 | 0.01 | Gaussian | 1.00 | 1 | 0.01 | 0.01 | 0.01 |
| A2-11 | Switching uncertainty | 0.26 | 0.26 | 0.26 | Rectangular | 1.73 | 1 | 0.15 | 0.15 | 0.15 |
| A2-13 | EEIRP summation error |  |  | TBD | TBD |  |  |  |  | TBD1 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  | 0.57 | 0.65 | TBD2 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  | 1.11 | 1.27 | 1.96*(TBD2) |

  - Transformation between coordinate system to 3GPP coordinate system, to align with other test cases

In the last RAN4 meeting, there were some discussions on converting the coordinate system of EEIRP mask to 3GPP coordinate system as defined in TS 38.141-2. As shown in the following figures, the converting between two coordinate might be very simple e.g. update the theta in EEIRP mask as -theta, however the updated coordinate will be not aligned with ITU recommendation which will make the regulatory requirement defined by WRC-23 not clear when it is interpreted outside of 3GPP. From our understanding, unless there are clear benefits identified to align the coordinate system between EEIRP mask and legacy 3GPP coordinate system, it’s not necessary to have such kind of converting.

Figure 4.14-1: Reference coordinate system

is the angle in the x/y plane, between the x-axis and the projection of the radiating vector onto the x/y plane and is defined between -180° and +180°, inclusive.  is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°, inclusive. Note that  is defined as positive along the down-tilt angle

Proposal 4: unless there are clear benefits identified to align the coordinate system between EEIRP mask and legacy 3GPP coordinate system, it’s not necessary to have such kind of converting.

Conclusions

In this contribution, we shared some further views on the EIRP mask requirement for band n104 and observations / proposals are made as following:

Proposal 1: to consider the manufacture declarations in Table 2.1-1 for different mechanical down-tilt and its the corresponding coverage angular range.

Observation 1: the limited number of testing beams could provide the stable measurement results compared with the finest beams set for measurements and the testing time could be reduced dramatically if the limited number of testing beams are adopted.

Observation 2: the different weighting factors under the 9 testing beams will have relatively bigger impacts since the measured EEIRP will differ roughly around 1.5dB between Set C and Set D/E, however the different weighting factor have quite limited impacts when the number of testing beams are up to 25.

Proposal 2: propose to specify the testing beams to align the conformance testing among vendors and FFS for weighting factors for each testing beams since these weighting factors are more or less dependent on the traffics distribution within the serving sector.

Observation 3: the EEIRP measurement results are almost the same when spatial sampling grid of theta and phi between 1° / 1° and 1° / 10°

Proposal 3: follow the legacy approach adopted for EIRP accuracy requirements as baseline, however considering the EEIRP mask requirements, some additional summation error due to the limited number of testing beams and spatial sampling grid for EEIRP mask measurement.

Proposal 4: unless there are clear benefits identified to align the coordinate system between EEIRP mask and legacy 3GPP coordinate system, it’s not necessary to have such kind of converting.

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- WRC-23 Provisional Final Acts, https://www.itu.int/dms_pub/itu-r/opb/act/R-ACT-WRC.15-2023-PDF-E.pdf

- R4-2406088,Way Forward for [110bis][311] NR_BS_RF, ZTE, approved.

- R4-2404757,On introduction of protection requirement for FSS UL service operating within 6425 to 7125 MHz

- R4-2409955, Way forward for [111][311] NR_BS_RF, Approved

- R4-2409118,On introduction of OTA spatial emission above the horizon requirement for band n104, Ericsson

- R4-2409073,Discussion on expected EIRP mask above horizon, Nokia

- R4-2409401,Expected EIRP requirements, Huawei, Hisilicon

- TS 37.941