3GPP TSG RAN4 Meeting #112																	R4-2413002

Maastricht, NL, 19 Aug. to 23 Aug. 2024

Agenda Item:	5.14.1

Source: 	Ericsson

Title:	Discussion on remaining open issues for FR2 SCell activation enhancements

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the remining open issues.

# Other enhancements

After the last meeting there are few proposals left for discussion concerning FR1 SCell activation. WF is captured below. In this contribution we provide our views on the proposals.

Issue 1-4: Other issues related with FR1 SCell activation enhancement

- FFS following proposals:

  - Proposal 1:

    - RAN4 to discuss if measurement period shall be considered as a condition to differentiate the requirements for FR1 SCell activation enhancement with L3 report.

    - Like the legacy FR1 known SCell activation, SCell activation delay requirement with L3 report shall be differentiated according to measurement period below or above 2400ms, and decide if AGC refinement or T/F tracking is needed.

  - Proposal 2:

    - RAN4 also consider applying enhancements of L3 reporting during SCell activation in FR1 to the case when only one SSB is transmitted in ssb-PositionInBurst.

    - RAN4 further extend the requirement applicability of 8.3.17 and 8.3.18 to the scenarios in FR1 where only one SSB is considered. If only one SSB is considered, Tuncertainty_MAC, Tuncertainty_SP, Tuncertainty_RRC and TRRC_delay are counted as zero, and the 3ms MACE CE decoding delay for TCI state activation is removed, i.e. the overall delay Tactivation_time is 7ms + TL3,report+ THARQ + TFineTiming + 2ms.

For proposal 1:

When the deactivated cycle measurement period is more than 160ms (i.e., measurement period is more than 2400ms), as per the legacy requirement, UE is allowed to get one additional sample for performing AGC in the SCell activation requirement. There was a proposal to follow the legacy case and allow one more sample for AGC for SCell activation when L3 measurement report is available.

However, we think the scenario is bit different from legacy scenario. The reason why UE may have measurement report available is UE may have completed the measurement quite recently and UE is waiting for the grant to send the report. One such scenario is shown in figure 1. The scenario shown in figure 1 do not need additional AGC sample. Due this reason, during the discussion we did not consider AGC for the L3 report based SCell activation.

Figure 1: Timeline for the example measurement and SCell activation

- Do not consider additional AGC sample for the measurement period more than 2400ms for L3 measurement based SCell activation.

For proposal 2:

When we have only one SSB, we think UE do not need to send a L3 report. We think NW can already indicate the TCI state along with SCell activation command as the TCI state is already known at the NW. Due to this, we do not see a need for l3 report based SCell activation enhancement for the scenario where only one SSB is present in the cell.

- No need to apply L3 based SCell activation when the single SSB is present.

# Summary and Conclusion

In this contribution we have provided our views on the FR2 SCell activation delay reduction and made following proposals:

- Do not consider additional AGC sample for the measurement period more than 2400ms for L3 measurement based SCell activation.

- No need to apply L3 based SCell activation when the single SSB is present.

# References

- RP-220977 “WID on Even Further RRM enhancement for NR and MR-DC”, Apple Oppo