3GPP TSG-RAN WG4 Meeting # 112	R4-2413368

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	8.2.5

Source: 	Nokia

Title: 	Impact of beamforming schemes on the coexistence of IMT with other services

1.	Introduction

ITU-R WP 5D is seeking information on whether detailed implementation of IMT Base Station (BS) multiuser spatial beamforming techniques, such as zero-forcing (ZF) or minimum mean-square error (MMSE) based schemes, would be necessary to improve the assessment of interference in the coexistence studies with other services performed in ITU. This study presents simulation results comparing the levels of interference measured at different spatial points (reflecting possible locations of victim terminals) with respect to the (aggressor) BSs due to different BS digital beamforming techniques. In this study, interference CDFs are measured at different spatial points for (1) MU-MIMO using zero-forcing (ZF) and (2) the MU-MIMO used in ITU coexistence studies (ITU MU-MIMO) as defined by the ITU-R M.2101 model [1] and the extended AAS model supporting sub-arrays [2].

2.	Discussion

2.1	Simulation Methodology

A detailed MIMO system level simulator was used to model the BS radiation for ZF and the ITU MU-MIMO beamforming where modeling included standard MU-MIMO scheduling using NR numerology. ZF MU-MIMO is applied in both azimuth and elevation. For the assessment of impact of these two MU-MIMO schemes, parameters of a 2.6 GHz IMT system were used. The study measures the downlink (DL) signal level (interference) seen at different spatial points for each beamforming technique. Table 1 describes the main system parameters assumed.

Table 1

Main IMT system parameters.

| Parameter | Base Station | UE |
| --- | --- | --- |
| Network topology, cell structure and characteristics |  |  |
| Centre frequency | 2.6 GHz |  |
| Channel bandwidth | 20 MHz |  |
| Network Configuration (Duplex Mode) | TDD |  |
| Cell radius | 0.8 km (suburban macro) | N.A. |
| Antenna height | 25 m | 1.5 m |
| Sectorization | 3 sectors | N.A. |
| User equipment density for terminals that are receiving simultaneously | N.A. | 3 UEs per sector |
| Frequency reuse | 1 | N.A. |
| Indoor user terminal usage | N.A. | 70% |
| Antenna characteristics |  |  |
| Antenna pattern model | Extended AAS model [2]. | Isotropic, -3 dBi gain |
| Element gain (Note 1) | 6.4 dBi | N.A. |
| Horizontal/vertical 3 dB beamwidth of single element | 90º for H 65º for V | N.A. |
| Horizontal/vertical front-to-back ratio | 30 dB for both H/V | N.A. |
| Antenna polarization | Linear/±45 degrees | N.A. |
| Antenna array configuration (Row × Column) | 4 x 8 elements | N.A. |
| Horizontal/Vertical radiating element/sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V | N.A. |
| Number of element rows in sub-array | 3 | N.A. |
| Vertical radiating element spacing in sub-array | 0.7 of wavelength of V | N.A. |
| Pre-set sub-array down-tilt | 3º | N.A. |
| Array ohmic loss (Note 1) | 2 dB | N.A. |
| Conducted power (before Ohmic loss) per antenna element/sub-array | 28 dBm | N.A. |
| Base station horizontal coverage range | ±60º | N.A. |
| Base station vertical coverage range | 90º -100º | N.A. |
| Mechanical down tilt | 6º | N.A. |
| Maximum base station output power/sector (e.i.r.p.) | 72.28 dBm | N.A. |
| Note 1: The element gain includes the ohmic loss. Therefore, ohmic loss is not needed for the calculation of the BS composite antenna gain and e.i.r.p. |  |  |

In the simulation, up to 3 of 10 available UEs per cell are MU-MIMO scheduled per slot in DL by a serving cell (21 total cells simulated) over 50 subframes (500 slots) where only the most recent interference measured at each measurement point is saved for each Monte Carlo drop. A minimum separation distance of 10 degrees (in azimuth and elevation) between the BS beams steered towards the served UEs is considered for MU-MIMO scheduling. The measurement points are equidistant from the center on a hemisphere above the IMT cell grid, as illustrated by the blue points in Figure 1, with the red points indicating the locations of the tri-sector sites. Given 100 Monte Carlo drops, 648 measurement points per drop, then 64800 snapshots of interference radiation are taken (64800 = 100 x 648) for each case modeled.

Figure 1: Measurement points in blue: equidistant from the center on a hemisphere above the IMT cell grid; locations of tri-sector sites in red.

2.2	Simulation Results

Figure 2 shows interference CDFs for ZF MU-MIMO and ITU MU-MIMO for AWGN channels. The delta of CDFs at the 5%-ile, 50%-ile, and 80%-ile points from Figure 2 are shown in Table 2 below. Since the interference delta CDF values are less than ~0.3 dB, the considered beam forming techniques have very similar impact to victim services.

Table 2: Interference CDF values for ZF and ITU MU-MIMO for AWGN channel

| Beamforming Type | Interference CDF values (dB) |  |  |
| --- | --- | --- | --- |
|  | 5%-ile | 50%-ile | 80%-ile |
| ZF MU-MIMO | -102.13 | -89.24 | -80.81 |
| ITU MU-MIMO | -102.36 | -89.48 | -81.08 |
| Delta CDF values | ~ 0.23 | ~ 0.24 | ~ 0.27 |

Similar study was conducted considering UMa channels for ZF MU-MIMO. Figure 3 shows the interference CDFs for ZF MU-MIMO and ITU MU-MIMO for this case. The delta of CDFs at the 5%-ile, 50%-ile, and 80%-ile points from Figure 3 are shown in Table 3 below. Since the interference delta CDF values are less than ~0.25 dB, the considered beam forming techniques have negligible difference in the impact to victim services.

Table 3: Interference CDF values for ZF and ITU MU-MIMO for UMa channel

| Beamforming Type | Interference CDF values (dB) |  |  |
| --- | --- | --- | --- |
|  | 5%-ile | 50%-ile | 80%-ile |
| ZF MU-MIMO | -100.36 | -87.53 | -79.5 |
| ITU MU-MIMO | -100.43 | -87.76 | -79.71 |
| Delta CDF values | ~ 0.07 | ~ 0.23 | ~ 0.21 |

Figure 2: Interference CDFs for ZF MU-MIMO and ITU MU-MIMO for AWGN channel and for minimum separation of 10 degrees for determining pairing.

Figure 3: Interference CDFs for ZF MU-MIMO and ITU MU-MIMO for UMa channel and for minimum separation of 10 degrees for determining pairing.

3.	Conclusion

This contribution provides simulation results for assessing the impact of different MU-MIMO schemes to the study of coexistence of IMT with other services. Simulations assume a 2.6 GHz IMT system and show the interference levels measured at different spatial points over AWGN channels and UMa channels for ZF MU-MIMO and ITU MU-MIMO beamforming techniques.

Since the interference delta CDF values are very similar (less than 0.3 dB), different beam forming techniques have similar impact to victim service, and there is no need for implementing more complex MU-MIMO models for coexistence studies with other services in ITU.

# References

[1]	Rec. ITU-R M.2101, “Modelling and simulation of IMT networks and systems for use in sharing and compatibility studies”, ITU-R.

[2]	R4-2410576, “LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, Ericsson.