3GPP TSG-RAN WG4 Meeting #112             	R4-2413438

Maastricht, Netherlands, 19th-23th, Aug, 2024

Agenda item:	5.23.8.1

Source:	Samsung

Title:	Discussion and simulation results for NTN enhancement

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the performance part of demodulation requirements for NTN enhancement was closed. The related agreement was captured in the WF as following [1]

- SAN demodulation requirement - General - The number of demodulation branches - Keep the previous agreement, i.e. both 1Rx and 2Rx, with the test applicability rule for support of different number of demodulation branches based on declaration. - Test setup for normal PUSCH with CP-OFDM for above 10 GHz bands - MCS - MCS 2 and MCS 12 - PTRS configuration - Not configure PTRS for test with MCS 2 and MCS 12 - Test setup for PUSCH repetition Type A for above 10 GHz bands - Test metric - BLER %1, where BLER is defined as residual BLER; i.e., ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. - Test setup for PUCCH for above 10GhZ - PUCCH format 0 - Keep case with first symbol is 12 with 2 symbols - PUCCH format 3 - Keep case#1 with first symbol is 0 with 14 symbols - Test setup for PRACH for above 10GHz - Test metric - The false alarm probability shall be less than or equal to 0.1% - The probability of detection shall be equal to or exceed 99%

In this contribution, the view on how to derive SNR is provided. Meanwhile, the ideal and impairment results are provided for requirement derivation.

# 2	SNR derivation

In the last meeting, the test setup for SAN demodulation requirement was finalized. For NTN enhancement WI, RAN4 has introduced requirement for PUSCH, PUCCH and PRACH requirement for above 10GHz in Ka-band, and also PUSCH with DMRS bundling requirement for coverage enhancement in NTN-FR1 S-band. Based on the current simulation results summary, the results for some of cases among companies are not aligned well, especially for PUSCH with DMRS bundling, also, there is no enough input for requirement derivation currently for some cases. Therefore, for those cases, the TBD was kept for requirement in current specification, and companies can still update the simulation results in the maintenance stage based on latest simulation assumption.

In Rel-15, how to derive SNR requirement, some rule was discussed and made the related agreement in R4-1904713 as following

Procedure to derive the performance requirements: - Only inputs that consist of a pair of ideal and impaired results can be taken into account. - If the ideal span <= [2] dB: - The AVERAGE impairment results can be used for the performance requirement with [] in the draftCRs/CRs; - Else if the ideal span is larger than [2] dB: - The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available: - The ultimate AVERAGE impairment results with corresponding ideal span <=2dB can be used for performance requirement with [] in the draftCRs/CRs. - Otherwise put TBD for the related performance requirements. - If the span of the impairment results after removal the outliers (if any) are larger than 4dB, then the procedure cannot be applied, related performance requirement remain TBD.

Meanwhile, for NTN WI in Rel-17, some rule also was considered for SNR values derivation in R4-2220161 as following

Procedure to derive the performance requirements: - Capture the following the derived SNR values with square brackets in the CRs to be endorsed for this meeting which are derived by applying the following steps in order: - Step 1: The existing SNR derivation procedure defined in [R4-1904713] [R4-19004714] is used, and the following SNR results are derived - Step 2: If the results cannot be derived by using the existing SNR derivation procedure [R4-1904713] [R4-19004714] after Step 1, i.e. the test cases with error code -103 as shown above, then just change the ideal span from 2dB to 2.5dB in the existing SNR derivation procedure, then the following SNR results with green are derived for those cases with error code -103: - Step 3: If SNR values still can’t be derived, remove the furthest results and average the remaining results, i.e. two companies results are allowed, but still with 2dB span threshold, then the following SNR results with green are derived for those cases with error code -103 after Step 2: - Companies can provide updated and more results for next meeting to further align the results. If results are well aligned, the existing SNR derivation procedure defined in [R4-1904713] [R4-19004714] should be used, otherwise further discussion on how to treat those cases with not well aligned results is not precluded.

Based on rule of Rel-15 of R4-1904713 or R4-19004714, it seems that only two companies’ results are available assuming to remove the furthest results in case the span with 2dB based on current simulation results summary, which is not difficult to verify the feasibility of requirements in the real network. Therefore, RAN4 should discuss the rule for how to derive the requirement for those cases with ideal simulation results span large than 2dB, in case there is no enough input with well aligned. The rule agreed in the R4-2220161 can be considered as a starting point.

Meanwhile, for some cases, even with relaxing the span from 2dB to [2.5dB], e.g., PUSCH requirement with DMRS bundling, only two companies’ results are available, additional marginal could be considered, the value of additional margin can be discussed in case-by-case manner.

Proposal 1:  Discussion the rule for how to derive the requirement for those cases with ideal simulation results span large than 2dB, in case there is no enough input with well aligned. The rule agreed in the R4-2220161 can be considered as a starting point, FFS on additional margin could be added.

PUSCH with DMRS bundling

In the previous meeting, the setup for PUSCH requirement with DMRS bundling was under discussion including the channel model and configured window size for joint channel estimation. For channel model, the existing channel used in NTN-FR1 are applied as NTN-TDLA100-200 compared the TDLA30-10, considering the channel coherent time with high doppler value, small value of window size for joint channel estimation are agreed for guaranteeing the phase continuous. However, there is no limitation of BS implementation to choose how many DMRS symbol within these window size. It is possible to only combine the DMRS within one slot regardless of the number of slot aggregation, or to combine all the DMRS within the configured window, pending on different implementation. Therefore, it is possible that the simulation results will be different given the assumption of large doppler value.  To reduce the performance difference, one possible solution is to update the simulation assumption to choose the small window size given the high doppler value. Consider the simulation assumption was agreed and companies has already provided the simulation result based on latest agreed simulation assumption, then, the requirement can only be derived based on current available simulation results, if it is not possible and agreed to refine the test parameters. Based on current available simulation results, the span is large than 3dB, to allow the different implementation, additional margin could be considered in case the results cannot be aligned well.

Proposal 2:  Additional margin could be considered for PUSCH requirement with DMRS bundling, in case the simulation results cannot be aligned well, where the span is large than 2dB. FFS to refine the test parameters

# 3	Simulation Results

In this section, the ideal and impairment simulation results based agreed test parameters for SAN demodulation requirement were provided.

## 3.1	PUSCH requirement

Table 1: ideal and impairment simulation results for PUSCH requirement with CP-OFDM

| Test case | waveform | SCS& BW | MCS | Channel | Mapping Type | Length Of symbol | Number of Tx and Rx | PTRS | Number of DMRS | SNR@ 70 (ideal) | SNR@70 (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CP-OFDM | 120KH 50MHz | 2 | NTN TDLC5-1200 | B | 10 | 1T2R | Disable | 1+1 (0,8) | -5.7 | -3.7 |
| 2 | CP-OFDM | 120KH 50MHz | 12 | NTN TDLC5-1200 | B | 10 | 1T2R | Disable | 1+1 (0,8) | 3.2 | 5.2 |
| 3 | CP-OFDM | 120KH 50MHz | 2 | NTN TDLC5-1200 | B | 10 | 1T1R | Disable | 1+1 (0,8) | -2.3 | -0.3 |
| 4 | CP-OFDM | 120KH 50MHz | 12 | NTN TDLC5-1200 | B | 10 | 1T1R | Disable | 1+1 (0,8) | 6.6 | 8.6 |

Table 2: ideal and impairment simulation results for PUSCH requirement with DFT-s-OFDM

| Test case | waveform | SCS& BW | RB | MCS | Channel | Mapping Type | Length Of symbol | Number of Tx and Rx | PTRS | Number of DMRS | SNR@ 70 (ideal) | SNR@70 (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM | 120KH 50MHz | 30 PRBs in the middle of test bandwidth | 2 | NTN TDLC5-3000 | B | 10 | 1T2R | Disable | 1+1 (0,8) | -5.5 | -3.5 |
| 2 | DFT-s-OFDM | 120KH 50MHz | 30 PRBs in the middle of test bandwidth | 2 | NTN TDLC5-3000 | B | 10 | 1T1R | Disable | 1+1 (0,8) | -2.3 | -0.3 |

Table 3: ideal and impairment simulation results for PUSCH requirement with repetition Type A in above 10GHz

| Test case | SCS& BW | RB | MCS (table3) | Channel | PUSCH Aggregation level | Mapping Type | Length Of symbol | Number of Tx and Rx | PTRS | Number of DMRS | SNR@BLER 1% (ideal) | SNR@BLER 1% (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 120KH 50MHz | 32 | 5 | NTN TDLC5-1200 | 2 | B | 10 | 1T2R | Disable | 1+1 (0,8) | -14.7 | -12.7 |
| 2 | 120KH 50MHz | 32 | 5 | NTN TDLC5-1200 | 2 | B | 10 | 1T1R | Disable | 1+1 (0,8) | -11.5 | -9.5 |

Table 4: ideal and impairment simulation results for PUSCH requirement with DMRS bundling

| Test case | SCS& BW | MCS (table1) | Channel | slot | Pusch-TimeDomainWindowLength | Mapping Type | Length Of symbol | Number of Tx and Rx | Number of DMRS | SNR@ 70 (ideal) | SNR@70 (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz 5MHz | 4 | NTN TDLA100-200 | 4 | 4 | A | 14 | 1T2R | 1+1 (2,11) | -6.6 | -4.6 |
| 2 | 15KHz 5MHz | 4 | NTN TDLA100-200 | 4 | 4 | B | 14 | 1T2R | 1+1 (0,8) | -6.2 | -4.2 |
| 3 | 30KHz 10MHz | 4 | NTN TDLA100-200 | 8 | 8 | A | 14 | 1T2R | 1+1 (2,11) | -9.2 | -7.2 |
| 4 | 30KHz 10MHz | 4 | NTN TDLA100-200 | 8 | 8 | B | 14 | 1T2R | 1+1 (0,8) | -9.0 | -7.0 |
| 5 | 15KHz 5MHz | 4 | NTN TDLA100-200 | 4 | 4 | A | 14 | 1T1R | 1+1 (2,11) | -2.6 | -0.6 |
| 6 | 15KHz 5MHz | 4 | NTN TDLA100-200 | 4 | 4 | B | 14 | 1T1R | 1+1 (0,8) | -2.1 | -0.1 |
| 7 | 30KHz 10MHz | 4 | NTN TDLA100-200 | 8 | 8 | A | 14 | 1T1R | 1+1 (2,11) | -5.6 | -3.6 |
| 8 | 30KHz 10MHz | 4 | NTN TDLA100-200 | 8 | 8 | B | 14 | 1T1R | 1+1 (0,8) | -5.3 | -3.3 |

## 3.2	PUCCH requirement

Table 5: ideal and impairment results for Format 0

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | Number of Symbol | ACK missed%1 (ideal) | ACK missed%1 (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 0 | 120KH 50MHz | Disabled | 1 | NTN-TDLC5-1200 | 9 | 1T2R | 1 | 1.85 | 3.85 |
| 2 | Format 0 | 120KH 50MHz | Enabled | 1 | NTN-TDLC5-1200 | 9 | 1T2R | 2 | -0.40 | 1.6 |
| 3 | Format 0 | 120KH 50MHz | Disabled | 1 | NTN-TDLC5-1200 | 9 | 1T1R | 1 | 6.81 | 8.81 |
| 4 | Format 0 | 120KH 50MHz | Enabled | 1 | NTN-TDLC5-1200 | 9 | 1T1R | 2 | 4.39 | 6.39 |

Table 6: ideal and impairment simulation results for Format 1

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | Number of Symbol | ACK missed%1 (ideal) | NACK to ACK %0.1 (ideal) | ACK missed%1 (impair) | NACK to ACK %0.1 (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 1 | 120KH 50MHz | Enabled | 2 | NTN-TDLC5-1200 | 1 | 1T2R | 14 | -8.84 | -7.26 | -6.84 | -5.26 |
| 2 | Format 1 | 120KH 50MHz | Enabled | 2 | NTN-TDLC5-1200 | 1 | 1T1R | 14 | -4.34 | -2.59 | -2.3 | -0.6 |

Table 7: ideal and impairment simulation results for Format 2

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | Number of Symbol | ACK missed%1 (ideal) | ACK missed%1 (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 2 | 120KH 50MHz | Disabled | 4 | NTN-TDLC5-1200 | 4 | 1T2R | 1 | -1.31 | 0.69 |
| 2 | Format 2 | 120KH 50MHz | Disabled | 4 | NTN-TDLC5-1200 | 4 | 1T1R | 1 | 3.63 | 5.63 |

Table 8: ideal and impairment simulation results for Format 2

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | Number of Symbol | BLER%1 (ideal) | BLER%1 (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 2 | 120KH 50MHz | Enabled | 22 | NTN-TDLC5-1200 | 9 | 1T2R | 2 | -3.35 | -1.35 |
| 2 | Format 2 | 120KH 50MHz | Enabled | 22 | NTN-TDLC5-1200 | 9 | 1T1R | 2 | 1.88 | 3.88 |

Table 9: ideal and impairment simulation results for Format 3

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | DMRS | Number of Symbol | 1%BLER (ideal) | 1%BLER (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 3 | 120KH 50MHz | Enabled | 16 | NTN-TDLC5-1200 | 1 | 1T2R | No additional DMRS | 14 | -5.01 | -3.01 |
| 2 | Format 3 | 120KH 50MHz | Enabled | 16 | NTN-TDLC5-1200 | 1 | 1T2R | With additional DMRS | 14 | -5.39 | -3.39 |
| 3 | Format 3 | 120KH 50MHz | Enabled | 16 | NTN-TDLC5-1200 | 1 | 1T1R | No additional DMRS | 14 | -0.93 | 1.07 |
| 4 | Format 3 | 120KH 50MHz | Enabled | 16 | NTN-TDLC5-1200 | 1 | 1T1R | With additional DMRS | 14 | -0.82 | 1.18 |

Table 10: ideal and impairment simulation results for Format 4

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | DMRS | Number of Symbol | 1%BLER (ideal) | 1% BLER (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 4 | 120KH 50MHz | Enabled | 22 | NTN-TDLC5-1200 | 1 | 1T2R | No additional DMRS | 14 | -2.88 | -0.88 |
| 2 | Format 4 | 120KH 50MHz | Enabled | 22 | NTN-TDLC5-1200 | 1 | 1T2R | With additional DMRS | 14 | -3.04 | -1.04 |
| 3 | Format 4 | 120KH 50MHz | Enabled | 22 | NTN-TDLC5-1200 | 1 | 1T1R | No additional DMRS | 14 | 1.91 | 3.91 |
| 4 | Format 4 | 120KHz 50MHz | Enabled | 22 | NTN-TDLC5-1200 | 1 | 1T1R | With additional DMRS | 14 | 2.17 | 4.17 |

## 3.3	PRACH requirement

Table 11: ideal and impairment simulation results for PRACH

| Test case | Format | SCS& BW | PRACH sequence length | Ncs | Logical sequence index | v | Number of Tx and Rx | Channel | Frequency offset | Time estimation tolerance | SNR@1 % Miss (ideal) | SNR@1 % Miss (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C2 | 120KH | 139 | 69 | 0 | 0 | 1T2R | NTN-TDLC5-1200 | 3000 | 0.13us | -11.04 | -9.04 |
| 2 | B4 | 120KH | 139 | 69 | 0- | 0 | 1T2R | NTN-TDLC5-1200 | 3000 | 0.13us | -15.38 | -13.38 |
| 3 | C2 | 120KH | 139 | 69 | 0 | 0 | 1T1R | NTN-TDLC5-1200 | 3000 | 0.13us | -5.53 | -3.53 |
| 4 | B4 | 120KHz | 139 | 69 | 0 | 0 | 1T1R | NTN-TDLC5-1200 | 3000 | 0.13us | -9.94 | -7.94 |

# 4	Conclusion

In this contribution, the view on SNR derivation of SAN demodulation requirements for NTN enhancement was provided. Meanwhile, the ideal and impairment simulation results are provided for requirement derivation.

the view on remaining issue of BS demodulation requirement for NTN enhancement was provided. Meanwhile, the ideal and impairment simulation results are provided for requirement derivation

Proposal 1:  Discussion the rule for how to derive the requirement for those cases with ideal simulation results span large than 2dB, in case there is no enough input with well aligned. The rule agreed in the R4-2220161 can be considered as a starting point, FFS on additional margin could be added.

Proposal 2:  Additional margin could be considered for PUSCH requirement with DMRS bundling, in case the simulation results cannot be aligned well, where the span is large than 2dB. FFS to refine the test parameters

# References

R4-2409876, WF on NR_NTN_enh_SAN_UE_demod