3GPP TSG-RAN WG4 Meeting #112	R4-2413452

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.22.4.2

Source: 	Qualcomm Incorporated

Title: 		RRM core requirements for LP-WUS/WUR

Document for:	Discussion

# Introduction

For release 19, a new work item on low-power wakeup signal (WUS) and wakeup receiver (WUR) for NR has been agreed with the following objectives:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

During RAN4#110bis and RAN4#111 meetings, few initial agreements [2][3] were reached on the scope of the RRM requirements. In this paper, we further discuss some aspects to be considered for RRM core requirements.

# RRM measurements relaxation/offloading

During the last RAN4 meeting, we discussed various measurement relaxation/offloading cases and agreed to the following:

| RRM measurement case index​ | MR serving cell measurement​ | MR neighboring cell measurement​ | WUR measurement​ |
| --- | --- | --- | --- |
| #1 Fully offloading case​ | Off ​ | Off: FFS the condition and the details​ | ON​ |
| #2 Relaxed case a​ | On with relaxation measurement​ | Off​ | ON​ |
| #3 Relaxed case b​ | On with relaxation measurement​ | On with relaxation measurement​ | ON​ |
| #4 Relaxed case c​ | Off ​ | On, FFS the condition and the details​ | ON​ |

- If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may not perform intra-frequency measurements​

- If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.​

The scenario of full measurement offloading case was agreed where the MR doesn’t perform serving cell and neighbor cell measurements (FFS: high priority carriers), while the WUR performs the serving cell measurements. In this case, since the UE doesn’t perform any neighbor cell measurements, this scenario should be applicable only when the UE is at the cell center and has low mobility. Currently, the UE is not requirement to perform the neighbor cell measurements when Srxlev and Squal values meet the configured thresholds SIntraSearchP, SIntraSearchQ and/or SnonIntraSearchP​ and SnonIntraSearchQ​

Observation 1: RAN4 currently specifies the conditions for when the UE is not required to perform neighbor cell measurements.

Proposal 1: RAN4 to specify full measurement offloading (i.e., serving cell measurements by the WUR and no neighbor cell measurements) when the following criteria are met:

- Srxlev > SIntraSearchP and Squal > SIntraSearchQ

- Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ

Scenarios #2 and #3 consider serving cell measurement at a relaxed rate. This would mean that some measurement samples are collected by the MR and others by the WUR. These samples will then be filtered to derive the measurement metric. Note that the MR performs measurements on the SSBs while the WUR performs measurements on the WUR. This poses implementation challenges as it’s not straightforward to combine the measurement samples across two radios. So, such scenarios should be precluded.

Observation 2: Combining measurements across the two radios, i.e., MR and WUR increases the implementation complexity of the UE.

Proposal 2: RAN4 to not consider combining serving cell measurements across the MR and WUR.

Proposal 3: Preclude scenario #2 and #3 for specifying RRM requirements.

Scenario #4 considers the serving cell measurements performed by the WUR while the neighbor cell measurements by the MR. We think this scenario nicely complements scenario #1 and can be applicable to the UEs at the cell edge where the UE needs to perform neighbor cell measurements to prepare for possible cell reselection. Whether the neighbor cell measurements are relaxed or not can be further discussed, but we think the existing mechanism for neighbor cell measurement relaxations can be applied in this case.

Observation 3: Scenario #4, i.e., serving cell measurements by the WUR and relaxed neighbor cell measurements by the MR compliments scenario #1 and may be useful for cell-edge scenarios.

Proposal 4: RAN4 to consider scenario #4, i.e., serving cell measurements by the WUR and relaxed neighbor cell measurements by the MR for RRM requirements specification.

Proposal 5: RAN4 to use, at-least, the existing neighbor cell measurement relaxation mechanisms for scenario#4.

That being said, UE cannot achieve much power savings with non-relaxed neighbor cell measurement. So, the criteria for WUR operation should be defined in a manner that neighbor cell measurements can be relaxed.

Overall, it looks like below:

Furthermore, from RAN4 perspective, RRM measurements offloading on the serving cell is considered only when the WUR and MR operates on the same carrier frequency. This will avoid complicated RF retuning of the WUR for measurements, thereby making it a low-cost, low power radio.

Proposal 6: RAN4 to specify RRM core requirements for serving cell measurement offloading to the WUR only for the case when WUR and MR are operating on the same carrier frequency.

Irrespective of which scenarios are agreed, RAN4 needs to discuss the following:

- The number of measurement samples needed for filtering.

- The number of consecutive times the UE needs to meet the entry/exit criteria.

Current SSB based measurement requirements for the MR specify filtering the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements and Nserv number of consecutive serving cell evaluations are used to prepare for possible cell re-selection. RAN4 can define similar procedure for entry/exit criteria evaluation as well. For instance, UE starts monitoring PO on the WUR when the entry criteria is met X consecutive times and UE stops monitoring PO on the WUR when the exit criteria is met Y consecutive times. This ensures that a sudden rise/drop in the measurement quantities doesn’t trigger the WUR entry/exit.

Proposal 7: RAN4 specifies the following for entry/exit criteria evaluation for WUS paging monitoring:

- The number of measurement samples needed for filtering.

- The number of consecutive times the UE needs to meet the entry/exit criteria.

# Conclusion

Observation 1: RAN4 currently specifies the conditions for when the UE is not required to perform neighbor cell measurements.

Proposal 1: RAN4 to specify full measurement offloading (i.e., serving cell measurements by the WUR and no neighbor cell measurements) when the following criteria are met:

- Srxlev > SIntraSearchP and Squal > SIntraSearchQ

- Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ

Observation 2: Combining measurements across the two radios, i.e., MR and WUR increases the implementation complexity of the UE.

Proposal 2: RAN4 to not consider combining serving cell measurements across the MR and WUR.

Proposal 3: Preclude scenario #2 and #3 for specifying RRM requirements.

Observation 3: Scenario #4, i.e., serving cell measurements by the WUR and relaxed neighbor cell measurements by the MR compliments scenario #1 and may be useful for cell-edge scenarios.

Proposal 4: RAN4 to consider scenario #4, i.e., serving cell measurements by the WUR and relaxed neighbor cell measurements by the MR for RRM requirements specification.

Proposal 5: RAN4 to use, at-least, the existing neighbor cell measurement relaxation mechanisms for scenario#4.

Proposal 6: RAN4 to specify RRM core requirements for serving cell measurement offloading to the WUR only for the case when WUR and MR are operating on the same carrier frequency.

Proposal 7: RAN4 specifies the following for entry/exit criteria evaluation for WUS paging monitoring:

- The number of measurement samples needed for filtering.

- The number of consecutive times the UE needs to meet the entry/exit criteria.

# References

[1] 	RP-240801, “Revised WID Low-power wake-up signal and receiver for NR (LP-WUS_WUR)”, vivo, RAN#103

[2] R4-2406367, “WF for RRM requirements for LP-WUSWUR”, vivo, RAN4#110bis

[3] R4-2410296, “WF for RRM requirements for LP-WUSWUR”, vivo, RAN4#111