3GPP TSG-RAN WG4 Meeting # 112	R4-2411652

Maastricht, Netherland, August 19th – 23rd, 2024

Source:	Meta Ireland, [OPPO, LGE]

Title:	TP on TR38.787: Operating bands and UE RF requirements for intra-band contiguous SL CA UE

Agenda item:			8.3.3.1, 8.3.3.2 & 8.3.3.3

Document for:	Approval

# Introduction

This contribution is a text proposal to add the operating bands and UE RF requirements for intra-band contiguous SL CA UE in TR38.787.

======== Start of TP in TR38.787 ========

# Transmitter characteristics for NR Sidelink supporting intra-band CA in ITS band

## Tx requirements for SL intra-band contiguous CA

### Maximum output power

For the intra-band contiguous SL CA operation, the additional PC2 NR SL CA UE define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 6.1.1-1: NR intra-band contiguous SL CA UE Power Class

| NR SL CA band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SL_n47B |  |  | 26 | +2/-33 | 23 | +2/-33 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: For intra-band SL CA UE, the maximum power requirement apply to the total transmitted power over all component carriers (per UE). NOTE 3: 3 refers to the transmission bandwidths (Figure 5.6-1 in TS38.101-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: Power class 3 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

### UE maximum output power reduction

For the basic parameters, RAN4 reuse the simulation assumptions in TR38.786 (Rel-18 enhanced NR sidelink) for NR intra-band contiguous CA UE. Other constraints for PSCCH/PSSCH/PSFCH/S-SSB can be assumed based on current RAN1’s agreement. Aslo, the following parameters are considered to evalute MPR requirements.

Table 6.1.2-1: Basic RF parameters for PC2 NR intra-band contiguous SL CA UE

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) |
| Maximum output power for aggregated CBW | 26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| ACLR | 31dBc |
| Carrier leakage | 25dBc |
| IQ image | 34dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

### UE additional maximum output power reduction

In RAN4 #111 Meeting, RAN4 agreed not to introduce PC2 A-MPR requirements for European regulation since the EU regulation is only to apply to the 10MHz CBW.

For the A-MPR analysis to comply the US regulation, RAN4 agreed to study and specipy the related PC2 A-MPR requirements with NS_52 after final FCC regulation for the additional emission limits is released in US.

### 6.1.4	Configured transmitted power

<Editor Note> The detail requirements are FFS.

### 6.1.5 Other Tx requirements

The other UE transmitter requirements will be reusing the requirements of PC3 NR intra-band contiguous SL CA UE in TR38.786 and TS38.101-1.

- Minimum output / transmit OFF power /transmit ON/OFF time mask

- Power control

- Transmit signal quality

- SEM/

- General spurious emissions/UE-to-UE coexistence SE

- Transmit intermodulation

## Tx requirements for SL intra-band non-contiguous CA

### 6.2.1	Maximum output power

### 6.2.2	UE maximum output power reduction

### 6.2.3	UE additional maximum output power reduction

### 6.2.4	Configured transmitted power

======== End of TP in TR38.787 ========