3GPP TSG-RAN WG4 #66Bis	R4-131957

Chicago, USA, 15th – 19th April 2013

Source: 	Ericsson, ST-Ericsson

Title:	System level simulation assumptions for LTE UE CRS-IM SI

Agenda Item:	9.8.1

Document for:	Approval

# Introduction

This paper provides simulation assumptions for enhanced performance requirements for LTE UE CRS-IM studies.

# System level parameters

System level simulations should focus on homogeneous deployments in 3GPP case 1 scenario. For traffic model, non-full-buffer model (FTP-like) are assumed as baseline.

Simulation assumptions for interference modelling are provided in Table 3. The simulation assumptions are fully reusing the simulation assumption defined for advanced receiver discussion [1]. One minor modification is change the full buffer in advanced receiver discussion to non-full-buffer.

Table 3: Simulation assumptions for interference modelling [1]

| Parameter |  | 3GPP Case 1 |
| --- | --- | --- |
| Bandwidth |  | 10 MHz |
| Carrier frequency |  | 2000 MHz |
| Cellular Layout |  | Hexagonal grid, 19 cell sites, 3 sectors per site |
| Inter-site |  | 500 m |
| Distance-dependent path loss |  | L = 128.1 + 37.6log10(R), R: km |
| Shadowing standard deviation |  | 8 dB |
| Shadowing correlation | Between cells | 0.5 |
|  | Between sectors | 1.0 |
| Penetration loss |  | 20 dB |
| Antenna pattern | Horizontal | degrees, |
|  | Vertical | degrees, Antenna height at the base station is set to 32m. Antenna height at the UE is set to 1.5m. |
|  |  | degrees |
|  | Combining method in 3D antenna pattern |  |
|  | Antenna gain + connector loss | eNodeB: 14 dBi, UE: 0 dBi |
| Total BS TX power (Ptotal) |  | 46 dBm |
| Minimum distance between UE and Cell |  | >= 35 meters |
| Hard handover hysteresis |  | 3 dB |
| Traffic model |  | Non-full buffer FTP traffic Model 1 from TR36.814 [5] |
| Network synchronization |  | Synchronized |
| PCI planning |  | Planned cell ID layout with 3-CRS shift patterns (“Shifted CRS”) |
| UE distribution |  | Uniformly distributed, all users outdoors, speed 3 km/h |
| Noise figure |  | 9 dB in UE |
| DL Transmission schemes |  | Closed-loop spatial multiplexing, up to 2 layers, QPSK/16QAM/64QAM |
| File size, S |  | 2 Mbytes (0.5 Mbytes optional) (one user downloads a single file) |
| Target average resource utilization (RU) |  | [0, 10%, 20%,30%,40%,50%, 100% (Low priority)] |
| User arrival rate λ |  | Poisson distributed with arrival rate λ |
| Possible range of λ |  | Each company to adjust λ to match target resource utilization |
| Geometry |  | TBD |
| UE receiver |  | MMSE-IRC without CRS-IM |
|  |  | MMSE-IRC with CRS-IM for 1st strongest interfering cell |
|  |  | MMSE-IRC with CRS-IM for 1st and 2nd strongest interfering cells |

# Summary

Interested companies are requested to provide the simulation results for interference profile/ modeling for RAN4#67 meeting.

# Reference

- 3GPP TR 36.829, Enhanced performance requirement for LTE User Equipment (UE), 3GPP, 2012

- RP-130393, “New Study Item proposal: CRS Interference Mitigation for Homogenous Deployments,” Ericsson, RAN #59.

- 3GPP TR 25.963, Feasibility study on interference cancellation for UTRA FDD User Equipment

- R4-115874, System level results for advanced performance requirements for LTE UE, Renesas Mobile Europe Ltd, Nov. 2011

- 3GPP TR36.814, Further advancements for E-UTRA physical layer aspects