3GPP TSG-RAN WG4 Meeting #78 NB-IoT Ad-Hoc            				        R4-160539

# Malta, 15-19 Feb. 2016

Title:	Feasibility and measurement result of B41 HPUE prototype

Source:	CMCC

Agenda Item:	9.1

Document for:	Discussion

- Introduction

The demand of B41 HPUE was discussed and proposed [1] in the last meeting. In order to study the feasibility of B41 HPUE, we have asked UE vendors to design and provide B41 HPUE prototype. In this paper we show the measurement results for them.

- Measurement results

Two types of B41 HPUE prototype (smart phones) provided by two UE vendors were measured. Basic RF requirement measurement results are summarized in table 1 and figure 1 (some of the requirements are still under testing).

Table 1. Measurement results

| Parameters | Criterions | Prototype 1 | Prototype 2 |
| --- | --- | --- | --- |
| RAT and band supported | N/A | CDMA:BC0/BC1/BC10 LTE: B25/ B26/ B41 | GSM，UMTS， E-UTRA (18 bands) |
| Max Output Power | 26 ± 2dBm | 24.96dBm | 25.87dBm |
| SAR | 1.6mW/g (US limit) | 0.75mW/g | Testing |
| E-UTRA ACLR | 30dB (Power class 3) | 39.73dB | 38.14dB |
| UTRA ACLR1 | 33dB (Power class 3) | 38.35dB | 36.06dB |
| UTRA ACLR2 | 36dB (Power class 3) | 45.96dB | 43.99dB |

Figure 1. Spurious emission measurement result for prototype 1

It can be seen that multi-mode multi-band smart phone can meet the power class 2 MOP requirement while ACLR, SAR and spurious emission still have margins compared to power class 3 requirements.

Observation 1: it is possible to implement B41 power class 2 in multi-mode multi-band smart phones

- Reference

[1] R4-157460	Use case and requirement for HPUE, CMCC