3GPP TSG RAN WG4 Meeting #80	R4-165454

Gothenburg SE, 22 - 26 Aug, 2016

Agenda Item:	6.7.4.2.2

Source: 	Nokia

Title: 	Ideal Simulation Results for NPUSCH Format 2

Document for:	Discussion

# Introduction

NPUSCH simulation assumptions for simulation result alignments without considering frequency error were agreed in RAN4#79AH [1].  In this paper, we provide the ideal simulation results for NPUSCH Format 2 based on the agreed simulation assumptions.

# Simulation Assumptions

For the NPUSCH format 2, RAN4 agreed to define the test cases, and related main configurations listed in Table 2-1. The test metric is:

- The DTX to ACK probability, i.e. the probability that ACK is detected when nothing was sent, shall not exceed 1%

- The ACK missed detection probability shall not exceed 1% at the given SNR

- The NACK to ACK probability shall not exceed 0.1% at the given SNR.

# Table 2-1: Simulation assumption for NPUSCH format 2

| Test number | BW | Singe-/Multi-tone | Sub-carrier spacing | repetition level | Modulation | Propagation Condition | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 180KHz | Single | 3.75KHz | [1],[16],[64] | BPSK | EPA5 | 1x 2 | TBD |
| 2 | 180KHz | Single | 15KHz | [1],[16],[64] | BPSK | EPA5 | 1 x 2 | TBD |
| Note 1: Assume the frequency offset value is [0] and channel estimation lengths are 16ms for 3.75kHz and 4ms for 15kHz |  |  |  |  |  |  |  |  |

# Simulation Results

Table 3-1 shows the NPUSCH Format 2 simulation results for

- The ACK missed detection probability shall not exceed 1% at the given SNR

Under the condition that The DTX to ACK probability, i.e. the probability that ACK is detected when nothing was sent, shall not exceed 1%.

Table 3-1 Simulation Results for ACK missed detection (≤1%)

| Simulation number | repetition level | SNR (dB) |
| --- | --- | --- |
| 1 | 1 |  |
|  | 16 |  |
|  | 64 |  |
| 2 | 1 | 6.0 |
|  | 16 | -4.63 |
|  | 64 | -10.14 |

In our simulation, for the SNR levels of 1% ACK missed detections shown in Tablel 3-1, there was no ACK to NACK, i.e., the NACK to ACK probability was zero.Thus, it looks 1% ACK missed detection should be the limiting factor under the assumed simulation assumptions.

# Conclusion

In this paper, we provided the ideal simulation results for NPUSCH Format 2 based on the agreed simulation assumptions. In our simulation, for the SNR levels of 1% ACK missed detections, the NACK to ACK probability was zero.Thus, it looks 1% ACK missed detection should be the limiting factor under the assumed simulation assumptions.

# References

- R4-79AH-0277, “Updated simulation assumptions for NB-IoT BS demodulation”, Huawei, Ericsson, Nokia, ZTE