3GPP TSG-RAN WG4 Meeting #82bis	R4-1702772

# Spokane, US, 3-7 April 2017

Source:	SoftBank Corp.

Title:	Enlarging maximum BWs of Band 8 in NR

Agenda item:	10.2.1.2

Document for:	Approval

1.  Introduction

In the last RAN4 meeting, it was proposed to migrate some 3G/4G bands including Band 8 to NR in Rel-15 time frame [1]. Looking back, it was argued to enlarge maximum BW of Band 8 to 15MHz/20MHz a couple of times in LTE but not realized so far.

At the time of writing, we are not sure how the maximum BW of a certain band works in NR context, esp. in the relation to flexible BW but judging from the past experiences of LTE, it is better to kick off the relevant discussion as early as possible, while no backward compatibility issue is foreseen within NR.

2.  Problem in LTE

The maximum BW of Band 8 has been 10MHz since LTE Rel-8, reflecting spectrum holdings of incumbent Band 8 owners at that time. But the relevant frequency ranges were allocated spanning more than 10MHz in various Region 3 countries and the allocations induced proposals to expand Band 8 to cover the allocated range with a single CC. The reason for withdrawal of the proposals comes from the fact that “legacy” LTE UE cannot recognize extended BW such as 15/20MHz: then even if someone manages to expand in 3GPP standard in a certain release, he/she has to give up using existing devices and this fact makes using the global band almost meaningless in terms of UE availability.

This implies that, in the initial phase or before someone starts to implement a band, it is not difficult to consider the change of BW of the band. One evidence is the addition of 1.4MHz/3MHz to Band 65 [2].

Thus, it is proposed to expand the maximum BW of Band 8 at least up to 20MHz in NR, based on current CA_8B scheme.

[Proposal-1] It is proposed to expand the maximum BW of Band 8 at least up to 20MHz in NR.

3.  A-MPRs for Japan

A cumbersome issue around Band 8 is to consider a protection toward Band 18/19 for Japan. As noted in 2UL of CA_8B technical report [3], it was unlikely to support 2UL for Japan mainly due to constraints of PUCCH positions in single band operation. But as NR, it is necessary to define relevant A-MPR for 5MHz - 15MHz for ensuring single band operation. The requirements are as below but apparently we should follow NR design in terms of Tx characteristics such as modulation scheme, BW/RB definition, etc.

Protection requirement: -40dBm/MHz @ 860 – 890MHz,

Range of Tx and BW:   900 – 915MHz, BW = 5MHz - 15MHz.

[Proposal-2] It is proposed to evaluate A-MPR necessary for Japan.

As long as the author recognizes, there is no other A-MPR required for Korea, China…but it is better to confirm if this understanding is correct.

[Proposal-3] It is proposed to confirm that no A-MPR is necessary other than for Japan.

4.  Maximum BW of legacy bands in NR

This section handles issues common to all the LTE bands migrated to NR, not always specific to Band 8 expansion.

In LTE, maximum BW of a system has been set to 20MHz and it has been mandated for a UE to support 20MHz transmission/reception. In NR, it is expected broader BW could be supported between 100 – 200MHz [4] in sub 6GHz.

One thing for sure is that, considering many intra-band contiguous CAs in LTE-A, there are demands of maximum BW more than 20MHz. Bands under Class B intra-band contiguous CA are also candidates for expansion including Band 8. Class C intra band contiguous CAs are defined for Band 1, 2, 3, 7, 38, 39, 40, 41, 42, (46), 66 and 70 while Class B is supported for Band 3, 5, 7, 8, 12, 23 and 27. (Red letters indicate bands for REL-15 NR proposals in [1].) Thus,

[Observation-1] There are demands for legacy bands to expand the maximum BW in NR.

On the other hand, a notion of flexible BW is under discussion [5]. This could remove/relax the constraint of backward compatibility of LTE mention in section 2 in the sense that a legacy UE supporting narrower maximum BW could be operated in a network with larger BW.

[Observation-2] Flexible BW could relax the current backward compatibility issue of LTE UE in terms of maximum BW expansion.

But the flexible BW could make some RAN4 RF requirements further complicated, esp. related to Tx and depending on number/position of RBs, esp. MPR/A-MPR. We should be careful on how the flexible BW is supported and impacts to RAN4 requirements. Practical FFT size and SCS could also give some limitations to the maximum BW.

All in all, at present we are not sure whatever happens to existing bands when the bands are migrated from LTE/UMTS/GSM to NR: it could be as per LTE, expanded to a certain extent or to the maximum possible BW of the band as long as it is below 100-200MHz. The proposal-1 includes “at least up to 20MHz” to reserve some further extension to be aligned with flexible BW discussion so it is proposed as below to make clear:

[Proposal-4] 20MHz upper bound in Proposal-1 should be revisited if general scheme of NR requires further extension of maximum BW.

5.  Conclusion

This paper proposes the expansion of Band 8 at least up to 20MHz in NR and gives some generic consideration on demands for potential expansions or relation to flexible BW. Proposals here are limited to Band 8 as:

[Proposal-1] It is proposed to expand the maximum BW of Band 8 at least up to 20MHz in NR.

[Proposal-2] It is proposed to evaluate A-MPR necessary for Japan.

[Proposal-3] It is proposed to confirm that no A-MPR is necessary other than for Japan.

[Proposal-4] 20MHz upper bound in Proposal-1 should be revisited if general scheme of NR requires further extension of maximum BW.

6.  Reference

[1]     R4-1702504	WF on NR spectrum, NTTdocomo,

[2]     RP-161988	Addition of 1.4 and 3MHz channel bandwidths for Band 65 in TS36.101(Rel-13 CR), HUGHES Network systems

[3]     TR 36.833-1-08   LTE Advanced intra-band contiguous Carrier Aggregation (CA) in Band 8

[4]     R4-1702374	WF on NR subcarrier spacing and Channel bandwidth, NTTdocomo

[5]     R4-1702091	Way forward on UE RF requirements scalability for flexible channel BW consideration, MediaTek

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"