3GPP TSG-RAN WG4 Meeting #82bis	R4-1703993

Spokane, WA, USA, 3rd – 7th April, 2017

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	TP to TR 37.843 - Names and definitions for OTA requirement compliance range

Agenda item:	9.12.2.8

Document for:	Approval

# 1	Introduction

During the last meeting RAN4#82, the following agreements on the names and definitions for transmitter OTA requirement compliance range [1] were captured in [2]:

OTA peak directions set(s): set(s) of beam peak directions within which certain TX OTA requirements are intended to be met, all OTA peak directions set(s) are subsets of the OTA coverage range

NOTE:	The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

OTA coverage range: a common range of directions within which TX OTA requirements that are neither specified in the OTA peak directions sets nor as TRP are intended to be met

In this document, we further clarify OTA coverage range and provide the text proposal to TR37.843 for the definition of OTA peak directions set(s) and OTA coverage range.

# 2	Discussion

According to Clause 5.1, TR 37.843, OTA requirements can be classified into either single direction or TRP type. Table 1 lists TX OTA requirements and provides their classification types. All single-direction TX OTA requirements are met in either the OTA peak direction set(s) and OTA coverage range.

Figure 1 illustrates an example of OTA coverage range that is derived from beam peak directions.

ϕ

θ

OTA coverage range

Figure 1: Example of OTA coverage range

| AAS BS requirement | OTA requirement type | Coverage range | Transmit mode | Notes |
| --- | --- | --- | --- | --- |
| Base station output power | Single direction | [OTA peak directions set] | FFS | EIRP accuracy requirement is already included as a core requirement in TS 37.105. |
| Output power dynamics |  |  |  |  |
| Transmit ON/OFF power |  |  |  |  |
| Frequency Error | Single direction | FFS | FFS |  |
| Time Alignment Error | Single direction | FFS | FFS |  |
| Modulation Quality (EVM) | Single direction | [OTA coverage range] | FFS |  |
| Unwanted emissions | TRP | n/a | FFS |  |
| Adjacent Channel Leakage Radio (ACLR) | TRP | n/a | FFS |  |
| Transmitter intermodulation |  |  |  |  |

Table 1: Overview of radiated TX requirements (from TR 37.843)

# 3	References

- R4-1610800, Way forward on OTA requirement compliance range, NTT DoCoMo

- R4-1702281, Names and definitions for OTA requirement compliance range, Nokia, Alcatel-Lucent Shanghai Bell

# 4	Text proposal

TR37.843 v0.2.0

--------------Start of text proposal-------------

# 5	3 Definitions, symbols and abbreviations

## 5.1	3.1 Definitions

{Unchanged definitions omitted}

OTA peak directions set(s): set(s) of beam peak directions within which certain TX OTA requirements are intended to be met, all OTA peak directions set(s) are subsets of the OTA coverage range

NOTE:	The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

[OTA coverage range: a common range of directions within which TX OTA requirements that are neither specified in the OTA peak directions sets nor as TRP are intended to be met]

--------------end of text proposal-------------