3GPP TSG-RAN WG4 Meeting #84bis	R4-1710402

Dubrovnik, Croatia, 09 - 13 October 2017

Agenda item:	10.1.1.2

Title: 	PDSCH simulation results for 8Rx with rank higher than 4

Source: 	Intel Corporation

Document for:	Discussion

- Introduction

The Release 15 Study Item “Study on LTE DL 8Rx antenna ports” has been approved and introduced in RAN#76 and the Study Item Description is given in [1] with an objective to study the performance gain with 8 Rx antenna compared to a smaller number of receive antennas, such as 4 Rx.

In RAN4#84 Meeting, 8Rx UE SI was extensively discussed. A Way Forward for 8Rx evaluation was agreed in [2], including simulation assumptions for PDCCH, 8×8 SDR tests, and PDSCH TM2/TM3/TM9 transmission modes with various MIMO layers and correlation levels. In this contribution, we provide the simulation results for the 8Rx PDSCH TM9 using 64QAM and 256QAM with layers higher than 4.

# Simulation Assumptions and Results

The simulation assumptions for TM9 with various MCS and MIMO layers are given in Table 1 below:

Table 1 Simulation assumptions for TM9 with 8 layers

| Test number | Transmission mode | Bandwidth and MCS and MIMO layers | Reference channel | Propagation condition | Correlation matrix and antenna config. |
| --- | --- | --- | --- | --- | --- |
| 1 – 6 | TM9 | 10 MHz 64QAM, MCS 17/18/19/20/21/22 8 layers | R.xx FDD | EPA5 | 8x8 Low |
| 7 – 9 | TM9 | 10 MHz 256QAM, MCS20 4/6/8 layers | R.xx FDD | EPA5 | 8x8 Low |

where the parameters for channel correlation models are given in Table 2.

Table 2 The  and  parameters for ULA MIMO correlation matrices

| Correlation Model |  |  |
| --- | --- | --- |
| Low Correlation | 0 | 0 |
| Medium Correlation A | 0.3 | 0.3874 |
| Medium Correlation B | 0.3 | 0.005154 |

Figure 1 shows the PDSCH TM9 simulation results with 8×8 antenna configuration and 8 layers (normalized throughput vs. SNR).

Figure 1 PDSCH TM9 simulation results with 8×8 antenna configuration and 8 layers

Observation 1: In general, for EVM=6% scenarios, the operating SNR range would be lower than 30dB. It implies that, according to Fig.1, only MCS17 can achieve the maximum configured throughput for the 64QAM + 8 layers combination.

Figure 2 shows the PDSCH TM9 simulation results with 8×8 antenna configuration and 256QAM modulation (MCS20).

Figure 2 PDSCH TM9 simulation results with 8×8 antenna configuration and 256QAM

Observation 2: Combination of 256QAM and 4-layer can fly, in terms of achieving the maximum configured throughput around 30dB; while maximum configured throughput for combination of 256QAM with 6-layer or 8-layer cannot be achieved around 30dB.

# Conclusions

In this contribution, we provide the simulation results for the 8Rx PDSCH TM9 using 64QAM and 256QAM with layers higher than 4.

Observation 1: In general, for EVM=6% scenarios, the operating SNR range would be lower than 30dB. It implies that, according to Fig.1, only MCS17 can achieve the maximum configured throughput for the 64QAM + 8 layers combination.

Observation 2: Combination of 256QAM and 4-layer can fly, in terms of achieving the maximum configured throughput around 30dB; while maximum configured throughput for combination of 256QAM with 6-layer or 8-layer cannot be achieved around 30dB.

# References

[1] RP-171491, “New SI proposal: Study on LTE DL 8Rx antenna ports”, Huawei, HiSilicon, 3GPP RAN #76, June 2017.

[2] R4-1708729, “Way forward on 8Rx simulation assumptions”, Huawei, HiSilicon, Sprint, August 2017