3GPP TSG-RAN WG4 Meeting #84bis                 	R4-1710718

Dubrovnik, Croatia, 9 – 13, October 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Reporting Criteria

Agenda item:	9.7.3

Document for:	Discussion

# 1	Introduction

In Nagoya AH meeting discussions related to capabilities of event triggering and reporting criteria. In this paper, we add our views to how to capture such requirements in 36.133 and 38.133.

# 2	Discussion

As stated in 36.133, the purpose of having requirements related number of event triggering and reporting criteria, is to set some limits on the number of different event, periodic, logged measurement and no reporting criteria the UE may be requested to track in parallel. It is assumed a similar requirement is needed in coming NR UE requirements specification 38.133.

## 2.1 Reporting criteria for 36.133

From LTE point of the measurements performed in NR will be categorized as Inter-RAT measurements. Therefore, it is necessary to introduce new NR inter-RAT reporting criteria requirements in 36.133.

Observation 1: New row for Ecat for NR inter-RAT NR needs to be introduced in 36.133.

As discussed, in NR there will likely be requirements related to cell identification and measurements as well as CSI-RS measurements. This means, that in addition to cell based reporting criteria as known from existing LTE requirements, there is a need for introduction of reporting criteria also for CSI-RS related events. This would need to be accounted when defining Ecat for NR inter-RAT.

Observation 2: The need for CSI-RS reporting needs to be account in Ecat for NR inter-RAT.

Exactly which reporting triggers RAN2 will define for NR is still under discussion in RAN2. Once these are settled the more detailed requirement can be defined in RAN4.

Introduction of NR related requirements could be done by introducing a new row in the existing table:

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Inter-RAT (NR) | TBD | Only applicable for UE with this (inter-RAT) capability. |

Whether there would need to be separate requirements for cell and CSI-RS based reporting criteria needs further discussion.

In [1] we provide a draft CR illustrating how the requirement could be captured in 36.133.

## 2.2 Reporting criteria for 38.133

For NR there would need to a similar requirement for UE related number of event triggering and reporting criteria, for the same reasons as in 36.133. For NSA, there is only a need to consider NR intra-frequency and inter-frequency related requirements.

There would be a need to introduce also SCell requirements in a similar manner as in LTE. Additionally, it needs be discussed whether there will be a need for separate cell and CSI-RS reporting criteria requirements, or whether one overall requirement covering both cells and CSI-RS would be sufficient.

However, the requirements will depend on what is decided in RAN2 concerning measurement configuration and reporting. As the text related to the UE reporting criteria requirements in 38.133 is likely to be closely related to how RAN2 defines the measurements configuration and reporting (trigger event, report types, cell and/or CSI-RS trigger and reporting events etc.) it seems necessary to wait RAN2 work.

# 3	Conclusion

In this paper, we presented our views related to capabilities of event triggering and reporting criteria and how to capture such requirements in 36.133 and 38.133. We observe for 36.133 it is possible to define requirements already now and observe:

Observation 1: New row for Ecat for NR inter-RAT NR needs to be introduced in 36.133.

Observation 2: The need for CSI-RS reporting needs to be account in Ecat for NR inter-RAT.

In [1] we have CR introducing requirements for 36.133. Requirements for 38.133 would need to wait for having stable RAN2 agreements concerning measurement configuration and reporting in NR.

# References

- R4-1710719, Reporting criteria for 36.133, Nokia, Alcatel-Lucent Shanghai Bell