3GPP TSG-RAN WG4 Meeting RAN4 #84bis                                                           R4-1710742

# Dubrovnik, Croatia, 9 - 13 October, 2017

Source: 	ZTE Corporation

Title:	PLL lock time

Agenda Item:	9.3.5

Document for:	Discussion

# Introduction

In RAN4 NR AH#3, how to implement 7.5 kHz shift for LTE-NR uplink subcarrier alignment was discussed intensively [1]. And one interesting question was also raised on how much RF retuning time can contribute to LTE-NR switch time.

In this contribution, we investigate the so-called “fast-lock” technique in PLL and provide the simulation results on the feasible lock time, and also show the measured results based on the available chipset equipped with the fast-lock technique. Both simulation and measurement show that 7.5 kHz shift on top of the RF frequency could be achieved within the required interval, e.g., 20us.

# Discussion

. “Fast-lock” feature is a very common technology and most of PLL ICs support this feature.

A typical fast-lock PLL circuit shows in Fig. 1.

- When a new frequency is loaded, the charge pump circuit receives an input to deliver 4 times the normal current per unit phase error, while an open drain NMOS on chip device switches in a second resistor (R2) to GND.

- Once locked on the correct frequency, the PLL will then return to standard, low noise operation.

- This transition does not affect the charge on the loop filter capacitors and is enacted synchronous with the charge pump output. This creates a nearly seamless change between fast-lock mode and standard mode.

# Figure 1, Fast-lock PLL circuit

# A simulation on the fast-lock technique on DCS1800 was conducted, and the simulated phase noise and lock time is shown as Fig. 2. With the fast-lock mode, the lock time for a jump from 1805MHz to 1880MHz is reduced from 120us down to 6us.

# Figure 2, Simulation on phase noise and lock time

As an example of mature commercial chips with the fast-lock, ultra-fast settling Frac-N PLL device ADF4193 & ADF4196 is a mature device with “FastLock” feature which is from Analog Devices, Inc.  Fig. 3 illustrates the block diagram of these two devices.

# Figure 3, Block diagram of ADF4193 and ADF4196

Fig. 4 shows measured results for DCS-1800 Tx synthesis lock time with 75MHz jump from 1815MHz to 1890MHz.

# Figure 4, Measured results on lock time with 75MHz jump from 1815MHz to 1890MHz

Fig. 5 illustrates measured results on lock time for GSM-900 with 40MHz jump from 1095MHz to 1055MHz.

# Figure 5, Measured results on lock time with 40MHz jump from 1095MHz to 1055MHz

It is noteworthy that 7.5kHz retuning is much less than 40MHz for GSM-900 RX and 75MHz for DCS-1800 and, hence it is reasonable to conclude that lock time for 7.5kHz offset should be less than 20us.

Observation: Mature fast-lock technique and devices enable 7.5kHz retuning with less than 20us.

# Conclusion

Based on the above discussions, we have the following observation on the feasible PLL lock time:

Observation: Mature fast-lock technique and devices enable 7.5kHz retuning with less than 20us.

# References

Final minutes report RAN4 NR AH#3.