3GPP TSG-RAN WG4 Meeting #84bis	R4-1710797

Dubrovnik, Croatia, 9 – 13 October 2017

Agenda Item:	6.2.3.2

Source:	Ericsson

Title:	RLM measurement period for non-BL CE UE

Document for:	Discussion

# 1	Introduction

RAN4#83 agreed with the way forward on RLM requirements for non-BL/CE UE as follow [1]:

- RAN4 will study the RLM evaluation period for BL/CE UE with 2Rx, when no DRX is used, for the following scenarios in Rel-14

  - CE Mode A Out-of-synch

  - CE Mode A In-synch

  - CE Mode B Out-of-synch

  - CE Mode B In-synch

- Companies are encouraged to provide the investigation results for the RLM evaluation period considering the effectiveness of receiver diversity in RAN4#84

  - Side condition:

    - CE Mode A: EPA5/ETU30 with SNR = -6dB

    - CE Mode B: EPA1/ETU1 with SNR = -15dB

Since RAN4#84 had not progress on this issue, we continue the discussion on the measurement period for non-BL/CE UE in Rel-14.

# 2	RLM requirements for non-BL UE

## 2.1	Discussion

Radio link monitoring (RLM) is the functionality to monitor the quality of the serving cell. For RLM, UE estimates the downlink radio link quality and compare it with the threshold Qout and Oin every radio frame. If the quality level is lower than Qout, UE judges it is out-of-synch. If the quality level is higher than Qin, then UE judges it is in-synch. For BL/CE UEs such as Category M1 or M2, Qout is derived from 10% BLER of the hypothetical MPDCCH transmission and Qin is derived from the 2% BLER of the hypothetical MPDCCH transmission, because BL/CE UEs should monitor MPDCCH instead of PCFICH/PDCCH.

The quality level evaluation is supposed to be based on the estimated SNR values from the cell-specific reference symbols. In TS36.133, the out-of-synch evaluation period and in-synch evaluation period for CE Mode A is specified as 400ms and 200ms, respectively, when no DRX is used. Similarly, the out-of-synch evaluation period and in-synch evaluation period for CE Mode B is specified as 4,000ms and 2,000ms, respectively, when no DRX is used. These L1 measurement periods are derived from the assumption UE has only single receiver antenna.

For non-BL CE UE, we have proposed to shorten the measurement period because of two receive antennas [2]. RAN4#83 and RAN4#84 discussed the concern to shorten the measurement period because very slow Doppler fading environment such as 1Hz assumed for CE Mode B needs longer measurement time to capture the longer coherent time. We think two receive antennas can shorten the measurement period by achieving the same measurement accuracy because of the receive diversity.

## 2.2	Simulation results

Table 1 is the simulation parameters used for evaluating RLM in UE Category M1.

Table 1	Simulation parameters for RLM for coverage enhancement UE with 2Rx.

| Parameter |  |
| --- | --- |
| System Bandwidth | 10 MHz |
| Channel model | AWGN, ETU30, EPA5, EPA1, ETU1 |
| Antenna configuration | 1x1, 1x2 |
| Antenna correlation | Low |
| RSRP sampling period | 10ms |
| L3 filtering | Disabled |

### 2.3.1	CE Mode A

Table 2 summarizes the L1 measurement period for 2Rx UE to keep the same RSRP error considering CE Mode A (SNR = -5dB). For example, the 5%-ile of RSRP error for ETU30 at SNR=-5dB is -2.79dB for 1Rx UE with 400ms measurement period. For 2Rx UE, the same RSRP error can be achieved with 220ms. It is observed from our simulation result that the measurement period could be reduced from 400ms to 130 - 220ms.

Considering the margin, we propose to reduce the measurement period by 2dB.

Proposal 1: For non-BL UE supporting CE Mode A, the L1 measurement periods for out-of-synch is 260ms and for in-synch is 130ms.

Table 2	Summary of RSRP measurement error for CE Mode A scenario.

|  |  |  |  | Measurement time |  |
| --- | --- | --- | --- | --- | --- |
| Channel model | SNR | Condition | RSRP Error [dB] | 1Rx (baseline) | 2Rx |
| ETU30 | -5dB | 5%-ile | -2.79 | 400ms | 220ms |
|  |  | 95%-ile | +0.07 | 400ms | 170ms |
| EPA5 | -5dB | 5%-ile | -3.21 | 400ms | 190ms |
|  |  | 95%-ile | +2.86 | 400ms | 130ms |

### 2.3.2	CE Mode B

Table 3 summarizes the L1 measurement period for 2Rx UE to keep the same RSRP error considering CE Mode B (SNR=-15dB). For example, the 5%-ile of RSRP error for EPA1 at SNR=-15dB is +0.79dB for 1Rx UE with 4000ms measurement period. For 2Rx UE, the same RSRP error can be achieved with 1,500ms. It is observed from our simulation result that the measurement period could be reduced from 4,000ms to 1,200 – 3,220ms.

Considering the margin, we propose to reduce the measurement by 20%.

Proposal 2: For non-BL UE supporting CE Mode B, the L1 measurement periods for out-of-synch is 3,200ms and for in-synch is 1,600ms.

Table 3	Summary of RSRP measurement error for CE Mode B scenario.

|  |  |  |  | Measurement time |  |
| --- | --- | --- | --- | --- | --- |
| Channel model | SNR | Condition | RSRP Error [dB] | 1Rx (baseline) | 2Rx |
| EPA1 | -15dB | 5%-ile | +0.79 | 4000ms | 1500ms |
|  |  | 95%-ile | +3.16 | 4000ms | 3100ms |
| ETU1 | -15dB | 5%-ile | -0.21 | 4000ms | 1200ms |
|  |  | 95%-ile | +2.11 | 4000ms | 3200ms |

# 3	Conclusion

Proposal 1: For non-BL UE supporting CE Mode A, the L1 measurement periods for out-of-synch is 260ms and for in-synch is 130ms.

Proposal 2: For non-BL UE supporting CE Mode B, the L1 measurement periods for out-of-synch is 3,200ms and for in-synch is 1,600ms.

# 4	References

R4-1706184, “Way forward on non-BL/CE UE RLM evaluation period”, Ericsson.

R4-1707674, “RLM requirements for non-BL CE UE in Rel-14”, Ericsson.