3GPP TSG-RAN WG4 Meeting #84bis	R4-1710844

Dubrovnik, Croatia, 9 – 13 October 2017

Source:	Ericsson, AT&T

Title:	TP to TR 36.715-02-01: CA_2DL_2A-14A_1UL_BCS0

Agenda item:	8.2.2

Document for:	Approval

# 1. Introduction

This contribution provides a TP for TR 36.715-02-01 to include the requirements for the 2DL/1UL CA combination CA_2DL_2A-14A_1UL_BCS0 as defined in RP-171026 [1].

# 2. Text Proposal for TR 36.715-02-01

##### ---Start of changes---

## 5.x	CA_2DL_2A-14A_1UL_BCS0

### 5.x.1	Operating bands for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-14 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |

### 5.x.2	Channel bandwidths per operating band for CA

Table 5.x.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_2A-14A | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  | 14 |  |  | Yes | Yes |  |  |  |  |

### 5.x.3	Co-existence studies

Table 5.x.3-1 gives the harmonic frequency limits for Band 2 and Band 14 2DL CA. As shown in the table, UL carrier of CA_2A-14A _BCS0 is not located at 2nd order harmonics or 3rd order harmonics of DL carriers.

# Table 5.x.3-1: DL harmonics frequency limits for CA of Band 2 and Band 14

| DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 768 | 1850 | 1910 |
| 2nd order DL harmonics frequency range (MHz) | 1516 to 1536 |  | 3700 to 3820 |  |
| 3rd order DL harmonics frequency range (MHz) | 2274 to 2304 |  | 5550 to 5730 |  |

As shown in Table 5.x.3-2, the harmonic frequencies of Band 2 and Band 14 in UL are away from the receive bands of interest in the DL and we can conclude that there is no issue on UL harmonic interference.

Table 5.x.3-2: UL harmonics frequency limits for CA of Band 2 and Band 14

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 |
| 14 | 788 | 798 | 758 | 768 | 1576 | 1596 | 2364 | 2394 |

### 5.x.4	ΔTIB,c and ΔRIB,c values

Table 5.x.4-1: ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-14A | 2 | 0.3 |
|  | 14 | 0.3 |

Table 5.x.4-2: ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2A-14A | 2 | 0 |
|  | 14 | 0 |

### 5.x.5	 MSD

No MSD values need to be defined for this combination.

---End of changes---

# Reference

[1]	RP-171026, “New WID on LTE Advanced inter-band CA Rel-14 for 2DL/1UL”, RAN#76, Qualcomm