3GPP TSG-RAN WG4 Meeting #84Bis	R4-1711145

Dubrovnik, Croatia, 9 – 13 October 2017

Agenda Item:	8.20.3.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Operating band unwanted emission requirements for microcell and picocell NB-IoT BS

Document for:	Approval

# 1.	Introduction

The WI proposal on Further NB-IoT enhancements was agreed in RAN#73 [1]. One of the objectives of the WI is to specify necessary support (appropriate eNB classes) for NB-IoT to be used in microcell, picocell, and femtocell deployments.

The transmitter and receiver requirements for microcell and picocell NB-IoT BS have been discussed in RAN4, and the way forwards have been agreed [2-5]. The only outstanding issue is the operating band unwanted emission requirements for microcell and picocell NB-IoT BS, where two options were listed in the agreed way forward [5].

This contribution provides a proposal on the operating band unwanted emission requirements for microcell and picocell NB-IoT BS to close the remaining issue.

# 2.	Discussion

The following two options to specify the operating band unwanted emission requirements for microcell and picocell NB-IoT BS were listed in the agreed way forward [5]:

1)	To apply the medium range and local area MSR with GSM/EDGE BS operating band unwanted emissions requirements also to the medium range and local area NB-IoT BS, respectively, for standalone operations.

2)	To apply the wide area NB-IoT BS operating band unwanted emissions requirements with power scaling to the medium range and local area NB-IoT BS for standalone operations.

While no disadvantage has been identified using option 1, using option 2 would probably result in the medium range and local area E-UTRA BS already deployed in the field need a hardware upgrade to deploy NB-IoT carrier with the BS. As such it is proposed to adopt option 1 above to apply the medium range and local area MSR with GSM/EDGE BS operating band unwanted emissions requirements also to the medium range and local area NB-IoT BS, respectively, for standalone operations.

# 3.	Conclusion and proposal

This contribution has provided a proposal on the operating band unwanted emission requirements for microcell and picocell NB-IoT BS to close the remaining issue.

Proposal:

To apply the medium range and local area MSR with GSM/EDGE BS operating band unwanted emissions requirements also to the medium range and local area NB-IoT BS, respectively, for standalone operations.

# References

[1]	RP-170852, “New WID on Further NB-IoT enhancements”, Huawei, HiSilicon, Neul.

[2]	R4-1705426, “Reference sensitivity for microcell NB-IoT BS”, Nokia, Alcatel-Lucent Shanghai Bell.

[3]	R4-1705427, “Receiver dynamic range for microcell NB-IoT BS”, Nokia, Alcatel-Lucent Shanghai Bell.

[4]	R4-1705428, “ACS/in-band blocking for microcell NB-IoT BS”, Nokia, Alcatel-Lucent Shanghai Bell.

[5]	R4-1708809, “WF on RF requirements for microcell and picocell NB-IoT BS”, Nokia, Alcatel-Lucent Shanghai Bell, Ericsson, Huawei.