3GPP TSG-RAN WG4 Meeting #84bis	R4-1711379

Dubrovnik, Croatia, 9th – 13th October 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	eAAS specification simplification

Agenda item:	8.27.1

Document for:	Approval

# 1	Introduction

During the last 3GPP RAN4 #84 meeting, a way forward on eAAS specification complexity was approved [1]. The way forward captures the following aspects of the conducted requirements which can potentially be considered to be removed or simplified for the eAAS (Rel-15 AAS) requirement:

- Support for both single RAT and MSR

For the OTA requirements, it should be further considered whether both single RAT and MSR is really needed. If UTRA should be enabled for such large arrays, supporting MSR only could be a possibility.

- Support for UTRA TDD

The current specification captures UTRA TDD. UTRA TDD has not evolved since release 8 and does not contain any advanced MIMO features. Receiver requirements assume only a single receiver.

- Simplified multiband support

Supporting multiband adds a certain complexity to the conducted specifications. From an OTA perspective, considering the aim to create black box requirements, it is not obvious why the emissions and blocking requirements should be differentiated based on the configuration of internal electronics.

- UTRA P-CPICH etc. Requirements

A group of requirements exist relating to the accuracy of transmitting pilot channels for UTRA. These requirements exist in the specification since times in which BS quantization accuracy could impact the pilot channel power. Any AAS BS that aims to reliably perform advanced MIMO, higher order modulation and achieve high SINR would by necessity have better quantization accuracy than these early BS, and thus this group of requirements are redundant for OTA tested systems. Although technically a method has been agreed how they could be introduced, removal of the requirements could contribute to simplifying the specifications.

In this document, we provide our views on each of the above aspects.

# 2	Discussion

Unlike hybrid requirements, OTA requirements will be applied to AAS BS with 8 or more transmitters according to TR 37.843. Our views on the above aspects are as follows:

- Support for both single RAT and MSR – taking into account UTRA and E-UTRA operation only, if UTRA operation is not supported in future AAS BS (with 8 or more transmitters) then only single RAT (E-UTRA) requirements are sufficient in the OTA AAS specification. However, for E-UTRA+NR operation, new MSR requirements (on the basis of E-UTRA/NR requirements only) need to be considered.

- UTRA TDD is already part of the AAS hybrid requirement and does not contain any advanced MIMO features to support AAS BS with more than 8 transmitters. Thus, it can be excluded from the OTA AAS requirements.

- Simplified multiband support – both single and multiband OTA AAS BS should be considered. However, multiband requirements can be considered as default assuming such a BS support at least two bands;

- UTRA P-CPICH etc. requirements are not needed [2].

Based on the above views, we propose the following:

Proposal 1: OTA AAS requirements should adopt single RAT (E-UTRA) requirements. For E-UTRA+NR operation new MSR requirements (on the basis of E-UTRA/NR requirements only) need to be considered.

Proposal 2:  Both single and multiband OTA AAS BS should be considered. However, multiband requirements can be considered by default assuming such a BS support at least two bands.

Proposal 3: UTRA P-CPICH requirements are not needed.

# 3	Conclusions

In this document, we propose the following

Proposal 1: OTA AAS requirements should adopt single RAT (E-UTRA) requirements. For E-UTRA+NR operation new MSR requirements (on the basis of E-UTRA/NR requirements only) need to be considered.

Proposal 2:  Both single and multiband OTA AAS BS should be considered. However, multiband requirements can be considered by default assuming such a BS support at least two bands.

Proposal 3: UTRA P-CPICH requirements are not needed.

# References

- R4-1708877, Way forward on eAAS specification complexity, Ericsson.

- R4-1708120, On the complexity of the eAAS specification, Ericsson.